3GPP TSG-RAN WG4 Meeting #114bis				                                                                                                                    R4-2505096

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.24.5

Source:	CMCC

Title:	WF on A-IOT device requirements

Document for:	Approval

Introduction

This way forward captures the agreements on A-IOT system parameters, device RF requirements and OTA test method. The summary document for RAN4#114 bis is R4-2504687.

The way forward agreed in previous RAN4 meeting is R4-2502858.

System parameters

- Topic 2-1: R2D bandwidth

Issue 2-1-1: R2D transmission bandwidth

Agreement:

- For RAN4 requirements, only define R2D transmission bandwidth configuration as 1PRB, 2PRB, 3PRB, 4PRB with 15KHz SCS.

Issue 2-1-2: R2D channel bandwidth

Agreement:

- Define R2D channel bandwidth with assumption of 90% spectrum utilization.

- Topic 2-2: D2R bandwidth

Issue 2-2-1: D2R bandwidth

Agreement:

- Define D2R channel bandwidth for A-IoT device:

- Based on 2SB transmission,

- Consider SFO impact

- Consider small frequency shift defined in RAN1

- D2R transmission bandwidth (2 times of 1SB), consider bit rate defined in RAN1

- Topic 2-3: Channel raster

Issue 2-3-1: R2D channel raster

Agreement:

- Consider following options for R2D channel raster

  - Option 1: define 10KHz as R2D channel raster (i.e. same as enhanced channel raster)

  - Option 2: Define channel raster considering the RB orthogonality between macro BS and A-IoT BS

  - Other options are not precluded.

- Topic 2-4: Others

Issue 2-4-2: Deployment scenario

Agreement:

- Do not capture the side condition of outdoor UE assumption in the specifications

Device RF requirements

- Topic 3-1: Transmit output power

Issue 3-1-1: Transmit output power

Agreement:

- Performance metric of transmit output power:

  - Backscattering loss

- Backscattering output power is defined as the minimum backscattered power of Device 1 at certain input CW Tx power

  - Minimum Backscattering power at device antenna= Input CW Tx power at device antenna-Backscattering loss

- For the Input CW Tx power to define requirements:

  - FFS on the whether to use one value or a range of CW Tx power

  - FFS on the value(s) of CW Tx power

- Consider following aspects to determine backscattering loss to define requirements:

  - modulation loss

  - antenna efficiency

  - backscattering circuitry loss

  - impairment margin

  - minimum distance to the reader

  - differential radar cross section in two different modulation states

  - differential reflection coefficient in two different modulation states

  - the spectral components of the backscatter signal to be included in the power measurement, e.g. only the 1st Lower SB and 1st Upper SB, excluding the reflected carrier itself

  - Other factors not excluded

- Topic 3-2: Modulation quality

Issue 3-2-1: SFO requirement

Agreement:

- 10% SFO is assumed as the minimum performance for devices.

- FFS on whether to define SFO requirement and how to test the SFO requirement if defined.

Issue 3-2-2: Modulation quality requirements

Agreement:

- Define modulation quality requirements for both BPSK and OOK.

- Consider to define same modulation quality metric for both BPSK and OOK if possible.

- Reference measurement process needs to be discussed

- Topic 3-3: Emission requirements

Issue 3-3-1: Emission requirements

Agreement:

- Discuss whether to define EIPR or TRP as performance metric for emission requirements

- Spurious emission: reuse legacy NR UE spurious emission requirement and further discuss the OOB boundary.

- FFS on CW maximum input power level for testing

- Topic 3-4: Reference sensitivity

Issue 3-4-1: Reference sensitivity

Agreement:

- Define minimum requirements of REFSENS based on link budget (target is [15m]) and measurement results reported by companies

- Consider to use inventory procedure for REFSENS testing, success rate ([90%]) or miss detection rate ([10%]) as the metric for inventory

- At least to define maximum gain direction sensitivity requirements

- FFS on how to different sensitivity requirements on different orientations

- Topic 3-5: Others

Issue 3-5-1: Maximum input power

Agreement:

- Define maximum input level, consider following options:

- Option 1: using [1m] distance assumption from reader to device

- Option 2: using square law appliance range

- Option 3: using dynamic range of device receiver

Issue 3-5-2: Occupied bandwidth

Agreement:

Further discuss whether to define occupied bandwidth

OTA test method

- Topic 4-1: General

Issue 4-1-1: testing frequencies

Agreement:

- Only test low, middle and high frequencies within a band and average the performance metrics from the three frequencies

- Check whether three frequencies are applicable for all requirements.

- Topic 4-2: Test setup

Issue 4-2-1: testing antenna and CW antenna relationship

Agreement:

Whole sphere scan is supported

The position of CW antenna is a set of fixed position(s) as baseline (depending on core requirements discussion).

- FFS on relative positions between reader antenna and CW

Issue 4-2-2: how to ensure the isolation between CW node and measurement antenna

Agreement:

- Further discuss power difference between backscatter signal and the CW signal leakage, e.g. 6dB, 12dB, etc.

- Encourage TE vendors provide the feasibility of TE extracting D2R power when CW source is transmitting.

Issue 4-2-3: 3D scan and measurement grid

Agreement:

- For TRP, TRS, EIRP and EIS requirements (depending on core requirements) measurement grid:

- For TRP and EIRP, use 30 degree as starting point

- For TRS and EIS, use 45 degree as starting point

Issue 4-2-4: tag mounting material

Agreement:

- The device needs to mount on the non-RF-reflective and non-RF-conductive material

- Topic 4-3: Position of device

Issue 4-3-1: Position of device

Agreement:

- If the device has a rectangular shape, the free space DUT orientation 1 specified in Table J.2-1 of TS 38101-2 is assumed as the default position of device.

- Front and back side is based on device declaration.

- Otherwise, the device positioning is based on device declaration.

- Topic 4-4: Minimum range/ QZ (Quiet Zone) size/QZ calibration

Issue 4-4-1: Ripple test for Quiet Zone

Agreement:

- Reuse the QZ Ripple test procedure as in 38.870

Issue 4-4-2: Quiet Zone and minimum range length

Agreement:

- Reuse the same QZ size and minimum range length as in 38.870. i.e. 30cm QZ size and 90cm minimum range length.

Issue 4-4-3: distance DCW2D and DMA2D

Agreement:

- Distance DCW2D equals to DMA2D, and minimum range length is referred to table in TR 38.870

- Topic 4-5: Test procedure

Issue 4-5-1: testing performance metric of reference sensitivity

Agreement:

- Use following as starting point:

- Measuring device’s correct response and correct respond timing relationship between R2D and the corresponding D2R transmission with a minimum mean power applied for device antenna.

- FFS on how to set the side condition including CW signal power.

Issue 4-5-2: Test procedure for receiver RF requirements

Agreement:

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Place DUT based on UE positioning guidelines.

- Step 3: Device is assumed charging during the measurement according to device declaration on the required energy conditions.

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal. The transmit power of signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device. FSS on the CW signal level.

- Step 5: Determine whether DUT can send the correct response in D2R channel within timing window, e.g., TR2D_min

- Step 6: Move to next measurement point and repeat Step 3-5 until complete all the testing point in the measurement grid.

- Step 7: FFS on performance metric. FFS on how to specify the performance metric.

Issue 4-5-3: Test procedure for Tx RF requirements

Agreement:

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Place DUT based on UE positioning guidelines.

- Step 3: Device is assumed charging during the measurement according to device declaration on the required energy conditions.

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal. The received power at the antenna of DUT is Pin. Pin is set as the power level of minimum reference sensitivity of device equal to or larger than “sensitivity+[X]dB”

- Step 5: Measure and record the backscattering signal level from DUT and calculate the backscattering power level transmitting from DUT antenna, i.e., Pout, based on the calibration data from step 1

- Step 6: Move to next measurement point and repeat Step 3-5 until complete all the testing point in the measurement grid.

- Step 7: Calculate the ratio of CW power level to D2R signal power (i.e., before and after backscattering) according to the equation: backscattering loss (dB) = Pout (TRP) – Pin (TRP)

- Step 8: FFS on performance metric. FFS on how to specify the performance metric.

- Topic 4-6: Energy source and energy harvesting

Issue 4-6-1: energy source and energy harvesting

Agreement:

- Device declaration on the required energy conditions.

- External energy source directly connected to the device with cable and/or battery is excluded.

- Topic 4-7: MU

Issue 4-7-1:MU assessment

Agreement:

- Use following as baseline for further discussion.

| UID | Description of uncertainty contribution | Details in clause |
| --- | --- | --- |
| Stage 2: DUT measurement (Figure 7.2-1, Figure 7.2-2) |  |  |
| 1 | Mismatch of transmitter chain | Reused |
| 2 | Insertion loss of transmitter chain | Reused |
| 3 | Influence of the measurement antenna cable | Reused |
| 4 | CW for backscatter measurement or R2D for sensitivity measurement: uncertainty of the absolute output level | FFS |
| 5 | Sensitivity measurement: output level step resolution | Reused |
| 6 | Measurement distance | Reused |
| 7 | Quality of quiet zone | Reuse the same QZ size and minimum range length as in 38.870. i.e. 30cm QZ size and 90cm minimum range length. |
| 8 | DUT sensitivity drift | FFS |
| 9 | Uncertainty related to the use of phantoms | No needs |
| 10 | Coarse sampling grid | - For TRP and EIRP, use 30 degree as starting point - For TRS and EIS, use 45 degree as starting point |
| 11 | Random uncertainty | Reused |
| 12 | Frequency Response | Reused |
| Stage 1: Calibration measurement, network analyzer method (Figure 7.3-1) |  |  |
| 13 | Uncertainty of network analyzer | Reused |
| 14 | Mismatch of transmitter chain | Reused |
| 15 | Insertion loss of transmitter chain | Reused |
| 16 | Mismatch in the connection of calibration antenna | Reused |
| 17 | Influence of the calibration antenna feed cable | Reused |
| 18 | Influence of the measurement antenna cable | Reused |
| 19 | Uncertainty of the absolute gain/radiation efficiency of the calibration antenna | Reused |
| 20 | Measurement distance | Reused |
| 21 | Quality of quiet zone | Reuse the same QZ size and minimum range length as in 38.870. i.e. 30cm QZ size and 90cm minimum range length. |
| Systematic Errors |  |  |
| 22 | Systematic Error related to TRS grids | Reused |