3GPP TSG-RAN WG4 Meeting #114bis   												         R4-2505099

Wuhan, P.R.China, 07th – 11th April, 2025

Agenda item:			7.1.3

Source:	Huawei, HiSilicon

Title:	WF on power domain enhancements

Document for:	Approval

- Topic #1: Power domain enhancements for single carrier

Sub-topic 1-1: Approaches to enable MPR reduction

Issue 1-1-1: RB based or CBW based extension

- WF

  - Symmetrical/ Asymmetric for maximum extension at each side are determined by 0.5*NRB

  - If a limited number of additional fixed values are introduced, the extension is determined by X*NRB at each side, where X < 0.5

  - The extended RBs are aligned with common RB grid

Issue 1-1-2: Fixed or flexible extension

- Agreement in main session:

  - Fixed value (0.5*CBW and/or 0.5*NRB) for Symmetrical/ Asymmetric by default for UE supporting the MPR reduction feature

    - In addition, consider a limited number of additional fixed values with capability

Issue 1-1-3: Derivation of new inner RB region in extended CBW

- WF

  - FFS in next meeting

Issue 1-1-4: Guard band for extended CBW

- Options

  - Option 1: No need to define multiple UE guard bands for initial CBW and extended CBW, UE should have one guard band per each side. All frequency segments within UE extended CBW and outside UE maximum transmission bandwidth (number of RBs in 38.101-1) act as UE total guard bands (i.e., UE initial guard band + additional guard band according to UE extended CBW NW indication), where this UE cannot be allocated and OOBE edges are the outer edges of UE total guard band.

  - Option 2: NRB based extension and associated guard band:

    - maximum bandwidth extension: The extended RB BW NRBext inherits the guard-bands (GBext) from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext.

    - flexible NRB-based extension: Similarly to the ceil(NRB/2) extension, the guard band mapping for the fractional NRB extension can inherit the guard band from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext

  - Option 3: The GBs of extended UE CBW should only be retained on the side that is far away from the original CBW. The GBs of extended UE CBW reuse the size of original GB of UE CBW.

  - Option 4: The GBs of the extended CBW shall be the remaining portion obtained by subtracting the RB numbers from the specified channel bandwidth when the extended CBW is selected from Table 5.3.2-1 in 3GPP TS 38.101-1.

  - Others

- WF

  - FFS in next meeting

Sub-topic 1-2: Applicable requirements

Issue 1-2-1: A-MPR handling

- Proposals

  - Option 1: Only apply the power domain enhancement scheme to channels which are not subject to A-MPR.

  - Option 2:

    - If A-MPR is specified for a given RB allocation X, then the UE performs

max(MPRenh(X), A-MPR(X))

    - If A-MPR is not specified for a given RB allocation, then the UE performs

max(MPR(X), A-MPR(X))

  - Option 3: In cases where additional emission should be met, the UE performs the max(MPR, A-MPR) function based on the specified A-MPR and the initial UE CBW MPR and thus set its backoff ignoring any MPR reduction.

  - Option 4: max(MPRenh(X), A-MPR(X)), where A-MPR(X) is determined based on a given RB allocation, and RB region with respect to initial CBW, WF and modulation etc, and MPRenh(X) is determined based on RB region/etc. within the extended UE CBW.

  - Others

- WF

  - Only apply the power domain enhancement scheme to channels which are not subject to A-MPR in Rel-19.

Issue 1-2-2: Whether the extended IBE is applied in the GB of original UE CBW

| Issue1 Issue 2 | Option1. Allocate the extended RBs from the GB of the original UE | Option2. Allocate the extended RBs from the edge of the UE CBW |
| --- | --- | --- |
| Option1. Individual evaluation of the extended RBs & Horizontal line | Case1 | Case2 |
| Option2. Combine evaluation of the extended RBs with the non-allocated RBs of the UE CBW & Stepped line | Case3 | Case4 |
| Note: It is assumed that extended RBs are RB aligned with the UE and BS RBs for the listed cases. |  |  |

- WF:

  - Case 1 and case 3 could be further down selected

  - GB of UE CBW may or may not be scheduled to other UEs

  - Extended IBE can be applied in the GB of UE CBW

    - Relaxation can be considered for the GB, based on calculation from the existing IBE formula from the last RB of original UE CBW

Issue 1-2-3: Derivation of the extended IBE

- Options

  - Option 1: Fixed IBE used for extended CBW at each side of UE CBW, the value is derived from the outmost allocated RB of UE CBW (illustrated in the figure below)

  - Option 2: Combine the evaluation of the extended RBs with the non-allocated RBs of the UE CBW: Stepped line from the first non-allocated RB to the last extended RB, NRB’ = NRB + extended RBs.

  - Option 3: Others

- WF

  - Discuss together with issue 1-2-2, and down select the options in next meeting.

Issue 1-2-4: Relaxation of the extended IBE

- Proposals

  - Proposal 1: For the IBE derivation for the extended CBW, use the general part of legacy IBE requirements and fix the EVM parameter to the one for BPSK. (Xiaomi)

  - Proposal 2: (vivo)

    - If the extended RBs are allocated from the GB of the original UE (case1/3), then the IBE in the GB should be relaxed to ensure the MPR of outer allocation reduces to the inner level:

      - Way 1: Skip the IBE evaluation for the extended RBs allocated in the GB.

      - Way2: Relax EVM in the IBE formula to that for BPSK for extended RBs.

IBE_general = max(PRB - 30 dB, )

- WF

  - For case 1 in issue 1-2-2, use BPSK EVM in the IBE formula for all extended RBs

  - For case 3 in issue 1-2-2, FFS the relaxation

Sub-topic 1-3: Signaling

Issue 1-3-1: NW signaling

- WF

  - FFS in next meeting

Issue 1-3-2: UE capability

- Agreement in main session:

  - An optional UE capability to indicate whether it supports the CBW extension based MPR reduction feature

  - To discuss the following aspects

    - Whether to have separate UE capabilities for scenario 1 and scenario 2

    - Whether to indicate multiple values of fixed extension capability

- Topic #2: MPR applicability for FR1 intra-band UL CA

Sub-topic 2-2: Intra-band non-contiguous CA

Issue 2-2-1: MPR for Single CC Fallback from Non-contiguous ULCA without dual PA support

- Proposals

  - By considering the limited and unfavourable use case for UL CA and potential extra required works, RAN4 not to update MPR for Single CC Fallback from Non-contiguous ULCA without dual PA support depending on UE capability. (CATT)

- Agreement in main session:

  - Not to update MPR for Single CC Fallback from Non-contiguous ULCA without dual PA support depending on UE capability.