3GPP TSG-RAN WG4 Meeting #114-bis	R4-2505100

Wuhan, China, 7th-11th April 2025

Title: 	           WF on HPUE RF requirements

Agenda Item:	7.1.1.1.2

Source: 	Qualcomm, Mediatek, Skyworks, LGE

Document for:	Approval

# 1. PC1.5 for intra-band contiguous UL CA

## 1.1 MPR values for intra-band contiguous ULCA with contiguous RB allocation (with TxD)

Agreement:

  - The MPR values for PC3(1Tx) and PC2(2Tx) listed in the below table is just for reference

| Modulation |  | PC3 (1Tx) |  | PC2 (2Tx) |  | PC1.5 (2Tx) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  | Handheld |  | Handheld |  | FWA1 |  |
|  |  | inner | outer | inner | outer | inner | outer | inner | outer |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 7 | 3.5 | 8 | 4.5 | 9.0 | 4 | 8.5 |
|  | QPSK | 2.5 | 7 | 3.5 | 8 | 4.5 | 9.0 | 4 | 8.5 |
|  | 16QAM | 2.5 | 7 | 3.5 | 8 | 4.5 | 9.0 | 4 | 8.5 |
|  | 64QAM | 5 | 7 | 6 | 8 | 7.0 | 9.0 | 6.5 | 8.5 |
|  | 256QAM | 7 | 7.5 | 8 | 8.5 | 9.0 | 9.5 | 8.5 | 9.0 |
| CP-OFDM | QPSK | 3.5 | 8 | 4 | 8.5 | 5.0 | 9.5 | 4.5 | 9.0 |
|  | 16QAM | 3.5 | 8 | 4 | 8.5 | 5.0 | 9.5 | 4.5 | 9.0 |
|  | 64QAM | 5 | 8 | 5.5 | 8.5 | 6.5 | 9.5 | 6.0 | 9.0 |
|  | 256QAM | 7 | 8 | 7.5 | 8.5 | 8.5 | 9.5 | 8.0 | 9.0 |
|  |  |  |  |  |  |  |  | NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |

## 1.2 MPR values for intra-band contiguous ULCA with non-contiguous RB allocation (with TxD)

  - The MPR values for PC3(1Tx) and PC2(2Tx) listed in the below table is just for reference

| Modulation |  | PC3 (1Tx) |  |  | PC2 (2Tx) |  |  | PC1.5 (2Tx) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  |  | Handheld |  |  | Handheld |  |  |
|  |  | inner | Outer12 | Outer23 | inner | Outer12 | Outer23 | inner | Outer12 | Outer23 |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 6 | 11.5 | 41 | 8.5 | 14.5 | [4.5-5] | [9-9.5] | [15.5-17] |
|  | QPSK | 2.5 | 6 | 11.5 | 41 | 8.5 | 14.5 | [4.5-5] | [9-9.5] | [15.5-17] |
|  | 16QAM | 3 | 6 | 11.5 | 41 | 8.5 | 14.5 | [4.5-5] | [9-9.5] | [15.5-17] |
|  | 64QAM | 5 | 6 | 11.5 | 6 | 8.5 | 14.5 | [6.5-7] | [9-9.5] | [15.5-17] |
|  | 256QAM | 6.5 | 6.5 | 11.5 | 7.5 | 8.5 | 14.5 | [8-8.5] | [9-9.5] | [15.5-17] |
| CP-OFDM | QPSK | 3.5 | 7 | 12 | 4.51 | 9 | 15.5 | [5-5.5] | [9.5-10] | [16.5-17] |
|  | 16QAM | 3.5 | 7 | 12 | 4.51 | 9 | 15.5 | [5-5.5] | [9.5-10] | [16.5-17] |
|  | 64QAM | 5 | 7 | 12 | 6 | 9 | 15.5 | [6.5-7] | [9.5-10] | [16.5-17] |
|  | 256QAM | 7.5 | 7.5 | 12 | 8.5 | 9 | 15.5 | [9-9.5] | [9.5-10] | [16.5-17] |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |  |  |  |  |  |  |

Agreement:

- Further discussion in upcoming meetings based on the tentative values in the above table.

- The MPR contributions from companies is captured in the spreadsheet below:

## 1.3 MPR values for intra-band non-contiguous ULCA (with dualPA)

Agreement:

- Further discussion in upcoming meetings

## 1.4 A-MPR applicability for intra-band contiguous ULCA with contiguous RB allocation

Qualcomm proposal:

For all waveform type, modulations and scs when Fedge, low - BWChannel_CA ≥ 2490.5 MHz, A-MPR = MPR

For all modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

if the RB allocation is an inner allocation as defined in clause 6.2A.2.1,  then A-MPR = MPR

Except for RBstart ≤ 0.33*BWchannel_CA/0.18MHz, AMPR= max (MPR, AMPRcc).

if the RB allocation is an outer allocation as defined in clause 6.2A.2.1,

then A-MPR = MPR+1.5dB for BW Class B A-MPR = MPR for BW class C for PC3 and PC2 and A-MPR=MPR+1.5dB for BW class C for PC1.5 UE handheld devices and A-MPR=MPR+1.5dB for BW class C for PC1.5 FWA devices.

Where

-	MPR is the MPR as defined in Table 6.2A.2.1-1, Table 6.2A.2.1-1a and Table 6.2A.2.1-1b for PC3 and PC2 respectively and the respective CA bandwidth class

-	 AMPRcc is defined as the PC3_A2, PC2_A4 or PC1.5_A6 AMPR in Table 6.2.3.2-2 for PC3, PC2 and PC1.5 respectively.

Agreement:

- FFS in next meeting

## 1.5 A-MPR for intra-band non-contiguous ULCA with non-contiguous RB allocation

Agreement:

- Further discussion in upcoming meetings

## 1.6 A-MPR values for intra-band non-contiguous ULCA (with dualPA)

Agreement:

- Further discussion in upcoming meetings

## 1.7 LS to RAN5 on PSD condition RAN4 assumed to derive MPR requirements

Online Agreement:

  - Send LS to RAN5 on the PSD condition (i.e., up to 6dB) RAN4 assumed to derive MPR/A-MPR requirements for PC1.5 intra band contiguous and non-contiguous ULCA, after all MPR/A-MPR values are agreed

    - RAN4 suggests that the test for PC2, PC3 and PC1.5 can be based on equal PSD assumption.

## 1.8 Re-organize 2Tx/3Tx Notes in clause 5 and 6

Agreement:

In case the connector count and power class notes are kept in clause 5 table notes for Rel-19:

- Connector count information in clause 5 table notes shall be replaced with reference to the UL configurations in clause 6 as example implementation below: