3GPP TSG-RAN WG4 Meeting #114bis		     R4-2505105

Wuhan, Hubei, China, 7th – 11th April, 2025

Agenda item:	7.19.5

Source: 	Qualcomm Incorporated

Title: 	WF on requirements for AI/ML air interface normative work

Document for:	Approval

# Introduction

This WF captures the agreements for the discussion carried out on AI/ML under the [114bis][126]NR_AIML_air thread.

# Agreements

## CSI reporting requirement framework for CSI prediction

### Agreements in ad-hoc session (R4-2505106)

Issue 1-1: Codebook to be used in the tests

Agreement:

- In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined

- use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions

Issue 1-2: Requirement baseline

Agreement:

  - Requirement to be introduced for Dopler domain basis N4=1

      - N4>1 can be further discussed in the future if needed

Issue 1-4: Test metric

Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4)

      - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/ML  [a given SNR] using the precoders configured according to the UE reports,

      - trnd is the throughput measured at SNRue_AI/ML  [the same SNR] with random precoding.

        - trnd is obtained with Rel-15 type I single panel codebook

      - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4

## RRM core requirement and testing framework for beam management

### Agreements in main session:

Issue 2-1: Metrics/KPIs for beam ID prediction

Agreement:

- Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported:

The successful rate for the correct prediction,

  - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB,

  - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation

    - FFS on N values

  - FFS on x values

Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Issue 2-2: Absolute RSRP accuracy

Agreement:

- Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least

  - FFS whether to apply to other beams

Issue 2-3: Relative RSRP accuracy

Agreement:

- RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined

  - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

    - Reported L1-RSRP can be predicted,

    - FFS whether reported L1-RSRP can be measured RSRP

  - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Issue 2-4: Measurement period for inference

Agreement:

- For measurement delay component of the prediction delay:

  - Use “legacy” L1-RSRP measurement as the baseline for Case 1

    - FFS whether same values for M,N,P should be reused

  - FFS for case 2

Issue 2-6: Test system setup

Companies are invited to bring further analysis on the test system setup.

At least the following aspects to be considered in the analysis:

    - Whether the test can/should reflect performance in the field and whether field data should be usable in the test

    - What channel model should be used

    - UE prediction model input (e.g. whether any spatial information like UE Rx beam, UE beam pattern, etc is used)

    - Pros and cons of proposed single AoA and multi AoA setups

For the multi AoA test setups, companies are invited to bring further analysis on the following aspects:

  - Define candidate test setups which are already feasible (e.g. based on eIFF, MPAC etc.)

  - Propose simplified channels suitable for the defined candidate test setup(s)

  - Validate the proposed simplified channels through AIML BM system level simulations, especially by comparing the performance with the non-simplified (theoretical) channels.

  - Assess the implementation of the simplified channels on the candidate systems, after the probe layout and the channel model have been clarified.

  - RRM core requirement and testing framework for Positioning accuracy enhancement

### Agreements in main session

Issue 3-2: LOS/NLOS indicator

Agreement:

the LOS/NLOS indicator is out of scope of WID. Stop discussions on it.

Issue 3-3: Rx-Tx time difference for case 3a

Agreement:

- Introduce gNB Rx-Tx time difference reporting accuracy requirements

    - Reuse legacy accuracy requirements

Issue 3-4: Report mapping for gNB Rx-Tx time difference

Agreement:

- Reuse the existing report mapping for gNB Rx-Tx time difference measurements