# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2505106

Wuhan, Hubei, China, 7th – 11th April, 2025

Agenda item:	7.19.5

Source: 	Qualcomm Incorporated

Title: 	Ad hoc minutes on AI/ML air interface

Document for:	Approval

# Introduction

The discussion on the AI/ML study is organized in two threads [126] and [127] in RAN4#114-bis. The ad-hoc meeting will discuss some of the topics from the moderator summary in [1].

# Discussion

## Requirements for CSI Prediction

### Sub-topic 1-1

Reporting codebook

Inline with RAN1 agreement, it should be confirmed that Rel-18 Doppler codebook will be used for AI/ML based reporting tests

Issue 1-1: Codebook to be used in the tests

- Proposals

  - Option 1: use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions

  - Option 2: further discuss

- Recommended WF

Option 1

Discussion:

R&S: Are we talking about PMI only or also CQI?

E///: should be only PMI. Only PMI prediction is discussed in RAN1

Agreement:

- In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined

- use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions

### Sub-topic 1-2

Requirement baseline

It is proposed to define requirements for Dopler domain basis N4=1, N4>1 might be introduced as an option.

Issue 1-2: Requirement baseline

- Proposals

  - Option 1: Requirement to be introduced for Dopler domain basis N4=1

    - N4>1 can be further discussed in the future if needed

  - Option 2: Others

- Recommended WF

  - Option 1

Discussion:

Agreement:

  - Requirement to be introduced for Dopler domain basis N4=1

      - N4>1 can be further discussed in the future if needed

### Sub-topic 1-3

Test baseline

Several companies brought suggested taking the Rel-18 PMI prediction test setup as baseline

Issue 1-3: Test baseline

- Proposals

  - Option 1: Take Rel-18 PMI prediction test setup as baseline

  - Details/parameters specific to AI/ML based predictions to be further discussed

  - Option 2: Following test setup to be taken as starting point (R4-2504539):

  - Option 3: others

- Recommended WF

  - Option 1

Also discuss if the proposal in Option 2 can be taken as baseline or more discussion is needed.

Discussion:

QC: Rel-18 to be taken as baseline might be premature. Rel-18 performance is similar to Rel-16 so we should differentiate more. With Ai/ML we see relatively high gains.

Samusng: to verify the requirement, we can discuss the basis test. We can refine the proposal for Option 2 for the UL slots. Based on RAN1 assumptions, both periodic and aperiodic are supported. We need to verity the AI/ML function so we focus on a simple setup.

E///: We also think it’s premature to consider Option 1, RAN1 is discussing the timeline for reporting. We can consider Option 1 for now but we might have to change.

### Sub-topic 1-4

Test metric

Several companies proposed to use relative throughputγas the metric for the requirements/tests. Some of the details also need further discussions

Issue 1-4: Test metric

- Proposals

  - Option 1: The requirements for AI/ML based CSI prediction(or AI/ML based PMI prediction) could be specified in terms of the ratio:

    - γAI/ML=tue_AI/MLtrnd

      - tue is 90 % of the maximum throughput obtained at [a given SNR, or a SNR range ] using the precoders configured according to the UE reports, and trnd is the throughput measured at [a given SNR, or a SNR range ] with random precoding.

        - trndis obtained with Rel-15 type I single panel codebook

      - FFS how γAI/ML is calculated:

        - Option 1: γ_(AI/ML) is calculated for each predicted CSI and then do the average within the CSI prediction window

        - Option 2: t_(ue_AI/ML) and t_rnd are calculated within the CSI prediction window first and then get the averaged γ_(AI/ML) value.

        - Option 3: FFS

  - Option 2: other proposals

- Recommended WF

  - Option 1

If Option 1 is agreed, it should be discussed whether Option 1 or 2 for how to calculate the throughput can be already agreed or this is left FFS (Option 3) for a future meeting

Discussion:

MTK: throughput should be measure over a larger window, if we take just this prediction window we cannot get an accurate result

R&S: general formula is fine but to make this a test we need to average over a large window because of fading. We have a fixed SNR for a certain throughput but it’s different for different UEs depending on the performance.

Nokia: same metric for PMI reporting should not be confused as same target requirement

Samsung: we agree in general but I would to mention we expect to achieve better performance. We should have an additional check that the gain is higher compared to the non-AI/ML based test.

oppo: if we want to simulate which of the options is better, we probably need to change N4=1. Companies could think about different options, if we have multiple samples within the prediction window, companies can compare the results. Maybe we can start N4=1 but we do not expect any difference between the options.

Samsung: it’snotclear how to define the prediction window

Agreement:

Requirements to be defined based on the ration(same metric as for the current PMI reporting requirements in 38.101-4)

    - γAI/ML=tue_AI/MLtrnd

      - tueAI/ML is 90 % of the maximum throughput obtained at [a given SNR ] using the precoders configured according to the UE reports,

      - trnd is the throughput measured at [the same SNR ] with random precoding.

        - trndis obtained with Rel-15 type I single panel codebook

FFS how γAI/ML is calculated, companies are invited to bring proposals in the next meetings

### Sub-topic 1-5

Simulation results and next steps

Several companies submitted simulation results and proposals for next steps (refinement of parameters, etc)

Issue 1-5: Simulation results and next stepts

- Proposals

  - Discuss the simulation results and next steps based on the simulation summary prepared by oppo

    - Simulation results

    - Refinement of parameters:and what to for the next steps

      - Introduce realistic channel estimation

      - Further discuss CSI-RS configuration, codebook configuration, Dopller, SNR, etc

      - Other proposals

- Recommended WF

  - To be discussed

Discussion on refinement of simulation parameters to be done mainly offline

Discussion:

Oppo: there is some performance gaps between the submitted results. Maybe we can check whether we can do some data sharing to check whether training sets are aligned. For the dataset maybe we could start with test data sharing. Some companies suggested we could check step 2 or throughput results.

Moderator: might be premature to start another data sharing effort because it takes time and this a single sided model. Maybe another round of alignment would be the first thing to try at this moment.

E///: we agree. SGCS alignment is not our goal. So far we used ideal channel estimation. We are using Rel-18 codebook. We should compare throughput with Rel-18 codebook and practical channel estimation.

Samsung: we agree with the moderator suggestion.. we do not have a clear criterion for SGCS comparison. Finally, we need to compare the throughput.

Apple: we would like to reemphasize that this exercise is just meant to have a better understanding of the basic models and whether we need a reference model or not. Going to throughput will be a much bigger step. We should first use SGCS or NMSE to understand how good alignment is. We need to first see whether it’s feasible to move to throughput prediction.

Oppo: we think that to kick off step 2 would be fine and we could also check throughput. For alignment, we need to look at the results. The throughput might not be that different. For 100Hz, the difference can be higher. If companies find that they are outliers, we can check offline if there are some issues like overfitting or data is not generated correctly.

Oppo to lead the offline discussion on further alignment and whether to move to step 2

### Sub-topic 1-6

Generalization

Several companies proposed to further study generalization issues and how to ensure that the UE performance does not degrade under different conditions.

Issue 1-6: Generalization

- Proposals

  - Option 1: Mix several static conditions to endure the generality of the prediction model

  - Option 2: Introduce tests with TDL-C300 or different number of CSI-RS ports, etc

  - Option 3: Introduce tests with CDL channel model if available

  - Option 4: further investigate the granularity of Doppler spread for which a new AI/ML model would be needed

- Recommended WF

  - Postpone discussion to future meetings

In the previous meeting it was agreed that generalization would be discussed later on when the requirements and tests details (e.g. conditions, etc) would become more clear. Companies can still consider the above proposals when discussing the definitions of the requirements and tests.

Discussion:

### Sub-topic 1-7

LCM

Several companies proposed to further LCM procedures while other companies propose to wait for further progress in other WG.

Issue 1-7: LCM

- Proposals

  - Option 1: Wait for further progress in RAN1/2 before continuing the discussion in RAN4

  - Option 2:

    - RAN4 to study the model-ID-based LCM requirements if localized models are considered for CSI prediction.

    - RAN4 to study delay requirements in Steps 4 and 5 of the LCM procedure based on RAN2’s 5-step approach.

    - RAN4 to discuss whether the delay components of LCM operations can vary depending on the specific CSI report configurations.

  - Option 3: RAN4 to discuss the testability of monitoring reporting options, and RAN4 could inform RAN1 if RAN1 does not make decision.

- Recommended WF

  - Option 1

Once the details become clear, RAN4 will discuss whether/how to define requirements for LCM. Above proposals can be considered in future discussions

Discussion:

## Requirements for Beam management

### Sub-topic 2-1

Beam ID prediction KPIs

The options for the beam prediction KPIs were downselected in the previous meeting, the KPIs for beam IDs still have to be agreed. The previously tentative agreement

-	Option 2: Beam prediction accuracy

-Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam"

-Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

-Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams"

-	Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

-Related measurement accuracy can be considered to determine x

-	Option 4: combinations of above options

Issue 2-1: Metrics/KPIs for beam ID prediction

Proposals

  - Option 1: Adopt only Option 3 as is

  - Option 2:Modified Option 3:

    - All top-K predicted beams have to be considered in the success rate evaluation

    - RSRP of each top-K predicted beam is to be compared with the RSRP of its equal ranked genie-aided beam - X

  - Option 3: Option 2 only with Top-K/1 (%)

  - Option 4: Option 2 and Option 3

  - Option 5: Wait for more simulation results to understand how the different options work

  - Option 6: Others (e.g. different distributions, etc): Normalized Discounted Cumulative Gain (nDCG)

- Recommended WF

  - Option 5

Discussion:

QC: we can check the simulation results, option 2 and 3 were simulated. Other metrics like modified option 3 or 6 have not been simulatedds

Nokia: modified option 3 is our proposal. The original only verifies top 1, we should check others. We have some results on modified option 3 and they show that this is a better fit to check the AI performance

Nokia: we cannot agree with the original option 3.

Vivo: in the last meeting we did downselection as much as possible. Maybe we can do further downselectoin. Even like this we have big variance in the results, we should try to find the root cause for this.

NTU: can we clarify what other information we are trying to get. We can do some analysis on how different options work. If we do sims, are we going to pick based on the options with better alignment? The requirements should have a meaning from a system performance perspective.

E///: we do not understand the purpose of the simulations. Different parameters will cause different results. we do not have any understanding of that is successful rate

QC: can we try option 2 with K/1 (option 3 above)

Nokia: if we look at the results, this metric does not look like a good metric. 2nd drawback is that we are not really verifying all the predictions. If RSRP is also predicted, it could be used as an additional metric. If it is not predicted, this is not a good metric.

QC: it’s not a major drawback, it can be addressed. Network could do further refinement and based on the report it could choose.

NTU: modified option 3, is an extension of original option 3. Option 3 is verified by both options. We can agree option 3 and then we can see if we want to check the 2nd and 3rd predicted beam.

Samsung: if we add this further discussion case it will be too complex. Question to Nokia, how will the network use top 2 or top 3 prediction results.

Xiaomi: RAN1 also has some discussions for this on LCM. We should align with the rAN1 design. We have concern about option 3. It’s an unfair comparison. For beam index, if any of the beam is close enough it is enough. Option 3 also needs some revision.

NTU: proposal is option 3. Further discuss if there should be any requirement on the predicted beams other than top 1.

### Sub-topic 2-2

Absolute RSRP accuracy applicability

Issue 2-2: Absolute RSRP accuracy

- Proposals

  - Option 1: Absolute RSRP accuracy requirement applies to all predicted beams (all beams in the report)

  - Option 2: Absolute RSRP accuracy requirement applies to only a subset of predicted beams

  - Option 3: Others

- Recommended WF

  - To be discussed

Discussion:

### Sub-topic 2-3

Relative RSRP accuracy

Relative RSRP accuracy has been discussed for a few meetings without any clear agreement

Issue 2-3: Relative RSRP accuracy

- Proposals

  - Option 1: The relative RSRP accuracy for AI/ML-based beam prediction = the ground truth of L1-RSRP of the predicted beam index i - the ground truth of the L1-RSRP of the Top-1 ideal beam index j

- Option 2: The relative RSRP accuracy for AI/ML-based beam prediction = Absolute L1-RSRP Accuracy of the predicted beam index i – Absolute L1-RSRP Accuracy of the best-predicted beam.

  - Option 3: Relative RSRP accuracy = (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest predicted value.

  - Option 4: Relative RSRP accuracy=predicted L1-RSRP of beam index i - ground truth of the beam index n ,where the beam index n owns the largest value

  - Option 5: relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest RSRP among the predicted beams.

  - Option 6: Relative RSRP accuracy for AI/ML BM can be defined as:

    - relative RSRP accuracy = (reported predicted RSRP of beam i – reported RSRP of beam j) – (reported RSRP of beam i – reported RSRP of beam j) where beam i is one of the top-K predicted beams and beam j is the beam with maximum measured RSRP

    - RAN4 to further discuss whether the reported RSRP of beam j can be predicted or measured or a combination of both predicted and measured samples.

  - Option 7

- Recommended WF

  - To be discussed

Discussion:

### Sub-topic 2-4

Measurement period for inference

Requirements for the measurement period are discussed by multiple companies and should be defined as core requirements

Issue 2-4: Measurement period for inference

- Proposals

  - Option 1: Use “legacy” L1-RSRP measurement as baseline

    - FFS whether same values for M,N,P should be reused

  - Option 2: Split case 1 and case 2

    - Reuse L1 measurement period for Case 1

    - Introduce configurable number of measurement occasions for case 2

  - Option 3: Measurement period should be configurable

  - Option 4: Others

- Recommended WF

  - To be discussed

Discussion:

### Sub-topic 2-5

Simulation results

Issue 2-5:	Simulation results

- Proposals

  - Option 1: Discuss the simulation results based on summary – to be prepared by vivo

    - Simulation results

    - Refinement of simulation assumptions (e.g. error modelling, etc)

- Recommended WF

  - To be discussed

Further align the error model?

Discussion:

Vivo: we do not know yet the root cause of the variance. We can have some offline discussion but we would need some initial direction.

MTK: as far as we know, some companies did the simulation on the scenario predicting CSI-RS based on SSB, others are just predicting SSB. Scenarios are different, first time is more useful. Can we agree to do just CSI-RS based on SSB. SSB should be a wider beam and CSI-RS should be a narrower beam.

NTU: we do not have a clear definition of set B(it’s just a subset of set A but it should be clear which ones are picked)

QC; we are doing the sims based on system but the test will actually be for a link level.

### Sub-topic 2-6

Test system basic setup

The basic test setup has been discussed for several meetings without much progress.

Issue 2-6: Test system setup

- Proposals

  - Option 1: Single AoA with UE antenna pattern knowledge

  - Option 2: Single AoA without any UE antenna pattern knowledge

  - Option 3: Multi AoA based on MPAC

  - Option 4: Multi AoA based on enhanced IFF RRM testing setup

    - Exact definition of enhanced IFF is FFS

- Recommended WF

  - To be discussed

Aspects to consider in the discussion based on the companies’ inputs:

Test objectives and data to be used

- whether the performance under synthetic channels can reflect the performance of AI/ML models in the real field.

- discuss what the testing goal is, to verify the model can work well in real field or just to test UE can train a model that works well under a specific scenario.

- If the testing goal is to verify the model can work well in real field, and the performance under a specific channel/scenario cannot reflect the performance in the real field, it is necessary to design a test system that is possible to use field data in the test.

UE models and impact on testing setup and performance:

There are currently three kinds of presumable UE patterns from the AI model input PoV.

UE pattern 1: No need to input Rx Beam information in the AI model.

In this case, it is expected that the UE Rx beam is pointing towards the direction of the measurement probe in an OTA chamber where the DL signals are transmitted.

UE pattern 2: One Rx Beam information is input to the AI model.

In this case, direction of the UE Rx beam is same as UE pattern 1 and that Rx Beam information is input to the AI model. TBD if the actual Rx beam pattern is necessary or it can be standardized.

UE pattern 3: Multiple Rx Beam information is input to the AI model.

In this case, for each Tx Beam ID in set B, UE will change its Rx beam direction to multiple directions and multiple L1-RSRP measurements are made per Tx Beam ID. Multiple Rx Beam information is input to AI model per Tx Beam ID.

Observation based on above models:

BM-Case 1 tests with UE pattern 1 and 2 (i.e. less than or equal to one Rx Beam information is input to the AI model) can be conducted with the following conditions.

- Rx beam pattern is standardized or,

- Rx beam pattern is measured during the evaluation.

UE beam management related issues

RAN4 to discuss and conclude whether following aspects shall be part of the UE AI/ML BM requirement and/or testing:

a) UE Rx antenna pattern knowledge,

b) UE Rx antenna performance,

c) UE Rx beam sweeping performance

d) UE Rx beam index (as input for the AI/ML model).

Discussion:

### Sub-topic 2-7

Test system channel model

In the previous meeting it was agreed that a CDL-based channel model will be taken as starting point.  Further details need to be discussed

Issue 2-7: Test system channel model

- Proposals

  - Option 1: “full” CDL model to be emulated in the test

  - Option 2: simplified CDL should be used

  - Option 3: others

- Recommended WF

  - To be discussed

If Option 2 is chosen, simplification methods should be discussed.

Discussion:

# References

[1] R4-2504684, “Topic summary for [114bis][126] NR_AIML_air_part1”, Moderator(Qualcomm Incorporated), RAN4#114-bis