3GPP TSG-RAN WG4 Meeting #114bis											R4-2505108

Wuhan, China, 7th ‒ 11th April, 2025

Agenda item:			7.20.3

Source:	Moderator (Ericsson)

Title:	Ad-hoc minutes for [114bis][127] NR_AIML_air_part2

Document for:	Information

# Introduction

,

# Topic #2: CSI compression two sided models

This topic addresses the study on two-sided CSI compression. It is divided into two sub-topics. The first sub-topic is about the simulation campaign investigating options 3 and 4 for requirement definition. The second sub-topic addresses issues relating to options 3 and 4 that are not directly related to the simulations.

## 2.1	Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503255 | Qualcomm | Observation 1: The SGCS obtained with separate encoder decoder training (QC encoder with Samsung’s frozen decoder) in option 3 with track 2 mixed dataset is almost identical to the SGCS obtained with Samsung’s joint encoder-decoder training. Observation 2: Option 3 (fully specified decoder) leads to good SGCS in, at least, the scenarios where encoder is trained with the same dataset as the decoder. Observation 3: Option 4a leads to good SGCS when Qualcomm’s “own” encoder, derived through a two-step approach from the labelled training dataset, is tested with Samsung decoder and mixed dataset. The feasibility of option 4a will depend on how Qualcomm’s “own” encoder performs other companies’ “nominal” decoders that were generated based on labelled dataset. - Note 1: “Labelled” dataset refers to (encoder input, encoder output) - Note 2: “Labelled dataset” was generated based on mixed dataset and Samsung reference encoder. Observation 4: In option 4b, - TE is expected to train its decoder based on reference encoder. - UE is expected to develop a nominal decoder that works well with the reference encoder and then train its encoder based on the nominal decoder. - There will be mismatch between TE and UE if the training dataset, i.e., channel information, is not specified. Observation 5: Training dataset, i.e., channel information, and reference encoder both need to be specified in option 4b. Observation 6: If channel information and reference encoder are specified in option 4b, then option 4b becomes a superset of option 4a where (channel information, encoder output) is specified. Proposal 1: RAN4 specifies both training dataset (channel information) and reference encoder in option 4 and focuses only on this option for the partially specified approach. Proposal 2: RAN4 also considers specifying a reference decoder in option 4 so that UE can derive its encoder through one step and TE can get a guideline to implement its decoder during the tests. |
| R4-2503258 | NTU | Proposal 1: Clarify the following for procedures of option 4a test decoder feasibility study: - What is the content of dataset from option 3 track 1 and option 3 track 2? - Whether option 3 track 1 has an encoder selected besides the selected fixed decoder - If we use the encoder/decoder selected from option 3 track 1, what’s the additional alignment needed given that we agreed to have “Each company is encouraged to train own encoder using the fixed decoder and their own dataset, and then test with every companies datasets, in order to check for consistency in results” procedure in option 3 discussion, which is the same as step 6 and 7. Similar issue is observed if we use dataset from option 3 track 2 - If the mixed dataset from track 2 is used, how to train the encoder and decoder based on it given that it only contains encoder input? - How to verify whether the following agreed aspects of option 4 are feasible or not? - Test repeatability should be ensured (variation among TE vendor implementations should be bound) - Other vendors should also be able to develop such a decoder and which can deliver similar performance Proposal 2: Clarify the following for procedures of option 4b test decoder feasibility study - How to select one encoder from companies’ own encoder in option 3 track 1? - Given that option 4b aims to select one reference encoder, why we need to check multiple encoder alignment in step 6 after one is selected already? - How to verify whether the following agreed aspects of option 4 are feasible or not? - Test repeatability should be ensured (variation among TE vendor implementations should be bound) - Other vendors should also be able to develop such a decoder and which can deliver similar performance More importantly, we want to ask for clarification what kind of feasibility is verified by the procedures captured in the WF. Proposal 3: Clarify the feasibility that can be verified by the agreed procedures and its limitation as follows: - Option 3 track 1: We verify the performance alignment of the selected test decoder and its own dataset is feasible. However, if we select a different decoder with a different dataset in the WI stage, the performance alignment may not be feasible, and RAN4 has to redo the feasibility study. - Option 3 track 2: We verify the performance alignment of the selected test decoder and the selected multiple source datasets is feasible. However, if we select a different decoder with different datasets in the WI stage, the performance alignment may not be feasible, and RAN4 has to redo the feasibility study. - Option 4a: We verify the performance alignment of test decoders trained with the selected dataset. However, if we select a different dataset in the WI stage, the performance alignment may not be feasible, and RAN4 has to redo the feasibility study. - Option 4b: We verify the performance alignment of test decoders trained with the selected reference encoder. However, if we select a different reference encoder in the WI stage, the performance alignment may not be feasible, and RAN4 has to redo the feasibility study. Proposal 4: Additional steps for option 4a and 4b feasibility study: - Define the decoder verification criterion from one of the following options (for 4a and 4b, respectively): (Option 4a: specify dataset) - 4a-1: RAN4 derive the threshold δ via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying z∈Zfgref(z),gtest(z)<δ, where Z is the set of decoder inputs (latent messages) in the specified dataset (from step 2), and f is the chosen loss/similarity function gref(z) is the test decoder output and gtest(z) is from the dataset. - 4a-2: RAN4 studies whether it is feasible to derive a test decoder satisfying gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z where the notations are the same as 4a-1, and Ytest is the test decoder output space. (Option 4b: specify reference encoder) - 4b-1: RAN4 derive the threshold δ and the encoder input X generation procedure via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying x∈Xfgref(z(x)),gtest(z(x))<δ, where z(x) is the reference encoder output (latent message) of encoder input x, and X is the sampled encoder input in the test decoder verification procedures. Note that RAN4 needs to specified the generation procedures of X and specifying - 4b-2: RAN4 studies whether it is feasible to derive a test decoder satisfying gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z where Ytest is the test decoder output space and Z is the reference encoder output space. - RAN4 check if the decoders derived from the above procedure can satisfy test repeatability requirement, i.e., the loss function delta between two different decoders when connecting to the same encoder, is within a RAN4 agreed margin. If yes, we can conclude that option 4 (at least one sub-option) is feasible. Proposal 5: Next steps for feasibility study of option 4c - Use the encoder input part of the dataset from option 3 feasibility study, use principle component analysis to derive eigenvectors and eigenvalues of the encoder input dataset. - RAN4 decides how many dominant eigenvectors, and the number of quantization bits for the coefficients of eigenvectors (sum to the number of bits in the latent message) to include in indexing procedure in proposal 3. - Companies propose any encoder-decoder pair, with the latent message reordering based on the procedures in proposal 3 and the dominant eigenvectors decided in step 1 - RAN4 check if the decoders derived from the above procedure can satisfy test repeatability requirement, i.e., the loss function delta between two different decoders when connecting to the same encoder (also following the latent message reordering procedure), is within a RAN4 agreed margin. If yes, we can conclude that option 4c is feasible. Proposal 6: The latent message reordering procedure is in the following: Step 1: Perform principle component analysis (PCA) on encoder input dataset, i.e., derive the SVD of the covariance matrix of the encoder input dataset. Step 2: Select the eigenvectors corresponding to the M largest eigenvalues as dominant eigenvectors. Step 3: Derive the coefficients of each of the decoder output on the (incomplete) basis composed of the selected M eigenvectors. Step 4: Suppose the latent message length is N. Distribute the N bits to represent the coefficients of the M eigenvalues proportional to the magnitude of the corresponding eigenvalues. Step 5: Quantize the coefficients to form the latent message corresponding to each decoder output. The mapping from the new latent message from these steps to the original latent message before reordering is added before decoder input. The mapping from the original encoder output to the new latent message from these latent messages is added to the encoder output. Proposal 7: Additional margin to account for the mismatch xref(z)-xtest(z)=minx∈Xtestxref(z)-x need to be considered when determining requirement. Proposal 8: Consider the test decoder selection criterion based on the agreed step, i.e., check whether performance alignment across contributing companies’ “own encoder”, which may have different structures, can be achieved. Therefore, companies can bring the test decoder proposal together with performance metrics achieved when connecting to different encoders with different structures, e.g., the structure options for test decoder discussion, but trained with the decoder input and output dataset from the proposed decoder. |
| R4-2503271 | Mediatek | Observation #1: Option 3 Track 1 SGCS results with frozen pretrained decoders using companies own datasets can in most cases closely match reference SGCS results where Encoder and Decoder are trained together (Reference). Observation #2: Dataset can have an impact on SGCS results for some shared frozen pretrained decoder models. Observation #3: Option 3 Track 2 SGCS results with frozen pretrained decoder (trained with mixed dataset) using own or mixed dataset can in most cases closely match reference SGCS results where Encoder and Decoder are trained together (Reference). Observation #4: Option 3 Track 2 SGCS results with frozen pretrained decoder (trained with mixed dataset) using own or mixed dataset can be poor in unquantized training where we suspect convergence issue with agreed hyperparameters. Observation #5: Option 4a SGCS results with using labelled dataset can closely match reference SGCS results where Encoder and Decoder are trained together (Reference). Observation #6: Option 4a SGCS results with frozen pretrained decoder from previous step can in most cases closely match reference SGCS results where Encoder and Decoder are trained together (Reference). Observation #7: Option 4a SGCS results with frozen pretrained decoder from previous step can be poor in unquantized training where we suspect convergence issue with agreed hyperparameters. |
| R4-2503410 | CATT | Observation 1: In Option 3 track 1, SGCS spans from 0.6868 to 0.7246 for un-quantized model and from 0.6557 to 0.7013 for quantized model. Observation 2: Option 3 track 1 is feasible when characteristic of training datasets and test datasets are aligned. Observation 3: In Option 3 track 2, SGCS is 0.7206 for un-quantized model and 0.713 for quantized model. Observation 4: CNN encoders with different number of ResNet blocks have slightly different SGCS. Observation 5: Option 4a and 4b will be essentially the same, i.e., equivalent to Option 4a, if mixed dataset is used to train own decoder in Option 4b and own encoder in both options. Observation 6: In Option 4a, SGCS is 0.7411 for un-quantized model and 0.6939 for quantized model when the mixed dataset is used; 0.7265 and 0.6722 respectively when own dataset is used. Observation 7: Compared with SGCS with the mixed dataset used to own encoder, a performance degradation, around 2%, is observed when own dataset is used. Observation 8: In Option 4b, SGCS is 0.749 for un-quantized model and 0.6841 for quantized model when own dataset is used. Proposal 1: Use own dataset to train own decoder and own encoder in Option 4b. Proposal 2: In Option 3, the encoder corresponding to the test decoder is selected as reference encoder. Proposal 3: FFS where to capture the reference encoder when defining performance requirements in WI, if exists. Observation 9: For both Option 3 track 1 and 2, it would be helpful to capture the training dataset. Proposal 4: Storing the training dataset in a dedicated folder can be considered for both Option 3 track 1 and 2. |
| R4-2503516 | Apple | Observation 1: The feasibility of Option 3 Test Decoder under Track 1 cannot be established for all the chosen test decoders due to the performance dependency on the testing dataset. That is, in the absence of an agreed training dataset, if each company trains each own encoder with own training dataset, then the testing performance at TE with the trained DUT will be testing-dataset dependent. DUT will perform differently depending on the testing dataset. From simulation results, there is a statistical difference between companies’ datasets. Observation 2: Performance of Own trained (mixed dataset) Encoder with Mixed Dataset trained Decoder is more consistent across various companies testing datasets. However, the performance on testing datasets that don’t belong to the training dataset (QC, CATT) is significantly degraded mirroring the issues observed in track1. Observation 3: To achieve reproducible tests and guarantee interoperability, the specification will need to capture the following aspects: - The latent space representation to be captured by the decoder - An aggregate dataset from UE vendors, featuring UE-preferred encoders in the form of {V, c}, aiming to enrich the latent space and enhance interoperability. Observation 4: Input CSI data distribution can be used to detect a monitoring event. By detecting monitoring events without ground truth CSI excessive overhead signaling can be avoided Observation 5: - SGCS Doesn't Capture Information or Capacity Loss Directly SGCS only measures angular similarity between CSI matrices, ignoring magnitude differences that impact channel quality. Information loss due to quantization, feedback delay, or imperfect CSI compression is not explicitly reflected in SGCS. Capacity loss is a function of channel power and spatial correlation, but SGCS does not account for power scaling effects, which are crucial in MIMO systems. - SGCS is Weakly Correlated with Throughput Throughput depends on spectral efficiency, which is determined by the achievable rate formula. SGCS does not directly relate to the determinant or eigenvalues of H, making it an incomplete indicator of spectral efficiency. The impact of CSI errors on MCS selection and beamforming is non-trivial, making it hard to infer actual throughput loss from SGCS degradation. - SGCS Does Not Capture CSI Feedback Imperfections Well Compression artifacts: If a UE sends quantized CSI feedback, the distortion in signal space may not align with angular deviations captured by SGCS. Interference and noise effects: SGCS does not distinguish between CSI errors due to channel noise versus quantization/compression, leading to ambiguities in interpreting feedback accuracy. Proposal 1: To establish the feasibility of the Test Decoder under Option 3 and Track1, each company should train its own encoder using the fully specified Test Decoder and its own training data. The trained encoder should then be tested with the Test Decoder using datasets from different companies and demonstrate performance consistency Proposal 2: Investigate a Channel State Information (CSI) metric that provides a statistical measure of how well CSI can be compressed, directly translating to SGCS performance. Using this approach, a dissimilarity metric between CSI training and testing datasets can be derived to predict or interpret SGCS results. One such metric could be Angular-Delay Channel Entropy (ADCE), which quantifies the uncertainty in CSI’s spectral distribution and reflects its compressibility. The ADCE can be defined as ADCE=ijPijlog2Pij, where Pij=Hij2¦¦H¦¦F2, and Hij=FdHFaH is a virtual channel representation in angular/delay domain which separates multipath components into resolvable delay taps and spatial angles. ADCE captures the sparsity or randomness of these components. Proposal 3: To determine the initial feasibility of test decoder under Option 3 and Track 2, each company to train Own Encoder with Test decoder with the mixed dataset. Subsequently each company tests the Enc/Dec pair with the mixed dataset to ensure performance consistency across all companies. Proposal 4: RAN4 to investigate potential performance loss due to the dataset distribution mismatch (field data distributions) when one or both parts of the AI/ML models are specified with a synthetic dataset. Proposal 5: RAN4 to investigate the similarity between Option 3 and Inter-vendor collaboration direction C (Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1) from RAN1 study Proposal 6: RAN4 to investigate how many test decoders need to be specified under Option 3. Identify the granularity of scenarios/deployments that will dictate a new test decoder. Proposal 7: To complete feasibility analysis under Option 3, the following question should be answered: Performance Consistency Across UE Encoder Implementations: Can the performance be maintained across different UE encoder implementations? Specifically, if the UE encoder uses a different AI/ML backbone (e.g., a Transformer-based architecture), does the system still achieve comparable results? Proposal 8: To align with real-world deployment scenarios and advance toward defining performance requirements, the feasibility analysis should be expanded to include at least two layers Proposal 9: RAN4 to employ the procedures shown in the flowchart for studying the feasibility analysis of test decoder Option 3. (It captures both Track-1 and Track-2) Proposal 10: The following criteria should establish the feasibility of the test decoder: - The test decoder (along with the encoder) should deliver reasonable throughput performance gain compared to e-type II in ideal conditions. RAN4 to extend the feasibility analysis to higher ranks and different payload sizes to determine consistency of performance improvement. FFS on how to quantify the acceptable performance gain. (performance-complexity tradeoff) - All UE preferred encoders should be aligned (work well) with the chosen test decoder Proposal 11: The following principles should guide the establishment of the alignment aspect of the feasibility analysis (steps 6-7): - A well-trained UE encoder with the same or higher complexity compared to the Ref encoder that the test decoder was trained with, should have at least similar performance with the reference performance. - FFS on how to define a criterion for the similarity/dissimilarity of all companies' performance results with respect to the reference performance, considering UE encoder complexity as well. Proposal 12: RAN4 to employ the detailed procedures shown in the flowchart for studying the feasibility analysis of test decoder Option 4a and Option 4b based on the aggregate dataset and database of test decoder models - The architecture of the test decoder is partially specified through some high-level parameters (to be agreed) while leaving room for vendor specific implementations - Interested companies implement a test decoder according to the partially specified agreement - The partially specified test decoder is trained from an aggregate data set of UE encoder latent space distributions (dataset is stored in a server). Multiple test decoders are trained from participating companies. The test decoders are stored in a database. - Each UE vendor tests its own preferred Encoder with the test decoders stored in the database - Analysis of results to determine feasibility of test decoder Proposal 13: Consider the following aspects regarding the different conditions for testing generalization for CSI AI/ML use: - Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports) - Various antenna spacings (e.g., 0.5 lambda, 0.8 lambda, etc) - Various antenna virtualization (TxRU mapping) - Various carrier frequencies and bands (e.g., 2GHz, 4.0GHz) - Various outdoor/indoor UE distributions for UMa/Umi - Various UE speeds. Consider the following aspects regarding the scalability aspect for generalization testing for CSI AI/ML use: - Various bandwidths (e.g., 20MHz, 50MHz) and/or frequency granularities, (e.g., size of subband), different layers - Various sizes of CSI feedback payloads Proposal 14: Investigate CSI similarity of input data distributions using Angular-Delay Channel Entropy (ADCE), as a statistical measure. This approach aims to reduce the reliance on more expensive monitoring procedures, such as those based on KPIs and ground truth availability, by initiating these procedures only when significant changes in data distribution are detected. The ADCE can be defined as ADCE=ijPijlog2Pij, where Pij=Hij2¦¦H¦¦F2, and Hij=FdHFaH is a virtual channel representation in angular/delay domain Proposal 15: RAN4 to investigate defining metrics for Monitoring CSI Input Distribution to Trigger Monitoring Procedures and their Application to Dynamic CSI Compression Proposal 16: For Monitoring CSI AI/ML Models investigate embedding a subset of known values, referred to as redundancy elements, into the CSI vector at the UE before encoding. These elements act as reference points that can be tracked throughout the encoding and decoding processes. The approach is designed to facilitate the monitoring of AI/ML models by providing a measurable and consistent benchmark for evaluating model performance. Proposal 17: We propose a two-stage approach for CSI model monitoring, first distinguishing whether performance degradation stems from the AI/ML model or external factors, and then isolating the specific issue within the model if identified as the cause. Proposal 18: RAN4 to investigate a performance monitoring framework to detect issues such as data drift, model degradation, and AI/ML model aging, triggering actions like re-training, re-configuration, or model switching. The system will ensure high detection accuracy, real-time monitoring, and proactive predictions, enabling reliable adaptation to changing channel conditions while maintaining optimal performance. Proposal 19: For a performance monitoring framework, event detection for data distribution drift and anomalous behavior is based on the output of a trained model that predicts performance deviations (the event). Consider the following cases as shown below: (1) Indirect Event Detection: AI/ML model for SGCS EST is trained to predict SGCS from latent space c. Subsequently a detection logic can trigger event detection (2) Direct Event Detection is based on AI/ML model to directly predict an event Proposal 20: Consider a new KPI for CSI compression that could offer an indication of spectral efficiency loss due to misalignment of precoders. It can be conceptually described as KPI:Closs=C​P=V-C​P=Vc where Vc is the corrupted reconstructed CSI Proposal 21: Investigate the benefits of defining a new KPI for CSI compression that is linked to capacity loss. For an SVD receiver this metric is approximated as Closs,SVD≈k=1Llog2ρρIkλk/L+1ρIk+vkHvck2, where ρIk=j=1,j≠kLvkHvck2 is defined as the inter-stream interference to layer k caused by the mismatch between Vc and V. Also ρ is the SNR and λk is the eigenvalue of the channel H'H For rank 1: the proposed metric becomes Closs=-log2v1Hvc1, while SGCS=v1Hvc122/vc122. Under ideal conditions the term ρIk = 0 due to the orthogonality. But under imperfection this term contributes to the denominator and the capacity decreases. Proposal 22: Investigate the feasibility of using the spectral efficiency KPI to determine the CQI adjustment for option 1b. |
| R4-2503773 | CMCC | Proposal 1: it is proposed to discuss if option 3 is feasible, whether option 4 is still needed. Proposal 2: it is proposed to discuss and allign on which part of training dataset are used for testing and/or inference . Proposal 3: it is proposed to discuss the “lifetime” of the data/model in a case by case manner. At least following cases are considered: - data/model captured in the TS - data/model uploaded as references for requirements definition - data/model uploaded for companies allignement |
| R4-2504006 | ZTE | Observation 1. We used a decoder from a company and train the encoder with our own dataset, the results without quantization and with quantization is about 0.72 and 0.66. Observation 2. From our simulation results, we use mixed dataset and five companies datasets to do test, the results of unquantization is about 0.71, and quantization is about 0.65 for mixed dataset. Observation 3. For track 2, we used a mixed dataset and our own dataset for training, and tested it with the mixed datset and our own dataset respectively. Firstly, we used the mixed dataset for training and the mixed dataset for testing. The results without quantization and with quantization are 0.61 and 0.59. We also used own dataset for training and tested it with the mixed dataset and our own dataset. The results without quantization and with quantization were 0.61, 0.63 and 0.58, 0.61 respectively. Observation 4. Option 4b is more likely option 3 if RAN4 reuse the mixed dataset as label data. Observation 5. Two different approaches are observed for option 4b, the firstly one is that DUT vendor directly uses reference model as implementation, and the second is that DUT vendor needs to develop new encoder based on the reference encoder. Observation 6. Since CSI compression is currently in the study item, we think it is better to capture these conclusion in the TR. Observation 7. The selection criteria needs to be based on practical perspective, such as complexity and performance. Proposal 1. Propose to consider a mixed dataset during training for option 4b as starting point. Proposal 2. Propose to capture the conclusion and reference model structure and parameters in TR. Proposal 3. Propose to follow RAN1’s conclusion on defining reference model structure and parameters. Proposal 4. Propose to specify reference model on RAN4 specification. |
| R4-2504026 | Vivo | Proposal 1: The feasibility study of Option 3, Option 4a-1 and Option 4b could work in parallel. Observation 1: For track 1, the SGCS results of Option 3 using models of Samsung, Huawei and CATT, using vivo’s dataset, are shown in Table 2.1-2 with quantization and Table 2.1-3 without quantization. The SGCS results of Option 4a/4b using Samsung model, using vivo’s dataset, are shown in Table 2.1-4 with quantization and Table 2.1-5 without quantization. Observation 2: For track 1 where the training dataset is different from the dataset for reference model, it seems that Option 4b has best validation performance, Option 4a has medium validation performance, while Option 3 has worst validation performance. Proposal 2: Suggested next steps for Option 4b: - Case 1 (Mandatory, target RAN4#114bis): For the case that UE directly uses reference encoder in field. - Take Samsung encoder and mixed dataset, train own decoder and then test own trained decoder with Samsung (reference) encoder. - Case 2 (Optional, maybe target RAN4#115): For the case that UE uses reference encoder to develop encoder in field. - Train a new decoder using the Samsung encoder and mixed dataset. Then train an "own" encoder with the newly trained decoder. - Then all companies share their "new" decoders that have been trained with the Samsung encoder and mixed dataset. - Test "own" encoder vs. All other companies’ decoders from step 2. (The other companies’ decoders here represent different TE vendors.) Observation 3: The SGCS results of Option 3/4a/4b using of Samsung models, are shown in Table 2.2-1 with quantization and Table 2.2-2 without quantization. The SGCS results of Option 3/4a/4b using of Nokia models, are shown in Table 2.2-3 with quantization. Observation 4: For track 2 where the training dataset is the same dataset for reference model, it seems that Option 3, 4a and 4b have similar validation performance. In field, the actual training dataset (field data) will be different from the synthetic dataset of reference model Proposal 3: Reference encoder needs to be specified in TS, due to the following aspects: - To be used as verification encoder in the TE test decoder verification. - No extra training procedure to train the verification encoder will be needed. - No performance degradation from imperfectly matched verification encoder will be caused. - To achieve better field performance. - To facilitate Rel-19 AI/ML study for reference model in RAN1 and RAN4. Proposal 4: Criterion for selection of reference encoder: reference encoder is selected from the encoder and decoder pair, from which the test decoder is selected. Proposal 5: It is more suitable for RAN4 to specify the reference encoder, since the encoder and decoder pairs are generated and selected in RAN4. Proposal 6: Reference dataset is beneficial to be specified, at least for TE test decoder verification. Proposal 7: In Option 4a, the model structure needs to be specified, to guarantee the performance of test decoder. Proposal 8: In Option 4a, dataset of raw channel or encoder input needs to be aligned, by aggregating the dataset of raw channel or encoder input from all companies, or choosing one dataset of raw channel or encoder input from all companies. Proposal 9: In Option 4b, channel generation method and reference decoder structure need to be in the spec. Proposal 10: There is no essential difference between reference model and reference dataset. Reference dataset may have some disadvantages compared to reference model, and the benefit of reference dataset over reference model is unclear. - Reference dataset would cause some performance loss compared to reference model. - The spec impact of writing model and dataset in the spec would be similar. - Reference dataset needs extra step compared to reference model, which is dataset generating from the aligned reference encoder and reference decoder. - The storage size of reference dataset is larger than reference model. - Reference model is more flexible than referend dataset for future release. Proposal 11: Similar feasibility conclusion through the simulation campaign of test options can be draw for both Option 3 and Option 4 (Option 4a and Option 4b). Observation 5: From initial results for field test, the generalization performance of AI/ML model trained by UMa simulation data on field data seems acceptable, which has similar performance as eType II codebook. The generalization performance of AI/ML model trained by CDL simulation data on field data is worse than AI/ML model trained by UMa simulation data. Observation 6: From initial results of field test, it is observed that - Directly use reference model in field (Case 2): similar performance on field data as eType II for one cell and performance loss compared to eType II is observed for another cell. - Finetuned encoder or decoder using field data against reference decoder/encoder (Case 2A-1, Case 2A-2): performance improved compared to directly use reference model (Case 2), but still has performance loss compared to fully trained by field data (Case1). Finetuned decoder (Case 2A-2) is better than finetuned encoder (Case 2A-1). - Pairing of finetuned encoder and finetuned decoder (Case 2A-3, Case 2A-4): performance loss in some cases considering mismatch on training data between UE and NW. Proposal 12: To achieve better field performance, the reference model should at least include the encoder part. Proposal 13: Using the mixed dataset for model training, including the mixing of TDL, CDL and UMa, while using the TDL dataset for RAN4 tests. Other mixing rules are not precluded. |
| R4-2504217 | Samsung | Observation 1: In Option 3 Track-1 (private encoder-only training), over different datasets, the SGCS performance degradation of training method-2 (encoder-only training) from training method-1 (Joint en/decoder trainings) is minor, i.e., 0.14%, 0.33%, 0.74%, 3.08% depending on different datasets. Proposal 1: Based on the experience of AI-CSI compression with CNN-based neural network, the criteria to determine “whether the selected reference decoder contains the same feature as the specific dataset” is: - The SGCS performance degradation of training method-2 (encoder-only training) from training method-1 (Joint en/decoder trainings) is within [1]% over a the specific dataset. Proposal 2: The above criteria in Proposal 1, RAN4 can identify the dataset may contain different feature from the one used for reference decoder training. Proposal 3: For Option 3 (specified reference decoder), the feasibility of Track-1 “private encoder-only training” can be confirmed under the following conditions: - The selected reference decoder shall be generalized by performing the training over the dataset with enough feature contained. - The dataset used for private training should have the feature distribution aligned with the one used for reference decoder, as much as possible. - Note: private encoder-only training is referred to: fixing reference decoder, and performing training for encoder in a private company-A dataset. Observation 2: In Option 3 Track-2 (encoder-only training over mixed dataset), companies provided decoders have quite similar SGCS performance, with standard deviation less than 0.01 for quantization case. Observation 3: In Option 3 Track-2 (encoder-only training over mixed dataset), a minor performance degradation can be expected for (1) encoder-only training by fixing the decoder, from (1) joint encoder/decoder training. Observation 4: In Option 3 Track-2 (encoder-only training over mixed dataset), by using appropriate dataset in addition to the mixed dataset, the SGCS can be further improved, at least for the following method: - Encoder-only retaining (The encoder is initialized with random weights). Discussion on Test decoder Option 4 Observation 5: Despite the similarity between Option 4 in RAN4 and Direction A in RAN1, the following difference should be noted: - In Option 4-1 (Direction A in RAN1) sharing dataset is used to derive practical encoder (to be used by UE), while in Option 4a (in RAN4) sharing dataset is used to derive test decoder (to be used by TE). - In Option 3a-1 (Direction A in RAN1) reference encoder is used to derive the practical encoder (to be used by UE), while in Option 4b (in RAN4) reference encoder is used to derive test decoder (to be used by TE). Proposal 4: Before Option 3 feasibility is confirmed or disconfirmed, the discussion on Option 4 can be deprioritized, by considering the factors including - to avoid parallel discussion with RAN1 Direction A down-selection. - the study on the feasibility of Option 4b can be speed up if Option 3 is confirmed. |
| R4-3504231 | Nokia | Observation 1: Even with further alignment on simulations, the performance of the testing phase in Option 3 Track 1 is not consistent while testing is done using different companies’ datasets. Observation 2: The SGCS values obtained in Option 3 Track 1 are not acceptable for several tests with different companies’ datasets, particularly with testing datasets from CATT and Qualcomm. Proposal 1: Based on our results, Option 3 Track 1 does not look like a feasible option. Therefore, RAN4 should down prioritize Option 3 Track 1. Observation 3: Compared to the SCGS performance from Option 3 Track 1, it is evident that Option 3 Track 2 results are more consistent among different companies’ models. Proposal 2: Since the performance when using a mixed dataset for the training of both encoder and decoder is better than when using different datasets, RAN4 should pursue Track 2 over Track 1 for the feasibility study of test decoder Option 3. Proposal 3: In the conclusion of the feasibility study of Option 3, and based on our results, we suggest selecting Nokia’s decoder from Track 2 as the test decoder together with the agreed mixed dataset Observation 4: Using a decoder trained for the purpose of training the UE vendor’s encoder as the test decoder violates the separation of knowledge between the UE and TE vendors. This creates potential problems in the determination of feasibility of Option 4a. Proposal 4: Clarify in Step 6 of the Option 4a feasibility procedure that the training of the “own decoder(s)” for the purpose of using it as a test decoder must be separated from the training of the “own encoder(s)” since the actions of different vendors (TE vendors and UE vendors, respectively) are being mimicked in this step. - Step 6: Companies bring results for independent training of “own encoder(s) and decoder(s)” with the selected dataset(s) - A decoder may be trained to support the training of the own encoder, but this decoder is distinct from the TE decoder used for performance assessment of Option 4a. - Performance alignment to be checked/discussed Observation 5: Use of the decoder in Step 5 of the Option 4b feasibility study procedure as both the TE test decoder and for training the UE encoder violates the assumption of independent training of the test decoder and UE encoder. Proposal 5: The dataset used in Step 5 of the feasibility study procedure for Option 4b is the company’s own dataset. Proposal 6: Adjust Step 6 in the feasibility study procedure for Option 4b to: - Step 6: Company trains “own encoder(s)” - A decoder may be trained to support the training of the own encoder, but this decoder is distinct from the decoder in Step 5. - The dataset used for the training is the company’s own dataset, but a dataset different from the one used in Step 5. Proposal 7: For Option 3, the corresponding encoder to the selected test decoder should be the candidate for reference encoder. Proposal 8: For Option 4b, encoder based approach, the selected encoder (from Option 3) should be the candidate for reference encoder. Proposal 9: For Option 4a, RAN4 needs to further discuss the selection criterion for reference encoder. Proposal 10: To verify test decoder and to define performance requirements, RAN4 needs to specify reference encoder. It should be captured in the TR conclusion as well as in the TS. Proposal 11: If RAN1 also specifies a reference encoder, there should be alignment between RAN1 and RAN4 reference encoder. |
| R4-2504236 | Oppo | Proposal 1: Principles to define test decoder(s) - to meet the minimum performance requirement in RAN4 tests - to be a simple design Proposal 2: For RAN4 test model alignment, the motivation should be clarified - Option1: It is expected that the alignment of minimum performance understanding can be achieved through results from different companies - Option2: It is expected to strictly align the output results of each company's AI/ML models through simulation Proposal 3: Prioritize discussing Option 3 first and then evaluate Option 4 if needed, rather than initiating parallel discussions on both Option 3 and Option 4. Observation 6: Several challenges that can prevent a seamless conversion of ONNX models to other formats, e.g., TensorFlow or PyTorch. Some main issues are listed below: - Lack of official conversation tools - Version Compatibility Issues - Conversion for custom or unsupported layers Proposal 4: To reconsider the format in which the test model is delivered, such as providing it in PyTorch format, thereby relax the challenges associated with version and format conversion. |
| R4-2504321 | Huawei, HiSilicon | Proposal 1: Use SGCS as a key metric for verifying the feasibility of procedures related to the standardization of full models. Proposal 2: Define SGCS margin by considering the following options. - Option 1: Average of SGCS - Option 2: SGCS @90% of CDF Proposal 3: Define multiple degrees of performance convergence based on SGCS margin: - SGCS margin > 0.1 indicates low - degree convergence. - SGCS margin within [0.05, 0.1] indicates medium - degree convergence. - SGCS margin < 0.05 indicates high - degree of convergence. Proposal 4: SGCS results for Track 1 with encoder model structure under current assumption are provided in Table 1. Proposal 5: SGCS results for Track 2 with encoder model structure under current assumption are provided in Table 1. Observation 3: In track 2, nearly no SGCS performance loss with fixed selected decoder using mixed dataset to retrain the encoder, assuming the encoder model structure is aligned between training and retraining. Observation 4: In track 2, around 0.05 SGCS performance loss is observed with fixed selected decoder using our own dataset to retrain the encoder. Proposal 6: Use track 2 to move forward by both considering encoder retraining under mixed dataset and companies own dataset. Proposal 7: SGCS results for lower-complexity CNN and transformer encoder are provided in Table 3. Observation 5: Under the same backbone, 0.03 SGCS performance loss is observed for encoder retraining with a lower-complexity CNN-based model structure. Observation 6: Under a different backbone from current CNN-based assumption, around 0.1 SGCS is observed for encoder retraining with a lower-complexity Transformer-based model structure. Proposal 8: Use lower complexity encoder model structure for testing/reference model structure alignment if AI-CSI turns to WI. Observation 7: Around 0.05 SGCS performance loss can be observed in Track 1 under mixed testing dataset. Observation 8: Around 0.1 SGCS performance loss can be observed in Track 1 under companies’ individual dataset. |
| R4-2504439 | Ericsson | Observation 1 For track 1, there is variation in performance between different companies that depends on the training set and the test set. Observation 2 Based on the Samsung decoder, option 3 track 1 seems basically feasible, at least where companies have similar datasets. Observation 3 The variation between the mixed testset performance and individual companies testsets for track 2 has decreased but is still significant. Observation 4 Variations between different companies datasets (and the mixed dataset) appears to be related to differing levels of correlation between the SGCS observed for different sub-bands. Observation 5 Based on working with the Samsung decoder as a frozen decoder, option 3 track 2 works very well if the UE vendor encoder is trained with the same mixed dataset as the test decoder. If trained with a different dataset, there is a small difference in performance. Observation 6 For option 4, if a frozen encoder is taken from track 1, a new decoder (trained on the frozen encoder) and encoder (trained on the new decoder) can be trained that show a small performance drop compared to the encoder/decoder trained directly. Observation 7 For option 4, if a frozen encoder is taken from track 2, a new decoder (trained on the frozen encoder) and encoder (trained on the new decoder) can be trained that show a small performance drop compared to the encoder/decoder trained directly. Observation 8 For option 4b, most likely a dataset needs to be standardized to enable reproducibility of the test decoder between different TE vendors. Proposal 1 If there is a need for it to be part of a TS or TR, then preferably the reference encoder should be part of a TS. However, it may be suitable that it is stored in the same managed location as the test decoder. Proposal 2 The reference encoder should be the encoder that is paired with, i.e. trained together with, the test decoder using the same training data. Proposal 3 Keep the test decoder and reference encoder together, in the RAN4 specifications. Or alternatively store and maintain the models together in the same place and refer from the RAN4/5 specifications. Proposal 4 If available, store the dataset used to create the test decoder such that it is linked to the test decoder and reference encoder and maintained across releases. Proposal 5 Discuss during development of the requirement whether to describe test and verification data by means of a channel model or an explicit dataset. Proposal 6 The study on option 3 should consider that a 3GPP internal method for capturing test decoder AI models is needed (for CSI compression and future models). Possible means to achieve this may be discussed if time allows. Proposal 7 The study should consider how a procedure for updating the test decoder would work, considering in particular whether updates should always be backward compatible or whether signalling of versions could be needed. Proposal 8 The study should consider what key characteristics should drive the discussion on structure for a real standardization exercise. Some characteristics include:  Backbone (CNN, transformer etc.)  Complexity target  Number of models (depending on scenarios etc.)  A metric and acceptable performance target  Ability of the agreed structure for the test decoder to allow for variation of real encoder/decoder implementation Proposal 9 Conclude that, if Option 3 is supported for performance testing, then the fully specified test decoder is the same as the decoder in the reference model for Direction C in RAN1 inter-vendor training collaboration discussion. Proposal 10 Conclude that, for a real WI, a sufficient diversity of simulation assumption sets would be needed to make the training dataset for the test decoder. Also, some steps to ensure alignment of different companies’ datasets are needed. Proposal 11 To test the feasibility of option 4, all companies should upload at least a “decoder 2” (decoder trained using frozen encoder and mixed dataset) and potentially also a “decoder 3” (decoder trained using frozen encoder and company’s own dataset). This will allow interoperability between UE vendor encoders and different TE vendor decoders to be emulated. Proposal 12 Merge options 4a and 4b to standardization of a dataset and standardization of a reference encoder. |

## 2.2	Open issues summary

### 2.2.1	Sub-topic 2-1 Simulation analysis for options 3 and 4

This sub-topic addresses the simulation analysis and the results provided for options 3 and 4. A summary of the simulation results is provided by Huawei and available in the inbox.

In this sub-topic, some potential preliminary observations based on the results that have been presented and observations in the submitted contributions are suggested. The intention is that, during the meeting, an attempt can be made to draw up preliminary observations that are mutually agreeable and also to highlight what further work is needed to progress the conclusions.

Green marking indicates that the observations were endorsed in the ad-hoc. Non-highlighted text will be reflected in the WF for focusing discussion in the next meeting, but is not endorsed.

Issue 2-1-1: Preliminary observations on datasets

The following are some moderator suggestions for observations about the dataset, to be discussed/refined during the meeting:

- Datasets consist of separate training data and separate test data.

- All companies have produced datasets based on the same simulation assumptions, however differences can be observed between performance with different companies datasets.

- The “mixed dataset” provides much more consistent results than training or testing using different individual company datasets

- The exercise demonstrates that in order to agree on test decoder(s) and or reference encoder(s), some effort is needed to align dataset(s).

Issue 2-1-2: Preliminary observations for option 3 track 1

The following are some moderator suggestions for discussion about option 3 track 1, to be discussed/refined during the meeting. The intention is not to draw final conclusions, but to endorse what observations can be made thus far on the results. The results numbers and conclusions will be updated when further results and analysis is available.

- The investigation on option 3 is based on using the agreed structure for the decoder and encoder

- It is possible for UE vendors to train an encoder based on another company’s frozen test decoder that is functional

- The performance of the encoder is very dependent on the alignment of the dataset used for training the encoder and the dataset used for training the frozen decoder, and the testing dataset

- The variation in the performance depending on the training datasets and test datasets can be as much as [20%] in SGCS. Further details are available in the results spreadsheet.

  - Preliminary observation; further results may be added/refined and the observation updated (for example, using the mixed dataset as in track 2)

- For option 3, track 1 to be useful, careful attention would be needed to the alignment of datasets

- Up to now, the same encoder structure is considered as used when creating the reference decoder; the impact of different encoder structure is not yet considered

Issue 2-1-3: Preliminary observations for option 3 track 2

The following are some moderator suggestions for discussion about option 3 track 2, to be discussed/refined during the meeting. The intention is not to draw final conclusions, but to endorse what observations can be made thus far on the results. The results numbers and conclusions will be updated when further results and analysis is available.

- There is reasonable SGCS convergence when companies train an encoder-decoder pair using the mixed dataset and test using the mixed testset

  - Around 1% variation in SGCS; see results spreadsheet for more details

  - Preliminary observation; further results may be added

- It is possible for UE vendors to train an encoder based on a frozen test decoder that is functional

- When training “own” encoder with the frozen decoder and testing, with all training (frozen decoder and encoder) and testing using the mixed dataset, there is good convergence in average SGCS results from results presented so far

  - Around [+- 2%] variation in SGCS from the average; see results spreadsheet for more details

  - Preliminary observation; further results may be added

- When training of frozen decoder is on the mixed dataset, “own” encoder is on a the mixed dataset and testing is on own testset, results have somewhat more divergence

  - A couple of initial results show 5%?? SGCS from average, but very limited data available.

- When training of frozen decoder is on the mixed dataset, “own” encoder is on a different dataset and testing is on a different dataset, results have somewhat more divergence

  - A couple of initial results show 12%?? SGCS from average, but very limited data available.

Companies are encouraged to provide more results on using mixed dataset frozen decoder and own dataset encoder training/testing.

- There may be some risk of overtraining the encoder

  - Clarify further why overtraining is a particular issue here

  - Not to be captured as observation for now

- Up to now, the same encoder structure is considered as used when creating the reference decoder by most companies; the impact of different encoder structure is not yet fully considered

  - A couple of initial results suggest similar SGCS can be obtained with lower complexity CNN, but SGCS would drop with a lower complexity transformer encoder

    - Further results are encouraged for the next meeting bring further simulation results with different encoder structures

Issue 2-1-4: Preliminary conclusions for option 4

The following are some moderator suggestions for discussion about option 4, to be discussed/refined during the meeting. The intention is not to draw final conclusions, but to endorse what observations can be made thus far on the results. The results numbers and conclusions will be updated when further results and analysis is available.

- For option 4b: It is feasible to train a functional “own” decoder based on the frozen encoder, and then a functional “own” encoder based on the “own” decoder when the mixed dataset is used

- For option 4a: For the mixed dataset, a similar observation to option 4b can be expected (in this case, the labelled dataset would be the same latent space as the selected encoder (created using joint training) acting on the mixed dataset, and so option 4a converges to option 4b)

- Results are convergent if the mixed training dataset and mixed test dataset is used, but more divergent if other test-sets are used

  - 2% divergence in SGCS observed from preliminary results for mixed set

  - Companies encouraged to contribute more results with mixed dataset for option 4b next meeting to confirm this observation

    - Bring results for SGCS when operating “own” encoder with “own” decoder from same company

  - Companies can also consider using own dataset when training “own” encoder (but always use mixed dataset for “own” decoder in this stage)

- Interoperability of “own” encoder with other company “own” decoders has not yet been examined.

  - Companies should upload their “own” decoder, trained using the mixed dataset, asap

  - Then check that “own” encoder works with other companies “own” (i.e., test) decoders

- Dataset 1: Dataset used for joint training of frozen encoder

- Dataset 2: Dataset used to train “own” decoder using frozen encoder

- Dataset 3: Dataset used to train “own” encoder using “own” decoder

- For next meeting:

  - Priority 1: Dataset 1 = dataset 2 = dataset 3 = mixed dataset

  - Priority 2: Dataset 1 = dataset 2 = mixed dataset, dataset 3 own dataset

- For future meeting:

  - For option 4b: Dataset 1 = mixed, dataset 2 = own dataset, dataset 3 = ??

- This is a first step where it is assumed that the mixed dataset is used for option 4b. There is an alternative where “own” dataset is used for option 4b; this implies training ”own” decoder based on own dataset. Take as a second step after mixed dataset 4b is assessed

- For option 4, consideration should also be given to the compatibility of “own” encoder that has a different structure to the agreed structure (lower complexity).

Issue 2-1-4a: Removing some simulations in order to increase focus:

Discuss whether the following can be downprioritized from simulations in order to increase the amount of results and ability to make observations for RAN4#115 and RAN4#116:

- No more results are expected Option 3 track 1

- No more results are expected Unquantized model results

- For option 3 track 2, use the Samsung decoder as the frozen decoder. Lower priority for Nokia decoder.

- For option 4, use the Samsung encoder as the frozen encoder. Lower priority for Nokia encoder.

Issue 2-1-8: New proposal – Option 4c

- Proposals

  - Add an option 4c, in which an encoder-decoder pair is evaluated based on latent message re-ordering considering the quantized dominant Eigenvectors (NTU proposals 5-7)

- Recommended WF

  - TBA

NTU: Option 4a, 4b we all need to agree a decoder or encoder. This makes option 4 almost the same as option 3. If we can already agree an encoder, why not just go back to option 3. The value of option 4 is no need to agree a structure, just specify some rules and make a matching encoder / decoder. We propose a latent message ordering to make sure that the latent message is ordered in a way that enables interoperability.

### 2.2.2 Sub-topic 2-2 Other considerations

In this section, considerations that do not directly relate to the simulation campaign are discussed:

Issue 2-2-1: What needs to be specified for option 3

During RAN4#114, the following agreement on reference model was made:

By definition, a test decoder is standardized for option 3. The need to define a reference encoder was discussed and the following agreement reached

Agreement:

- To verify the performance of CSI compression test decoder for calibration of test equipment, agree to define the reference encoder including encoder structure and parameters in RAN4

  - FFS on whether to specify it in TS or TR, and capture the conclusion in the TR conclusion part

  - FFS on criterion for selection of reference encoder

  - FFS on decision whether RAN4 or RAN5 will specify it

- Proposals

  - For the reference encoder (already agreed) take the encoder corresponding to the test decoder (CATT proposal 2, Vivo proposal 4, Nokia proposal 7, Ericsson proposal 2)

  - FFS where to capture reference encoder (CATT proposal 3)

  - Specify the reference encoder in the TS (Vivo proposal 3, Ericsson proposal 1)

  - Specify the reference encoder in TR and TS (Nokia proposal 10)

  - Specify the reference encoder in RAN4 (Vivo proposal 5)

  - Store the reference encoder together with the decoder and refer to it from TS/TR as appropriate (Ericsson proposal 1, 3)

  - Training dataset (CATT proposal 4, Vivo proposal 6, Ericsson proposal 4 if available)

  - Either test and verification datasets or test and verification channel models (Ericsson proposal 5)

  - Do not specify option 4 if option 3 is feasible (CMCC proposal 1)

  - Test decoder should meet the minimum performance requirement in RAN4 and be simple (Oppo proposal 1)

- Recommended WF

  - The encoder that corresponds to the standardized test decoder is stored within 3GPP

    - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS)

    - FFS whether several encoders would be captured (e.g. due to different complexity)

  - Check whether it is agreeable to assume that the training dataset for the standardized test decoder is stored within 3GPP, if available

    - Capture for example in case UE or network vendors could find access to the dataset used for the test encoder useful for their developments

    - Capture in wF for next time

  - Discuss further also test dataset