3GPP TSG-RAN4 Meeting #114bis	                                                                 R4-2505112

Wuhan, China, 7th – 11th April 2025

Agenda item:			7.12.7

Source:	CMCC

Title:	WF on UE RF requirements for NR ATG

Document for:	Approval

Introduction

This document captures the agreements on Rel-19 ATG UE RF requirements in RAN4#114bis.

Topic #1: R19 UE RF for inter-band CA

Issue 1-1: MSD for CA_n3A-n39A with n39 UL

Agreement:

- MSD requirements from linear average values based on companies’ simulation results are given in the tables below.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | MSD Table X | >ACLR2 |

Table X CA_n3-n39 with band n39 UL MSD requirements for different output powers

| Output power | CMCC | Huawei | LGE | Qualcomm | ZTE | Linear Average |
| --- | --- | --- | --- | --- | --- | --- |
| 23dBm | 3.6 | 3.4 | 5.5 | 5.8 | 4.71 | 4.7 dB |
| 24dBm | 4.2 | 3.57 | 6.3 | 6.3 | 5.1 | 5.2 dB |
| 25dBm | 4.8 | 3.74 | 7.1 | 6.8 | 5.54 | 5.8 dB |
| 26dBm | 5.5 | 3.94 | 7.9 | 7.3 | 6.03 | 6.3 dB |
| 27dBm | 6.3 | 4.16 | 8.7 | 8.0 | 6.58 | 7.0 dB |
| 28dBm | 7.1 | 4.42 | 9.5 | 8.6 | 7.19 | 7.7 dB |
| 29dBm | 7.9 | 4.72 | 10.3 | 9.4 | 7.85 | 8.4 dB |
| 30dBm | 8.8 | 5.07 | 11.1 | 10.1 | 8.55 | 9.1 dB |
| 31dBm | 9.7 | 5.49 | 11.9 | 11.0 | 9.3 | 10.0 dB |
| 32dBm | 10.5 | 5.96 | 12.7 | 11.8 | 10.1 | 10.7 dB |
| 33dBm | 11.5 | 6.51 | 13.5 | 12.8 | 10.92 | 11.6 dB |
| 34dBm | 12.4 | 7.13 | 14.3 | 13.7 | 11.78 | 12.5 dB |
| 35dBm | 13.3 | 7.84 | 15.1 | 14.7 | 12.66 | 13.3 dB |
| 36dBm | 14.3 | 8.63 | 15.9 | 15.7 | 13.57 | 14.2 dB |
| 37dBm | 15.2 | 9.50 | 16.7 | 16.8 | 14.49 | 15.2 dB |
| 38dBm | 16.2 | 10.46 | 17.5 | 17.9 | 15.43 | 16.1 dB |
| 39dBm | 17.2 | 11.50 | 18.3 | 19.0 | 16.38 | 17.1 dB |
| 40 dBm | 18.2 | 12.63 | 19.1 | 20.2 | 17.34 | 18.1 dB |

Issue 1-2: How to incorporate MSD requirements for different power levels into the spec.

Agreement:

- Use the following tables to incorporate MSD requirements for different power levels into the spec

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | MSD Table X | >ACLR2 |

Table X MSD requirements for different output powers

| Output power | MSD (dB) |
| --- | --- |
| 23dBm | [TBD] |
| 24dBm | [TBD] |
| … | … |
| 39dBm | [TBD] |
| 40 dBm | [TBD] |

Issue 1-3: MSD for CA_n3A-n39A with n3 UL

Agreement:

- MSD requirements will be decided based on the average values among companies’ simulation results.

  - Encourage companies to provide results next meeting.

Issue 1-4: Case 4 related issues..

Agreement at RAN4#114: - ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam is possible for specific band combination. - Antenna array type for inter-band CA is highly depending on frequency range of operating bands and UE implementation. - Option 1: Case 4 UE implementation is possible for CA_n3-n39 that frequency ranges between two bands are close to each other. - Option 2: Case 4 UE implementation is possible for CA_n3-n39 that DL frequency separation between two bands are less than [X=FFS, e.g, 500 MHz].

Agreement:

- Case 4 UE implementation is possible for CA_n3-n39

Topic #2: R19 UE RF for UL-MIMO

Agreement:

- Preclude the single antenna port related requirement for TxD for ATG UE

- Keep the single antenna port related requirements with UL-MIMO for ATG UE

- ULFPTx mode still optionally applies for ATG UE to support different implementation

  - Support ULFPTx mode without specification impact except to clarify ATG UE maximum output power in the section “UE maximum output power for UL MIMO”.