3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2505113

Wuhan, China, 7th - 11th April, 2025

Title: 	WF on HPUE RF requirements for NR-NTN

Agenda Item:	7.10.5

Source: 	Samsung

Document for:	Approval

Topic #1: RF requirements

Sub-topic 1-1: Power classes and UE types

Issue 1-1-2: Power boosting option for PC2

Agreement:

- FFS on the power boosting option for PC2, including

  - Whether or not and how to define power boosting for PC2 as proposed above.

- The power boosting discussion not to impact the requirements discussion for PC2 without power boosting, e.g. MPR/AMPR, for PC2.

Sub-topic 1-2 Tx requirements

Issue 1-4: General

Agreement:

- As a baseline approach, existing Tx requirements from the TN specifications are re-used for the high-power NTN UE.

Issue 1-5: MOP and tolerance

Agreement:

- Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance for PC1 and PC1.5 for NR-NTN Non-Redcap HPUE.

Issue 1-6: SEM

Agreement:

- Use agreed baseline approach for SEM.

  - Apply SEM from NR-NTN PC3 to other PCs for NR-NTN as baseline.

- No need to further discuss the FCC 47 CFR part 25 specification in clause 202 in this work item in this release.

Issue 1-7-1: PC2 and PC1.5 ACLR

Agreement:

- Re-use existing TN ACLR:

  - For 2Tx PC1.5: Reuse TN PC1.5 ACLR

  - For 1Tx and 2Tx PC2: Reuse TN PC2 ACLR

Issue 1-7-2: PC1 ACLR

Tentative agreement:

- PC1 ACLR is [34] dBc

  - Companies to further evaluate this, and to be confirmed in next meeting.

Issue 1-8-1: PC2/1.5 MPR

Ad-hoc agreement:

- Agree to re-use TS 38.101-1 MPR for PC2 and PC1.5

  - TS 38.101-1 Table 6.2.2-2 applies to NR-NTN PC2 HPUE MPR with 1Tx

  - TS 38.101-1 Table 6.2D.2-1 applies to NR-NTN PC2 HPUE MPR with 2Tx

  - TS 38.101-1 Table 6.2D.2-3 applies to NR-NTN PC1.5 HPUE MPR with 2Tx

Issue 1-8-2: PC1 MPR

Way forward:

- To collect MPR analysis and determine MPR for PC1 in next meeting.

Issue 1-8-3: MPR/A-MPR simulation assumptions:

Ad-hoc agreement:

- For PC1, ACLR used in PA calibration should be aligned with ACLR requirements.

Issue 1-9: A-MPR

Ad-hoc agreements:

- Discuss the A-MPR in the draft CRs in next meeting.

Sub-topic 1-3 Rx requirements

Issue 1-10: General discussion of RSD analysis

Ad-hoc agreements:

- In addition to Tx leakage to Rx, Rx non-linearity needs to be considered in RSD evaluation.

Issue 1-11: General discussion of RSD of PC1.5 and PC1

Ad-hoc agreements:

- Companies agree to bring more analysis for the RSD values for PC1.5/PC1 non-handheld UE.

  - Further check these in next meeting.

Issue 1-13: RSD requirements for PC2 on n255

Ad-hoc agreements:

- For n255 PC2:

|  | 5MHz | 10MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| RSD@1Tx | [0.8] | [0.8] | [0.9] | [0.9] |
| RSD@2Tx | [1.6] | [1.6] | [1.7] | [1.7] |

Issue 1-14: RSD requirements for PC1.5 and PC1.

Way forward:

- Encourage companies to further discuss the following proposals:

  - Option 1: For PC1.5 2Tx (MTK)

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 2.2 | 2.2 | 2.2 | 2.2 |
| n255 | 3.2 | 3.2 | 3.3 | 3.3 |
| n254 | 2.2 | 2.2 | 2.2 |  |

  - Option 2: For PC1 1Tx (MTK)

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | [3] | [3] | [3] | [3] |
| n254 | [3] | [3] | [3] |  |

Issue X: Proposal to allow power reduction beyond certain duty cycle

Way forward:

- Encourage companies to further discuss the following proposal:

  - Option 1: For NR NTN PC2 and PC1.5, and IoT NTN PC2, allow but not mandate UE to reduce it’s transmit power when 50% Tx duty cycle in a radio frame is exceeded. (Qualcomm)

Topic #2: Co-existence study

Issue 2-1: Simulation assumptions:

Ad-hoc agreements:

- To use the R4-2504331 as baseline text for the TR.

Topic #3: Impacts on TR and TS from HPUE

For TS/TR impacts:

Way forward:

- To create a new sub section [6b] in TR 38.863 to accommodate the NTN HPUE co-ex studies.

- To capture NR-NTN HPUE RF requirements analysis in TR 38.863.

The agreed work split provided below for information:

| Impacted sections on TS 38.101-5 | Work split |
| --- | --- |
| Big CR for TS 38.101-5 | Samsung |
| 6.2 Transmitter power |  |
| 6.2.1 UE maximum output power | Samsung |
| 6.2.2 MPR |  |
| 6.2.3 A-MPR |  |
| 6.2.4 Configured transmitted power |  |
| 6.5 Output RF spectrum emissions |  |
| 6.5.2.2 Spectrum emission mask | Xiaomi |
| 6.5.2.3 Additional spectrum emission mask |  |
| 6.5.2.4.1 NR ACLR | Huawei/Hisilicon |
| 6.5.3.3 Additional spurious emissions |  |
| TBA |  |
| 7.3 Reference sensitivity |  |
| 7.3.2 Reference sensitivity | Qualcomm |
| TBA Please suggest if anything missing. |  |
| Impacted sections on TS 36.102 | Work split |
| Big CR TS 36.102 | vivo |
| 6.2A Transmitter power for category M1 | vivo |
| 6.2A.1 UE maximum output power |  |
| 6.2A.2 MPR |  |
| 6.2A.3 A-MPR |  |
| 6.2A.4 Configured transmitted power |  |
| 6.2B Transmitter power for category NB1 and NB2 |  |
| 6.2B.1 UE maximum output power |  |
| 6.2B.2 MPR |  |
| 6.2B.3 A-MPR |  |
| 6.2B.4 Configured transmitted power |  |
| 6.5A Output RF spectrum emissions for category M1 | Mediatek |
| 6.5A.2.2 Spectrum emission mask |  |
| 6.5A.2.4.1 NR ACLR |  |
| 6.5A.3.3 Additional spurious emissions |  |
| 6.5B Output RF spectrum emissions for category NB1 and NB2 |  |
| 6.5B.2.2 Spectrum emission mask |  |
| 6.5B.2.4.1 NR ACLR |  |
| 6.5B.3.3 Additional spurious emissions |  |
| 7.3A Reference sensitivity for category M1 | OPPO |
| 7.3A.2 Reference sensitivity |  |
| 7.3B Reference sensitivity for category NB1 and NB2 |  |
| 7.3B.2 Reference sensitivity |  |
| Impacted sections on TR 38.863 | Work split |
| Big CR for TR 38.863 | Samsung |
| New 6b Co-existence study for NTN HPUE |  |
| 6b.1 Co-existence simulation scenarios | Samsung We will prepare and invite all companies to co-sign |
| 6b.2 Co-existence simulation assumption |  |
| 6b.3 Co-existence simulation methodologies |  |
| 6b.4 Co-existence simulation results |  |
| 6b.5 Summary of co-existence study |  |
| 7.4.2 NTN UE Tx requirements (NR-only) |  |
| 7.4.2.1 General | CATT |
| 7.4.2.2.1 MOP |  |
| 7.4.2.2.2 MPR/AMPR | Apple We expect this session (esp. AMPR analysis) requires some joint-effort. Let's submit CRs and then to merge into Apple's draft CR, and co-sign. |
| 7.4.2.2.6 SEM |  |
| 7.4.2.2.7 ACLR | Ericsson |
| 7.4.2.2.8 Spurious emissions |  |
| TBA |  |
| 7.4.3 NTN UE Rx requirements (NR-only) |  |
| 7.4.3.2.2 Refsense | ZTE We expect this session (esp. RSD analysis) requires some joint-effort. Let's submit CRs and then to merge into ZTE's draft CR, and co-sign. |