3GPP TSG-RAN WG4 Meeting #114bis	 R4-2505127

# Wuhan, China, 7-11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.764 | CR | <CR#> | rev | <Rev#> | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TR 36.764 to capture agreement for B252 introduction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | EchoStar, Boost Mobile Network |  |  |  |  |  |  |  |  |  |
| Source to TSG: | - |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_FDD_S_band-Core |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | draftCR to capture agreements for Introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) WI |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capturing agreements for B252 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreements on definition of NTN band B252 are not captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- Start of changes ---

Unneffected parts ommited

- Change 1 -

# 5	Common band agnostic aspects

## 5.1	System parameters

### 5.1.1	Operating bands

IoT NTN operation is designed in the operating bands defined in Table 5.1.1-1.

Table 5.1.1-1 E-UTRA operating bands for IoT NTN operation

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 254 | 1610 MHz | - | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| 253 | 1668 MHz | - | 1675 MHz | 1518 MHz | - | 1525MHz | FDD |
| 252 | 2000 MHz | - | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256 |  |  |  |  |  |  |  |

### 5.1.2		Channel arrangements

#### 5.1.2.1	Channel arrangement for category M1

Table 5.1.2.1-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 254 | 100 | 2483.5 | 228571 | 228571 –<1>- 228735 | 1610 | 261339 | 261339 –<1>- 261503 |
| 253 | 100 | 1668 | 228501 | 228501-<1>-228570 | 1518 | 261269 | 261269 -<1>-261338 |

NOTE:	In addition to the above, the following was agreed during RAN4#108-bis:

-	The need for additional flexibility in channel raster for IoT NTN LTE bands has been identified, especially to accommodate future deployments of NB-IoT NTN and eMTC NTN together with NR NTN around existing services, taking into account possible guard bands for protection of other services, Doppler, and for deployment within the same SAN (i.e. same FFT).  This issue could be further discussed independently of band 253, with more general applicability to all IoT NTN LTE bands.

-	Capture this aspect in the TR.

-	FFS whether to capture a note in the specification

Table 5.1.2.1-2: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 254 | 873.5 MHz |
| 253 | -150 MHz |

NOTE:	As highlighted in RAN4#108-bis, based on operator input, there is a strong desire to consider flexible TX-RX separation for the band itself, and moreover for NTN FR1 bands, provided the frequency separation is sufficiently large.

The following conclusion was reached during RAN4#109:

Conclusion: Stop discussion on this issue in this WI.

NOTE:	The channel arrangements for B252 is defined in clause 8.2.1.

Unneffected parts ommited

- Change 2-

# 8	FDD band B252 (IoT-NTN FDD S_band)

## 8.1	Regulation

Table 8.1-1 shows the service allocation for the frequency range in the ITU-R Radio Regulations [xx] and Table 8.1-2 shows the the FCC frequency allocation table for the frequency range [xx].  From Table 8.1-1 and Table 8.1-2, there is some deviation in how MSS S-band is allocated in region 2 versus in all other regions. Per ITU, the Region 2 MSS S-Band had been allocated to UL 1980-2025 MHZ and DL 2160-2200 MHz. In the case of North America region (US, Canada, Mexico) the MSS band is allocated to UL 2000 MHz -2020 MHz and DL 2180 MHz -2200 MHz which is not exactly aligned to n256 definition, i.e., UL 1980 MHz – 2010 MHz, DL 2170 MHz – 2200 MHz. The 3GPP NTN  S-band B252 (UL 2000 MHz -2020 MHz and DL 2180 MHz -2200) is defined for consistency across North America and Region 2 as a whole.

Table 8.1-1: Allocation of 2000-2020 MHz and 2180–2200 MHz in the ITU-R Radio Regulations

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 980-2 010 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.351A 5.388 5.389A 5.389B 5.389F |  |  |
| 2 010-2 025 FIXED MOBILE 5.388A 5.388B 5.388 | 2 010-2 025 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.388 5.389C 5.389E | 2 010-2 025 FIXED MOBILE 5.388A 5.388B 5.388 |
| 2 170-2 200 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.351A 5.388 5.389A 5.389F |  |  |
| 5.388: The frequency bands 1 885-2 025 MHz and 2 110-2 200 MHz are intended for use, on a worldwide basis, by administrations wishing to implement International Mobile Telecommunications (IMT). Such use does not preclude the use of these frequency bands by other services to which they are allocated. The frequency bands should be made available for IMT in accordance with Resolution 212 (Rev.WRC-15)* (see also Resolution 223 (Rev.WRC-15)*). 5.389A: The use of the bands 1 980-2 010 MHz and 2 170-2 200 MHz by the mobile-satellite service is subject to coordination under No. 9.11A and to the provisions of Resolution 716 (Rev.WRC-2000)** 5.389B: The use of the frequency band 1 980-1 990 MHz by the mobile-satellite service shall not cause harmful interference to or constrain the development of the fixed and mobile services in Argentina, Brazil, Canada, Chile, Ecuador, the United States, Honduras, Jamaica, Mexico, Paraguay, Peru, Suriname, Trinidad and Tobago, Uruguay and Venezuela. 5.389F: In Algeria, Cape Verde, Egypt, Iran (Islamic Republic of), Mali, Syrian Arab Republic and Tunisia, the use of the frequency bands 1 980-2 010 MHz and 2 170-2 200 MHz by the mobile-satellite service shall neither cause harmful interference to the fixed and mobile services, nor hamper the development of those services prior to 1 January 2005, nor shall the former service request protection from the latter services. |  |  |

Table 8.1-2: FCC frequency allocation for 2000-2020 MHz and 2180–2200 MHz

The following are the applicable FCC regulatory requirements:

#### 47 CFR § 25.202

According to paragraph (a)(4)(ii), the following frequencies are available for use by the 2 GHz Mobile-Satellite Service:

- 2000-2020 MHz: User-to-Satellite Link;

- 2180-2200 MHz: Satellite-to-User Link.

According to paragraph (f) on emission limitations, except for SDARS terrestrial repeaters and as provided for in paragraph (i), the mean power of emissions shall be attenuated below the mean output power of the transmitter in accordance with the schedule set forth in paragraphs (f)(1) through (f)(4) of this section:

- In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 50 percent up to and including 100 percent of the authorized bandwidth: 25 dB;

(2)  In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 100 percent up to and including 250 percent of the authorized bandwidth: 35 dB;

(3)  In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 250 percent of the authorized bandwidth: An amount equal to 43 dB plus 10 times the logarithm (to the base 10) of the transmitter power in watts;

(4)  In any event, when an emission outside of the authorized bandwidth causes harmful interference, the Commission may, at its discretion, require greater attenuation than specified in paragraphs (f) (1), (2) and (3) of this section.

#### 47 CFR § 25.216

According to paragraph (e) on limits on emissions from mobile earth stations for protection of aeronautical radionavigation-satellite service, the e.i.r.p density of emissions from mobile earth stations with assigned uplink frequencies between 1990 MHz and 2025 MHz shall not exceed −70 dBW/MHz, averaged over any 2 millisecond active transmission interval, in frequencies between 1559 MHz and 1610 MHz. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations between 1559 MHz and 1605 MHz shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations between 1605 MHz and 1610 MHz manufactured more than six months after Federal Register publication of the rule changes adopted in FCC 03-283 shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval.

It should be noted that for the TN Band 23, which is studied and documented in [xx], the FCC regulation part 25.252 on ATC (Ancillary Terrestrial Component) for satellite service was referred to derive the RF requirements. However, since B252 is defined as an NTN band, ATC related requirements are not applicable for this band.

The following figure 8.1-1 shows an overview of the NR TN bands adjacent to the NTN B252 band.

Figure 8.1-1: Adjacent TN bands to NTN band n252 in North America

As shown in Figure 8.1-1, the B252 is adjacent to LTE bands B25 DL and B2 DL, and is overlap with B70 DL and B66 DL. The B25 DL and B2 DL protections in terms of UE-to-UE coexistence perspective is addressed in section 8.2.2.2.2.  The UE-to-UE coexistence between B252 and B70, and between B252 and B66 is described in section 8.2.2.2.3.

## 8.2	UE requirements

### 8.2.1	Channel arrangements

#### 8.2.1.1 	Channel arrangement for category M1

Table 8.2.1.1-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 252 | 100 | 2180 | 228301 | 228301 –<1>- 228500 | 2000 | 261069 | 261069 –<1>- 261268 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

#### 8.2.1.2 	Tx-Rx frequency separation

##### 8.2.1.2.1	Tx-Rx frequency separation for Cat. M1

Table 8.2.1.2.1-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 252 | 180 MHz1 161.4 to 198.6 MHz2 |
| NOTE 1: Default TX-RX separation. NOTE 2: The verification of flexible TX-RX frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3A. |  |

Table 8.2.1.2.1-2: TX – RX carrier centre frequency separation for REFSENS verification

| E-UTRA Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| 252 | 1.4 MHz | 161.4 MHz, 198.6 MHz |

##### 8.2.1.2.2	Tx-Rx frequency separation for Cat. NB1 and NB2

Table 8.2.1.2.2-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 252 | 180 MHz1 160.2 to -199.8 MHz2 |
| NOTE 1: Default Tx-Rx separation. NOTE 2: The verification of flexible TX-RX frequency separation within this range is limited to reference sensitivity. |  |

Table 8.2.1.2.2-2: TX – RX carrier centre frequency separation for REFSENS verification

| E-UTRA Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| 252 | 0.2 MHz | 160.2 MHz, 199.8 MHz |

### 8.2.2	UE transmitter characteristics

#### 8.2.2.1 	Maximum output power for category M1 and NB1/NB2

Table 8.2.2.1-1: UE Power Class for category M1

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 253 |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |

Table 8.2.2.1-2: UE Power Class for category NB1 and NB2

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| [253] | 23 | +/-2 | 20 | +/-2 |

#### 8.2.2.2 	Emission requirements and NS values

##### 8.2.2.2.1	Additional spurious emission requirements to protect GNSS frequency

As described in section 8.1, B252 shall protect the aeronautical radionavigation-satellite service in the frequency range of 1559 MHz and 1610 MHz. A separate NS value (NS_ZZ) is assigned to signal the protection of GNSS frequency. The additional spurious emission requirement for GNSS frequency protection is defined in the following Table 8.2.2.2.1-1.

Table 8.2.2.2.1-1: Additional spurious emission requirement to protect GNSS frequency 
(NS_ZZ requirement)

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |
| 1559≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

##### 8.2.2.2.2	UE-to-UE coexistence spurious emission requirements to protect B2 and B25

To protect B2 and B25, the following two different additional spurious emission requirements are defined, each is associated to an NS value, and a corresponding A-MPR.

  - -30 dBm/MHz (NS_XX)

  - -40 dBm/MHz (NS_YY)

This requirement applies based on coordination between operators and subject to regional/national regulation. From UE perspective, UE applies this requirement based on the received NS value.

NOTE: 	No A-MPR is needed to meet -40 dBm/MHz protection for the B2 and and B25 DL for B252 operation with UE Cat. M1, Cat. NB1/NB2 as described in 8.2.4.

The emission requirement for GNSS frequency protection is included as part of NS_07N and NS_08 requirements to avoid sending multiple NS flag when NS_07N or NS_08N is broadcast.

The following table 8.2.2.2.2-1 and 8.2.2.2.2-2 describe the additional requirement for NS_YY and NS_XX for Cat. M.

Table 8.2.2.2.2-1: Additional requirements for "NS_YY" for Cat.M

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE | Description |
| --- | --- | --- | --- | --- |
|  | 1.4MHz |  |  |  |
| 1559≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval | For GNSS frequency protection |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |  |
| 1930 ≤ f ≤1995 | -40 | 1MHz |  | For UE-to-UE coexistence |

Table 8.2.2.2.2-2: Additional requirements for "NS_XX" for Cat.M

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE | Description |
| --- | --- | --- | --- | --- |
|  | 1.4MHz |  |  |  |
| 1559≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval | For GNSS frequency protection |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |  |
| 1930 ≤ f ≤1995 | -30 | 1MHz |  | For UE-to-UE coexistence |

The following table 8.2.2.2.2-3 and 8.2.2.2.2-4 describe the additional requirement for NS_YY and NS_XX for Cat. NB1 and Cat. NB2.

Table 8.2.2.2.2-3: Additional requirements for "NS_YY" for Cat. NB1, Cat. NB2

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE | Description |
| --- | --- | --- | --- | --- |
|  | 1.4MHz |  |  |  |
| 1559≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval | For GNSS frequency protection |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |  |
| 1930 ≤ f ≤1995 | -40 | 1MHz |  | For UE-to-UE coexistence |

Table 8.2.2.2.2-4: Additional requirements for "NS_XX" for Cat. NB1, Cat. NB2

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE | Description |
| --- | --- | --- | --- | --- |
|  | 1.4MHz |  |  |  |
| 1559≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval | For GNSS frequency protection |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |  |
| 1930 ≤ f ≤1995 | -30 | 1MHz |  | For UE-to-UE coexistence |

##### 8.2.2.2.3	UE-to-UE coexistence spurious emission requirements for B70 and B66

The following spurious emission requirement for UE-to-UE coexistence applies for B70 and B66.

| NR NTN satellite Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| B252 | E-UTRA Band 70 | FDL_low | - | FDL_high | NA | NA | 1 |
|  | E-UTRA Band 66 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE 1: The co-existence between n252 and band 70 is subject to regional/national regulation. |  |  |  |  |  |  |  |

### 8.2.3	UE receiver characteristics

#### 8.2.3.1		Reference Sensitivity

##### 8.2.3.1.1		Reference Sensitivity for Cat.M1

Table 8.2.3.1.1-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 252 | -102.2 | FDD |

Table 8.2.3.1.1-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 252 | -103 | HD-FDD |

##### 8.2.3.1.2		Reference Sensitivity for Cat.NB1, Cat. NB2

Table 8.2.3.1.2-1: Reference sensitivity Cat. NB1/NB2

| NTN Band | REFSENS (dBm) |
| --- | --- |
| 252 | - 108.2 |

#### 8.2.3.2		Blocking requirements

##### 8.2.3.2.1		In-band blocking requirements

###### 8.2.3.2.1.1		In-band blocking requirements for Cat. M1

Table 8.2.3.2.1.1-1: In-band blocking for Cat.M1

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 256, 255, 254, 253, 252 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

##### 8.2.3.2.2	Out-of-band blocking requirements

###### 8.2.3.2.2.1	Out-of-band blocking requirements for Cat.M1

RAN4 agrees the Out-of-band blocking requirement is decoupled from the implementation assumption of UE duplexer. The requirement does not preclude any implementation of UE duplexer architecture.

Table  7.4.3.4.2-1 describes the agreed out-of-band blocking requirement for B252.

Table 8.2.3.2.2.1-1: Out-of-band blocking for B252

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| B252 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |

### 8.2.4	A-MPR

### 8.2.4.1	AMPR and NS value

NS values and their associated A-MPR for Cat. M1 and Cat. NB1/NB2 are defined in the following Table 8.2.4.1-1 and Table 8.2.4.1-2, respectively.

Table 8.2.4.1-1: NS value and associated A-MPR for B252 Cat. M1 UE

| Network signalling label | Requirements (clause) | NR satellite Band | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- |
| NS_ZZ | 8.2.2.2.1 | B252 |  | N/A |
| NS_XX | 8.2.2.2.2 | B2521 |  | N/A |
| NS_YY | 8.2.2.2.2 | B2521 |  | N/A |
| NOTE 1: The NS_XX and NS_YY are signalled based on coordination between operators and subject to regional/national regulation |  |  |  |  |

UETable 8.2.4.1-2: NS value and associated A-MPR for B252 Cat. NB1 E and Cat. NB2 UE

| Network signalling label | Requirements (clause) | NR satellite Band | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- |
| NS_ZZ | 8.2.2.2.1 | B252 |  | N/A |
| NS_XX | 8.2.2.2.2 | B2521 |  | N/A |
| NS_YY | 8.2.2.2.2 | B2521 |  | N/A |
| NOTE 1: The NS_XX and NS_YY are signalled based on coordination between operators and subject to regional/national regulation |  |  |  |  |

## 8.3 BS requirements

To Be Completed

--- End of changes ---