3GPP TSG-RAN WG4 Meeting #114-bis									                    R4-2505143

Wuhan, China, 7th-11th April 2025

Agenda item:			7.1.3

Source:	Skyworks, Qualcomm, Murata, Nokia, Samsung, Huawei, MediaTek, Vivo, ZTE (…)

Title:	WF on HPUE MSD simplification

Document for:	Approval

# 1		Introduction

This WF captures agreements for 1UL and 2UL MSD simplification based on proposals [1..10].

The goal of the HPUE MSD simplification is not to match the “legacy”/previously agreed MSD requirements (i.e., HPUE LUTs do not aim at achieving the best possible fitting of previously agreed MSD requirements). It is a way to reduce RAN4 MSD analysis workload, remove inconsistencies across band combinations and minimize errors by adopting a single set of assumptions.

# 2		WF on General Aspects

It is expected that CR for HPUE MSD topic is agreed in August RAN4#116 meeting when Rel-19 is completed.

Until that in RAN4 #115 and #116 basket WI work continues without any change.

If CR for 1 band 1 ULCC HPUE MSD topic is approved in September RAN#109 and if an exception to extend the Rel19 HPUE Baskets WIs are allowed after RAN#109, then the Rel19 HPUE Basket WIs may be revised to reflect that the new HPUE MSD rules can be applied,

NR-U is not part of this WF.

When the CR for HPUE MSD is implemented (after RAN#109)

- All specified PC2 MSD tables and PC1.5 MSD tables remain untouched in TS 38.101-1 and TS38.101-3

- If for some new PC2/1.5 CA configuration there is a need to define a MSD differently from LUTs it can be further discussed how to apply the exception case for the specific band combinations, but the expectation is that this is a rare case.

Handling of the Rel-19 baskets during the extension after CR for HPUE MSD is implemented (after RAN#109):

- The procedure is unchanged for the default power class (i.e.PC3): MSD must be studied, and, where applicable, the default PC3 MSD is captured in the TPs for TR

# 3		WF on 1 band 1 UL CC HPUE MSD Simplification

Agreement 1 on simplification Look-Up Tables (LUTs):

- Scope: In Rel-19, the 1 band UL 1CC HPUE MSD simplification only applies to PC2 1Tx, PC2 2Tx and PC1.5 achieved with 2Tx. Other Rel-19 1UL HPUE power classes are out of scope, e.g. PC1.

- For new Rel-19 inter-band CA combinations, and for all valid 1UL HPUE configurations, specify the 1 band 1 UL CC HPUE MSD requirements due to uplink harmonic, harmonic mixing, cross-band isolation caused by the same aggressor UL band as:

PCx MSD = PC31Tx MSD + MSD where,

- x = 2 1Tx, 22Tx, or 1.52Tx and,

MSD is specified in three LUT columns with a maximum MSD (“MSDmax”) of 3, 6 and 9dB for harmonic, cross band isolation, harmonic mixing. One example of such LUT is captured below. Coarser granularity and other tabulated values are not precluded.

| PC31Tx MSD (dB) | MSDmax (dB) |  |  |
| --- | --- | --- | --- |
|  | 3 | 6 | 9 |
| 0.1≤ ≤0.3 | 0.3 | 0.4 | 1.0 |
| 0.4≤ ≤0.6 | 0.5 | 0.8 | 1.8 |
| 0.7≤ ≤0.9 | 0.7 | 1.1 | 2.5 |
| 1.0≤ ≤1.9 | 1.2 | 2.1 | 4.1 |
| 2.0≤ ≤2.9 | 1.6 | 2.7 | 5.1 |
| 3.0≤ ≤3.9 | 1.8 | 3.3 | 5.9 |
| 4.0≤ - ≤4.9 | 2.0 | 3.8 | 6.5 |
| 5.0≤ - ≤6.9 | 2.3 | 4.5 | 7.3 |
| 7.0≤ - ≤9.0 | 2.5 | 5.0 | 7.9 |
| ≥10 | 3 | 6 | 9 |

- and where the mapping of the 1UL MSD type to the “MSDmax” LUT is specified with the following companion table.

| MSD type |  | MSDmax (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3 | 6 | 9 |
| 1UL | UL harmonic | PC21Tx | PC22Tx | [PC1.52Tx] |
|  | Harmonic mixing | PC21Tx | PC22Tx | PC1.52Tx |
|  | Cross-band isolation | PC21Tx | PC22Tx | PC1.52Tx |

- It should be noted that for some band combinations, LUTs may represent a slight tightening of the PC22Tx/PC1.52Tx MSD requirements, especially for UL harmonic, while for other band combinations LUTs may represent a slight relaxation of the PC22Tx/PC1.52Tx MSD requirements, especially for harmonic mixing.

- Agreement 2 on handling band combinations without MSD for PC3 but with small PC2/PC1.5 MSD:

- For new HPUE band combinations, do not specify the MSD requirements for 1UL HPUE PC21Tx PC22Tx and PC1.5 (2Tx).

- For legacy PC3 band combinations (i.e. band combinations specified prior to the agreement of the MSD simplification CR for 1UL HPUE), where no PC3 MSD is specified, companies are encouraged to browse through these band combinations and bring correction CRs if 1UL HPUE MSD requirements are needed.

Background on the companion table.

This table is needed for two reasons:

  - to anticipate future 1UL power classes and / or new MSD requirements. For example, at the time of writing, in Rel-19 there are no PC1.52Tx MSD requirements due to UL harmonic interference. Hence, no mapping exists for this case yet. In anticipation of these new 1UL CC HPUE MSD requirements, it is proposed to capture this mapping using square brackets.

  - to facilitate the possible introduction of a single “unified” mapping table for both 1UL and 2UL HPUE MSD simplification [9]. An extension of the 1UL companion table is anticipated to address 2UL mapping cases of Rel-19 different power classes.

# 2			WF on 2 band 2 UL CC HPUE MSD Simplification

Agreement:

- 2UL HPUE MSD requirements can be mapped to a separate LUT with maximum of [8] columns, e.g. at least with ΔMSDmax = [3], 6, 9, 12, 15, [18, 24, 30] dB.

- Other values are not precluded and the need for additional columns is FFS.

- FFS on optimized mapping to avoid penalizing 2UL combos with too high ΔMSDmax.

# 3			References

- R4-2503196 On Rel-19 HPUE CA DC MSD consideration, 3GPP TSG-RAN WG4 Meeting # 114b, Wuhan, China, Apple.

- R4-2503153 Discussion on HPUE MSD, 3GPP TSG-RAN WG4 Meeting # 114b, Wuhan, China, Huawei, Hisilicon.

- R4-2503362 Discussion on MSD simplification of HPUE CA/DC UE, 3GPP TSG-RAN WG4 Meeting # 114b, Wuhan, China, MediaTek Korea Inc.

- R4-2503215 Further discussion on HPUE MSD rules, 3GPP TSG-RAN WG4 Meeting # 114b, Wuhan, China, Murata Manufacturing Co Ltd.

- R4-2503225 On 1UL/1CC HPUE MSD rules, 3GPP TSG-RAN WG4 Meeting # 114b, Wuhan, China, Nokia, Samsung.

- R4-2504118 MSD beyond PC3, 3GPP TSG-RAN WG4 Meeting # 114b, Wuhan, China, Qualcomm France.

- R4-2503169 Simplification of HPUE 2UL DeltaMSD, 3GPP TSG-RAN WG4 Meeting # 114b, Wuhan, China, Skyworks Solutions Inc.

- R4-2503238 On 1UL HPUE MSD simplifications, 3GPP TSG-RAN WG4 Meeting # 114b, Wuhan, China, Skyworks Solutions Inc.

- R4-2503239 Unified HPUE 1UL2UL MSD simplification proposals for Rel-19, 3GPP TSG-RAN WG4 Meeting # 114b, Wuhan, China, Skyworks Solutions Inc., Samsung.

- R4-2503839 Discussion on HPUE MSD simplification, 3GPP TSG-RAN WG4 Meeting # 114b, Wuhan, China, Vivo.