3GPP TSG-RAN4 Meeting #114bis	R4-2505149

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	vivo, Ericsson

Title:	TP for TR 38.774 on LP-WUS UE RF considerations

Agenda item:			7.26.2.1

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.774 to add considerations and agreements for LP-WUS RF aspects.

2. Reference

[1] TR 38.774 v0.2.0

# 3. Text Proposal

<Start of Text Proposal>

7.1.2	Rx SNR evaluations

To derive SNR performance of LP-WUS, it was agreed to select OOK-4 M=4 under AWGN channel model with 1% MDR without repetition as worst case.

<Skip>

7.1.5	ASCS requirements

Adjacent subcarrier selectivity (ASCS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured channel frequency in the presence of adjacent in-band NR signal(s). The in-band LP-WUS and NR signal should be same PSD and be separated by a given frequency offset (guard RB) between LP-WUS and NR, and the NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs.

<End of Text Proposal>