3GPP TSG RAN WG4 Meeting #114-bis	R4-2505152

Wuhan, Hubei, China, 7th – 11th April, 2025

Agenda item:			7.19.5

Source:	vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT

Title:	Updated simulation assumptions for AI/ML BM

Document for:	Approval

- Introduction

In this contribution, the simulation assumption for AI/ML based BM case-1 is further updated based on [1]. Companies are encouraged to provide simulation results to evaluate the impact of the measurement error based on the performance metrics in section 5.

- Simulation assumptions for baseband error

Link level simulation (LLS) assumptions and System level simulation (SLS) assumptionsfor evaluation of measurement error on AI/ML based BM case-1 performance are defined in Table 1-6.

Table 1 Baseline Link Level Simulation assumptions for AI/ML in beam management evaluations

| Parameter | Value |
| --- | --- |
| Carrier frequency | 30GHz |
| Subcarrier spacing | 120kHz |
| Channel model | Case 1: NLOS channel - Baseline: - TDL-C, DS=100ns - Optional: - CDL-C, DS=100ns |
|  | Case 2: AWGN channel |
| BS antenna configurations | - Option 1: - One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline. - Number of Tx beams is 32 - Option 2: - Number of TX beams: 1 - Baseband errors are independent for each TX beam |
| BS antenna element radiation pattern | Option1: Table 2 Option 2: N/A |
| BS antenna height and antenna array down-tilt angle, if needed (Option 1 above) | Option1: 25m, 110° Option 2: N/A |
| Beamforming characteristic of the BS pattern | Use same assumption for System-level simulation assumption defined Table 6 |
| UE antenna configurations | Option 1: Panel structure: (M, N, P) = (1, 4, 2), - 2 panels (left, right) with (Mg, Ng) = (1, 2) as baseline - 1 panel as optional Number of Rx beams is up to UE Option 2: 2 Rx |
| UE antenna element radiation pattern | Option 1: Table 3 Option 2: N/A |
| UE moving speed | 3km/h |
| Reference signal | SSB or CSI-RS |
| DRX | No |
| Number of samples in L1 averaging | Baseline: - 1 sample Optional: - 3 samples - 5 samples |
| Note 1: TXRU weights mapping for BS and UE is up to company. |  |

Table 2 BS antenna radiation pattern

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8dBi |

Table 3 UE antenna radiation pattern model

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in dim (dB) |  |
| Antenna element radiation pattern in dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |
| Note: are in local coordinate system. |  |

Table 4 Cell-specific parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 |
| SSB periodicity | ms | 20 |
| SSB bandwidth | RB | 20 |
| SSB SCS | kHz | 120 |
| RB Utilization | % | 100 |
| Data Modulation | - | QPSK |
| CP Length | - | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| Relative Delay of 1st Path (synchronous) | µs | 0 |
| SNR | dB | -3 Note: Simulation with other SNR levels are not precluded, e.g., for determining the SNR for ideal measurement. |
| Note: When analog beamforming is used in LLS, SNR value of above row represents the post beamforming SNR of the strongest Tx-Rx beam pair. For other Tx-Rx beam pairs, there are two options. Option 1: Apply error distribution from the post beamforming SNR of the strongest Tx-Rx beam pair to all Tx-Rx beam pair’s measurements in the SLS Option 2: Generate error distribution separately for each individual Tx-Rx beam pair’s SNRs and apply to the corresponding Tx-Rx beam pair measurement in the SLS. When analog beamforming is not used in LLS, Apply error distribution based on the SNR of above row to the strongest Tx-Rx beam pair’s measurements in the SLS Option 1: Apply error distribution from the SNR of above row to all Tx-Rx beam pairs’ measurements in the SLS Option 2: Generate error distribution separately for each individual Tx-Rx beam pairs’ SNR values and apply to the corresponding Tx-Rx beam pair measurements in the SLS. SNR of the strongest Tx-Rx beam pair can be fixed or not fixed. |  |  |

Table 5 CSI-RS configuration parameters

| Parameter | Value |
| --- | --- |
| CSI-RS bandwidth | 48 PRBs; |
| CSI-RS SCS | 120 kHz |
| Periodicity | 5 ms (optional); 40 ms |
| CSI-RS configuration: <X,D,N,CDM>, where: X=number of CSI-RS ports, D=density [RE/RB/port], N=number of OFDM symbols in the same slot | <32,1,4,FD-CDM2> |

Table 6 Beamforming characteristic of the BS pattern for System-Level simulation assumption

| Parameter | Value |
| --- | --- |
| Beam pattern legend for Set A and Set B |  |
| Azimuth and elevation angle ranges | 32 beams with grid of 4 elevation angles from (100°to 150°) and 8 azimuth angles from (-60°to 60°) Note: here the 0 degree in Elevation refers to +Z axis |
| AoDs of Set A and Set B (°) | - Set A: - 4 elevation DFT beams: [106.25, 118.75, 131.25, 143.75] - 8 azimuth DFT beams: [-52.5, -37.5, -22.5, -7.5, 7.5, 22.5, 37.5, 52.5] - Set B [(azimuth, elevation)]: - [(-52.5, 143.75), (-37.5, 131.25), (-22.5, 118.75), (-7.5, 106.25), (7.5, 143.75), (22.5, 131.25), (37.5, 118.75), (52.5, 106.25) |

- Simulation assumptions for RF error

The impact of RF errors should be considered. Following assumptions can be used to generate RF errors.

- Truncated Gaussian distribution under ±4.5 dB RF error (μ=0,  σ2=4.5) is used

  - Each TX beam are Independent

- Reference model

For better alignment of the performance between companies, it would be better to perform simulations based on a reference model. The model description in Fig. 1 is an example of reference model for BM-case 1 performance evaluation, while it is not the intention to preclude other reference models.

Fig. 1 Reference model description

- Performance metrics

At least the following performance characteristics are to be provided

KPI 1: Beam prediction accuracy

- The successful rate for the correct prediction which is considered as maximum ground truth RSRP among top-K predicted beams is larger than the ground truth RSRP of the strongest genie-aided beam – x dB, where K=1,2,3,4,5, X= 1, 2, 3dB

KPI 2: Absolute RSRP accuracy

- 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ground truth L1-RSRP of the same beams

Note 1: the ‘ground truth’ underlined refers to:

- Ideal value without consideration of measurement error (for Case 1 and Case 2a defined in clause 6)

- Measurement result with consideration of measurement error (for Case 2b and Case 3 defined in clause 6)

Note 2:

90%-tile L1-RSRP difference = max (abs (95%-tile L1-RSRP), abs (5%-tile L1-RSRP))

- Simulation procedures

We provide the following procedures for companies to perform simulations to evaluate the impact of measurement error in BM prediction Sceanario-1: spatial domain prediction with 32 beams in Set A, Set B is subset of Set A and contains 8 beams for measurement.

The following cases are treated as the candidate Options:

- Case 1: No error will be considered in training dataset, model input during inference and ground-truth.

- Case 2:

  - Case 2a No error will be considered in training dataset and ground-truth. Error will be considered in model input during inference

  - Case 2b No error will be considered in training dataset and ground truth for training. Error will be considered in model input during inference and ground-truth for inference

- Case 3: Error will be considered in training dataset, model input during inference and ground-truth.

For Case 1, the simulation procedure includes:

- Companies to generate ideal L1-RSRP dataset from the SLS assumption defined in Table 6 and TR 38.843 Table 6.3.1-1.

- Use subset samples of the ideal dataset generated from the 1st step for training models.

- Filter UEs in the other subset samples of the ideal dataset (none overlap samples with the subset dataset in 2nd step) which SNR> -3dB (use this SNR range as the starting point) to derive the ideal dataset for inference

Note: UE distribution is 100% outdoor

- Use the ideal dataset for inference and to evaluate the performance metric assuming that the ground truth is without the measurement error

For Case 2a and Case 2b, the simulation procedure includes:

- Companies to generate ideal L1-RSRP dataset from the SLS assumption defined in Table 6 and TR 38.843 Table 6.3.1-1.

- Use subset samples of the ideal dataset generated from the 1st step for training models.

- Filter UEs in the other subset samples of the ideal dataset (none overlap samples with the subset dataset in 2nd step) which SNR> -3dB (use this SNR range as the starting point) to derive the ideal dataset for inference.

Note: UE distribution is 100% outdoor

- Use the LLS simulation assumptions defined in Table 1-5 to generate L1-RSRP difference as the baseband errors.

- Use the Simulation assumptions for RF error defined in Clause 3 to generate RF errors.

- Add the measurement errors generated from LLS into the ideal SLS dataset to derive the dataset with measurement errors. The measurement errors will be:

  - Alt 1: Only baseband errors are included

  - Alt 2: Only RF errors are included

  - Alt 3: Both baseband errors and RF errors are included

- Use the dataset with measurement errors for inference and to evaluate the performance metric assuming that

- Case 2a: the ground truth is with the measurement error (The same Alt of error as selected in Step 5)

Case 2b: the ground truth is without the measurement error

For Case 3, the simulation procedures include:

- Companies to generate ideal L1-RSRP dataset from the SLS assumption defined in Table 6 and TR 38.843 Table 6.3.1-1.

- Use subset samples of the ideal dataset generated from the 1st step with added measurement error for training models (The same Alt of error as selected in Step 6).

- Filter UEs in the other subset samples of the ideal dataset (none overlap samples with the subset dataset in 2nd step) which SNR> -3dB (use this SNR range as the starting point) to derive the ideal dataset for inference.

Note: UE distribution is 100% outdoor

- Use the LLS simulation assumptions defined in Table 1-5 to generate L1-RSRP difference as the baseband errors.

- Use the Simulation assumptions for RF error defined in Clause 3 to generate RF errors.

- Add the measurement errors generated from LLS into the ideal SLS dataset to derive the dataset with measurement errors. The measurement errors will be:

  - Alt 1: Only baseband errors are included

  - Alt 2: Only RF errors are included

  - Alt 3: Both baseband errors and RF errors are included

- Use the dataset with measurement errors for inference and to evaluate the performance metric assuming that the ground truth is with the measurement error (The same Alt of error as selected in Step 6)

- References

- R4-2502969, Updated simulation assumptions for beam prediction, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT

- TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)