3GPP TSG-RAN WG4 Meeting # 114bis	R4-2505163

Wuhan, China, 7th – 11st April, 2025

Source:	Huawei, HiSilicon

Title:	TP for TR 38.719-02-01 on introduction of CA_n40A-n71A

Agenda item:	6.3.3

Document for:	Approval

1 Background

This contribution provides text proposal on the NR band combination CA_n40A-n71A.

2 Text Proposal

---Start of changes---

5.x	CA_n40-n71

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination of band nX+nY

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band nX+nY

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n40A-n71A | CA_n40A-n71A | n40 | n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |

5.x.1.3	UE co-existence studies for 1 band UL

Table 5.x.1.3-1 CA_n40A-n71A UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n40 | UL12 | UL2 | UL33 | UL4 | UL5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 2300 | 4600 | 6900 | 9200 | 11500 | MSD type |
| n71 | fLow | fHigh | 2400 | 4800 | 7200 | 9600 | 12000 |  |
| DL1 | 617 | 652 | N/A | No | No | No | No | UL harmonic |
| DL23 | 1234 | 1304 | No | N/A | No | N/A | N/A | Harmonic mixing |
| DL34 | 1851 | 1956 | No | No | N/A | No | N/A |  |
| DL4 | 2468 | 2608 | No | N/A | N/A | N/A | N/A |  |
| DL54 | 3085 | 3260 | No | No | N/A | N/A | N/A |  |
| Analysis |  |  | There is no UL/DL harmonics/haromic mixing issue for this combo |  |  |  |  |  |
| UL/DL |  | nY | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 663 | 1326 | 1989 | 2652 | 3315 |  |
| nX | fLow | fHigh | 698 | 1396 | 2094 | 2792 | 3490 |  |
| DL1 | 2300 | 2400 | N/A | No | No | No | No | UL harmonic |
| DL23 | 4600 | 4800 | No | N/A | No | N/A | N/A | Harmonic mixing |
| DL34 | 6900 | 7200 | No | No | N/A | No | N/A |  |
| DL4 | 9200 | 9600 | No | N/A | N/A | N/A | N/A |  |
| DL54 | 11500 | 12000 | No | No | N/A | N/A | N/A |  |
| Analysis |  |  | There is no UL/DL harmonics/haromic mixing issue for this combo |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range |  |  |  |  |  |  |  |  |
| Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. |  |  |  |  |  |  |  |  |
| When the gap between ULx and DLy frequency range is from 0Hz to n*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. |  |  |  |  |  |  |  |  |
| Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz |  |  |  |  |  |  |  |  |
| Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.x.1.3-2 CA_n40A-n71A UL/DL cross band isolation analysis

| Bands3 | n40 |  | n71 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 2300 | 2400 | 663 | 698 |
| fDL (MHz) | 2300 | 2400 | 617 | 652 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 100 | 5 | 35 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 2200 | 2500 | 628 | 733 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 2100 | 2600 | 593 | 768 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 2000 | 2700 | 558 | 803 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1900 | 2800 | 523 | 838 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1800 | 2900 | 488 | 873 |
| Analysis | There is no cross band isolation issue |  | There is no cross band isolation issue |  |
| Note 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: |  |  |  |  |
| - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. |  |  |  |  |
| - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency |  |  |  |  |
| - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible |  |  |  |  |
| Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges |  |  |  |  |
| while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.x.1.4	∆TIB,c and ∆RIB,c values

For CA_n40-n71, the TIB,c and RIB,c values are given in the tables below referring to CA_n28-n40.

Table 5.x.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n40-n71 | 0.3 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.x.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n40-n71 | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.x.1.5	REFSENS requirements

There is no MSD requirements for this combo.

5.x.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirements for CA_n40-n71.

5.x.2	Specific for 2 bands UL CA

5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n40A-n71A | 23 | +2/-3 |

5.x.2.2	UE co-existence studies for 2 bands UL

Table 5.x.2.2-1 lists Band n40 + Band n71 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2.2-1: Band n40 and Band n71 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1737 | 1602 | 2963 | 3098 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3902 | 4137 | 1074 | 904 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5263 | 5498 | 3626 | 3796 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6202 | 6537 | 411 | 206 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3204 | 3474 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7563 | 7898 | 4289 | 4494 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5926 | 6196 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 492 | 252 | 8937 | 8502 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2506 | 2811 | 5874 | 5504 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4952 | 5192 | 9863 | 10298 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6589 | 6894 | 8226 | 8596 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.x.2.2-1, there is no IMD issue for CA_n40-n71

5.x.2.3	REFSENS requirements

Based on the assumption of restricted frequency range for CA_n40-n71, there is no MSD issue due to intermodulation interference.

---End of changes---

Reference

[1]	RP-241833, “Revised WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells”, Ericsson, ZTE, Huawei, HiSilicon