3GPP TSG-RAN WG4 Meeting # 114-bis					R4-2505169

Wuhan Meeting, Apr. 7th – Apr. 11th, 2025

Title:	TP to TR 38.719-02-01 CA_n5-n48

Source:	Nokia, Verizon

Agenda item:	6.3.3

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.719-02-01 to add CA_n5A-n48B, CA_n5B-n48B and CA_n5B-n48A with uplink CA variants of BCS 4&5. Note that additional tables are added for TR completeness, regardless if the requirements have been placed in the specifications.

************************************* Start of TP*****************************************

## 5.6		CA_n5-n48

### 5.6.1	Common for 1 band UL and 2 bands UL CA

#### 5.6.1.1	Operating bands for CA

Table 5.6.1.1-1:  CA band combination of band n5+n48

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

#### 5.6.1.2	Channel bandwidths per operating band for CA

Table 5.6.1.2-1: Supported bandwidths per CA band combination of band n5+n48

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n48A | CA_n5A-n48A | n5 | n5 channel bandwidths in Table 5.3.5-1 of 38.101-1 | 4 and 5 |
|  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |
| CA_n5A-n48B | CA_n48B CA_n5A-n48A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  | CA_n48B CA_n5A-n48A CA_n5A-n48B | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n48 | CA_n48B_BCS 4 and 5 |  |
| CA_n5B-n48A | CA_n5B CA_n5A-n48A CA_n5A-n48A | n5 | CA_n5B_BCS 4 and 5 | 4 and 5 |
|  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5B-n48B | CA_n5B CA_n48B CA_n5A-n48A CA_n5A-n48B | n5 | CA_n5B_BCS 4 and 5 | 4 and 5 |
|  |  | n48 | CA_n48B_BCS 4 and 5 |  |
| CA_n5B-n48(2A) | CA_n5B CA_n5A-n48A | n5 | CA_n5B_BCS 4 and 5 | 4 and 5 |
|  |  | n48 | CA_n48(2A)_BCS 4 and 5 |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. |  |  |  |  |

#### 5.6.1.3	UE co-existence studies for 1 band UL

Table 5.6.1.3-1 summarizes frequency ranges where harmonics and/or harmonic mixing occur for CA_n5-n48.

Table 5.6.1.3-1: UL harmonics and harmonic mixing analysis

| UL/DL |  | n48 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 3550 | 7100 | 10650 | 14200 | 17750 |  |
| n5 | fLow | fHigh | 3700 | 7400 | 11100 | 14800 | 18500 |  |
| DL1 | 869 | 894 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 1738 | 1788 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 2607 | 2682 |  |  | N/A |  | N/A |  |
| DL4 | 3476 | 3576 | D | N/A | N/A | N/A | N/A |  |
| DL5 | 4345 | 4470 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is collision detected from downlink band n5 4th harmonic mixing with n48 uplink. |  |  |  |  |  |
| UL/DL |  | n5 | UL13 | UL2 | UL3 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 824 | 1648 | 2472 | 3296 | 4120 |  |
| n48 | fLow | fHigh | 849 | 1698 | 2547 | 3396 | 4245 |  |
| DL1 | 3550 | 3700 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 7100 | 7400 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 10650 | 11100 |  |  | N/A |  | N/A |  |
| DL4 | 14200 | 14800 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 17750 | 18500 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |

Based on Table 5.6.1.3-1:

For the 4th DL harmonic-mixing falling on the downlink, the BSD for CA_n5-n48 for PC3 should be same as the defined CA_n26-n48. This is because the new higher channel bandwidth introduced for band n48 BCS 4 and 5 is the same as the BCS 0 and 1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n48 | n5 | 10 | 15 | 25 (RBstart=0) | 5 | 5.7 | NOTE 8 | UL1/DL4 |

Table 5.6.1.3-2 summarizes frequency ranges where cross-band isolation issues may occur for both UL bands into the other band DL.

Table 5.6.1.3-2: Cross-band isolation analysis

| Bands | n5 |  | n48 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 824 | 849 | 3550 | 3700 |
| fDL (MHz) | 869 | 894 | 3550 | 3700 |
| CBW (MHz) | 5 | 25 | 5 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 799 | 874 | 3450 | 3800 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 774 | 899 | 3350 | 3900 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 749 | 924 | 3250 | 4000 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 724 | 949 | 3150 | 4100 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 699 | 974 | 3050 | 4200 |
| Analysis | There is no overlap of the band n5 ACLR range with the band n48 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n48 so no cross-band MSD is needed. |  | There is no overlap of the band n48 ACLR range with the band n5 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n5 so no cross-band MSD is needed. |  |

Based on Table 5.6.1.3-2:

There is no uplink band overlapping with the downlink in ACLR range. The existing diplexer isolation should provide sufficient rejection.

Table 5.6.1.3-3 summarizes frequency ranges where IMD products caused by n48 UL band with 2CC intra-band UL CA may occur for n48B.

Table 5.6.1.3-3: Intra-band ULCA IMD overlap with the other DL band analysis.

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n48 fUL | 3550 | 3700 | Order | flow | fhigh |
| n5 fDL | 869 | 894 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 20 | 100 |  | 20 | 100 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 40 | 200 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 3450 | 3800 |  | 60 | 300 |
| IMD5 (3-2) | fULlow-2*Max2CCBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 3350 | 3900 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 3250 | 4000 |  | 7000 | 7500 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 3150 | 4100 |  | 6900 | 7600 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 3050 | 4200 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 2950 | 4300 |  | 10550 | 11200 |
| Analysis | No issues found |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

Based on Table 5.6.1.3-3:

There are no issues for uplink CA_n48B

Table 5.6.1.3-4 summarizes frequency ranges where IMD products caused by n5 UL band with 2CC intra-band UL CA may occur for n5B.

Table 5.6.1.3-4: Intra-band ULCA IMD overlap with the other DL band analysis.

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n5 fUL | 824 | 849 | Order | flow | fhigh |
| n48 fDL | 3550 | 3700 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 10 | 25 |  | 10 | 25 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 20 | 50 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 799 | 874 |  | 30 | 75 |
| IMD5 (3-2) | fULlow-2*Max2CCBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 774 | 899 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 749 | 924 |  | 1623 | 1723 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 724 | 949 |  | 1598 | 1748 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 699 | 974 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 674 | 999 |  | 2447 | 2572 |
| Analysis | No issues found |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

Based on Table 5.6.1.3-4:

There are no issues for uplink CA_n5B.

#### 5.6.1.4	∆TIB,c and ∆RIB,c values

For CA_n5-n48, the ∆TIB,c and ∆RIB,c values have already been specified.

#### 5.6.1.5	REFSENS requirements

There is no need for new REFSENS requirements.

#### 5.6.1.6	OOB blocking exception requirements

There is no need for OOB blocking exception.

### 5.6.2	Specific for 2 bands UL CA

#### 5.6.2.1	Maximum output power for inter-band CA

Table 5.6.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n48B | 23 | +2/-3 |

#### 5.6.2.2	UE co-existence studies for 2 bands UL

Table 5.6.2.2-1 lists Band n5 + Band n48 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.6.2.2-1: Band n5 and Band n48 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2701 - 2876 |  | 4374 - 4549 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1852 - 2052 |  | 6251 - 6576 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5198 - 5398 |  | 7924 - 8249 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1003 - 1228 |  | 9801 - 10276 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 5402 - 5752 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6022 - 6247 |  | 11474 - 11949 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8748 - 9098 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13351 - 13976 |  | 154 - 404 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8952 - 9452 |  | 4553 - 4928 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15024 - 15649 |  | 6846 - 7096 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12298 - 12798 |  | 9572 - 9947 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

The requirements are already found in the specification.

Table 5.6.2.2-2 lists Band n5 + Band n48 2 bands UL CA(3CC) triple beat IMD analysis for Band n5- Band n48

Table 5.6.2.2-2: Band n5 and Band n48 for 3CC UL IMD products

| Band / CA1 | n5 |  | CA_n48B |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 824-849 |  | 3550-3700 |  |
| fDL | 869-894 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -100 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 724-824 |  | 849-949 |  |
| Analysis | There is an issue of IMD3, but no requirements, since the bands are not in same or adjacent bandgroups |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. NOTE 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

#### Based on Table 5.6.2.2-2:

The bands are not of the same or adjacent band groups, so Note 1 applies.

#### 5.6.2.3	REFSENS requirements

There is no need for new REFSENS requirements.

************************************* End of TP******************************************