3GPP TSG RAN WG4 Meeting #114bis	R4-2505213

Wuhan, China, 07 April 2025 – 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.922 | CR |  | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 38.922 Removal of references to internal TRs and Tdocs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_IMT_4400_7125_14800MHz |  |  |  |  |  | Date: |  |  | 2025-03-19 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Removal of references to internal TRs and TDOCs is to address the comments from RAN plenary |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of references to Tdocs and internal 3GPP TRs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current TR shall not pass the TR checker from MCC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2,3,4,5,6,7, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	(Void)

[3]	(Void)

[4]	(Void)

[5]	(Void)

[6]	(Void)

[7]	(Void)

[8]	(Void)

[9]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”

[10]	3GPP TS 36.104: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”

[11]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

[12]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[13]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain"

[14]	3GPP TR 38.921: “Study on International Mobile Telecommunications (IMT) parameters for 6.425 – 7.025 GHz, 7.025 – 7.125 GHz and 10.0. – 10.5 GHz”

[15] 	C A Balanis, “Antenna Theory Analysis and Design”, John Wiley and sons

[16] 	H. Jin et al., "Massive MIMO Evolution Toward 3GPP Release 18," in IEEE Journal on Selected Areas in Communications, vol. 41, no. 6, pp. 1635-1654, June 2023

[17] 	Christopher Mollen, Ulf Gustavsson, Thomas Eriksson, Erik G Larsson, “Out of Band radiation Measure for MIMO arrays with beamformed transmission “, Proc. IEEE Int. Conf. Commun., May 2016.

[18] 	Erik Larsson, Liesbet Van der Perre, “Out of band Radiation for Antenna arrays Clarified”, arXiv : 1802.02745v1, Feb 2018

[19] 	E. Sienkiewicz, N. McGowan, B. Göransson, T. Chapman and T. Elfström, "Spatially dependent ACLR modelling," 2014 IEEE Conference on Antenna Measurements & Applications (CAMA), Antibes Juan-les-Pins, France, 2014, pp. 1-4,

[20] 	A. S. Tehrani, H. Cao, S. Afsardoost, T. Eriksson, M. Isaksson and C. Fager, "A Comparative Analysis of the Complexity/Accuracy Trade off in Power Amplifier Behavioural Models," in IEEE Transactions on Microwave Theory and Techniques, vol. 58, no. 6, pp. 1510-1520, June 2010

[21] 	C. Mollén, U. Gustavsson, T. Eriksson and E. G. Larsson, "Spatial Characteristics of Distortion Radiated From Antenna Arrays With Transceiver Nonlinearities," in IEEE Transactions on Wireless Communications, vol. 17, no. 10, pp. 6663-6679, Oct. 2018.

[22] 	Y. Zou et al., "Impact of Power Amplifier Nonlinearities in Multi-User Massive MIMO Downlink,” 2015 IEEE Globecom Workshops, San Diego, CA, USA, 2015, pp. 1-7.

[23] 	P Nandin et al, “Web Lab: A Web based set up for PA digital distortion and Characterisation”, IEEE Microwave Magazine, vol 16, no1, p 138- 140, Feb 2015

[24]	(Void)

[25]	(Void)

[26]	(Void)

[27]	(Void)

[28]	ITU-R Recommendation M.2101: " Modelling and simulation of IMT networks and systems for use in sharing and compatibility studies"

[29]	(Void)

[30]	(Void)

[31]	(Void)

[32]	(Void)

[33]	(Void)

[34]	(Void)

<End of Change 1>

<Start of Change 2>

# 4	4400 - 4800 MHz frequency range

## 4.1	General parameters

The general parameters can be extracted from requirements defined for NR operating band n79.

Figure 4.1-1: NR band definition in the  4.4 – 5.0 GHz frequency range

### 4.1.1	Duplex mode

RAN4 considered TDD as the current duplexing candidate. An enhancement of TDD duplexing, via allowing the simultaneous existence of non-overlapping downlink and uplink sub-band at the BS side within a TDD carrier in a conventional TDD band (i.e., sub-band non-overlapping full duplex), was studied in Rel-18. RAN4 Rel-19 normative work for SBFD operation at the BS side within a TDD carrier is on-going and the requirements and conformance aspects for Rel-19 SBFD work item can be tracked through the list of impacted specifications captured in the respective Work Item description.

<End of Change 2>

<Start of Change 3>

# 5	7125 - 8400 MHz frequency range

## 5.1	General parameters

### 5.1.1	Duplex mode

Even though FDD is not precluded, most likely TDD will be used in this frequency range. This frequency range is adjacent to existing TDD bands, e.g. n104 (6425 – 7125 MHz) and n79 (4400 – 5000 MHz), making also SBFD as a candidate duplexing method. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specifications captured in the respective Work Item description.

### 5.1.2	Channel Bandwidth

While a number of channel bandwidth will be specified for this frequency range, 100 MHz is considered as a typical channel bandwidth. Higher channel bandwidths compared to 100MHz are not precluded for this range. Annex A entails additional information on the impact of higher channel bandwidth on different RF parameters.

<End of Change 3>

<Start of Change 4>

### 5.4.2	UE antenna characteristics

The outcome of the RAN WG4 study for collecting technical background information relevant for the frequency range 7 to 24 GHz indicated that the frequency range 7.125 - [10-13] GHz would have "FR1 like" requirements. Therefore, the UE is expected to have a conducted interface assuming an isotropic radiation pattern antenna and without beamforming.

<End of Change 4>

<Start of Change 5>

### 6.1.3	Co-existence simulation results

#### 6.1.3.1	Urban Macro scenario

##### 6.1.3.1.1	Downlink

For Urban macro scenarios, simulation results for DL throughput loss with baseline assumption ACLR = 45 dBc at BS and ACS = 33 dBc at UE are summarized in Table 6.1.3.1.1-1/2/3/4/5. The following results are based on the FR1 like UE with omni-directional antenna and no beamforming.

Table 6.1.3.1.1-1: DL throughput loss for 14800 – 15350 MHz [Relative ACIR value]

| Company | Throughput loss | Relative ACIR offset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | -2 | -3 | -4 | -6 | -8 | -9 | -10 | -12 | -15 |
| Nokia | Average throughput loss in % (15 GHz) | 0.17 |  | 0.28 |  | 0.45 |  | 0.70 |  | 1.06 | 1.55 |
|  | 5%-tile throughput loss in % (15 GHz) | 0.00 |  | 0.18 |  | 0.35 |  | 1.00 |  | 1.38 | 3.53 |
| ZTE | Average throughput loss in % (15 GHz) | 0.33 | 0.41 |  | 0.52 | 0.67 | 0.85 |  | 1.09 |  |  |
|  | 5%-tile throughput loss in % (15 GHz) | 2.50 | 2.75 |  | 3.41 | 4.38 | 4.97 |  | 5/89 |  |  |

Table 6.1.3.1.1-2: DL throughput loss for 14800 – 15350 MHz [Absolute ACIR value]

|  | ACIR (dB) | 5 | 10 | 15 | 17 | 18 | 19 | 20 | 21 | 25 | 30 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Apple | Average throughput loss in % |  |  | 0.47 |  |  |  | 0.232 |  |  | 0.49 | 0.026 |
|  | 5%-tile throughput loss in % |  |  | 0.623 |  |  |  | 0.247 |  |  | 0.054 | 0.006 |
| CATT | Average throughput loss in % | 0.043 | 0.026 | 0.015 |  |  |  | 0.0082 |  | 0.0042 | 0.0023 |  |
|  | 5%-tile throughput loss in % | 0.16 | 0.15 | 0.14 |  |  |  | 0.11 |  | 0.10 | 0.046 |  |
| MediaTek | Average throughput loss in % | 7.41 | 4.61 | 2.74 |  | 1.97 |  |  | 1.39 | 0.86 | 0.44 |  |
|  | 5%-tile throughput loss in % | 35.10 | 22.03 | 12.45 |  | 8.57 |  |  | 5.90 | 3.78 | 1.70 |  |
| Vivo | Average throughput loss in % |  | 3.11 | 1.79 |  |  |  | 0.91 |  | 0.46 | 0.24 |  |
|  | 5%-tile throughput loss in % |  | 17.71 | 10.13 |  |  |  | 6.21 |  | 2.67 | 1.49 |  |
| Qualcomm | Average throughput loss in % | 6.4 | 3.8 | 2.2 |  |  |  | 1.2 |  | 0.7 | 0.4 |  |
|  | 5%-tile throughput loss in % | 34.5 | 20.4 | 10 |  |  |  | 5.4 |  | 2.5 | 1.5 |  |
| Ericsson | Average throughput loss in % |  |  |  | 0.0134 | 0.0121 | 0.01 | 0.009 | 0.008 |  |  |  |
|  | 5%-tile throughput loss in % |  |  |  | 8 | 7 | 6.24 | 5.25 | 4.6 |  |  |  |
| Huawei | Average throughput loss in % | 5.63 | 3.29 | 1.83 |  |  |  | 0.97 |  | 0.49 | 0.23 |  |
|  | 5%-tile throughput loss in % | 25.58 | 16.08 | 9.49 |  |  |  | 5.24 |  | 2.64 | 1.21 |  |
| Samsung | Average throughput loss in % | 4 | 3 | 2 |  |  |  | 1 |  | 0 | 0 |  |
|  | 5%-tile throughput loss in % | 34 | 20 | 12 |  |  |  | 7 |  | 3 | 2 |  |

In addition, coexistence simulation results for 3 active UEs and Indoor Probability 0% are summarized in Table 6.1.3.1.1-3. and 6.1.3.1.1-4 below.

Table 6.1.3.1.1-3: DL throughput loss for 14800 – 15350 MHz [Relative ACIR with 3 active UEs]

| Company | Throughput loss | Relative ACIR offset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | -2 | -3 | -4 | -6 | -8 | -9 | -10 | -12 | -15 |
| Nokia | Average throughput loss in % (15 GHz) | 0.31 |  | 0.51 |  | 0.82 |  | 1.29 |  | 2.00 | 3.02 |
|  | 5%-tile throughput loss in % (15 GHz) | 0 |  | 0 |  | 2.11 |  | 2.11 |  | 2.99 | 4.86 |

Table 6.1.3.1.1-4: DL throughput loss for 14800 – 15350 MHz [Absolute ACIR with Indoor Probability 0%]

| Company | Throughput loss | Absolute ACIR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| Ericsson | Average throughput loss in % (15 GHz) | 0.015 | 0.014 | 0.0125 | 0.01 | 0.01 | 0.009 | 0.008 |
|  | 5%-tile throughput loss in % (15 GHz) | 11 | 10 | 8 | 7.5 | 7.2 | 6.7 | 5.5 |

Based on the above simulation results, the required DL ACIR for 14800 – 15350 MHz for urban macro scenarios are in summarized in Table 6.1.3.1.1-5. The range of ACIR is derived as 20-26 dB, and the range of 22-24 dB by excluding the highest/lowest.

Table 6.1.3.1.1-5: DL simulation results for 14800 – 15350 MHz

|  | ACIR (dB) |
| --- | --- |
| Company | Urban macro |
| Nokia | -12 dB (relative) |
| Apple | 28-30 |
| CATT | 24 |
| MediaTek | 23-24 |
| Vivo | 22 |
| Qualcomm | 22 |
| Ericsson | 20-24 |
| ZTE | -8 dB (relative) |
| Huawei | 21 |
| Samsung | 24-25 |
| [Average in range] | 20-26 |
| [Excluding the highest/lowest] | 22-24 |

##### 6.1.3.1.2	Uplink

For Urban macro scenarios, simulation results for UL throughput loss with baseline assumption ACLR = 30 dBc at UE and ACS = 45 dBc at BS are summarized in Table 6.1.3.1.2-1/2/3/4/5. The following results are based on the FR1 like UE with omni-directional antenna and no beamforming.

Table 6.1.3.1.2-1: UL throughput loss for 14800 – 15350 MHz [Relative ACIR value]

| Company | Throughput loss | Relative ACIR offset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | -2 | -3 | -4 | -6 | -8 | -9 | -10 | -12 | -15 |
| Nokia | Average throughput loss in % (15 GHz) | 0.07 |  | 0.12 |  | 0.20 |  | 0.30 |  | 0.46 | 0.68 |
|  | 5%-tile throughput loss in % (15 GHz) | 1.14 |  | 2.21 |  | 3.52 |  | 3.55 |  | 3.61 | 3.73 |
| ZTE | Average throughput loss in % (15 GHz) | 0.26 | 0.30 |  | 0.34 | 0.39 | 0.46 |  | 0.55 |  |  |
|  | 5%-tile throughput loss in % (15 GHz) | 6.36 | 7.20 |  | 8.95 | 9.10 | 9.36 |  | 10.98 |  |  |

Table 6.1.3.1.2-2: UL throughput loss for 14800 – 15350 MHz [Absolute ACIR value]

|  | ACIR (dB) | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 20 | 22 | 23 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Apple | Average throughput loss in % |  |  | 0.6 |  |  |  |  | 0.3 | 0.13 | 0.099 | 0.079 | 0.06 | 0.022 |
|  | 5%-tile throughput loss in % |  |  | 0.59 |  |  |  |  | 0.22 | 0.14 | 0.131 | 0.1 | 0.1 | 0.01 |
| CATT | Average throughput loss in % |  |  | 0.027 |  |  |  |  | 0.018 | 0.01 |  |  | 0.0055 | 0.0024 |
|  | 5%-tile throughput loss in % |  |  | 0.088 |  |  |  |  | 0.0032 | 0.0001 |  |  | 3.1e-5 | 9.7e-6 |
| MediaTek | Average throughput loss in % |  |  | 2.12 |  |  |  |  | 1.15 | 0.60 |  |  | 0.3 | 0.14 |
|  | 5%-tile throughput loss in % |  |  | 8.48 |  |  |  |  | 4.05 | 1.94 |  |  | 0.75 | 0.16 |
| Vivo | Average throughput loss in % |  |  | 0.80 |  |  |  |  | 0.38 | 0.18 |  |  | 0.08 | 0.04 |
|  | 5%-tile throughput loss in % |  |  | 5.77 |  |  |  |  | 2.82 | 0.93 |  |  | 0.14 | 0 |
| Qualcomm | Average throughput loss in % |  |  | 1.4 |  |  |  |  | 0.7 | 0.4 |  |  | 0.18 | 0.09 |
|  | 5%-tile throughput loss in % |  |  | 7.4 |  |  |  |  | 2.8 | 1.8 |  |  | 0.7 | 0.4 |
| Ericsson | Average throughput loss in % | 0.0088 | 0.0078 | 0.0068 | 0.006 | 0.0052 | 0.0046 | 0.004 |  |  |  |  |  |  |
|  | 5%-tile throughput loss in % | 6 | 5.66 | 5 | 4.6 | 4.22 | 3.6 | 3.3 |  |  |  |  |  |  |
| Huawei | Average throughput loss in % |  |  | 3.57 |  |  |  |  | 2.00 | 1.08 |  |  | 0.56 | 0.27 |
|  | 5%-tile throughput loss in % |  |  | 8.59 |  |  |  |  | 5.63 | 3.58 |  |  | 2.30 | 1.28 |

In addition, coexistence simulation results for 3 active UEs and Indoor Probability 0% are summarized in Table 6.1.3.1.2-3.and 6.1.3.1.2-4 below.

Table 6.1.3.1.2-3: UL throughput loss for 14800 – 15350 MHz [Relative ACIR with 3 active UEs]

| Company | Throughput loss | Relative ACIR offset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | -2 | -3 | -4 | -6 | -8 | -9 | -10 | -12 | -15 |
| Nokia | Average throughput loss in % (15 GHz) | 0.23 |  | 0.33 |  | 0.48 |  | 0.72 |  | 1.08 | 1.63 |
|  | 5%-tile throughput loss in % (15 GHz) | 1.81 |  | 3.40 |  | 3.47 |  | 5.06 |  | 6.14 | 9.49 |

Table 6.1.3.1.2-4: UL throughput loss for 14800 – 15350 MHz [Absolute ACIR with Indoor Probability 0%]

| Company | Throughput loss | Absolute ACIR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| Ericsson | Average throughput loss in % (15 GHz) | 0.02 | 0.017 | 0.015 | 0.013 | 0.012 | 0.01 | 0.009 |
|  | 5%-tile throughput loss in % (15 GHz) | 12.52 | 11 | 10 | 8.2 | 7.2 | 6 | 5 |

Based on the above simulation results, the required UL ACIR for 14800 – 15350 MHz for urban macro scenarios are in summarized in Table 6.1.3.1.2-5. Many results suggest ACIR around e.g. 13-15dB excluding highest and lowest values.

Table 6.1.3.1.2-5: UL simulation results for 14800 – 15350 MHz

|  | ACIR (dB) |
| --- | --- |
| Company | Urban macro |
| Nokia | -9 (relative) |
| Apple | 22-24 |
| CATT | 20 |
| MediaTek | 15-17 |
| Vivo | 12 |
| Qualcomm | 13 |
| Ericsson | 11-14 |
| ZTE | -10 (relative) |
| Huawei | 18 |

<End of Change 5>

<Start of Change 6>

## 6.2	General parameters

### 6.2.1	Duplex mode

There is no defined 3GPP band for the 14800 - 15350 MHz frequency range. Even though FDD is not precluded, TDD is assumed as a baseline duplexing in this frequency range. SBFD can be a candidate duplexing method for this frequency range. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in the respective Work Item description.

### 6.2.2	Channel Bandwidth

While a number of channel bandwidth will be specified for this frequency range, 200 MHz is considered as a typical channel bandwidth. Other channel bandwidths compared to 200MHz are not precluded for this range.

<End of Change 6>

<Start of Change 7>

### 6.5.2	UE antenna characteristics

#### 6.5.2.1	General considerations

One of the key issues for this range is the UE RF architecture. While existing FR1 system parameters are defined for frequencies up to 7.1GHz, same FR1 principles are already assumed for e.g. 7.1-8.4GHz, which is however below the considered range of 14.8-15.3GHz. At the same time, the lower bound for the FR2 starts at 24.25GHz, which is notably higher than 15.3GHz. A choice for a particular UE RF architecture at these frequency ranges should consider implementation feasibility, e.g. physical constraints in case of handheld devices, and anticipated performance, such as coverage. The capabilities of PA technology in the FR2 frequency range motivated 3GPP to consider a UE architecture which relies on panels performing analog beam-forming, which was also considered as a feasible option from the handheld device implementation perspective. In contrast, panels and analog beam-forming are not feasible for lower FR1 frequencies.

In the context of the UE RF architecture, better UL coverage at FR2 frequencies was one of the key driving factors to consider the UE RF design with panels performing analog beam-forming. An FR2-like array antenna system incorporates analog beamforming into the array antenna to increase the coherent EIRP gain, thus helping to reduce the path loss between the UE and the base station. Furthermore, the FR2 array antenna, LNAs, PAs, beamformers, and PMIC are integrated into a single package, small form factor of which was also considered as a feasible option from the handheld device implementation perspective. As for the frequencies around 10-15GHz, previous work studied feasibility of FR2 principles in hand-held devices operating in that frequency range. One of the main challenges is the size of potential FR2 panel and, as noted in previous work, "… we can see that the antenna module doubles in size going from 28 GHz to 20 GHz and further doubles at 14 GHz". Figure 6.5.2.1-1 presents an example of the patch width and ground plane extension based on the microstrip patch array design assuming four elements with half-wavelength spacing.

Figure 6.5.2.1-1: Patch array dimensions vs. frequency

For more detailed information on the potential array antenna size operating at the considered frequency of 15GHz, Figure 6.5.2.1-2 below presents several example cases with different antenna arrangements accounting for the typical spacing between elements of the array antenna of half wavelength. As can be seen from the figure for a typical inter-element antenna spacing, the total antenna array size becomes much larger comparing to the FR2 sizes.

Figure 6.5.2.1-2: Estimated antenna size at 15GHz

Figure 6.5.2.1-3 below illustrates an example of a survey of the internal area used by four different commercially available 5G devices for FR1 (based on individual company contributions in RAN4).  The FR2 area is calculated based a single 4x1 reference design, the configuration used as the basis for current FR2 spherical coverage requirements.  Additionally, an estimate of the required 12-15GHz antenna array area is provided for comparison, similarly based on a 4x1 microstrip patch array with half-wavelength spacing. So, assuming dedicated apertures, an estimated 4x1 array at 12-15 GHz would represent an additional area growth on the order of 30-70% relative to antenna area usage currently needed to support FR1.

Figure 6.5.2.1-3: A survey of internal area usage for FR1, FR2 and estimated 13-15GHz antennas

In addition, support of analog beamforming in 15GHz might impose a substantial growth and volume requirements dedicated for cellular antennas.  Furthermore, the nature of such an array might constrain the dimensions leading to implementation challenges in densely integrated consumer devices that can be of different form factors. For instance, Figure 6.5.2.1-4 shows an example of the PCB of a foldable phone, from which one can see that it might be challenging to mount single patch antennas on four edges or the backside of the smartphone considering the larger array size and the already-packed smartphone space. In other words, it can be difficult for a hand-held to assume that there will be extra space in all required physical dimensions making it challenging, at least from the packing perspective, to put FR2-like panels supporting the 15GHz range.

Figure 6.5.2.1-4: Main PCB of a foldable mobile phone.

<End of Change 7>

<Start of Change 8>

# 7	Additional information on AAS

## 7.1	Array antenna model

### 7.1.1	Overview

A parameterized array antenna model has been developed over time in 3GPP. The model has been used for numerous studies in RAN4, including AAS, NR, HST, IAB, etc. The model has been adopted in other forums outside RAN4 and is also described in RAN1 and in ITU-R in recommendation M.2101.[28].

The model is defined around a set of equations, which rely on a set of input parameters to describe the array antenna. At a high level the model can be described as shown in Figure 7.1.1-1.

Figure 7.1.1-1: Array antenna model overview

Parameters can be divided into different categories:

-	General parameters, which are parameters that will be required for the simulator (spatial angles, considered wavelength).

-	Element parameters used to model the radiating elements.

-	Sub-array parameters used to model the sub-array.

-	Array parameters used to model the array.

The model will produce gain normalized radiation pattern for given q, j angles defined in the range 0≤θ≤180 degrees and -180≤φ≤180 degrees.

The wavelength, ld is related to the design of the array and is fixed by design, while the excitation wavelength, le may vary as function of considered frequency within a specific operating band.

### 7.1.2	Parameters

The input parameters required to describe the antenna is summarized and described in Table 7.1.2-1.

Table 7.1.2-1: Model input parameters

| Category | Parameter | Description | Note |
| --- | --- | --- | --- |
| General | ld | Design wavelength for array antenna in meters | The wavelength is fixed and selected for a given design frequency. The wavelength will not vary within a given operating band for a given design. |
|  | le | Array excitation wavelength in meters | This wavelength varies within the considered band. |
|  |  | Vertical angle in degrees |  |
|  |  | Horizontal angle in degrees |  |
|  | Ptx | Total conducted power in dBm | For a dual polarized antenna, the total conducted power is calculated over MxNx2 ports |
|  | mech | BS mechanical down-tilt angle in degrees | The mechanical down-tilt angle is a deployment parameter selected to maximize coverage within a given coverage area. The mechanical tilt can be applied as a coordinate transformation as described in TR 36.814, subclause A.2.1.6.2. |
| Element | Am | Element front-to-back ratio in dB |  |
|  | SLAv | Element side-lobe suppression in dB |  |
|  | 3dB | Element horizontal half power beamwidth in degrees |  |
|  | 3dB | Element vertical half power beamwidth in degrees |  |
|  | GE,max | Element peak gain in dBi | This parameter is related to the selection of 3dB, 3dB and LE, where GE,max=DE,max-LE and DE,max is related to 3dB, 3dB and array lattice unit area for the element. |
|  | LE | Element loss in dB |  |
| Sub-array | Msub | Number of element rows in sub-array |  |
|  | dv,sub | Vertical element separation in sub-array in meters |  |
|  | subtilt | Electrical pre-set sub-array down-tilt angle in degrees | The pre-set sub array down-tilt is a fixed design parameter for a base station. It is envisaged as a passive fixed (non-varying) electrical tilt within the sub-array elements. The deployment configuration (including mechanical tilt) for a base station is dependent on the environment. Thus, a same base station with fixed pre-set subarray tilt could also be used across different environments. The value of the pre-set tilt is determined based on the intended deployment scenarios and targeted user distribution and coverage range. |
| Array | M | Number of sub-array rows in array |  |
|  | N | Number of sub-array columns in array |  |
|  | dh | Horizontal sub-array separation in array in meters |  |
|  | dv | Vertical sub-array separation in array in meters | This parameter is related to Msub and dv,sub. |
|  | etilt | Electrical beam direction down-tilt angle in degrees |  |
|  | escan | Electrical beam direction scan angle in degrees |  |
|  | r | Array excitation correlation factor | For wanted signal ρ=1, while for unwanted emission the correlation falls of as function of frequency offset from carrier within the interval 0≤ρ<1. |

The total conducted power, Ptx at the Transceiver Array Boundary (TAB) is related to Total Radiated Power (TRP) in logarithmical scale as:

Ptx=10log102MN+Pn

TRP=Ptx-LE

Where, Pn is the power per transmitter branch in dBm. Total conducted power is defined as the total power for all ports, including two orthogonal polarizations.

The equivalent isotropic radiated power (EIRP) is calculated in dBm using the model as:

EIRPθ,φ=Ptx+AAθ,φ

The peak EIRP can also be derived from parameters as:

EIRP0=Ptx+GE,max+10log10Msub+10log10MN

When co-existence is evaluated in RAN4, the focus is on the main beam pointing towards the UE, but for other compatibility scenarios, e.g., with other services, the focus is not necessarily just the main beam. Spatial regions outside the main beam may also be highly relevant. Since the sub-array topology will affect the radiating characteristics in the sidelobe region, the model needs to be extended to provide the ability to model the sidelobe region characteristics correctly with reasonable complexity.

For the case where single element array geometries is considered (Msub=1), the extended array model collapses to the original model described in former 3GPP studies.

The antenna element separation is an important parameter that must be selected with care. Obviously, the value is static for a specific base station design, whereas the compatibility analysis may need to cover an entire frequency band. Typically, the antenna is designed to support given element separations for the highest frequency to avoid grating lobes for lower frequencies, but other design principles can be used for base stations supporting multiple bands or very wide operating bands.

Some parameters, such as the element beamwidths are related to the peak element gain via the element loss. This means that these parameters must be selected carefully.

Considering base stations are optimized for various factors including performance, cost, and coverage, it is expected that sub array configurations are relevant for these bands as well as a set of physical antenna elements are combined to form a logical element. The model comprises of a basic element pattern which is then combined appropriately based on the equations to form the sub array pattern and the composite pattern. Since dual polarized elements are used in typical base stations, it is sufficient to model each polarization separately as considered in the specific model. The models are selected so that they are simple and representative to model base station performance with sufficient confidence. The element pattern is based on a simple gaussian beam which has a flat sidelobe level. The Gaussian pattern is sufficiently wide and cover most of the regions of interest, especially in the elevation domain. At high elevation angles, the flat sidelobe level is sufficient to model the side lobes of the antenna element which are significantly lower than the main beam. Thus, the proposed extended antenna model to model sub arrays is sufficient to model the beamforming capability of IMT base stations in considered frequency ranges.

Another aspect also to consider is the performance and coexistence simulator complexity. The antenna model provides is reasonable complex and produce a gain normalized radiation pattern. If parameters are selected properly, no additional directory normalization is required which will reduce complexity and save processing capacity and simulation time.

The antenna model has support to model the array response outside the wanted carrier bandwidth, within adjacent channels using the correlation factor, r. When the wanted signal is considered r is equal to 1. For unwanted emissions outside the wanted signal bandwidth values within the range 0≤ρ<1can be considered. Further details on the frequency response of r require further investigations.

<End of Change 8>

<Start of Change 9

### 7.3.2	Adjacent channel modelling

#### 7.3.2.1	Correlation roll-off based model

##### 7.3.2.1.1	Modelling overview

The array antenna model adopted by 3GPP in previous studies creates the composite pattern as a multiplication between the element factor (or sub-array element factor) and array factor. The array antenna model was first introduced in TR 37.840 [26] in the early days of AAS for BS. Based on the antenna model, the average radiation pattern produced by an array antenna was evaluated and an analytical expression for the average radiation pattern was established.

The composite radiation gain pattern for pure LOS channel can be expressed in logarithmical scale as:

AAθ,φ=Asubθ,φ+10log101+ρm=1Mn=1Nwm,nvm,n2-1		(Eq. 7.4.1-1)

, where r is the correlation factor defined in the interval 0≤ρ≤1. Relevant parameter values for the frequency range 4400 to 4800 MHz can be found in subclause 4.4.1.2 and for 7125 to 8400 MHz in subclause 5.4.1.2, and for 14800 to 15350 MHz in subclause 6.5.1.2.

It can be noticed that the average antenna gain for the wanted signal where r is equal to 1, the composite array gain is produced (GE,max + 10log10(Msub) + 10log10(MN) dBi), while for unwanted emission away from the carrier where r is equal to 0 only the gain of the element/sub-array is produced (GE,max + 10log10(Msub) dBi). For sharing studies, it would be relevant to consider r being represented as a function of frequency.

The impact of array signal correlation to directivity and gain response have been evaluated for different array antenna structures considered for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz considering 10 degrees down-tilt beam steering direction. The results are plotted in Figure 7.3.2.1.1-1.

Figure 7.3.2.1.1-1: Vertical radiation pattern considering different array geometries and correlation factor.

The array factor impact due decorrelation can be analysed for all antenna geometries separately. The array factor drops gradually for low correlation as shown in Figure 7.3.2.1.1-2.

Figure 7.3.2.1.1-2: Array factor decorrelation characteristics plotted as function of correlation.

It can be noticed that the average gain drops gradually when the array correlation reduces. At array correlation 0.5 the array factor has reduced approximately 3 dB.

The characteristic is visualized in Figure 7.3.2.1.1-2 can be used to translate measured directivity drop to correlation factors to use for modelling purposes.

<End of Change 9>