3GPP TSG-RAN WG4 Meeting #114-bis									                    R4-2505214

Wuhan, China, 7th-11th April, 2025

Agenda item:			6.3.3

Source:	Skyworks, (…)

Title:	WF on handling optional channel bandwidth across releases

Document for:	Approval

# 1		Introduction

This WF captures agreements for how to handle CBW which change from “optional support” in Release N to “mandatory support” in Release N+1 based on proposals [1].

This change may introduce new smallest and new greatest CBW. For inter-band CA band combination specified with BCS4/5, new MSDs test points may be needed, or legacy MSD test points may need to be updated. Some rules were agreed on how to handle such changes for 1UL MSD test point in [2] but were not discussed for MSD due to dual UL IMD interference. Hence this WF proposes rules on MSD test point management for 2UL IMD.

# 2		Background

It is some companies’  understanding that channel bandwidth (CBW) which are specified as “optional support” in Rel-18 using Note 4 and Note 12 in Table 5.3.5-1 are supposed to change “automatically” to “mandatory support” in Release-19. Yet in Release 19.1.0, all optional CBW inherited from Release 18 remain optional.

It should be noted that changing optional CBW to mandatory support may introduce new greatest and new smallest CBW as highlighted in yellow and blue respectively in Table 1 below [1] and listed below:

New greatest “mandatory” CBW:

-	100MHz CBW for Band n46, n96, n102

-	35MHz CBW for Band n71,

New smallest “mandatory” CBW:

-	3MHz for Bands n26, n28, n31, n85, n100,

-	5MHz for Bands n41 and n90.

Table 1: Rel-18 optional CBW ported as in Rel 19.0.0 Table 5.3.5-1.

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n26 | 15 | 34 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |  |
| n28 | 15 | 34 | 5 | 10 | 15 | 207 | 257 | 307 |  | 404,7 |  |  |  |  |  |  |  |
| n31 | 15 | 34 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n41 | 15 |  | 54,11 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |
| n46 | 30 |  |  | 105 |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 1004 |
|  | 60 |  |  | 105 |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 1004 |
| n71 | 15 |  | 5 | 10 | 15 | 20 | 2512 | 3012 | 3512 |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 2512 | 3012 | 3512 |  |  |  |  |  |  |  |  |
| n72 | 15 | 34 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n7910 | 30 |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 704 | 80 | 90 | 100 |
|  | 60 |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 704 | 80 | 90 | 100 |
| n85 | 15 | 34 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
| n90 | 15 |  | 54 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |
| n96 | 30 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 1004 |
|  | 60 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 1004 |
| n100 | 15 | 34 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n102 | 30 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 1004 |
|  | 60 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 1004 |
| n106 | 15 | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 11: Not all frequency positions of 5 MHz carriers are possible due limitations of the SSB position relative to the 5 MHz channels. 5 MHz channels with Fc such that 2499+N*1.2 ≤Fc<2499.3+N*1.2MHz for 0≤N<157 are not compatible with SSB positions and cannot be used for 5 MHz n41. NOTE 12: This UE channel Bandwidth is optional for uplink in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Introducing new smallest/greatest CBW impacts MSD requirements for inter-band CA band combinations specified with support of BCS4/5.

# 3		WF#1 on handling optional CBW across Releases

Agreement 1:

- Change across Releases from “optional support” to “mandatory support” is evaluated on a case-by-case basis. Interested companies are invited to share their views on the following options for the following CBW:

- 3MHz CBW:

  - option 1: remains optional across Releases for all bands except for band n106 where mandatory support is kept across Releases,

  - option 2: changes to mandatory support in Release 19, but this CBW is not accounted for MSD analyses.

- 5MHz CBW for band n41/n90:

  - option 1: remains optional across Releases

  - option 2: changes to mandatory support in Release 19

  - option 3: remove this CBW from band n41/n90.

- 35MHz CBW for band n71

  - Option 1: UL remains optional across releases

  - Option 2: changes UL to mandatory support in Release 19

Agreement 2:

It is agreed that 100MHz CBW for band n46, n96 and n102 becomes mandatory support in Release 19.

# 2			WF#2 on MSD test point maintenance for BCS4/5 due to new smallest/greatest CBW

Agreement:

- Impact on 1UL MSD test points: agreements from [2] and captured in RAN4 PRD apply.

- Impact on 2UL MSD test points: At the exception of 3MHz, and [7MHz] CBW, an MSD test point is needed when a new smallest CBW is enabled by BCS4/5 even if CBW is optional support in current Release.

# 2			WF#3 on MSD evaluation

Agreement: interested companies are invited to evaluate the following MSDs [1]

CA_n5A-n71A:

- Consider changing 1st test point to 2nd optional test point based on [2],

- Evaluate the following band n5 MSD requirements,

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | [FFS] | >ACLR2 |

CA_n71A-n85A:

PC3: Consider voiding footnotes 6 and 7 in Table 7.3A.6-1.

PC2: Consider voiding footnotes 4 and 5 in Table 7.3A.6-1a-1	.

CA_n77A-n102A, CA_n78A-n102A:  Evaluate the following PC3 cross-band isolation MSD requirements.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n102 | n77 | 5975 | 100 | 30 | 270 (RBstart=0) | 4195 | 10 | [FFS] | >ACLR2 |
| n102 | n78 | 5975 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | [FFS] | >ACLR2 |

CA_n3A-n74A, CA_n3A-n75A:

- Consider the following change of 1st test point to 2nd optional test point based on [2].

- Evaluate the following band n74 and band n75 MSD requirements,

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n74 | 1730 | 40 | 15 | 50 (RBstart=0) | 1515.5 | 5 | [FFS] | >ACLR2 |
| n3 | n75 | 1730 | 40 | 15 | 50 (RBstart=0) | 1514.5 | 5 | [FFS] | >ACLR2 |

# 3			References

- R4-2503240 Cross-band isolation exception cases and corrections, 3GPP TSG-RAN WG4 Meeting # 114b, Wuhan, China, Skyworks Solutions, Inc.

- R4-2310321 WF on guidelines on checks/specification updates required when BCS4/5 introduces a new smallest or largest CBW, 3GPP TSG-RAN WG4 Meeting # 107, Incheon, South Korea, Skyworks Solutions, Inc.