3GPP TSG-RAN WG4 Meeting #114-bis									                    R4-2505215

Wuhan, China, 7th-11th April 2025

Agenda item:			6.2.2

Source:	Skyworks, Apple, (…)

Title:	WF on intra-band ENDC configurations

Document for:	Approval

# 1		Introduction

This WF captures agreements to clarify the meaning of the E-UTRA “N/A” configurations specified for the intra-band contiguous MSD test points [1].

# 2		Background

Table 1 shows the Rel-19 MSD test points (1UL and 2UL), and the corresponding restrictions on single switched uplink allowed. For 1UL, the MSD is specified for the LTE anchor PCG due to transmission in the NR SCG, and hence it was agreed that the LTE uplink configurations are specified as “N/A” to indicate that MSD is specified only due to the NR band transmitter chain non-linearities.

Some concerns were raised that these “N/As” may be problematic for conformance since the throughput measurements require an LTE uplink connectivity.

During this meeting, one company commented that band combinations which are restricted to “only single switched uplink operation” may not be problematic. Theses cases are highlighted in blue in Table 1.

However, the ambiguity remains for band combinations where dual-uplink transmissions are allowed, as highlighted in yellow in Table 1.

Table 1: Intra-band contiguous EN-DC MSD test points - Table 7.3B.2.1-1

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode | Single UL allowed |
| DC_(n)3AA | 3 | N/A | 15 | N/A | 1842.5 | 6.2 | FDD | Only single switched UL is supported |
|  | n3 | 1770.0 | 30 | 10 (RBstart = 150) | 1865.0 | N/A |  |  |
| DC_(n)5AA | 5 | N/A | 5 | N/A | 871.5 | 5.2 | FDD | Only single switched UL is supported |
|  | n5 | 839 | 20 | 20 (RBend = 105) | 884 | N/A |  |  |
| DC_(n)12AA | 12 | N/A | 5 | N/A | 733.5 | 4.5 | FDD | Only single switched UL is supported |
|  | n12 | 711 | 10 | 20 (RBend = 51) | 741 | N/A |  |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 | FDD |  |
|  | n71 | 678 | 10 | 10 (RBend = 51) | 632 | 29.4 |  |  |
| DC_(n)71AA4 | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |  |
|  | n71 | 678 | 101 | 10 (RBend = 51) | 634.5 | 29.1 |  |  |
| DC_(n)71AA3 | 71 | N/A | 10 | N/A | 642.0 | 21.3 |  |  |
|  | n71 | 673.0 | 20 | 5 (RBstart = 0) | 627.0 | N/A |  | No |
| DC_(n)71AA4 | 71 | N/A | 5 | N/A | 639.5 | 6.8 |  |  |
|  | n71 | 670.5 | 151 | 5 (RBstart = 2) | 627.0 | N/A |  |  |
| DC_(n)71AA5 | 71 | N/A | 5 | N/A | 634.5 | 6.4 |  |  |
|  | n71 | 670.5 | 15 | 5 (RBstart = 0) | 624.5 | N/A |  |  |

This WF aims at clarifying the meaning of “N/A” to RAN5.

# 3		WF on intra-band ENDC configurations

Agreement:

RAN4 to send an LS to RAN5 clarifying the meaning of the E-UTRA “N/A” configurations specified in Table 7.3B.2.1-1. It is proposed to share the following recommendations [1], other views are not precluded:

For 1UL MSD intra-band ENDC MSD test points specified in Table 7.3B.2.1-1 where the E-UTRA configurations are specified as “N/A”:

- E-UTRA uplink carrier frequency “Fc (UL)”: RAN4 recommends configuring the E-UTRA Fc (UL) according to the E-UTRA band’s default Tx-Rx frequency separation,

- E-UTRA uplink RB allocation “Lcrb”: RAN4 recommends configuring the E-UTRA UL RB allocation according to the UL configuration specified for the E-UTRA band’s REFSENS of the corresponding DL CBW.

- E-UTRA transmission power: RAN4 recommends configuring the E-UTRA transmit power as anchor agnostic, or preferably at minimum output power.

# 3			References

- R4-2503242 Clarifications on valid 1UL E-UTRA configuration for EN-DC intra-band contiguous MSD, 3GPP TSG-RAN WG4 Meeting # 114b, Wuhan, China, Skyworks Solutions, Inc.