3GPP TSG-RAN WG4 Meeting #114bis	R4-2505216

Wuhan, Hubei, China, 7th – 11st April, 2025

Title:  WF for UL CA_n28-n71

Agenda Item:	6.1

Source: 	Huawei, HiSilicon, ….

Document for:	Approval

Background

Recently, some countries in the Middle East launched the 600MHz spectrum (band n71) for operators [1] [2]. As these operators own the legacy 700MHz network (B28/n28), there is a demand to enable the UL CA_n28-n71 feature. As everyone know, there are some difficulties for UE vendors to support the low-low band combination. In addition to that, UL CA_n28-n71 may face more difficulties, e.g. DL Sensitivity degradation for both bands due to intermodulation, 5MHz gap between two UL bands etc.

In this meeting, one contribution [3] provided some analysis for UL CA_n28-n71 based on three antenna architectures, but companies may need more time to check and contribute UL CA_n28-n71 based on other RF architectures.

In this way forward, we’d like to summarize the open issues and potential solutions for UL CA_n28-n71

Way forward

Proposal 1: RAN4 can investigate the following RF architectures in order to enable the UL CA_n28-n71.

Option 1: Three-antenna RF architecture below as an example. (NOTE: it isn’t precluded to implement Full band duplexer for band n28.)

Option 2: Two-antenna RF architecture with two triplexers.

Option 3: Other RF architectures are not precluded, e.g. studies in TR38.872 on CA_n28-n105.

Proposal 2: RAN4 can investigate the impacts on the following RF requirements in order to enable the UL CA_n28-n71.

- The attenuation between UL band n71 and UL band n28.

- MSD for both DL band n28 and n71 due to IMD3 and IMD5.

- Potential revisit on the existing MSD due to cross band isolation.

- Delta Tib and Rib

- MPR requirements based on the investigation whether intermodulation interference will excceed the SEM or spurious emission requirements.

- The band protection requirements of -50dBm/MHz.

NOTE: Whether to specify 2UL is based on the outcome of the studies.

Proposal 3: it’s encouraged for companies to evaluate the following 2nd test point MSD for DC_28_n71.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | 28 | 680.5 | 35 | 15 | 20 (RBstart=168) | 760.5 | 5 | FFS | >ACLR2 |

Reference

[1] https://www.gsma.com/connectivity-for-good/spectrum/saudi-arabia-leads-the-way/

[2] https://developingtelecoms.com/telecom-technology/wireless-networks/18302-e-uae-tests-6-ghz-and-600-mhz-bands-on-its-5g-network.html

[3] R4-2503687, TP for TR 38.719-02-01 on introduction of CA_n28A-n71A, Huawei, HiSilicon