3GPP TSG-RAN WG4 Meeting #114bis	R4-2505234

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.26.5

Source:	vivo

Title:	WF for [114bis][131] NR_LPWUS_UERF

Document for:	Approval

Introduction

This is WF for Rel-19 LP-WUS UE RF.

Topic #1: General and system parameters

Sub-topic 1-1 General and system parameters

Issue 1-1-1: Any RF impacts in RAN2 LS R2-2501394

Agreement:

- No RF impact corresponding to RAN2 LS R2-2501394

Issue 1-1-3: Operation bands for LP-WUS in TR 38.774

Agreement:

  - n47, n46 ,n96 and n102 should be removed for LP-WUS.

Topic #2: REFSENS, ASCS and ACS requirements

    - Sub-topic 2-1 FR1 SNR, NF and IM for REFSENS requirements

Issue 2-1-1: SNR for OOK-based LP-WUS

- Proposals

  - Proposal 1: Conclude SNR for OOK-based LR.  (Moderator)

| FR1 OOK-based receiver | SNR | RM coding | Length of information bits | ADC | CRC | Note | REFSENS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Huawei | -4dB for both | 8/16/32 | 5bit? | 4-bit | No | SNR: -2.15 dB (8 bit), -4.63 dB (16 bit), -5.94 dB (32 bit) |  |
| Sony | -7 to -3dB |  |  |  |  |  |  |
| Spread trum | -3 |  |  |  |  | SNR last meeting |  |
| vivo | -6.5dB | 16/20 | 5bit | 4-bit | No | SNR: -6.5dB for 16bit RM coding without Te or 20bit RM coding with Te |  |
| CATT | -3.3dB |  |  |  |  | SNR last meeting | -96.3/-93.3 |
| CMCC | -2.5dB |  |  |  |  | SNR last meeting |  |
| ZTE | -1.5dB | in TS38.212 Clause 5.3.3.3 | 5bit | 4-bit | No | SNR -1.5dB (without ML), -7.5dB (with ML) | 9dB higher than MR |
| Nokia | >RAN1 SNR |  |  |  |  | SNR last meeting |  |
| E/// | -1.5 dB |  |  |  |  | SNR last meeting |  |
| QC | -4.0 dB |  |  |  |  | SNR as side condition. Assume RAN1: NF + IM = 17.0 dB, and 14.0 dB? |  |
| LGE | -7 to 8 dB |  |  | 4bit |  | SNR last meeting |  |

Agreements:

- Companies are encouraged to provide the simulation results with the assumption of the RM coding of 8bit, 16bit and 32bit to align simulations in RAN4. It is targeted to conclude SNR value for envelope detection and OFDM sequence detection next RAN4 meeting.

Issue 2-1-5: How to map two LP-WUR types with UE capability

Agreement:

  - RAN4 define two LP-WUR types (Type 1 and Type 2), no need 1-to-1 mapping between waveform and LP-WUR type.

  - Requirement applicability should be clearly defined.

  - RAN4 need to further discuss how to design the RMC for each type, how to ensure the performance of each type properly.

Sub-topic 2-2 FR2 SNR, NF and IM for REFSENS requirements

Issue 2-2-2: Diversity gain value for FR2 LP-WUR

Agreement:

- [-3]dB diversity gain is assumed for FR2 LP-WUR REFSENS derivation

Issue 2-2-3: NF+IM for FR2 LP-WUR

Agreements:

  - RAN4 further discuss FR2 LPWUR, NF+IM value.

Issue 2-2-4: Spherical coverage for FR2 LP-WUR

Agreement:

- The coverage matching requires the same spherical coverage percentile of WUR and MR for different FR2 power classes.

- FFS on whether to skip the test of FR2 LP-WUR for spherical coverage.

Sub-topic 2-3 ASCS simulation and requirements

Issue 2-3-1: ASCS requirements for OOK-based receiver

Agreement:

- For Type1 UE requirements, which are derived based on the assumption that demodulation of OOK content of LP-WUS is targeted

  - 1 RB guard for ASCS and 0.5dB de-sens

- For Type2 UE requirements, which are derived based on the assumption that demodulation of OFDM content of LP-WUS is targeted

  - 0 RB guard for ASCS and 0 dB de-sens

- For Type 1 and Type2 UE are not limited to specific LP-WUS waveform.

- FFS how to define test case and requirement applicability for Type 1 and Type 2

  - FFS how type 1 and type 2 capability related to RAN1 UE feature

- A UE can only support either Type 1 or Type 2 but not both for LP-WUR

Issue 2-3-3: Channel bandwidth for ASCS Test case

Agreements:

  - ASCS requirement should be applied to all supported UE CBW. Conformance test case and potential down-selection will be defined by RAN5.

  - For conformance testing, it is expected that not all CBW need to be tested.

Issue 2-3-4: whether high in-band signal (both LP-WUS and NR) case is considered for alternative ASCS requirements

Agreements:

  - FFS For ASCS, whether additional test case that the wanted signal is 14dB higher than LP-WUS REFSENS, i.e., REFSENSLP_WUR + 14 dBm, should be considered or not.

Sub-topic 2-4 ACS simulation and requirements

Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB for SET-1 (worse NF) LP-WUR

- Proposals

  - Proposal 1: check the following input from companies and converge the range for set-1 LR.  (Moderator)

|  | vivo | CMCC | ZTE | Huawei | Nokia | E/// | Sony | QC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACS (dB) | 28 | 22-24 | 24 | 11 (Note1) | 22 | Note 2 | Higher wanted signal (Note3) | Higher wanted signal Note4 |
| Guard RB (as test parameter, not requirements) | 1RB | 1RB | 1RB | Less RB | 2RBs |  |  |  |

Note 1: For LP-WUR type aimed at moderate power consumption, the guard RBs could be derived with better phase noise profile and the interferer level could similar to that of MR.

Note 2: The same set of the inteference signal and wanted signal defined for both two set of the LP-WUS requirments. Further discuss the number of guard RB for two set of requirment.

Note 3: A higher wanted signal can be specified for OOK type LP-WUR ACS requirement to balance the power consumption and interference mitigation capability.

Note 4: LPWUS ACS is specified as (assuming higher wanted signal is higher than refsens by 14 dB). DgMR is the diversity gain used for legacy PDSCH REFSENS derivation
ACSLPWUS=  NFlegacy-NFLPWUS+10logCBWBWLPWUS+ACSlegacy-DgMR

  - Proposal 2: Whether consider a higher the wanted signal for set-1 ACS requirements, e.g., LP REFSENS+ >14dB.  (Moderator)

WF:

- Encourage companies to update the simulation analysis based on latest agreements. Target to conclude ACS requirements and test parameters next meeting.

Issue 2-4-2: Whether adopt the same wanted power level for Set-2 LR?

Agreement:

- The same set of the interference signal level defined for both two types of the LP-WUS requirements.

Issue 2-4-4: Case 1 ACS test parameter

Agreements:

Specify case 1 and case 2 ACS test parameters for Type 1 and Type 2.

  - Define interferer as BWinterferer of 5 MHz.

Topic #3: Other RF requirements

Sub-topic 3-1 IBB, OBB and Intermodulation requirements for UE RF

Issue 3-1-1: IBB, OBB requirements

Agreements:

- Same interference level as MR. further discuss and decide next meeting on relaxed wanted signal.

Topic #4: Testability issues

Sub-topic 4-1 Testability for UE RF requirements

Issue 4-1-1: Whether LR can be tested directly

Agreements:

- Down select below two approaches for LP-WUR UE RF requirements:

- Option 1: stick to 1% MDR LP-WUS as performance metric, but with a note in TS that LP-WUR ON is not mandated.

  - The note depends on RAN1 conclusion

- Option 2: update it to 1% MDR of PDCCH triggered by LP-WUR

Issue 4-1-4: UE test mode for LP-WUR

Agreements:

- Test mode design as well as the details of test mode for LP-WUS verification can be left to RAN5 with RAN4 inputs.

Topic #5: CR work splitting for UE RF (for approval)

Issue 5-1-1: CR work splitting

TS 38.101-1

| Requirements | Title | Leading company |
| --- | --- | --- |
| General aspects | Clause 2, References Clause 3, Definitions, symbols and abbreviations Clause 4, General | vivo |
| Operation band | 5.2M Operating bands for LP-WUS/WUR | CATT |
| Bandwith (RBs) | 5.3A UE channel bandwidth for LP-WUS/WUR | CATT |
| REFSENS | 7.3M Reference sensitivity for LP-WUS/WUR General REFSENS | Nokia, Huawei |
| Max input level | 7.4M Maximum input level for LP-WUS/WUR | E/// |
| ACS and ASCS | 7.5M Adjacent channel and subcarrier selectivity for LP-WUS/WUR ACS ASCS | QC, E///, Nokia, Huawei |
| Blocking | 7.6M Blocking characteristics for LP-WUS/WUR In-band blocking Out-of-band blocking Narrow band blocking | Huawei |
| Spurious response | 7.7M Spurious response for LP-WUS/WUR | ZTE |
| Intermodulation | 7.8M Intermodulation characteristics | ZTE, CMCC |
| Spurious emissions | 7.9M Spurious emissions for LP-WUS/WUR | ZTE, OPPO |
| Annex A Measurement channels | DL reference measurement channels | Nokia |

TR 38.774 TP owners for detailed sub-clauses

| Requirements | Title | Leading company |
| --- | --- | --- |
| General aspects |  | vivo |
| System parameters | Operation band Channel raster | CATT |
| Clause 6 Simulations (including typical simulation results and observations) | 6.3.1 General 6.3.2 SNR simulations | vivo |
|  | 6.3.3 ASCS simulations | vivo |
|  | 6.3.4 ACS simulations | vivo |
|  | 6.3.5 other aspects | vivo |
| RF requirements (including RAN4 technically discussions/considerations and final conclusion) | 7.1.1 General 7.1.2 Rx SNR evaluations | Nokia, Huawei |
|  | 7.1.3 Architecture and NF considerations | Nokia, Huawei |
|  | 7.1.4 REFSENS requirements | Nokia, Huawei |
|  | 7.1.5 ASCS requirements | QC, E///, Nokia, Huawei |
|  | 7.1.6 ACS requirements | QC, E///, Nokia, Huawei |
|  | 7.1.7 IBB requirements | Huawei |
|  | 7.1.8 OBB requirements | Huawei |
|  | 7.1.9 other Rx requirements | ZTE, CMCC |
| Clause 8 Testability | 8.1.1 General Framework | Nokia, Samsung |
|  | 8.1.2 other | Nokia, Samsung |
| Others? | Other clause needed ? | vivo |

TS 38.101-2 [due to the progress, will be discussed next meeting]

| Requirements | Title | Leading company |
| --- | --- | --- |