3GPP TSG- WG4 Meeting # 115	R4-2505321

# St Julian’s, Malta, 19th – 25th May 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on HPUE MSD

Agenda Item:	7.1.2.1.1

Document for:	Approval

Introduction

In RAN4#114bis, it was agreed in the WF [1] that for 1 band 1UL CC MSD, to specify three LUT columns with MSDmax of 3, 6 and 9dB respectively, with companion table mapping the HPUE MSD type to MSDmax.

| MSD type |  | MSDmax (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3 | 6 | 9 |
| 1UL | UL harmonic | PC21Tx | PC22Tx | [PC1.52Tx] |
|  | Harmonic mixing | PC21Tx | PC22Tx | PC1.52Tx |
|  | Cross-band isolation | PC21Tx | PC22Tx | PC1.52Tx |

For 2 band 2UL CC HPUE MSD IMD, it was agreed that

- 2UL HPUE MSD requirements can be mapped to a separate LUT with maximum of [8] columns, e.g. at least with ΔMSDmax = [3], 6, 9, 12, 15, [18, 24, 30] dB. - Other values are not precluded and the need for additional columns is FFS. - FFS on optimized mapping to avoid penalizing 2UL combos with too high ΔMSDmax.

This paper is to provide further thinking concerning the HPUE MSD rules in Rel-19.

Simplification for 1 band 1UL CC HPUE MSD

One example of LUT columns for 1 band 1UL CC HPUE MSD was captured in [1] as follows.

| PC31Tx MSD (dB) | MSDmax (dB) |  |  |
| --- | --- | --- | --- |
|  | 3 | 6 | 9 |
| 0.1≤ ≤0.3 | 0.3 | 0.4 | 1.0 |
| 0.4≤ ≤0.6 | 0.5 | 0.8 | 1.8 |
| 0.7≤ ≤0.9 | 0.7 | 1.1 | 2.5 |
| 1.0≤ ≤1.9 | 1.2 | 2.1 | 4.1 |
| 2.0≤ ≤2.9 | 1.6 | 2.7 | 5.1 |
| 3.0≤ ≤3.9 | 1.8 | 3.3 | 5.9 |
| 4.0≤ - ≤4.9 | 2.0 | 3.8 | 6.5 |
| 5.0≤ - ≤6.9 | 2.3 | 4.5 | 7.3 |
| 7.0≤ - ≤9.0 | 2.5 | 5.0 | 7.9 |
| ≥10 | 3 | 6 | 9 |

However, for 1UL MSD, the granularity of PC31Tx MSD for the LUT columns are too fine. The gaps of values between the lines are too small as 0.2dB. And it is kind of over-designed for 1UL HPUE MSD LUT.

Observation 1: The gaps of values between different lines are too small as 0.2dB in the example LUT columns in WF R4-250514. And it is over-designed for 1 UL HPUE MSD simplification.

Coarser granularity as follows should be defined. After all, the LUTs may represent a tightening or relaxation of MSD requirements.

Table 1 Example for 1UL MSD LUT columns

| PC3 MSD (dB) | MSDmax (dB) |  |  |
| --- | --- | --- | --- |
|  | 3 | 6 | 9 |
| 0< <1.0 | 0.7 | 1.1 | 2.5 |
| 1.0≤ <4.0 | 1.8 | 3.3 | 5.9 |
| 4.0≤ <7.0 | 2.3 | 4.5 | 7.3 |
| ≥7 | 3 | 6 | 9 |

With the updated example LUT columns above, we can achieve fewer tightening MSD requirements.

Proposal 1: Coarser granularity of PC31Tx MSD for 1 UL HPUE MSD LUT columns should be defined. The values gap between adjacent lines should not be smaller than 0.5dB.

Simplification for HPUE IMD

As per conclusions in [1], it is companies’ consensus that 4 LUT columns with different ΔMSDmax should be defined for PC2 IMD with 4 different IMD orders: 6, 9, 12, 15dB mapped to IM2, IM3, IM4 and IM5 respectively. Similarly, the other 4 LUT columns can be defined for PC1.5 IMD. It is possible that the 4 values of ΔMSDmax for PC2 IMD can be partially overlapped with the 4 values of ΔMSDmax for PC1.5 IMD. How to determine the ΔMSDmax for PC1.5 IMD is the key point in this meeting for the issue.

Observation 2: The values of 6, 9, 12, 15dB are defined as ΔMSDmax in the LUT columns for PC2 IMD2/3/4/5.

Proposal 2: At most 4 values of ΔMSDmax should be defined in the LUT columns for PC1.5 IMD2/3/4/5.

The coefficient related to IMD order for each uplink component applies to the value of diplexer, antenna switch, duplexer for both main path and diversity path, and band A/B PA forward /reverse mixing, resulting in MSD value for the band combination. Compared to PC3 IMD calculation, the main difference lies in output power for the two uplink components with PC1.5 in one band combination. It is noted that for a band combination with two bands, it could be implemented either with 2Tx or 3Tx.

Table 7.3A.5-1: IMD for PC3 CA NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 Table 7.3A.5-1b: IMD for PC1.5 CA NOTE 1: This band combination is specified for inter-band UL CA with UL MIMO or Tx diversity capabilites, and the transmitter shall be set at min (+23 dBm, PCMAX_L,f,c) for the band with single Tx antenna connector as defined in clause 6.2A.4, and set at min (+27.8 dBm, PCMAX_L,f,c) for the band with two Tx antenna connectors as defined in clause 6.2H.3 or 6.2L.3.4.

- 23+27.8 is used as the power configuration in the 3Tx PC1.5 IMD calculation where band A is 23dBm and band B is 24.8dBm+24.8dBm with 2Tx

Using CA_n3-n78 as an example, for PC 1.5 IMD with 3Tx, according to the RF front end architecture in [6], compared to 2Tx PC2 MSD analysis, there is one more forward IMD and reverse IMD from the 2nd PA of band n78 in the other branch for 3Tx PC1.5. The isolation of antenna, PCB and diplexers, etc, between the two branch is quite large, and there is barely a little contribution from the 2nd PA to the PC1.5 MSD value. Therefore, only 24.8dBm output power from the band with 2Tx is considered.

Figure 1 IMD MSD for CA_n3-n78 with 1Tx or 2Tx @ n78

The ΔMSD between PC1.5 and PC2 does not exhibit a clear or consistent pattern based on the existing requirements. Unlike PC2, the MSD for PC1.5 may vary depending on whether 3Tx or 2Tx is used, primarily due to differences in PA configurations for the constituent bands. Additionally, particularly for high-order intermodulation distortion (IMD), such as IMD4, which can result from components like 2f2-2f1 or 3f2-f1, the positions of the IMD products and the PA output power are key factors that contribute to the significant differences in ΔMSD between PC1.5 and PC2.

In [2] it was also observed that more than one PA configuration may be considered for PC1.5 MSD, including 26dBm+26dBm and 23dBm+27.8dBm, etc. Different PA configurations can be reflected in various UE RF-FE architectures. Originally, companies calculate MSD values based on various UE architectures, resulting in different IMD values. Furthermore, there is no capability signalling designed to inform the network which PA configuration (UE architecture) the UE applies to the specific band combination. In this case, the PA configuration-agnostic IMD simplification way need to be considered. For each IMD order of PC1.5, the maximum or average value calculated based on 26dBm+26dBm and 23dBm+27.8dBm can be defined as the ΔMSDmax. In addition, for high-order IMD, the maximum value resulted from the positions/indexes of the IMD product should be considered as the ΔMSDmax.

Proposal 3: The ΔMSDmax for PC1.5 IMD should be defined de-coupled with PA configuration(s).

Proposal 4: For each IMD order of PC1.5, the following factor should be considered to define as the ΔMSDmax:

- the Maximum value resulted from the position/index of IMD product.

In the slides prepared by Skyworks in April meeting for 2UL MSD rules – Summary of worst-case (highest MSD) mapping, values are proposed by companies. Quote those for PC1.5 as follows.

Table 2 Quoted 2UL IMD MSD LUT for PC1.5

| MSD order |  | MSDmax (dB) [LUT] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 12 | 18 | 24 | 27 | 30 | 35 |
| 2UL | IMD2 | PC1.52Tx PA configuration: 26+26 ~11dB |  |  |  |  |  |
|  | IMD3 |  | PC1.52Tx PA configuration: 26+26 | PC1.53Tx ~19dB |  |  |  |
|  | IMD4 |  |  | PC1.52Tx PA configuration: 26+26 | PC1.52Tx PA configuration: 23+27.8 |  |  |
|  | IMD5 |  |  |  |  | PC1.52Tx PA configuration: 26+26 | PC1.52Tx PA configuration: 23+27.8 |

According to the value calculated based on the formula in Skyworks[3] and vivo[4], and the inputs from Apple[5] and Murata[8], the value of 12, 18 and 24dB could be considered as ΔMSDmax for IMD2/3/4 in PC1.5 MSD requirements. However, these proposed values exhibit a relatively larger deviation and are more relaxed compared to existing requirements. In [10], a simplified formula was proposed for PC1.5 in comparison to PC2, i.e.

MSDPC1.5, IMx = MSDPC2, IMx+ [1.5dB margin] + (N-1)*1.8, where N represents the IMD order.

Applying this formula yields rounded MSD values of 9, 15, 18, and 24 dB for IMD2/3/4/5 in PC1.5, respectively. It should be noted that these derived values demonstrate better accuracy when compared to current requirements, which can be observed in the Annex IMD tables.

Proposal 5: For PC1.5 MSD, the suggested ΔMSDmax values are 9, 15, 18, and 24 dB for IMD2/3/4/5 respectively.

Accordingly, the following “MSDmax” LUT companion table is proposed for PC2/PC1.5 2UL IMD MSD requirements.

Table 3 Proposed 2UL IMD MSD LUT for PC2 and PC1.5

| MSD order |  | MSDmax (dB) [LUT] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 6 | 9 | 15 | 18 | 24 |
| 2UL | IMD2 | PC2 |  | PC1.5 |  |  |  |
|  | IMD3 |  | PC2 |  | PC1.5 |  |  |
|  | IMD4 |  |  | PC2 |  | PC1.5 |  |
|  | IMD5 |  |  |  | PC2 |  | PC1.5 |

In [3], 3dB is proposed as a candidate ΔMSDmax for PC2 where the victim’s band achieves PC2 with 1Tx. The additional value is also based on the special PA configuration and should be removed.

Proposal 6: The candidate ΔMSDmax of 3dB for PC2 should be removed to decouple with PA configuration.

Besides, it was also discussed in the last meeting, whether unified LUT columns should be specified for 1UL and 2UL HPUE MSD. The advantage is obvious that both ΔMSD and ΔMSDmax can be reused for all types of HPUE MSD, especially for those ΔMSDmax values shared by PC2 and PC1.5, e.g 9dB and 15dB in Table 3. However, given that only 3, 6 and 9dB of ΔMSDmax are defined for 1UL HPUE MSD, the ΔMSD values are too small compared to that for 2UL HPUE MSD(~ 24dB for 2UL). The granularity for ΔMSD in 2UL is far too fine for 1UL. Therefore, it is more proper to define separate LUT columns for 1UL and 2UL HPUE MSD.

For 1UL HPUE MSD simplification, Table 1 would be the reference. The LUT columns for 2UL IMD simplification, [9] provide an example with the granularity accommodating the large step between lines for high-order IMD.

Table 4 Quoted from [9] as the reference for 2UL IMD LUT columns

Proposal 7: It is proposed to define separate LUT columns for 1UL and 2UL HPUE MSD, and the granularity for ΔMSD is not proper to apply to both LUTs.

Conclusion

In this contribution we discussed on HPUE MSD rules, according to the analysis, we have the following observations and proposals:

Observation 1: The gaps of values between different lines are too small as 0.2dB in the example LUT columns in WF R4-250514. And it is over-designed for 1 UL HPUE MSD simplification.

Observation 2: The values of 6, 9, 12, 15dB are defined as ΔMSDmax in the LUT columns for PC2 IMD2/3/4/5.

1 band 1UL CC MSD

Proposal 1: Coarser granularity of PC31Tx MSD for 1 UL HPUE MSD LUT columns should be defined. The values gap between adjacent lines should not be smaller than 0.5dB.

2UL – IMD MSD

Proposal 2: At most 4 values of ΔMSDmax should be defined in the LUT columns for PC1.5 IMD2/3/4/5.

Proposal 3: The ΔMSDmax for PC1.5 IMD should be defined de-coupled with PA configuration(s).

Proposal 4: For each IMD order of PC1.5, the following factor should be considered to define as the ΔMSDmax:

- the Maximum value resulted from the position/index of IMD product.

Proposal 5: For PC1.5 MSD, the suggested ΔMSDmax values are 9, 15, 18, and 24 dB for IMD2/3/4/5 respectively.

LUT columns design

Proposal 6: The candidate ΔMSDmax of 3dB for PC2 should be removed to decouple with PA configuration.

Proposal 7: It is proposed to define separate LUT columns for 1UL and 2UL HPUE MSD, and the granularity for ΔMSD is not proper to apply to both LUTs.

- References

[1] R4-2505143, WF on HPUE MSD simplification, RAN4#114bis, April, 2025

[2] R4-2417794, Simplification of HPUE 2UL IMD MSDs, Skyworks Solutions, Inc., RAN4#113, November, 2024

[3] R4-2503169, Simplification of HPUE 2UL DeltaMSD, Skyworks, RAN4#114bis, April, 2025

[4] R4-2503839, Discussion on HPUE MSD simplification, vivo, RAN4#114bis, April, 2025

[5] R4-2503196, On Rel-19 HPUE CA DC MSD consideration, Apple, RAN4#114bis, April, 2025[6] R4-2307146, On UE MSD requirements for FWA UE supporting 3T for two bands, Huawei, RAN4#107, May, 2023

[7] RP-232937 SR of 4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC, OPPO, RAN#102, December, 2023

[8] R4-2503215, Further discussion on HPUE MSD rules, Murata, RAN4#114bis, April, 2025

[9] R4-2504118, MSD beyond PC3, Qualcomm France, RAN4#114bis, April, 2025

[10] R4-2503153, Discussion on HPUE MSD, Huawei, HiSilicon

- Annex

A.1 IMD delta_MSD comparisons

Reference Table 1: Delta_MSD comparison for IMD2

|  |  | Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | PC3 MSD | PC2 MSD (dB) | PC2 DeltaMSD to PC3 | PC1.5 MSD (dB) | PC1.5 DeltaMSD to PC3 | Duplex mode |  |
| CA_n1-n77 | n1 | 1950 | 5 | 25 | 2140 | 29.8 | 35.8 | 6 |  |  | FDD | IMD2 |
|  | n77 | 4090 | 10 | 50 | 4090 |  | N/A |  |  |  | TDD | N/A |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 26 | 32.10 | 6.1 | 35.2 | 9.2 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 |  | N/A |  |  |  | TDD | N/A |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 26 | 32.1 | 6.1 | 37.5 | 11.5 | FDD | IMD24 |
|  | n77 | 3790 | 10 | 50 | 3790 |  | N/A |  |  |  | TDD | N/A |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 31 | 34.33 | 3.33 | 40 | 9 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 |  | N/A |  |  |  | TDD | N/A |
| CA_n70-n77 | n70 | 1702.5 | 5 | 25 | 2002.5 | 31 | 37 | 6 | 41.8 | 10.8 | FDD | IMD2 |
|  | n77 | 3705 | 10 | 50 | 3705 |  | N/A |  |  |  | TDD | N/A |
| CA_n3-n77 | n3 | 1740 | 5 | 25 | 1835 | 26 | 31.9 | 5.9 |  |  | FDD | IMD2 |
|  | n77 | 3575 | 10 | 50 | 3575 |  | N/A |  |  |  | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 26 | 31.9 | 5.9 |  |  | FDD | IMD2 |
|  | n78 | 3575 | 10 | 50 | 3575 |  | N/A |  |  |  | TDD | N/A |
| CA_n25-n78 | n25 | 1855 | 5 | 25 | 1935 | 26 | 32.10 | 6.1 |  |  | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 |  | N/A |  |  |  | TDD | N/A |

Reference Table 2: Delta_MSD comparison for IMD3

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | PC3 MSD | PC2 MSD (dB) | PC2 DeltaMSD to PC3 | Duplex mode |  |
| CA_n18-n41 | n18 | 820 | 5 | 25 | 865 | 15.6 | 24.6 | 9 | FDD | IMD3 |
|  | n41 | 2505 | 5 | 25 | 2505 |  | N/A |  | TDD | N/A |

Reference Table 3: Delta_MSD comparison for IMD4

|  |  |  | Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) |  | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | PC3 MSD | PC2 MSD (dB) | PC2 DeltaMSD to PC3 | PC1.5 MSD (dB) | PC1.5 DeltaMSD to PC3 | Duplex mode |  |
| CA_n1-n774 | n1 | 1950 |  | 5 | 25 | 2140 | 8.0 | 17.8 | 9.8 |  |  | FDD | IMD4 |
|  | n77 | 3710 |  | 10 | 50 | 3710 |  | N/A |  |  |  | TDD | N/A |
| CA_n1-n78 | n1 | 1950 |  | 5 | 25 | 2140 | 8 | 17.8 | 9.8 |  |  | FDD | IMD4 |
|  | n78 | 3710 |  | 10 | 50 | 3710 |  | N/A |  |  |  | TDD | N/A |
| CA_n2-n77 | n2 | 1900 |  | 5 | 25 | 1980 | 5 | 19.10 | 14.1 | 26.4 | 21.4 | FDD | IMD4 |
|  | n77 | 3720 |  | 10 | 50 | 3720 |  | N/A |  |  |  | TDD | N/A |
| CA_n5-n77 | n5 | 844 |  | 5 | 25 | 889 | 8.3 | 18.6 | 10.3 | 25.5 | 17.2 | FDD | IMD44,13 |
|  | n77 | 3421 |  | 10 | 50 | 3421 |  | N/A |  |  |  | TDD | N/A |
| CA_n7-n77 | n7 | 2540 |  | 5 | 25 | 2660 | 7.1 | [15.8] | 8.7 | 29.7 | 22.6 | FDD | IMD4 |
|  | n77 | 3870 |  | 10 | 50 | 3870 |  | N/A |  |  |  | TDD | N/A |
| CA_n25-n77 | n25 | 1900 |  | 5 | 25 | 1980 | 8.0 | 19.1 | 11.1 | 25.8 | 17.8 | FDD | IMD4 |
|  | n77 | 3720 |  | 10 | 50 | 3720 |  | N/A |  |  |  | TDD | N/A |
| CA_n41-n71 | n41 | 2614 |  | 5 | 25 | 2614 |  | N/A |  |  |  | TDD | N/A |
|  | n71 | 665 |  | 5 | 25 | 619 | 11 | 16.3 | 5.3 | 25.4 | 14.4 | FDD | IMD4 |
| CA_n3-n41 | n3 | 1740 |  | 5 | 25 | 1835 | 8.2 | 18.4 | 10.2 |  |  | FDD | IMD4 |
|  | n41 | 2657.5 |  | 10 | 50 | 2657.5 |  | N/A |  |  |  | TDD | N/A |
| CA_n3-n77 | n3 | 1765 |  | 5 | 25 | 1860 | 8 | 18.5 | 10.5 |  |  | FDD | IMD4 |
|  | n77 | 3435 |  | 10 | 50 | 3435 |  | N/A |  |  |  | TDD | N/A |
| CA_n3-n78 | n3 | 1765 |  | 5 | 25 | 1860 | 8 | 18.5 | 10.5 |  |  | FDD | IMD4 |
|  | n78 | 3435 |  | 10 | 50 | 3435 |  | N/A |  |  |  | TDD | N/A |
| CA_n5-n78 | n5 | 844 |  | 5 | 25 | 889 | 8.3 | 18.6 | 10.3 |  |  | FDD | IMD4 |
|  | n78 | 3421 |  | 10 | 50 | 3421 |  | N/A |  |  |  | TDD | N/A |
| CA_n8-n77 | n8 | 897.5 |  | 5 | 25 | 942.5 | void | 15.5 |  |  |  | FDD | IMD4 |
|  | n77 | 3635 |  | 10 | 50 | 3635 |  | N/A |  |  |  | TDD | N/A |
| CA_n8-n78 | n8 | 897.5 |  | 5 | 25 | 942.5 | 8.3 | 15.5 | 7.2 |  |  | FDD | IMD4 |
|  | n78 | 3635 |  | 10 | 50 | 3635 |  | N/A |  |  |  | TDD | N/A |
| CA_n18-n77 | n18 | 827.5 |  | 5 | 25 | 872.5 | 8.4 | 17.5 | 9.1 |  |  | FDD | IMD416 |
|  | n77 | 3355 |  | 10 | 50 | 3355 |  | N/A |  |  |  | TDD | N/A |
| CA_n26-n78 | n26 | 836.5 |  | 5 | 25 | 881.5 | 11.1 | 23.8 | 12.7 |  |  | FDD | IMD4 |
|  | n78 | 3391 |  | 10 | 50 | 3391 |  | N/A |  |  |  | TDD | N/A |
| CA_n30-n77 | n30 | 2310 |  | 5 | 25 | 2355 | 8.0 | 17.6 | 9.6 |  |  | FDD | IMD4 |
|  | n77 | 3487.5 |  | 10 | 50 | 3487.5 |  | N/A |  |  |  | TDD | N/A |

Reference Table 4: Delta_MSD comparison for IMD5

|  |  | Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | PC3 MSD | PC2 MSD (dB) | DeltaMSD | PC1.5 MSD (dB) | PC1.5 DeltaMSD to PC3 | Duplex mode |  |
| CA_n2-n774 | n2 | 1885 | 5 | 25 | 1965 | 8 | 20.0 | 12 | 28 | 20 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 |  | N/A |  |  |  | TDD | N/A |
| CA_n5-n772 | n5 | 826.5 | 5 | 25 | 871.5 | 5.5 | Void | Void | 24.3 | 18.8 | FDD | IMD5 |
|  | n77 | 4177.5 | 10 | 50 | 4177.5 |  |  |  | N/A |  | TDD | N/A |
| CA_n25-n77 | n25 | 1885 | 5 | 25 | 1965 | 5 | Void | Void | 29.8 | 24.8 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 |  |  |  |  |  | TDD | N/A |
| CA_n66-n77 | n66 | 1760 | 5 | 25 | 2160 | 5.0 | 11.27 | 6.27 | 26 | 21 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 |  | N/A |  |  |  | TDD | N/A |
| CA_n70-n77 | n70 | 1697.5 | 5 | 25 | 1997.5 | 5.0 | 18.4 | 13.4 | 27.9 | 22.9 | FDD | IMD5 |
|  | n77 | 3545 | 10 | 50 | 3545 |  | N/A |  |  |  | TDD | N/A |
| CA_n71-n77 | n71 | 681.5 | 5 | 25 | 635.5 | 5.5 | 11.4 | 5.9 | 16 | 10.5 | FDD | IMD513 |
|  | n77 | 3361.5 | 10 | 50 | 3361.5 |  | N/A |  |  |  | TDD | N/A |

A.2 Harmonic delta_MSD comparisons (only the 1st test points are illustrated)

Reference Table 5: Delta_MSD comparison for Harmonic

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | PC2 MSD_1Tx | DeltaMSD | PC2 MSD_2Tx | DeltaMSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) | (dB) | (dB) | (dB) |  |  |
| n3 | n78 | 5 | 15 | 12 | 10 | 23.9 | 27.1 | 3.2 | 32.3 | 8.4 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 5 | 15 | 12 | 10 | 23.9 | 26.9 | 3 | 31.9 | 8 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 5 | 15 | 12 | 10 | 23.9 | 26.9 | 3 | 31.9 | 8 | NOTE 2 | UL2/DL1 direct-hit |
| n8 | n41 | 5 | 15 | 8 | 10 | 13 | 15.9 | 2.9 | 20 | 7 | NOTE 2 | UL3/DL1 direct-hit |
| n71 | n25 | 5 | 15 | 8 | 5 | 6.9 | 9.4 | 2.5 | 12.9 | 6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 6 | 10 | 10.8 | 13.6 | 2.8 | 17.8 | 7 | NOTE 4 | UL4/DL1 direct-hit |
| n8 | n79 | 5 | 15 | 5 | 10 | 12 | 14.9 | 2.9 | 19 | 7 | NOTE 2 | UL5/DL1 direct-hit |
| n71 | n77 | 5 | 15 | 5 | 10 | 10.4 | 13.2 | 2.8 | 17.4 | 7 | NOTE 5 | UL5/DL1 direct-hit |

A.3 Harmonic mixing delta_MSD comparisons (only the 1st test points are illustrated)

Reference Table 6: Delta_MSD comparison for Harmonic mixing UL1/DL2

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | MSD_1Tx | deltaToPC3 | MSD_PC1.5 | deltaToPC3 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) |  |  |  |  |  |
| n77 | n2 | 10 | 15 | 25 | 5 | 6.7 | 9.2 | 2.5 | 11.8 | 5.1 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 5 | 5.7 | 8.1 | 2.4 | 10.7 | 5 | NOTE 4 | UL1/DL2 |
| n77 | n25 | 10 | 15 | 25 | 5 | 5.6 | 7.9 | 2.3 | 10.6 | 5 | NOTE 4 | UL1/DL2 |
| n78 | n3 | 10 | 15 | 25 | 5 | 5.7 | 8.1 | 2.4 | 10.7 | 5 | NOTE 4 | UL1/DL2 |

Reference Table 7: Delta_MSD comparison for Harmonic mixing UL1/DL3

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | MSD_1Tx | deltaToPC3 | MSD_2Tx | deltaToPC3 | MSD_PC1.5 | deltaToPC3 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) |  |  |  |  |  |  |  |
| n25 | n713 | 5 | 15 | 25 | 5 | 26.8 | 29.5 | 3 | 34.5 | 7.7 |  |  | NOTE 4 | UL1/DL3 |
| n40 | n28 | 5 | 15 | 6 | 5 | 37.8 | 40.8 | 3 |  |  | 43.8 | 6 | NOTE 7 | UL1/DL3 |
| n41 | n186 | 10 | 15 | 25 | 5 | 26.3 | 29.3 | 3 |  |  |  |  | NOTE 7 | UL1/DL3 |

Reference Table 8: Delta_MSD comparison for Harmonic mixing UL1/DL4

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | MSD_1Tx | deltaToPC3 | MSD_PC1.5 | deltaToPC3 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) |  |  |  |  |  |
| n77 | n5 | 10 | 15 | 25 | 5 | 5.7 | 8.1 | 2.4 | 10.7 | 5 | NOTE 5 | UL1/DL4 |
| n77 | n8 | 10 | 15 | 25 | 5 | 5.7 | 8.1 | 2.4 |  |  | NOTE 5 | UL1/DL4 |
| n78 | n5 | 10 | 15 | 25 | 5 | 5.7 | [8.1] | 2.4 |  |  | NOTE 5 | UL1/DL4 |
| n78 | n8 | 10 | 15 | 25 | 5 | 5.7 | 8.1 | 2.4 | 10.6 | 4.9 | NOTE 5 | UL1/DL4 |
| n78 | n26 | 10 | 15 | 25 | 5 | 5.4 |  |  | 10.1 | 4.7 |  | UL1/DL4 |

Reference Table 9: Delta_MSD comparison for Harmonic mixing UL1/DL5

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | MSD_1Tx | deltaToPC3 | MSD_PC1.5 | deltaToPC3 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) |  |  |  |  |  |
| n77 | n12 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n77 | n85 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n78 | n28 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n79 | n8 | 10 | 15 | 25 | 5 | 25 | 28.0 | 3 | 31 | 6 | NOTE 1 | UL1/DL5 |

Reference Table 10: Delta_MSD comparison for Harmonic mixing ULx/DLy

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | MSD_1Tx | deltaToPC3 | MSD_2Tx | deltaToPC3 | MSD_PC1.5 | deltaToPC3 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) |  |  |  |  |  |  |  |
| n41 | n39 | 10 | 15 | 8 | 5 | 4.3 | 6.1 | 1.8 |  |  | 10.5 | 6.2 | NOTE 9 | UL3/DL4 |
| n25 | n41 | 5 | 15 | 25 | 10 | 1.3 | 2.2 | 0.9 | 3.2 | 1.9 |  |  | NOTE 11 | UL4/DL3 |
| n39 | n41 | 5 | 15 | 6 | 5 | 9.3 | 10.6 | 1.3 |  |  |  |  | NOTE 8 | UL4/DL3 |
| n41 | n77 | 10 | 15 | 6 | 10 | 8.9 | 11.6 | 2.7 |  |  | 14.5 | 5.6 | NOTE 8 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 10 | 9.3 | 12.1 | 2.8 |  |  | 14.9 | 5.6 | NOTE 8 | UL4/DL3 |
| n77 | n7 | 10 | 15 | 12 | 5 | 14.7 | 17.7 | 3 |  |  | 17.5 | 2.8 | NOTE 3 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 12 | 5 | 10.4 | 13.2 | 2.8 |  |  | 16.1 | 5.7 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 10 | 11.7 | 14.5 | 2.8 |  |  | 16.1 | 4.4 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 10 | 11.7 | 14.5 | 2.8 |  |  | 17.5 | 5.8 | NOTE 3 | UL2/DL3 |
| n78 | n40 | 10 | 15 | 12 | 10 | 14.7 | 13.2 | -1.5 |  |  | 16.1 | 1.4 | NOTE 3 | UL2/DL3 |

Reference Table 11: Delta_MSD comparison for Cross band isolation

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | PC3 MSD | PC2-1Tx MSD | Delta_PC2-PC3 | PC2- 2Tx MSD | Delta_PC2-2Tx-PC3 | PC1.5 MSD | Delta_PC1.5-PC3 | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) |  | (dB) |  |  |  |  |  |  |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 23 | 26 | 3 | 32.3 | 9.3 |  |  | ACLR1 |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 28.1 | 31.1 | 3 |  |  | 37.9 | 9.8 | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 31.4 | 34.4 | 3 |  |  | 41.2 | 9.8 | ACLR2 |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 8.2 | 10.9 | 2.7 | 15.9 | 7.7 |  |  | ACLR2 |
| n2 | n66 | 1900 | 20 | 15 | 50 (RBstart=56) | 2112.5 | 5 | 0 | 0.7 | 0.7 | 0.9 | 0.9 |  |  | >ACLR2 |
| n3 | n1 | 1760 | 50 | 15 | 50 (RBstart=220) | 2112.5 | 5 |  | 0.8 |  | 1.1 |  |  |  | >ACLR2 |
| n3 | n7 | 1760 | 50 | 15 | 50 (RBstart=220) | 2622.5 | 5 |  | 0.5 |  | 0.7 |  |  |  | >ACLR2 |
| n7 | n1 | 2525 | 50 | 15 | 45 (RBstart=0) | 2167.5 | 5 |  | 0.8 |  | 1.1 |  |  |  | >ACLR2 |
| n7 | n3 | 2525 | 50 | 15 | 45 (RBstart=0) | 1877.5 | 5 | 0.6 | 1.1 | 0.5 | 1.5 | 0.9 |  |  | >ACLR2 |
| n7 | n40 | 2525 | 50 | 15 | 45 (RBstart=0) | 2397.5 | 5 | 3.7 |  |  |  |  |  |  | >ACLR2 |
| n25 | n41 | 1760 | 40 | 15 | 40 (RBstart=176) | 2501 | 10 |  | 0.8 |  | 1 |  |  |  | >ACLR2 |
| n25 | n66 | 1895 | 40 | 15 | 40 (RBstart=176) | 2112.5 | 5 |  | 0.7 |  | 0.9 |  |  |  | >ACLR2 |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 3.3 | 4.7 | 1.4 |  |  |  |  | >ACLR2 |
| n40 | n3 | 2350 | 100 | 30 | 270 (RBstart=0) | 1877.5 | 5 |  | 0.6 |  |  |  | 1.2 |  | >ACLR2 |
| n41 | n1 | 2546 | 100 | 30 | 270 (RBstart=0) | 2167.5 | 5 | 18.1 | 20.8 | 2.7 |  |  | 23.5 | 5.4 | >ACLR2 |
| n41 | n3 | 2546 | 100 | 30 | 270 (RBstart=0) | 1877.5 | 5 | 0.6 | 2.3 | 1.7 |  |  | 3.9 | 3.3 | >ACLR2 |
| n41 | n25 | 2546 | 100 | 30 | 270 (RBstart=0) | 1992.5 | 5 | 0.6 | 1.6 | 1 |  |  | 2.8 | 2.2 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1917.5 | 5 | 1.6 | 2.7 | 1.1 |  |  | 6.7 | 5.1 | >ACLR2 |
| n41 | n66 | 2546 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 10.5 | 13.1 | 2.6 |  |  | 7.7 | -2.8 | >ACLR2 |
| n41 | n77 | 2640 | 100 | 30 | 270 (RBstart=3) | 3305 | 10 | 8.3 | 10.5 | 2.2 |  |  | 13.3 | 5 | >ACLR2 |
| n41 | n79 | 2640 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 |  | 3.1 |  |  |  |  |  | >ACLR2 |
| n66 | n2 | 1760 | 40 | 15 | 216 (RBstart=0) | 1932.5 | 5 | 1.2 | 1.9 | 0.7 | 3.3 | 2.1 |  |  | >ACLR2 |
| n66 | n25 | 1757.5 | 45 | 15 | 240 (RBstart=2) | 1932.5 | 5 | 1.4 | 2.2 | 0.8 | 3.8 | 2.4 |  |  | >ACLR2 |
| n66 | n41 | 1760 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 0.4 | 0.8 | 0.4 | 1 | 0.6 |  |  | >ACLR2 |
| n66 | n70 | 1760 | 40 | 15 | 216 (RBstart=0) | 1997.5 | 5 |  | 1.9 |  | 3.3 |  |  |  | >ACLR2 |
| n70 | n66 | 1702.5 | 15 | 15 | 75 (RBstart=4) | 2112.5 | 5 |  | 0.4 |  | 0.5 |  |  |  | >ACLR2 |
| n77 | n2 | 3350 | 100 | 30 | 270 (RBstart=0) | 1987.5 | 5 |  | 1.0 |  |  |  | 1.8 |  | >ACLR2 |
| n77 | n7 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 4.5 | 6.5 | 2 |  |  | 9.0 | 4.5 | >ACLR2 |
| n77 | n25 | 3350 | 100 | 30 | 270 (RBstart=0) | 1992.5 | 5 |  | 1.0 |  |  |  | 1.8 |  | >ACLR2 |
| n77 | n30 | 3350 | 100 | 30 | 270 (RBstart=0) | 2357.5 | 5 |  | 1.0 |  |  |  | 1.8 |  | >ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 10 | 4.5 | 6.5 | 2 |  |  | 9.0 | 4.5 | >ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 4.5 | 6.5 | 2 |  |  | 2.2 | -2.3 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 | 4.5 | 6.5 | 2 |  |  | 9.0 | 4.5 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2640 | 100 | 4.5 | 6.5 | 2 |  |  | 9.0 | 4.5 | >ACLR2 |
| n77 | n66 | 3350 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 |  | 1.0 |  |  |  | 1.8 |  | >ACLR2 |
| n78 | n7 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 4.5 | 6.5 | 2 |  |  | 9.0 | 4.5 | >ACLR2 |
| n78 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2395 | 10 | 4.5 | 6.5 | 2 |  |  | 9.0 | 4.5 | >ACLR2 |
| n78 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 4.5 | 1.2 | -3.3 |  |  | 2.2 | -2.3 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 | 2 | 5 | 3 |  |  | 8 | 6 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4450 | 100 | 2 | 5 | 3 |  |  | 8 | 6 | >ACLR2 |
| n79 | n41 | 4450 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 |  | 3.5 |  |  |  |  |  | >ACLR2 |
| n79 | n783 | 4450 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | 2.6 | 5.6 | 3 |  |  | 8.6 | 6 | >ACLR2 |
| n79 | n783 | 4450 | 100 | 30 | 270 (RBstart=0) | 3750 | 100 | 2.6 | 5.6 | 3 |  |  | 8.6 | 6 | >ACLR2 |