3GPP TSG- WG4 Meeting # 115	R4-2505322

# St Julian’s, Malta, 19th – 25th May 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on LB-LB switching

Agenda Item:	7.7.3

Document for:	Approval

Introduction

In RAN4#114bis, the WF [1] for Low band carrier aggregation via switching was approved with the following issues.

Issue 1: Switching period capability - The switching period capability can be defined per band pair per band combination. Issue 2: Switching period location - Await outcome of RAN1 design Issue 3: Time mask for switching - Define the Tx on-off time mask requirements - FFS on how to define the Rx switching requirements to reflect where the switching period occurs - RAN4 has understanding that RAN1 will take switching period and timing advance into account for RAN1 scheduling pattern for low band CA via switching Issue 4: Considerations related to the Tx on-off time mask - Option 1: Specify transmit on/off time mask for FDD UL carrier/Pcell for switching between LB(FDD)-LB(SDL) - Option 2: Specify on->off time mask for switching from FDD to SDL carrier and specify off->on time mask for switching from SDL to FDD carrier - Other options are not precluded

In RAN1 April meeting, the conclusion for issue 2 Switching period location was concluded.

Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, down-select one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - Alt 3: Switching gap is always assumed at the SDL carrier

This paper is to provide further thinking concerning the time mask and related capability for LB-LB switching in Rel-19.

Time mask for LB-LB switching

It was agreed in the last RAN1 meeting on switching period for switching from SDL to FDD. RAN4 has agreed to specify Tx on-off time mask requirements. However, SDL as DL carrier is borrowed to demonstrate switching period location. Timing advance between FDD carrier and SDL carrier is taken into consideration. In the semi-static switching pattern, the DL transmissions configured on the SDL avoid the OFDM symbols occupied by switching period. Switching period does not impact the start symbol(s) of the transmission on FDD band (switch-to), illustrated in Figure 1, and the vacant parts in the carrier of the following figures are the time domain resource without transmission configuration.

For switching from FDD to SDL, Alt.1 and Alt. 3 are shown in the following Figure 2 and Figure 3, respectively. Whether RAN4 captures the examples in Figure 2 and/or Figure 3 is up to RAN1’s further discussion.

Figure 1 Time mask for low band CA via switching from SDL carrier to FDD carrier, where the switching period is located in SDL carrier

Figure 2 Time mask for low band CA via switching from FDD carrier to SDL carrier, where the switching period is located in FDD carrier

Figure 3 Time mask for low band CA via switching from FDD carrier to SDL carrier, where the switching period is located in SDL carrier

Proposal 1: The Tx on-off time masks for LB-LB switching show:

- The transmissions configured on the SDL/FDD avoid the OFDM symbols occupied by switching period in the semi-static switching pattern.

- TA between FDD carrier and SDL carrier, if SDL is borrowed in the Tx time mask.

UE capability related to LB-LB switching

In the WID, the example band combination is CA_n5-n29. But before the approved WI, companies discussed more about CA_n12-n29. CA_n12-n29 is quite special that the frequency isolation between band n12 UL and n29 DL is only 1MHz. Thus, it is difficult to design filter for n12+n29. Con-current operation for (n12UL, n29DL) is not supported. It requires network scheduling restriction.

The capability simultaneousRxTxInterBandCA is designed for TDD-TDD and TDD-FDD inter-band NR CA. And for FDD-FDD/ FDD-SDL inter-band NR CA, simultaneous Rx-Tx is mandatorily supported without signaling. However, the case is different at least for CA_n12-n29 via switching.

Observation 1: The capability simultaneousRxTxInterBandCA is designed for TDD-TDD and TDD-FDD inter-band NR CA. And for FDD-FDD/ FDD-SDL inter-band NR CA, simultaneous Rx-Tx is mandatorily supported without signaling.

Meanwhile, con-current operation for (n12DL, n29DL) is not supported either, since Rel-19 is to specify the switching between case 1 and case 2, and there is no dualRx for the two bands.

Figure 4 Frequency range illustration for CA_n12-n29

The definition of carrier aggregation is Aggregation of two or more component carriers in order to support wider transmission bandwidths [2]. For inter-band DL CA, the simultaneous Rx operation for both the DL bands must be supported, resulting in wider bandwidth. Therefore, for Rel-19 low band FDD-SDL inter-band CA via switching that is not capable of con-current Rx operation, the capability should be reported separately from BandCombinationList for inter-band CA.

Observation 2: Normally for inter-band DL CA band combination, con-current Rx operation for both the DL bands is supported.

Proposal 2: For Rel-19 sub-1GHz FDD-SDL band combinations that is not capable of con-current Rx operation, the capability should be separately reported from BandCombinationList for inter-band CA.

The UE capability should reflect the UE RF front-end architecture. The con-current operation of Rx-Tx, Rx-Rx depends on the UE architectures.

(a) The UE architecture for CA_n12-n29 that simultaneous operation (n12DL, n29DL) is not supported

(b) The UE architecture for CA_n12-n29 that simultaneous operation (n12DL, n29DL) is supported

Figure 5 UE RF front-end architectures for inter-band FDD-SDL CA_n12-n29

The filters for band n12 and n29 are separately designed in the reference UE RF front-end architecture in Figure 5(a) The UE architecture for CA_n12-n29 that simultaneous operation (n12DL, n29DL) is not supported in Rel-19. Some companies also proposed that in the future release the concurrent Rx on the two DL bands should be supported. The capability is related to scheduling restriction. The capability design should be scalable, not only reflect different UE architectures, but also apply to the requirements in different releases, and the inter-band CA with more than two bands.

Observation 3: The capability of low band CA via switching is also related to scheduling restriction.

The sub1GHz FDD-SDL low band CA switching capability design should take scheduling restriction into consideration. For FDD-SDL inter-band CA:

- Simultaneous (FDD-DL, SDL-DL)

- Simultaneous (FDD-UL, SDL-DL)

Proposal 3: The sub1GHz FDD-SDL low band CA switching capability design should take scheduling restriction into consideration.

Proposal 4: The capability design should be scalable, not only reflect different UE architectures, but also apply to the requirements in different releases, and the inter-band CA with more than two bands.

For Rel-19 low band FDD-SDL CA via switching, the capability should be per-BC reported. When network receives the low band FDD-SDL CA via switching capability, the RRC configuration of the switching pattern should be configured. The switching pattern enables the TDM of the operations on the two bands.

Proposal 5: When network receives the per-BC low band CA via switching capability, the RRC configuration of the switching pattern should be configured.

Conclusion

In this contribution we discussed on low band CA via switching, according to the analysis, we have the following observation and proposals:

Observation 1: The capability simultaneousRxTxInterBandCA is designed for TDD-TDD and TDD-FDD inter-band NR CA. And for FDD-FDD/ FDD-SDL inter-band NR CA, simultaneous Rx-Tx is mandatorily supported without signaling.

Observation 2: Normally for inter-band DL CA band combination, con-current Rx operation for both the DL bands is supported.

Observation 3: The capability of low band CA via switching is also related to scheduling restriction.

Proposal 1: The Tx on-off time masks for LB-LB switching show:

- The transmissions configured on the SDL/FDD avoid the OFDM symbols occupied by switching period in the semi-static switching pattern.

- TA between FDD carrier and SDL carrier, if SDL is borrowed in the Tx time mask.

Proposal 2: For Rel-19 sub-1GHz FDD-SDL band combinations that is not capable of con-current Rx operation, the capability should be separately reported from BandCombinationList for inter-band CA.

Proposal 3: The sub1GHz FDD-SDL low band CA switching capability design should take scheduling restriction into consideration.

Proposal 4: The capability design should be scalable, not only reflect different UE architectures, but also apply to the requirements in different releases, and the inter-band CA with more than two bands.

Proposal 5: When network receives the per-BC low band CA via switching capability, the RRC configuration of the switching pattern should be configured.

- References

[1] R4-2505223, WF on UE RF requirements for LB CA via switching, Apple, RAN4#114bis, April, 2025.

[2] TS 38.101-1