3GPP TSG- WG4 Meeting # 115	R4-2505324

# St Julian’s, Malta, 19th – 25th May 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on simultaneous Rx-Tx

Agenda Item:	6.11.2

Document for:	Approval

Introduction

In the approved revised WID on simultaneous Rx-Tx [1], one of the objectives is

- Align the specification treatment of simultaneous Rx/Tx capability for CA, SUL, MR-DC and NR-DC band combinations.

In details, we need to align the treatment of simultaneous Rx/Tx capability for inter-band CA in TS38.101-1 and EN-DC in TS38.101-3 in Rel-19.

Besides, this paper provides further analysis based on the WF about how to handle the exceptional cases for FDD-FDD and FDD-SDL.

Discussion

Alignment on the specification treatment for simultaneous Rx/Tx capability

For TDD-FDD inter-band CA, RAN4 has agreed and endorsed to capture in TS 38.101-1 in [2] that

Unless stated otherwise, simultaneous Rx/Tx capability is mandatory for FR1+FR1 FDD-TDD CA combinations.

The specification treatment for FDD-TDD CA band combination for simultaneous Rx-Tx capability in TS 38.101-1 should be applied to the FDD-TDD inter-band EN-DC band combination in TS 38.101-3.

Proposal 1: The specification treatment for FDD-TDD CA band combination for simultaneous Rx-Tx capability in TS 38.101-1 should be applied to the FDD-TDD inter-band EN-DC band combination in TS 38.101-3.

In RAN4#114, the WF [3] was approved that

- To clarify the handling of simultaneous Rx-Tx interaction between network and UE for band combinations with different types of duplex mode - For TDD-SDL band combination, simultaneous Rx-Tx is mandatory with signalling. Consider to reuse the capability of simultaneousRxTxInterBandCA referring to TDD-FDD. - For FDD-FDD band combinations, simultaneous Rx-Tx is mandatory without signalling. - Deviations to this due to UL CA_n5-n8 is TBD - For FDD-SDL band combinations, simultaneous Rx-Tx is mandatory without signalling. - Deviations to this due to LB-LB RX switching including LB-LB configurations in RP-242532 are TBD.

In [4], the conclusions above was captured in TS 38.101-1

Unless stated otherwise, simultaneous Rx/Tx capability is mandatory for FR1+FR1 FDD-TDD and TDD-SDL CA combinations. Simultaneous Rx/Tx capability is mandatory without signaling for FR1+FR1 FDD-FDD and FDD-SDL CA combinations.

The specification treatment for TDD-SDL, FDD-FDD and FDD-SDL CA band combination for simultaneous Rx-Tx capability in TS 38.101-1 should be applied to the FDD-TDD inter-band EN-DC band combination in TS 38.101-3.

Proposal 2: The specification treatment for TDD-SDL, FDD-FDD and FDD-SDL CA band combination for simultaneous Rx-Tx capability in TS 38.101-1 should be applied to the FDD-TDD inter-band EN-DC band combination in TS 38.101-3.

We provide the companion draftCR for TS 38.101-3 in [5] to reflect the treatment alignment between the specifications for simultaneous Rx/Tx capability for band combinations with the duplex mode.

Discussion on FDD-FDD and FDD-SDL inter-band CA

Current state for simultaneous Rx-Tx of inter-band CA in spec:

For TDD-TDD and TDD-FDD inter-band CA, taking CA_nA-nB for example, the capability simultaneousRxTx does not differentiate the two cases. Just both supported or neither.

- simultaneous (nAUL, nBDL)

- simultaneous (nBUL, nADL)

In RRM requirement Applicability of requirements for scheduling availability in TS 38.133 clause 3.6.9

The scheduling availability requirements in clause 8.1.7.1, 8.1.7.2, 8.5.7.1, 8.5.7.2, 8.5.8.1, 8.5.8.2, 9.5.6.1, 9.5.6.2, 9.8.6.1, and 9.8.6.2 assumes that the UE is not configured with simultaneous UL/DL between two FR1 bands if the UE does not have the capability of supporting simultaneousRxTxInterBandCA.

For FDD-FDD inter-band CA, it was concluded that simultaneous Rx-Tx is mandatory without signalling. The exceptions should be considered. The frequency range of UL in band n8 and DL in band n5 is overlapped, and it is impossible to implement simultaneous (n5DL, n8UL). Meanwhile, simultaneous (n5UL, n8DL) can be supported by default. In RAN4#113, RAN4 sent LS [9] to RAN2 to ask necessary modification in RAN2 spec and/or add UE capability to enable the functionality of different simultaneous Rx-Tx capability for (n5UL, n8DL) and (n8UL, n5DL) in CA_n5-n8 with two FDD bands frequency range partially overlap.

Figure 1 Two possible operations illustration for CA_n5-n8

Continue discuss UL CA for CA_n5-n8 in Rel-19:

In RAN4#114bis, the Chair guidance is to continue the discussions of CA n5-n8 in the simultaneous Rx-Tx WI and no up-scope of discussion is allowed.

RAN4 conducted studies in Rel-18 on enhancements for 700/800/900MHz band combinations. The studies concluded that three options are feasible for implementing the CA_n5-n8 band combination, as detailed in TR 38.872.:

1) Full band n5 and n8 RF filters implementation with option 1 and option2: - Option 1: Only support 1UL/2DL CA. Single UL in n5 - Option 2: Support both 1UL/2DL and 2UL/2DL CA. Non-concurrent n5 DL and n8 UL 2) Dedicated RF filters implementation with partial frequency range - Option 3: Support both 1UL/2DL and 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL without any scheduling restriction on both bands.

In [6], it was pointed out that standardizing Option 2 in Rel-19 may require a future-proof signalling design to support that the capabilities for the simultaneous operation in case more similar NR FDD-FDD inter-band CA band combinations where UL and DL from different FDD bands partially overlap are introduced, as requested in the original LS [9]. The only workload to RAN2 would be to introduce a capability differentiating simultaneous Rx-Tx capability for (nA UL, nB DL) and (nA DL, nB UL). If some day, the dedicated filter is implemented and enable the simultaneous (nA UL, nB DL). The forward-compatibility should be considered. Without simultaneous Rx-Tx between n5 DL and n8 UL, the scheduling restriction would be applied according to RRM spec. No switching behaviour is required in Rel-19.

Proposal 3: To support UL CA for CA_n5-n8 in Rel-19, a future proof capability differentiating simultaneous Rx-Tx capability for (nA UL, nB DL) and (nA DL, nB UL), is required.

In addition, in [6], the note solution was raised to pre-define 2UL/2DL CA_n5-n8 in the Rel-19 TS 38.101-1 specification, which is: NOTE 15: Uplink is only in n5 for one uplink CA_n5-n8. 2UL/2DL CA_n5A-8A is supported using non-concurrent n5 DL and n8 UL. However, to coordinate with RRM spec for scheduling availability in TS 38.133, it may not be a neat solution, from the perspective of network.

Proposal 4: The Note to support using non-concurrent n5 DL and n8 UL for UL CA_n5-n8 may not be coordinated with scheduling availability from the perspective of network.

However, CA_n5-n8 would not be the only FDD-FDD BC that hardly supports simultaneous Rx-Tx. For band combination without sufficient separation between DL carrier in one band and UL carrier in the other band, simultaneous DL-UL operation is difficult. In [7], two operators in North America identify two sub-1GHz to be aggregated, CA_n26-n106 with 2MHz isolation between n26 DL and n106 UL. How to design the duplexer for the BC is questionable.

Observation 1: The FDD-FDD band combination CA_nA-nB without sufficient frequency separation between band nA UL and band nB DL can hardly support simultaneous operation of (nA UL, nB DL).

For FDD-SDL inter-band CA, it was concluded that simultaneous Rx-Tx is mandatory without signalling. For Rel-19 low band carrier aggregation via switching [8], there is no simultaneous Rx-Tx between FDD UL and SDL. From the duplexer design perspective, the frequency separation between the bands does not pose a feasibility challenge in general. Things are quite similar to that for FDD-FDD without sufficient isolation between nA UL and nB DL in CA_n12-n29.

Observation 2: FDD-SDL band combination in Rel-19 low band carrier aggregation via switching without sufficient frequency separation results in duplexer design difficulty, and makes it hard to implement simultaneous Rx-Tx for FDD UL and SDL.

In [7], CA_n28-n67 was also proposed for CA via switching, n67 DL (738-758MHz) overlaps with n28 UL (703-748MHz). The reason for non-simultaneous Rx-Tx is same as that for CA_n5-n8 (n5DL, n8UL).

Conclusion

In this contribution we further the rules for simultaneous RxTx for FR1, according to the analysis, we have the following observations and proposals:

Proposal 1: The specification treatment for FDD-TDD CA band combination for simultaneous Rx-Tx capability in TS 38.101-1 should be applied to the FDD-TDD inter-band EN-DC band combination in TS 38.101-3.

Proposal 2: The specification treatment for TDD-SDL, FDD-FDD and FDD-SDL CA band combination for simultaneous Rx-Tx capability in TS 38.101-1 should be applied to the FDD-TDD inter-band EN-DC band combination in TS 38.101-3.

Proposal 3: To support UL CA for CA_n5-n8 in Rel-19, a future proof capability differentiating simultaneous Rx-Tx capability for (nA UL, nB DL) and (nA DL, nB UL), is required.

Proposal 4: The Note to support using non-concurrent n5 DL and n8 UL for UL CA_n5-n8 may not be coordinated with scheduling availability from the perspective of network.

Observation 1: The FDD-FDD band combination CA_nA-nB without sufficient frequency separation between band nA UL and band nB DL can hardly support simultaneous operation of (nA UL, nB DL).

Observation 2: FDD-SDL band combination in Rel-19 low band carrier aggregation via switching without sufficient frequency separation results in duplexer design difficulty, and makes it hard to implement simultaneous Rx-Tx for FDD UL and SDL.

- References

[1] RP-242088, Revised WID Simultaneous RxTx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-19, RAN#105, September, 2024.

[2] R4-2420505, drafCR to 38.101-1: On Rel-19 simultaneous Rx-Tx, RAN4#113, November, 2024.

[3] R4-2502840, WF on simultaneous Rx-Tx, RAN4#114, February, 2025.

[4] R4-2505226, draftCR to 38.101-1: Simultaneous Rx-Tx, RAN4#114bis, April, 2025.

[5] R4-2505325, draftCR to 38.101-3 Simultaneous Rx-Tx, RAN4#115, May, 2025

[6] R4-2505124, Reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx, RAN4#114bis, April, 2025.

[7] R4-2411176, Motivation for a new work item on LB+LB CA based on switching in Rel-19, Apple, , Skyworks Solutions Inc., BT plc, TELUS, Bell Mobility, Anterix, Southern Linc, RAN4#112.

[8] RP-243317, New WID on low band carrier aggregation via switching, RAN#106

[9] R4-2420410, LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx, RAN4#113