3GPP TSG-RAN WG4 Meeting #115	R4-2505340

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.17.3

Source:	MediaTek Inc.

Title:	Discussion on UE demodulation requirements with 8Rx MMSE-IRC receiver

Document for:	Discussion

# Introduction

In this contribution, we share our perspectives on the remaining issues in [1].

- Discussion

CQI calculation algorithm and Receiver coverage Agreement: - Companies are encouraged to provide suggestions on methodology for deciding CQI reporting requirements in next meeting. - Decision to have one or two sets of requirements is postponed to next meeting when more simulation results are available. Configuration for CQI requirements Proposals: - Option 1: - Rank 2 - SINR = -1dB - Gamma =2 - BLER = 0.02 - Option 2: - Rank2 - SINR=-2dB - Gamma=2.1

We believe it is challenging to establish a methodology for determining CQI reporting requirements due to the significant variation in gamma ratios across different companies' results, which is a common occurrence. Based on our detail simulation results in [2], when SINR is -1 dB, the difference of gamma value for baseline and simplified receiver are quite small as shown in Table 1. Therefore, we think it is feasible to define one set of requirements applied to both baseline and simplified receiver with gamma = 2, BLER = 0.02 and SINR = -1dB.

Table 1. Difference of gamma values for baseline simplified receiver when SINR = -1dB

|  | difference of gamma between baseline simplified receiver when SINR = -1dB |
| --- | --- |
| FDD | 0.12 |
| TDD | 0.17 |

Observation 1: Based on our simulation results, the difference of gamma value for baseline and simplified receiver are quite small, which is smaller than 0.2.

Proposal 1: Define one set of CQI requirements for baseline and simplified receiver.

Proposal 2: For both FDD and TDD, use SINR = -1dB, gamma =2 and BLER = 0.02 to define CQI requirements.

Test applicability to skip 2/4Rx requirements Proposals - Option 1: For UEs supporting 8Rx, 2Rx and/or 4Rx, if UE passes 8Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH), the 2 and/or 4Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH) can be skipped, no matter same test configurations as 2RX/4RX are defined or not. - Option 2: To consider if UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. - Option 3: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. - Option 4: Introduce test applicability under inter-cell interference and under intra-cell inter user interference to skip a 2/4Rx requirement if at least the following applies for each considered 8Rx test: - UE support both 8Rx and 2/4Rx - The UE passes an 8Rx requirement with same rank and interference configuration as the 2/4Rx requirement. - Option 5: RAN4 shall have the following applicability rules: - UE that passes Rank2 ICI test case with 8Rx shall skip Rank1 ICI requirements with 4Rx. - UE that passes Rank2 MU-MIMO test case with 8Rx shall skip Rank1/2 MU-MIMO requirements with 4Rx. Release independency Agreement: - The new Rel-19 requirements shall at least be release independent from Rel-18. Remaining proposals: - Option 1: The new Rel-19 requirements shall also be release independent from Rel-17. - Option 2: The new Rel-19 requirements shall also be release independent from Rel-17. The requirements shall be mandatory for Rel-18 and optional from Rel-17.

Given that processing for 8Rx is more complex compared to 2Rx/4Rx, we support Option 1 as it reduces the testing load on the UE.

Proposal 3: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

In Rel-18, 8Rx requirements (without considering ICI and MU-MIMO scenario) are agreed to be release independent from Rel-17. The Rel-19 8Rx considering ICI and MU-MIMO scenario could be treated as an extension of Rel-18 requirements, where the implementation of 8Rx inherently considers scenarios with interference. Therefore, we support that new Rel-19 requirements shall also be release independent from Rel-17.

Proposal 4: The new Rel-19 requirements can be release independent from Rel-17.

# Conclusion

In this contribution, we present our views on UE requirements for devices equipped with an 8Rx MMSE-IRC receiver. Our proposals are outlined below:

Observation 1: Based on our simulation results, the difference of gamma value for baseline and simplified receiver are quite small, which is smaller than 0.2.

Proposal 1: Define one set of CQI requirements for baseline and simplified receiver.

Proposal 2: For both FDD and TDD, use SINR = -1dB, gamma =2 and BLER = 0.02 to define CQI requirements.

Proposal 3: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Proposal 4: The new Rel-19 requirements can be release independent from Rel-17.

# Reference

- R4-2504694, Way forward for [114bis][322] NR_demod_Ph5_Part2_UE, Nokia

- R4-2505341, Simulation results for UE demodulation requirements with 8Rx MMSE-IRC receiver, MediaTek