3GPP TSG-RAN WG4 Meeting #115	R4-2505343

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.1.4

Source:	MediaTek Inc.

Title:	Discussion on demodulation and CSI requirements for 6Rx UE

Document for:	Discussion

# Introduction

Based on the updated WID [1], RAN4 should define performance requirements for 6Rx UE as outlined below. In this contribution, we present initial evaluations and perspectives on the unresolved issues in the WF [2].

6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - 6 MIMO layers performance requirements will be only defined for FWA. - 4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices. - Specify the SDR requirements to support 6Rx

- Discussion

## 2.1 PDSCH requirements

Test scenarios

Issue 2-2: Test scenarios - Way forward - Option 1: Test scenarios without interference: PDSCH Mapping Type A - Option 2: Test scenarios with interference - PDSCH demodulation requirements with inter-cell interference - PDSCH demodulation requirements with intra-cell inter-user interference

For testing scenarios, we prefer not to consider interference. In TS38.101-4, there were many requirements considering interference for 2Rx and 4Rx UE. RAN4 can introduce applicability rules for UE supporting not only 6Rx to verify the interference rejection capability.

Proposal 1: RAN4 consider only “Test scenarios without interference: PDSCH Mapping Type A”.

6 layers requirements

Issue 2-6: Propagation channel and MIMO correlation for 6 MIMO layers - Way forward - Option 1: TDLA 30-10 ULA Low - Option 2: TDLD 30-5 ULA Low Issue 2-8: DMRS configuration for 6 MIMO layers case - Way forward - Option 1: Rel-15 Type 1 DMRS 2+2 - Option 2: Rel-15 Type 1 DMRS 2+0 - Option 3: Rel-18 eType1 DMRS 1+1

For propagation channel, we are OK to use TDLA30-10 as 6 MIMO layers is considered.  Besides, we propose using Rel-15 2+2 to define requirements for FWA 6R6L as Rel-18 enhanced DMRS needs additional UE capabilities.

Proposal 2: Use TDLA30-10 ULA low to define 8T6R6L FWA requirements.

Proposal 3: Use Rel-15 2+2 DMRS configuration to define 8T6R6L FWA requirements.

Similar to the 8-layer case, the Rel-15 2+2 DMRS configuration occupies symbols #2, 3, 10, and 11 for 6-layer transmission. Therefore, if RAN4 agrees to define 8T6R6L FWA requirements with Rel-15 2+2 DMRS configuration, TRS symbols can be configured in symbols #5 and #9.

RAN4 agreed to use 6D+4G+4U in the configuration of special slot for 6Rx requirements at the last meeting. There are only two symbols for PDSCH by considering two PDCCH symbols and two DMRS symbols. We propose not to schedule PDSCH in special slot for 6-layer case as what RAN4 did in 8Rx requirements.

Proposal 4: If Rel-15 2+2 DMRS configuration is used to define 8T6R6L FWA requirements,

- Configure TRS symbols in symbols #{5, 9}

- Not to schedule PDSCH in special slot

Issue 2-12: MCS for 6 MIMO layers case - Way forward - Option 1: MCS13 (Table 2) - Option 2: MCS 13 as starting point (Table 1) - Option 3: 256QAM - Option 4: 64QAM (MCS Table 1) with the same MCS for both CW - Option 5: MCS17 as starting point (Table 1)

Regarding MCS, several options have been proposed at the last meeting. We show some preliminary results based on Table 1 in Annex to assist RAN4 narrowing down the available options.

Figure 1. Simulation results for FWA 8T6R6L with different MCS options (ULA Low and TDLA30-10)

The required SNRs achieving 70% maximum throughput are summarized in Table 1. It can be shown that the required SNR for MCS 13 and MCS 17 are 12.66 dB and 18.63 dB, respectively, which are in a reasonable SNR range and testable. However, the required SNR for 256QAM is 32.82 dB, which is quite high. Therefore, we propose not to consider 256QAM for 8T6R6L requirements.

Table 1. Summary for the required SNR achieving 70% maximum throughput (8T6R6L)

| Device Type/Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| FWA/Rank 6 | 40 / 30 | MCS13 (MCS Table 1) | TDLA30-10 | 8x6 ULA Low | 70 | 12.66 |
| FWA/Rank 6 | 40 / 30 | MCS17 (MCS Table 1) | TDLA30-10 | 8x6 ULA Low | 70 | 18.63 |
| FWA/Rank 6 | 40 / 30 | MCS20 (MCS Table 2) | TDLA30-10 | 8x6 ULA Low | 70 | 32.82 |

Observation 1: When considering 256QAM (MCS20 in Table 2), the required SNR achieving 70% maximum throughput is 32.82 dB, which is quite high.

Proposal 5: Do not consider 256QAM in FWA 8T6R6L requirements.

Observation 2: For FWA 8T6R6L requirements, the required SNRs for MCS 13 and MCS 17 are 12.66 dB and 18.63 dB, respectively, which are in a reasonable SNR range and testable.

Proposal 6: Consider MCS13 or MCS 17 in Table 1 to define FWA 8T6R6L requirements.

4 layers requirements

Issue 2-4: Test cases for 4 MIMO layers for FWA and Handheld UE - Way forward - Option 1: One single test case for 4 MIMO layers that is applicable for both FWA and Handheld UE - Option 2: Two separate test cases for 4 MIMO layers for FWA and Handheld UE - Option 2a: One test case with ULA Low for FWA; Another test case with ULA Medium with β = 0.00718 for Handheld UE - Other options

Handheld UE and FWA differ significantly in terms of form factor and mobility. Handheld UE, such as foldable phones or tablets, are designed to be compact and portable, allowing users to carry them easily and use them on the go. These devices are characterized by their high mobility, enabling seamless connectivity across various locations and supporting features like handovers between cells. In contrast, fixed wireless access equipment is typically larger in form factor and designed for stationary use, providing a stable and reliable internet connection to fixed locations such as homes, offices, and buildings. These devices are often mounted on buildings or installed in specific locations. Based on the distinct properties for FWA and handheld UE, the MIMO correlation and propagation channel should be different when deriving the requirements for 6Rx FWA and handheld UE.

Observation 3: Handheld UE and FWA exhibit significant differences in form factor and mobility.

Proposal 7: Define two sets of 4 layers with different MIMO correlation and propagation channel, one is for FWA and another is for handheld UE.

Issue 2-11: Antenna configuration for 4 MIMO layers case - Way forward - Option 1: 4Tx6Rx - Option 2: 8Tx6Rx

It is possible to transmit 4 layers with 4T6R and 8T6R. Following legacy RAN4 rules, we propose to use the same number for layer and Tx number, i.e., 4T for 4 layers requirements.

Observation 4: In current TS38.101-4, the number of layer and the number of transmitter antenna are the same in PDSCH requirements.

Proposal 8: Consider 4T6R4L to define 4 layers PDSCH requirements.

Issue 2-7-1: Propagation channel for 4 MIMO layers - Way forward - Option 1: TDLA30-10 - Option 2: TDLC300-100 - Option 3: TDLA30-10 and TDLC300-100

For FWA 4 layers requirements, we suggest using the same TDLA30-10 ULA Low as FWA 6 layers requirements. To reduce the testing burden, we can introduce test applicability rules to let UE skip FWA 4 layers requirements if it can pass the FWA 6 layers requirements.

Proposal 9: Consider TDLA30-10 ULA Low to define FWA 4 layers requirements.

Proposal 10: Introduce test applicability rules to let UE skip FWA 4 layers requirements if it can pass the FWA 6 layers requirements.

Spatial multiplexing relies on accurate channel state information (CSI) to separate the layers at the receiver. In high Doppler scenarios, the channel changes rapidly, making it difficult to maintain accurate CSI. In high Doppler scenarios, i.e., TDLC300-100, lower-layer transmission (e.g., fewer spatial streams or more robust modulation and coding schemes) is often preferred because it is more resilient to channel variations. Therefore, we propose to use TDLA30-10 to define handheld UE 4 layers requirements.

Observation 5: It is not suitable for high layer transmission under rapidly changing channels, e.g., TDLC300-100.

Proposal 11: Consider TDLA30-10 ULA Low to define handheld UE 4 layers requirements.

Issue 2-7-2: MIMO correlation for 4 MIMO layers - Way forward - Option 1: ULA Medium RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β425*β925*β425*β125*1 - Option 1a: with β = 0.00718 - Option 1b: with β = 0.005154 - Option 2: ULA Low

As for handheld UE with smaller form factor compared to FWA, we suggest leveraging legacy pragmatic approach used in RAN4. The spatial correlation coefficient in correlation matrices between the i-th and j-th antenna elements is expressed as rij=ρdijΔ2, where dij=(i-j)Δ(N-1) represents the separation between the antenna elements, and ρ denotes the correlation at a reference element separation of Δ. Additionally, N is the total number of antennas.

To be more specific, we suggest using β = 0.07179 for 6Rx (N = 6) to attain the same correlation value as 2Rx, 4Rx and 8Rx between 1st and 2nd antenna, i.e., β2Rx=β4Rx1/9=β6Rx1/25=β4Rx1/36. We show the proposed MIMO correlation for 6Rx in Table 2, where the correlation values for 2Rx, 4Rx and 8Rx in Table 2 are captured in current TS38.101-4.

Table 2. Illustration of proposed MIMO correlation for 6Rx and existing MIMO correlation for 2Rx, 4Rx and 8Rx

|  | 2 Rx antennas | 4 Rx antennas | 6 Rx antennas | 8 Rx antennas |
| --- | --- | --- | --- | --- |
| UE correlation RUE | 1ββ*1 | 1β19β39ββ19*1β19β39β49*β19*1β19β*β49*β19*1 | 1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β425*β925*β425*β125*1 | 1β149β449β949β1649β2549β3649ββ125*1β149β449β949β1649β2549β3649β425*β149*1β149β449β949β1649β2549β925*β449*β149*1β149β449β949β1649β1625*β949*β449*β149*1β149β449β949β2549*β1649*β949*β425*β149*1β149β449β3649*β2549*β1649*β949*β425*β149*1β149β*β3649*β2549*β1649*β949*β425*β149*1 |
| UE correlation matrix with different β | Medium in Table B.2.3.1.2-1 | Medium A in Table B.2.3.1.2-1 |  | Medium B in Table B.2.3.1.2-1 |

Proposal 12: Consider ULA medium with beta = 0.07179 for handheld UE 4 layers requirements.

Issue 2-13: MCS for 4 MIMO layer case - Way forward - Option 1: MCS13 (Table 2) : 64QAM with CR = 567/1024 - Option 2: MCS 13 as starting point (Table 1) : 16QAM with CR = 490/1024 - Option 3: 256QAM - Option 4: 64QAM (MCS Table 1) - Option 5: MCS17 as starting point (Table 1): 64QAM with CR = 438/1024 - Option 6: MCS13 and MCS 17 as starting point

Regarding MCS, several options have been proposed. Figures 2, 3, and 4 present preliminary results derived from Table 1 in the Annex, aimed at assisting RAN4 in narrowing down the available options.

Figure 2. Simulation results for FWA 4T6R4L with different MCS options (ULA Low and TDLA30-10)

| Figure 3. Simulation results for handheld UE 4T6R4L with different MCS options, with α = 0.3 and β= 0.005154, TDLA30-10 | Figure 4. Simulation results for handheld UE 4T6R4L with different MCS options, with α = 0.3 and β= 0.07179, TDLA30-10 |
| --- | --- |

We present some preliminary simulation results for 4L requirements in Table 3. For FWA 4T6R4L, it can be shown that all MCS, including MCS13 (Table 1), MCS 17 (Table 1) and MCS20 (Table 2) are testable. Therefore, we suggest RAN4 considering MCS20 (Table 2) to define FWA 4T6R4L requirements. As for handheld UE, only MCS 13 (Table 1) are testable when considering either “α= 0.3 and β= 0.005154” or “α = 0.3 and β= 0.07179”.

Observation 6: For FWA 4T6R4L, it can be shown that MCS13 (Table 1), MCS 17 (Table 1) and MCS20 (Table 2) are testable when considering ULA low TDLA30-10.

Proposal 13: Consider MCS20 (Table 2) to define FWA 4T6R4L requirements.

Observation 7: For handheld UE 4T6R4L requirements, only MCS 13 (Table 1) are testable when considering either “α= 0.3 and β= 0.005154” or “α = 0.3 and β= 0.07179”.

Proposal 14: Consider MCS 13 (table 1) to define handheld UE 4T6R4L requirements.

Table 3. Summary for the required SNR achieving 70% maximum throughput (4T6R4L)

| Device Type/Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| FWA/Rank 4 | 40 / 30 | MCS13 (MCS Table 1) | TDLA30-10 | 4x6 ULA Low | 70 | 8.81 |
| FWA/Rank 4 | 40 / 30 | MCS17 (MCS Table 1) | TDLA30-10 | 4x6 ULA Low | 70 | 12.74 |
| FWA/Rank 4 | 40 / 30 | MCS20 (MCS Table 2) | TDLA30-10 | 4x6 ULA Low | 70 | 24.11 |
| Handheld UE/Rank 4 | 40 / 30 | MCS13 (MCS Table 1) | TDLA30-10 | 4x6 ULA with α = 0.3 and β= 0.005154 | 70 | 20.81 |
| Handheld UE/Rank 4 | 40 / 30 | MCS17 (MCS Table 1) | TDLA30-10 | 4x6 ULA with α = 0.3 and β= 0.005154 | 70 | NA |
| Handheld UE/Rank 4 | 40 / 30 | MCS20 (MCS Table 2) | TDLA30-10 | 4x6 ULA with α = 0.3 and β= 0.005154 | 70 | NA |
| Handheld UE/Rank 4 | 40 / 30 | MCS13 (MCS Table 1) | TDLA30-10 | 4x6 ULA with α = 0.3 and β= 0.07179 | 70 | 23.55 |
| Handheld UE/Rank 4 | 40 / 30 | MCS17 (MCS Table 1) | TDLA30-10 | 4x6 ULA with α = 0.3 and β= 0.07179 | 70 | NA |
| Handheld UE/Rank 4 | 40 / 30 | MCS20 (MCS Table 2) | TDLA30-10 | 4x6 ULA with α = 0.3 and β= 0.07179 | 70 | NA |

## 2.2 SDR requirements

Issue 3-3: Static channel and precoding for 6Rx - Proposals - Option 1: Introduce new 6Rx static channel. The effective channel matrix, which is a combination of the introduced 6Rx static channel and precoding matrix, should maintain good orthogonality across multi-layers. The effective channel matrices should be full rank and leading no zero input at the receiving antenna. - Option 2: - For 1T6R (Random precoding): H=111111 - For 2T6R (Random precoding): H=111j-jj111-jj-j - For 4T6R (Random precoding): H=111111111-1-1-122j-2j22j3010j305j-33010j-22j2j-22j-3010j305j-3010j - For 8T6R (Fixed precoding with i1,1=i1,2=i2=0): H=34*11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

The current specification, TS38.101-4 Annex B.1, outlines static propagation conditions for configurations with 2, 4, and 8 Rx, which are applied to PDSCH requirements (for LOS channels), CQI requirements (for AWGN channels), and SDR requirements. Consider a MIMO system with Nt transmit antennas and Nr receive antennas. The system can be represented by the following equation:

Y = HPX+N = HeffX + N

where:

  - Y is the Nr×1 received signal vector

  - N is the Nr×1 noise vector

  - H is the Nr×Nt static channel matrix

  - P is the Nt×L precoder

  - X is the L×1 transmitted signal vector

A multi-layer signal is conveyed through an effective channel matrix which is the result of combining the static channel with the precoding matrix. It is noteworthy that all effective channel matrices utilized in the current requirements are fully ranked, also ensuring that no combination of static channel and precoding matrix results in zero input at the receiving antenna. This guarantees exceptional orthogonality across the multiple signal layers.

After checking static channel matrices in Option 2 above, it satisfies the properties mentioned in Option 1, which is “The effective channel matrices should be full rank and leading no zero input at the receiving antenna”. Therefore, we support to define static channel matrices for 6Rx with Option 2 with minor modifications by removing the scaling factor in static channel for 8T6R.

Proposal 15: Support to define the following static channel matrices for 6Rx:

- For 1T6R (Random precoding):

H=111111

- For 2T6R (Random precoding):

H=111j-jj111-jj-j

- For 4T6R (Random precoding):

H=111111111-1-1-122j-2j22j3010j305j-33010j-22j2j-22j-3010j305j-3010j

- For 8T6R (Fixed precoding with i1,1=i1,2=i2=0):

H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

Issue 3-5: SDR requirements for 1024QAM - Agreement - Define SDR requirements for 1024QAM with 2 layers - FFS Define SDR requirements for 1024QAM with 4 layers

To assess the feasibility of introducing SDR 1024QAM 4-layer requirements for 6Rx, RAN4 must first finalize the 6Rx static channel specifications. Without this conclusion, running simulations will not be possible.

Proposal 16: FFS SDR 1024QAM 4-layer requirements after 6Rx static channels are determined.

## 2.3 CSI requirements

Issue 4-1: CQI test for 6Rx - Way forward - Option 1: Not define CQI test - Option 2: Define CQI test - Option 2a: Only define CQI requirements with rank 4 under static channel - Option 2b: Define the following CQI test - Periodic CQI reporting requirements under AWGN - Wideband CQI reporting requirements with inter-cell interference under fading conditions Issue 4-3: RI test - Way forward - Option 1: Not define RI test for 6 layers - Option 2: Define RI test for 6 layers

For the CSI requirements, we believe there is minimal difference between 6Rx and 2Rx, as well as between 4Rx and 8Rx. Consequently, we suggest not defining specific CQI and RI requirements

Proposal 17: Not to define CQI requirements for 6Rx.

Proposal 18: Not to define RI requirements for 6Rx.

# Conclusion

In this contribution, we provide our views about performance requirements for 6Rx UEs. Observations and proposals are summarized as follows.

Proposal 1: RAN4 consider only “Test scenarios without interference: PDSCH Mapping Type A”.

Proposal 2: Use TDLA30-10 ULA low to define 8T6R6L FWA requirements.

Proposal 3: Use Rel-15 2+2 DMRS configuration to define 8T6R6L FWA requirements.

Proposal 4: If Rel-15 2+2 DMRS configuration is used to define 8T6R6L FWA requirements,

- Configure TRS symbols in symbols #{5, 9}

- Not to schedule PDSCH in special slot

Observation 1: When considering 256QAM (MCS20 in Table 2), the required SNR achieving 70% maximum throughput is 32.82 dB, which is quite high.

Proposal 5: Do not consider 256QAM in FWA 8T6R6L requirements.

Observation 2: For FWA 8T6R6L requirements, the required SNRs for MCS 13 and MCS 17 are 12.66 dB and 18.63 dB, respectively, which are in a reasonable SNR range and testable.

Proposal 6: Consider MCS13 or MCS 17 in Table 1 to define FWA 8T6R6L requirements.

Observation 3: Handheld UE and FWA exhibit significant differences in form factor and mobility.

Proposal 7: Define two sets of 4 layers with different MIMO correlation and propagation channel, one is for FWA and another is for handheld UE.

Observation 4: In current TS38.101-4, the number of layer and the number of transmitter antenna are the same in PDSCH requirements.

Proposal 8: Consider 4T6R4L to define 4 layers PDSCH requirements.

Proposal 9: Consider TDLA30-10 ULA Low to define FWA 4 layers requirements.

Proposal 10: Introduce test applicability rules to let UE skip FWA 4 layers requirements if it can pass the FWA 6 layers requirements.

Observation 5: It is not suitable for high layer transmission under rapidly changing channels, e.g., TDLC300-100.

Proposal 11: Consider TDLA30-10 ULA Low to define handheld UE 4 layers requirements.

Proposal 12: Consider ULA medium with beta = 0.07179 for handheld UE 4 layers requirements.

Observation 6: For FWA 4T6R4L, it can be shown that MCS13 (Table 1), MCS 17 (Table 1) and MCS20 (Table 2) are testable when considering ULA low TDLA30-10.

Proposal 13: Consider MCS20 (Table 2) to define FWA 4T6R4L requirements.

Observation 7: For handheld UE 4T6R4L requirements, only MCS 13 (Table 1) are testable when considering either “α= 0.3 and β= 0.005154” or “α = 0.3 and β= 0.07179”.

Proposal 14: Consider MCS 13 (table 1) to define handheld UE 4T6R4L requirements.

Proposal 15: Support to define the following static channel matrices for 6Rx:

- For 1T6R (Random precoding):

H=111111

- For 2T6R (Random precoding):

H=111j-jj111-jj-j

- For 4T6R (Random precoding):

H=111111111-1-1-122j-2j22j3010j305j-33010j-22j2j-22j-3010j305j-3010j

- For 8T6R (Fixed precoding with i1,1=i1,2=i2=0):

H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

Proposal 16: FFS SDR 1024QAM 4-layer requirements after 6Rx static channels are determined.

Proposal 17: Not to define CQI requirements for 6Rx.

Proposal 18: Not to define RI requirements for 6Rx.

# Reference

- RP-250780, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei, HiSilicon

- R4-2504750, Way Forward for [114bis][325] NR_ENDC_RF_Ph4_Demod_6Rx, Huawei, HiSilicon

# Annex: Simulation Assumptions

Table 1. Simulation assumptions for PDSCH requirements

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD pattern |  |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  |  | 30/40 |
| MCS |  |  | MCS13 (MCS Table 1) MCS17 (MCS Table 1) MCS20 (MCS Table 2) |
| MIMO layer and Antenna configuration |  |  | Rank 4: 4T6R Rank 6: 8T6R |
| Propagation condition and antenna correlation |  |  | TDLA30-10 Low |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols (maxLength) for DL |  | Rank 4: Single-symbol Rank 6: Double-symbol |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| Overhead |  |  | 0 |
| N1 and N2 configurations for 8Tx cases |  |  | For 8Tx test: (N1,N2) = (4,1), (O1, O2) = (4,1) For 4Tx test: (N1,N2)=(2,1), (O1, O2) = (4,1) |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 6 layers cases |
| Number of HARQ Processes |  |  | 8 |
| HARQ ACK/NACK bundling |  |  | Not configured |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Test metric |  |  | SNR@70% max TP |