3GPP TSG-RAN WG4 #115	R4-2505351

St. Julians, Malta, May 19-23, 2025

Agenda item:	7.24.4.2		Other RRM Requirements

Source:	Qualcomm Incorporated

Title:						On additional core requirements for NR MIMO Phase 5

Document for:	Discussion

# Introduction

In RAN4 #112, the work on Rel-19 MIMO Phase 5 was started. Several agreements were made to identify impacts on RRM requirements, and several open issues were outlined in [1].

In this paper we provide our views for the open issues identified in the way forward in [1] in RAN4 #114bis.

# Discussion

## UE Reporting Enhancements for CJT Deployments

In the previous RAN4 meetings first discussions about the accuracy requirements for delay and frequency offset reporting were started. Background for this discussion is whether one or multiple samples are needed to provide sufficient accuracy for the reporting, since, depending on the number of required samples, core or performance requirements are impacted in TS 38.133. To investigate the measurement performance, simulation assumptions for delay and frequency offset reporting have been agreed.

One agreement in RAN4 #114bis was that the report resolutions for delay and frequency offset should be set to MD =256 and MFO = 256.

Issue 1-1: On the simulation assumption for delay and frequency offset for CJT calibration

Agreement:

- Update the report resolution to (MD = 256, MFO = 256).

The main discussion in RAN4 #114bis was whether core requirements for delay and frequency offset reporting should be defined. Based on the submitted simulation results, it was agreed that core requirements should be defined, however the number of samples is still FFS in RAN4.

Issue 1-2: How to define RRM core requirements for delay and frequency offset reporting based on the simulation results for CJT calibration?

Agreement:

- Define core requirements for delay and frequency offset reporting for

  - Measurement period (FFS number of samples)

  - Scheduling restriction (if identified)

RAN4 should define what accuracy requirements should be achieved. Based on the simulation assumptions agreed in [2], a TDD system with carrier frequency of 3.5 GHz can be assumed. Under these assumptions, the quantization step sizes are Δt = 0.5 CP / 255 = 0.5 * 2.3 ms / 255 = 4.5 ns and Δf = 0.1ppm of 3.5 GHz / 255 = 1.37 Hz for the most accurate case of MD = 256 and MFO = 256.

For DL, no requirements for coherency are defined. However, requirements for coherent UL MIMO are defined in 38.101-1 as the maximum allowable difference of relative phase error over a time window. The allowable difference of relative phase error is defined as 40 over a time window of 20 ms. We think that this requirement can be also used as a reference for phase coherency in time and frequency domain for delay and frequency offset reporting.

Observation 1: Requirements for coherent UL MIMO are defined in TS 38.101-1 as the maximum allowable difference of relative phase error over a time window. The allowable difference of relative phase error is defined as 40 over a time window of 20 ms.

Proposal 1: RAN4 should discuss whether 40 can be used as a reference for phase coherency in time and frequency domain for delay and frequency offset reporting.

Assuming 40 as reference for phase coherency, we derive the accuracy requirements for delay and frequency offset reporting.

- Assuming τn0 [seconds] is the timing misalignment between TRPs. For a SCS = ∆f and a subcarrier index k, the phase rotation in frequency domain is then given by 2πk∆fτn0.

- Assuming en [ppm] is the frequency offset between TRPs. For a symbol duration T, a symbol index l and a carrier frequency fc, the phase rotation in time direction is given by 2πenfclT.

For a phase rotation of 40 in time domain, we can now do the following calculation:

- 40 corresponds to 0.35 rads. T=130 kHz, l=14 for one slot = 0.5 ms. If we want to achieve a phase coherency across 20 ms, then l=560.

- 0.35=2πenfclT=2πenfcl30 kHz=2πenfc56030 kHz⇒enfc=0.35∙30 kHz560 ∙ 2π≈3 Hz

For a phase rotation of 40 in frequency domain, we can do the following calculation:

- 40 corresponds to 0.35 rads. ∆f=30 kHz, k=12 for one PRB. If we want to achieve a phase coherency of one sub-band of 8 PRB, then k=96.

- 0.35=2πk∆fτn0=192π∙30 kHz ∙ τn0⇒τn0=0.35192π∙30 kHz ≈19 ns

Observation 2: If 40 is assumed as a reference for phase coherency across a time duration of 20 ms and a sub-band size of 8 PRBs, the required accuracy requirements for delay offset and frequency offset reporting are 19 ns and 3 Hz, respectively.

Proposal 2: RAN4 should discuss whether an accuracy of 19 ns and 3 Hz for delay offset, and frequency offset reporting can be considered as accuracy requirement to derive how many samples for measurement are required.

Based on reporting resolution of MD = MFO = 256, it has been derived above that the quantization step sizes are Δt = 4.5 ns and Δf = 1.37 Hz for a time offset of 0.5 CP and a frequency offset of 0.1 ppm, respectively. Based on these quantization step sizes the accuracy requirements above translate to a bin accuracy for the delay offset of roughly 4 bins and for the frequency offset of roughly 2 bins.

In order to understand the currently reported accuracy in more detail, we review the simulations results provided in [5] by MediaTek and in [6] by Ericsson. Figure 2-1 shows the comparison of the reported quantiles for 0.5 CP time offset and 30 kHz SCS

Figure 2-1: Reported Quantiles [5] and [6] for 0.5 CP Time Offset, 30 kHz SCS

Since MD = 256 corresponding to a time offset of one CP, the expected delay offset report is 128 corresponding to the 0.5 CP time offset. In both papers [5] and [6] it seems that the error with respect to 128 is reported. That means for most of the UEs a very small value should be seen. The definition of the quantile is slightly different. Ericsson reports the absolute error in order, i.e., the 5% quantile is always lower than the 5% quantile while MTK seems to calculate the true error value and then drop the sign, i.e., the 5% quantile values in Figure 2-1 are representing negative values. Another very interesting observation is that the reported values by Ericsson depend slightly on the number of samples but are independent on SNR. In the MTK results it is seen that both number of samples as well as an increase in SNR reduces the inaccuracy in the delay offset report. Overall, the measurement quality in the results reported by Ericsson is significantly better.

Observation 3: The results reported by MediaTek and by Ericsson for delay offset reporting show significant differences in measurement accuracy, dependence of accuracy on the number of samples and dependence of accuracy on the SNR.

Another question that has not yet been addressed is whether also core requirements for phase offset reporting should be defined.

Proposal 3: RAN4 should discuss whether core requirements for phase offset reporting should be defined.

## Asymmetric DL sTRP / UL mTRP

In RAN4 #114bis, two main agreements were achieved for asymmetric DL sTRP / UL mTRP, namely that timing requirements will be defined both for FR1 and FR2 and that UL TCI state switching based on SRS does not have impact on RRM requirements.

Issue 2-4: For Scenario#1 in FR2, whether to update the RRM requirements for 2TA enhancement for FR2?

Agreement:

- RAN4 to define timing requirements for FR1 as well as FR2.

Issue 2-5: Whether to define UL TCI state activation requirement based on SRS?

Agreement: No RRM impacts on TCI state switching requirement.

However, several discussion points remained open in RAN4 #114bis [1]. In this contribution we provide our view to the open issues identified in RAN4 #114bis. In RAN4 #114, two scenarios to be covered by RAN4 have been identified

- Scenario #1: in this scenario, the UE is not expected to receive SSB from UL TRP(s)

- Scenario #2: in this scenario, the UE expects to receive SSB from UL TRP(s)

One of the open discussion points in RAN4 114bis was whether the existing RAN1 agreement of a single downlink timing reference is also applied for Scenario #2 (Issue 2-1).

Issue 2-1: For Scenario#2, how to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Agreement:

- Wait for RAN1 reply to the LS on whether the existing agreement for a single downlink timing reference is extended to Scenario#2.

For that purpose, an LS has been sent to RAN1 to ask about their view whether a second downlink timing reference should be introduced [2]. RAN1 has already agreed that the UE maintains two downlink reference timings for the Scenario #2, i.e., when the UE expects to receive SSB from the UL TRP(s).

Observation 4: RAN1 has agreed that the UE maintains two downlink reference timings for the Scenario #2, i.e., when the UE expects to receive SSB from the UL TRP(s).

For Scenario #1, it was already agreed in RAN4 #114 that the uplink transmission timing takes place in (NTA + NTA offset) x Tc before reception of the first detected path of one of the corresponding downlink reference signal(s). This agreement for Scenario #1 needs to be extended to Scenario #2 in a way that addresses that the downlink timing reference can be changed.

Proposal 4: For Scenario #2 the uplink transmission timing for PUCCH/PUSCH/SRS takes place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signals (s) of the reference cell associated with DL/joint TCI state and the first TAG or of the reference cell associated with joint/UL TCI state and the second TAG.

One of the open questions for Scenario #1 is how to handle autonomous timing adjustment (Issue 2-2).

Issue 2-2: For Scenario#1, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Way forward:

- Discuss whether gradual timing adjustment applies to both TAGs or not.

- Discuss whether gradual timing adjustment applies to DL/UL TRP or also to UL TRP.

The topic was discussed in RAN1 #118bis and RAN1 119bis without reaching a clear agreement. However, it seems clear that there is no RAN1 spec impact. In our understanding	the above-mentioned autonomous timing adjustment is the gradual timing adjustment defined in TS 38.133. RAN4 should discuss how gradual timing adjustment should be applied in scenario #1 when there is just one downlink reference signal but two uplink TAGs. Our view is that the same gradual timing adjustment should be applied for both TAGs in case of Scenario #1. For Scenario #2, RAN1 has agreed to support a second downlink reference timing. In this case, two gradual timing adjustments should be applied by the UE, one for each TAG based on the corresponding downlink reference timing.

Proposal 5: For Scenario #1 gradual timing adjustment should be applied with the same value for both TAGs.

Proposal 6: For Scenario #2 two independent gradual timing adjustments with potentially different values should be applied to both TAGs.

In Issue 2-3 it was discussed whether RACH transmission requirements need to be updated for the case of 2 TA.

Issue 2-3: For Scenario#1, whether other updates are needed for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Way forward:

- Discuss whether RACH transmission requirements need to be updated.

We believe the discussion was triggered by considerations of non-perfect synchronization between different TRPs. In this case the RACH timing derived from the single downlink timing reference may not be perfect for the RACH transmission to the UL TRP. However, we do not see how the RACH timing can be corrected to work universally. Hence, we do not prefer to add any additional correction to the RACH timing

Proposal 7: The legacy timing requirements for RACH should be reused for Scenario #1.

In Issue 2-6 the question was raised whether new UL TCI state switching requirements need to be defined for Scenario #2 when two downlink reference timings are maintained. The existing requirements apply for the case of separate uplink TCI states and joint TCI states and support up to four different pathloss reference signals

Issue 2-6: Whether to define UL TCI state activation requirement for Scenario#2?

Way forward: discuss the following options in the next meeting for Scenario#2

- Option 1: No RRM impacts on TCI state switching requirement.

- Option 2: Legacy SSB based UL TCI state activation requirement can be re-used.

Since the agreement in RAN1 is to reuse Rel-17 and Rel-18 unified TCI framework, there can only be up to two UL TRPs transmitting downlink pathloss reference signals. Hence, we think the existing requirements should cover also Scenario #2.

Observation 5: Since the agreement in RAN1 is to reuse Rel-17 and Rel-18 unified TCI framework, there can only be up to two UL TRPs transmitting downlink pathloss reference signals.

Proposal 8: RAN4 should not introduce new UL TCI state switching requirements for Scenario #2.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #114bis. Our observations and proposals are summarized as follows:

UE reporting enhancement for CJT deployments

Observation 1: Requirements for coherent UL MIMO are defined in TS 38.101-1 as the maximum allowable difference of relative phase error over a time window. The allowable difference of relative phase error is defined as 40 over a time window of 20 ms.

Proposal 1: RAN4 should discuss whether 40 can be used as a reference for phase coherency in time and frequency domain for delay and frequency offset reporting.

Observation 2: If 40 is assumed as a reference for phase coherency across a time duration of 20 ms and a sub-band size of 8 PRBs, the required accuracy requirements for delay offset and frequency offset reporting are 19 ns and 3 Hz, respectively.

Observation 3: The results reported by MediaTek and by Ericsson for delay offset reporting show significant differences in measurement accuracy, dependence of accuracy on the number of samples and dependence of accuracy on the SNR.

Proposal 2: RAN4 should discuss whether an accuracy of 19 ns and 3 Hz for delay offset, and frequency offset reporting can be considered as accuracy requirement to derive how many samples for measurement are required

Proposal 3: RAN4 should discuss whether core requirements for phase offset reporting should be defined.

Asymmetric DL sTRP/UL mTRP

Observation 4: RAN1 has agreed that the UE maintains two downlink reference timings for the Scenario #2, i.e., when the UE expects to receive SSB from the UL TRP(s).

Proposal 4: For Scenario #2 the uplink transmission timing for PUCCH/PUSCH/SRS takes place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signals (s) of the reference cell associated with DL/joint TCI state and the first TAG or of the reference cell associated with joint/UL TCI state and the second TAG.

Proposal 5: For scenario #1 gradual timing adjustment should be applied with the same value for both TAGs.

Proposal 6: For Scenario #2 two independent gradual timing adjustments with potentially different values should be applied to both TAGs.

Proposal 7: The legacy timing requirements for RACH should be reused for Scenario #1.

Observation 5: Since the agreement in RAN1 is to reuse Rel-17 and Rel-18 unified TCI framework, there can only be up to two UL TRPs transmitting downlink pathloss reference signals.

Proposal 8: RAN4 should not introduce new UL TCI state switching requirements for Scenario #2.

# References

- R4-2504903, WF on RRM requirements for NR_MIMO_Ph5_Part2, RAN4 #114bis, April 2025

- R4-2504955, Collection of simulation assumptions for CJT calibration in NR MIMO Ph5 Part2, RAN4 #114bis, April 2025

- R4-2502667, LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP / UL mTRP, RAN4 #114, February 2025

- R1-2503091, Draft Reply to RAN4 LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP / UL mTRP, RAN1 #120bis, April 2025

- R4-2504603, Discussion on R19MIMO for other RRM requirements, MediaTek Inc, RAN4 #114bis, April 2025

- R4-2503935, Discussion on CJTC reporting and asymmetric UL TRP requirements, Ericsson, RAN4 #114bis, April 2025