3GPP TSG-RAN WG4 #115	R4-2505353

St. Julians, Malta, May 19-23, 2025

Agenda item:	7.27.3.1		On-demand SSB SCell operation

Source:	Qualcomm Incorporated

Title:	On core requirements for on-demand SSB SCell operation

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 work on Rel-19 NES was started and the work plan has been agreed in [1]. Several agreements have been made for on-demand SSB, and additional open issues have been identified in [2].

In this paper we provide our view for the open issues identified in the way forward in [2] in RAN4 #114bis.

# Discussion

Good progress was made in RAN4 #114bis on several aspects of OD-SSB [2]. In the following subsections we address the open topics.

## OD-SSB based deactivated SCell L3 Measurement

### Case #1: OD-SSB only

In RAN4 #114bis discussions for several open issues for the deactivated SCell measurements took place. Issue 1-1-1 was related to the definition of requirements during the fast measurement window. Sub-issue 1 addresses whether a lower bound based on on-demand periodicity is needed. Sub-issue 2 is related to the prioritization of measurements of single OD-SSB over legacy SSB carriers.

Issue 1-1-1: OD-SSB based deactivated SCell measurement – requirement in FMW

Sub-issue 1: Lower bound for measurement in FMW

Agreement

Introduce UE capability on whether a lower bound is needed, i.e.,

  - The lower bound is not needed (default value), or,

  - A lower bound is needed and based on OD-SSB periodicity of [10] ms

    - Further confirm whether [10] is feasible, and only one value

Sub-issue 2: CSSF

Agreement

- When NW configures single OD-SSB carrier with multiple legacy SSB carriers, CSSF =1 for OD-SSB layer

  - Extend the FMW window time by one OD-SSB periodicity

  - Longer measurement delay of other frequency layers with AO-SSBs are expected

For sub-issue 1, we see the potential benefit of having a lower bound in certain implementations but we do not believe that a lower bound is absolutely necessary. Hence, a UE that does not require this lower bound should also be able to support no lower bound.

Observation 1: A lower bound based on a specific OD-SSB periodicty may provide additional room for optimization in the UE implementation but may also not be required.

Proposal 1: RAN4 should allow to configure a lower bound for measurement in FMW but support the case that no lower bound is needed.

In our understanding sub-issue 2 was only a tentative agreement in RAN4 #114bis but did not get finally discussed due to lack of time. However, we can confirm the tentative agreement from our point of view.

Observation 2: Sub-issue 2 of Issue 1-1-1 was only a tentative agreement in RAN4 #114bis but did not get finally discussed due to lack of time. However, we can confirm the tentative agreement from our point of view.

In our option the most important open issue is the SCell measurement procedure in Issue 1-1-2. The question is how to define the intervall T2, i.e. the start point and the minival intervall durationion of T2. The most controversial discussion in RAN4 #114bis was the definition of the start point. So far in our understanding the implicit assumption of the start point of T2 was that it starts after the fast measurement window. However, in RAN4 #114bis option 2a was proposed that the start point of T2 is the time point of OD-SSB activation.

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure

Sub-issue 1: The start point of T2

Wayforward

The start point of T2 is

- Option 1:

  - The time point of OD-SSB deactivation

- Option 2:

  - The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify.

- Option 2a:

  - The time point of OD-SSB activation

Sub-issue 2: The minimum interval(T2) between multiple FMWs

Wayforward

- Option 1: 5* measCycleSCell

- Option 1a: 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity)

- Option 1b: 5*max (measCycleSCell, DRX cycles)

- Option 2: Minimum measurement period defined in existing spec.

Option 3: The time period of known Scell condition but with OD-SSB periodicity for FR1

In the following we summarize our understanding of the expected behavior for Case #1 and use Figure 2-1 as an illustration. In case #1 and deactivated SCell, OD-SSB is activated at one point in time. Afterwards the UE does measurements based on OD-SSB periodicity. Once the UE sent the first measurement report to the network in our understanding expected network behavior is that the network deactivated OD-SSB again. However, the network does not have to do this and can continue to transmit OD-SSB.

Observation 3: Once the UE sent the first measurement report to the network, expected network behavior is that the network deactivated OD-SSB again.

Observation 4: However, the network does not have to deactivate OD-SSB and can continue to transmit OD-SSB. In this case the UE falls back to legacy measurement requirements based on measCycleScell.

Figure 2-1: Illustration of Time T2

In case that the network deactivates OD-SSB, the concern from UE vendors is that the network activates OD-SSB vry early again and by that enforces the UE to do fast measurement again. Hence, time interval T2 was introduced to avoid this network behavior. The missing agreement is how to define the time T2 yet. We also understand the view from the infra vendors to avoid a very dynamic definition of T2. Hence, a definition based on the time when the UE sends the L3 measurement report has been ruled out.

Observation 5: Infra vendors do not prefer a UE-dependent definition of T2. Hence, a definition based on the time when the UE sends the L3 measurement report has been ruled out.

In our opinion all three remaining options for how to define the start time of T2 are suitable and differ only in details of the definition. Generally, we prefer option 1 and Option 2a compared to option 2 since the start time is clearly defined by a signal coming from the network.

Observation 6: Option 1 and Option 2a have the advantage over Option 2 that they define the start time of T2 based on a signal coming from the network. Option 2 instead is defined based on a time defined in the specification.

The main difference between Option 1 and Option 2a is that in the UE implementation also Tend needs to be considered and applied in the case of Option 2a. If Tend is a single time, both options are equivalent, however, we believe that multiple times Tend may be defined depending on whether cell identification is needed in the FMW or not. In this case we believe UE implementation of Option 2a could become more complicated.

Observation 7: The implementation of Option 2a could become more complicated than Option 1 in case that multiple times will be Tend defined, depending on whether cell identification in the FMW is needed or not.

Proposal 2: RAN4 should adopt Option 1 (time point of OD-SSB deactivation) as the definition of the start time of T2.

Although expected behavior is that the network deactivates OD-SSB once it has received the L3 measurement report, it may take a whole till the network does so. Hence, there may still be some OD-SSB being transmitted after the measurements report has been received as illustrated in Figure 2-1. Once the MAC CE deactivation is received in the network, the UE starts time interval T2. To simplify UE implementation, the length of the time T2 should be identical for all cases. In our opinion Option 1b would serve this purpose well and is already a time duration that is used in UE implementation.

Proposal 3: RAN4 should adopt Option 1b (5*max (measCycleSCell, DRX cycles)) as the definition of the minimum interval of T2.

The network should only be allowed to activate OD-SSB again after the time interval T2. If the network activates OD-SSB within T2, the UE should be allowed to ignore OD-SBB till the end of the time interval T2.

Proposal 4: If the network activates OD-SSB within T2, the UE should be allowed to ignore OD-SBB till the end of the time interval T2.

### Case #2: Always-on SSB and OD-SSB

In RAN4 #114bis, two main agreements were made for case #2.

The first one says that UE follows legacy requirements if the UE has already sent a valid measurement report. If no report has been sent, the SCell is unknown and fast measurement procedure for case #1 applies which is yet to be fully defined as discussed in the previous section.

Issue 1-2-1: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

Agreement

    - The UE follows legacy requirements based on measCycleSCell if UE has sent a valid measurement report before OD-SSB activation indication within the time window as follow.

      - max(5*measCycleSCell,  5*DRX cycles) for FR1, or

      - 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2.

    - Otherwise, the same approach in Case 1 can be reused.

    - Note: The agreement is applied when OD-SSB and AO-SSB are in the same frequency and with same spatial relation.

    - Note 2: T2 discussion is a separate issue.

The second agreement is related to Alt-Time C1 as defined by RAN1. In RAN1 #120, RAN1 discussed the relation in terms of time domain location between OD-SSB and always-on SSB and sent an LS to RAN4 [3]. In this LS the time and frequency location of OD-SSB and always-on SSB are clarified. Alt Time C-1 is an extension of the previous RAN4 agreement that the effective measurement periodicity is half of the OD-SSB periodicity. Consequently, in RAN4 #114bis it was agreed that the effective measurement periodicity (EMP) is the periodicity of the union of the always-on SSB and OD-SSB and equals one of the existing SSB periodicities defined in the 5G NR specifications.

Issue 1-2-2: Requirement for RAN1 Alt Time-C1

Agreement:

- RAN4 to update the agreement of EMP for deactivated SCell measurement and SCell activation to apply to RAN1 Alt Time-C1 scenario.

  - The EMP is the periodicity of the union of AO-SSB and OD-SSB and equals one of legacy SSB periodicities defined in the specification.

  - FFS for other procedures.

Alt Time-C2 as defined in [3] results in a non-periodic time domain pattern of always-on SSB and OD-SSB. In the discussions in RAN4 #114bis it remained open whether requirements should be defined (Issue 1-2-3).

Issue 1-2-3: Requirement for RAN1 Alt Time-C2

Agreement

- Option 1: Define the requirement based on OD-SSB periodicity.

- Option 2: Not to define requirement.

We do not see a strong use case for such a configuration. It can result in an effective SSB burst periodicity less than 5 ms if no additional restrictions are applied. This is not a periodicity supported by SSB in the 5G NR specification. Hence, we prefer not to define requirements for Alt Time-C2.

Proposal 5: RAN4 should not define requirements for Case #2 when the union of always-on SSB and OD-SSB yields to a non-periodic time domain pattern (Alt Time-C2).

As can be seen in the LS to RAN4 [3], RAN1 agreed that always-on SSB and OD-SSB can be location on different non-overlapping frequencies within the same BWP. This is the case when always-on SSB is a CD-SSB on the synchronization raster. In this case, OD-SSB is not on the sync-raster. RAN1 further agreed in RAN1 #120 that this scenario is subject to a separate UE capability and the UE is not required to measure both always-on SSB and OD-SSB. In RAN4 #114bis it remained open whether requirements should be defined for that case (Issue 1-2-4)

Issue 1-2-4: Requirement for scenario 3 (OD-SSB and always-on SSB are within different frequencies)

Agreement

RAN4 to first discuss the RAN4 impact based on RAN2 LS R2-2503015.

- Option 1: RAN4 to define requirement for different frequencies

- Option 2: RAN4 not to define requirements for different frequencies

RAN1 agreed in RAN1 #120 that this scenario is subject to a separate UE capability and the UE is not required to measure both always-on SSB and OD-SSB:

RAN1 #120 Agreement

Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB,

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

  - On-demand SSB is not on sync raster

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: Additional conditions

  - Subject to separate UE capability

  - Note: UE is not required to measure both AO-SSB and OD-SSB

In this case the UE should follow the agreement in Issue 1-2-1 for the deactivated SCell L3 measurement and requirements should not be defined based on the combination of measurements.

Observation 8: For the case that always-on SSB and OD-SSB are on different frequencies within the same BWP, RAN1 has agreed the UE is not required to measure both SSBs.

Proposal 6: For the case that always-on SSB and OD-SSB are on different frequencies within the same BWP, agreement in Issue 1-2-1 can be applied as well, and requirements should not be defined based on the combination of measurements.

In RAN2 #129bis, RAN2 agreed on an additional working assumption for this scenario and sent a LS to RAN4 [6]:

In RAN2#129b, RAN2 discussed how to configure L3 measurement when always-on SSB (A-SSB) and on-demand SSB (OD-SSB) have different center frequency, and made the following Working Assumption:

Working assumption:

- When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB.

Note: The WA was made from RRC signaling perspective. RAN2 relies on RAN4/1 to decide whether to support the scenario when A-SSB and OD-SSB have different center frequencies or not.

RAN1 has already agreed that the center frequencies for always-on SSB and on-demand SSB are within the same BWP. Then it is unclear why a new servingCellMO needs to be introduced and measurements on both center frequencies cannot be handled by one servingCellMO. The RAN2 agreement overrides basic Rel-15 assumption that one serving cell has exactly one servingCellMO. For RedCap, there are already multiple servingCellMO per serving cell, but these measurements objects are for different bandwidth parts.

Observation 9: Since always-on SSB and on-demand SSB are within the same BWP, it is unclear why a new servingCellMO needs to be introduced. The RAN2 agreement overrides basic Rel-15 assumption that one serving cell has one servingCellMO.

Proposal 7: RAN4 should discuss whether introduction of a new servingCellMO is needed.

## MAC CE Timeline

RAN4 replied to a RAN1 LS regarding the MAC CE timeline as follows in [4].

- RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective.

  - Note 1: This does not imply UE shall be ready to process the OD-SSB from time instance A from RAN4 requirement perspective.

- Meanwhile, RAN4 is discussing the additional UE processing/preparation time for OD-SSB reception for RAN4 requirement.

Figure 2-1 illustrates the MAC CE timeline. A MAC CE triggering OD-SSB is received in slot #n and the corresponding PDSCH is acknowledged in slot #(n+m). in this slot, the MAC CE is processed and the information contained in the MAC CE is available in the UE. This delay is unrelated to the OD-SSB and applies to each MAC CE. In case MAC CR triggers OD-SSB, the delay of m slots till the HARQ feedback is sent to the network can remain unchanged in the timeline for MAC CE triggering OD_SSB. According to the regular MAC CE timeline, additional 3 ms are left to the UE to get ready for the reception of the OD-SSB. Hence, Tmin is the minimal time when the UE needs to be ready to process the OD-SSB.

Figure 2-2: MAC CE Timeline for OD-SSB

It is reflected in the Reply LS to RAN1 that the UE may need more time to be ready to process OD_SSB. RAN4 should discuss this additional processing/preparation time for OD-SSB [4].

Observation 10: The UE may need more time than given by the regular MAC CE timeline for aperiodic signals to be ready to process OD-SSB.

The additional processing time may depend on the spectrum (FR1 or FR2), the time domain and/or frequency domain location of OD-SSB and always-on SSB and other configuration parameters of OD-SSB.

Observation 11: The additional processing time may depend on the spectrum (FR1 or FR2), the time domain and/or frequency domain location of OD-SSB and always-on SSB and other configuration parameters of OD-SSB.

In RAN4 #114bis it was agreed that different UE capabilities for the deactivated SCell measurement is introduced, see Issue 1-4-2. An additional processing time of 5 ms was already agreed. The other two values under discussion are [2 ms] and [3 ms]. The difficulty in defining the timeline is that it needs to cover the worst-case scenario. Also, implementation may improve over time. Hence, defining a UE capability with a large span makes sense. 2 ms and 5 ms could therefore be a good decision for the UE capability.

Issue 1-4-2: Additional Processing time for OD-SSB reception – value

Agreement

UE applies the additional processing timing only in deactivated SCell measurement.

- Note: The additional UE processing timing is not applied to SCell activation procedure.

The additional processing timing is T_processing, the value of T_processing:

- Introduce different values with UE capability: [2 or 3]ms and 5ms.

Proposal 8: RAN4 should introduce UE capability for MAC CE based OD-SSB activation in a single carrier with values of 2 ms and 5 ms.

Observation 12: It should be noted that the discussion for the timeline considered only OD-SSB activation for a single carrier. It still needs to be analyzed whether this processing timeline can also be applied for OD-SSB activation of multiple carriers.

In the agreement on Issue 1-4-2 it is noted that the additional processing time is not applied to SCell activation procedure. We believe that this requires further clarification.

In RAN1#120bis it was also agreed that OD-SSB can be indicated by MAC CE for scenario #3B in case #1 and case #2. Scenario #3B is the case when the SCell is already activated. RAN1 agreed that in this case the MAC CE is only updating the transmission parameters.

RAN1 #120bis Agreement

For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE.

- Scenario #3B and Case #1

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

- Scenario #3B and Case #2

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

However, in this case the measurement needs to be reprogrammed by the UE following the new OD-SSB periodicity. This processing requires additional processing time in the UE.

Observation 13: For scenario #3B, additional processing time is required in the UE in case the OD-SSB periodicity is changed by MAC CE

Proposal 9: RAN4 should apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates.

In RAN4 #114bis it was already discussed whether additional processing time is needed for OD-SSB deactivation (Issue 1-4-3). In RAN1 #120 bis, RAN1 already sent an LS related to deactivation of OD-SSB asking RAN4 whether the working assumption in RAN1 can be confirmed [5].

Issue 1-4-3: Additional Processing time for OD-SSB deactivation

Wayforward

- FFS RAN4 to consider defining the same timeline for activating and deactivating OD-SSB

In RAN1 #120 bis, RAN1 already sent an LS to RAN4 related to deactivation of OD-SSB [5].

RAN1 #120bis Agreement

- Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B.

  - Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’.

    - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB.

      - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB

    - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - (Working assumption) T=T_min

    - RAN4 to confirm the above two working assumptions

    - Nslotsubframe,μ is based on the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

The LS from RAN1 is asking whether the UE can expect that OD-SSB is not transmitted in time instance B’ when time instance B’ is T = m+3Nslotsubframe,μ+1 slots after the slot where UE receives a signaling from gNB to deactivate OD-SSB. Essentially, the ask by RAN1 is whether the UE can be fully aware in time instance B’ that no OD-SSB is transmitted in this time instance. As for MAC CE based activation and MAC CE based parameter updates, the MAC CE based deactivation needs to cover the worst case of all possible scenarios. In case that network stops transmission, but UE has not processed MAC CE deactivation command quickly enough, the UE will measure an OD-SSB time location that does not carry OD-SSB anymore. This would mess up filtering in case of L3 measurements and L1 measurements

Observation 14: In case that network stops transmission, UE has not processed MAC CE deactivation command quickly enough, the UE will measure an OD-SSB time location that does not carry OD-SSB anymore. This will mess up measurements.

Certainly, MAC CE deactivation timeline will be shorter in most cases than MAC CE activation timeline in most cases. However, to keep the timeline specification simple, we suggest that RAN4 defines the same timeline for activating and deactivating OD-SSB based on MAC CE.

Proposal 10: To keep the specification simple, RAN4 should define the same timeline for activating and deactivating OD-SSB based on MAC CE.

## OD-SSB based SCell Activation

Issue 1-3-2: The known cell condition – period of known condition

- Wayforward

Reuse the legacy known/unknown cell condition period for FR1 and FR2, i.e. max(5*measCycleSCell, 5*DRX cycles) in FR1 or 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4  in FR2.

In Issue 2-1-1 it was discussed whether requirements for multiple SCell activation should be introduced.

Issue 2-1-1: OD-SSB based multiple SCells activation

Wayforward

- Option 1:

  - RAN4 to define OD-SSB based multiple SCells activation delay requirements.

    - The limitation is needed

    - It doesn’t impact the WI’s completion.

- Option 2:

  - Not define multiple OD-SSB carriers requirement in this release.

In our view, RAN4 should also focus on multiple SCell activation since multiple SCell activation is a mandatory feature. The other Scell activation approaches are optional features in Rel-16, Rel-17 and Rel-18 with less relevance in our view given current deployment status. RAN4 should also distinguish in those features whether OD-SSB is used for deactivated SCell measurement or acceleration of SCell activation. For Rel-17 Fast SCell activation, an aperiodic TRS is applied. For that feature, OD-SSB should only be used for deactivated SCell measurement.

Proposal 11: RAN4 should focus next on OD-SSB based SCell activation delay requirements for multiple SCell activation since multiple SCell activation is a mandatory feature.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #114bis. Our observations and proposals are summarized as follows:

Observation 1: A lower bound based on a specific OD-SSB periodicty may provide additional room for optimization in the UE implementation but may also not be required.

Proposal 1: RAN4 should allow to configure a lower bound for measurement in FMW but support the case that no lower bound is needed.

Observation 2: Sub-issue 2 of Issue 1-1-1 was only a tentative agreement in RAN4 #114bis but did not get finally discussed due to lack of time. However, we can confirm the tentative agreement from our point of view.

Observation 3: Once the UE sent the first measurement report to the network, expected network behavior is that the network deactivated OD-SSB again.

Observation 4: However, the network does not have to deactivate OD-SSB and can continue to transmit OD-SSB. In this case the UE falls back to legacy measurement requirements based on measCycleScell.

Observation 5: Infra vendors do not prefer a UE-dependent definition of T2. Hence, a definition based on the time when the UE sends the L3 measurement report has been ruled out.

Observation 6: Option 1 and Option 2a have the advantage over Option 2 that they define the start time of T2 based on a signal coming from the network. Option 2 instead is defined based on a time defined in the specification.

Observation 7: The implementation of Option 2a could become more complicated than Option 1 in case that multiple times will be Tend defined, depending on whether cell identification in the FMW is needed or not.

Proposal 2: RAN4 should adopt Option 1 (time point of OD-SSB deactivation) as the definition of the start time of T2.

Proposal 3: RAN4 should adopt Option 1b (5*max (measCycleSCell, DRX cycles)) as the definition of the minimum interval of T2.

Proposal 4: If the network activates OD-SSB within T2, the UE should be allowed to ignore OD-SBB till the end of the time interval T2.

Proposal 5: RAN4 should not define requirements for Case #2 when the union of always-on SSB and OD-SSB yields to a non-periodic time domain pattern (Alt Time-C2).

Observation 8: For the case that always-on SSB and OD-SSB are on different frequencies within the same BWP, RAN1 has agreed the UE is not required to measure both SSBs.

Proposal 6: For the case that always-on SSB and OD-SSB are on different frequencies within the same BWP, agreement in Issue 1-2-1 can be applied as well, and requirements should not be defined based on the combination of measurements.

Observation 9: Since always-on SSB and on-demand SSB are within the same BWP, it is unclear why a new servingCellMO needs to be introduced. The RAN2 agreement overrides basic Rel-15 assumption that one serving cell has one servingCellMO.

Proposal 7: RAN4 should discuss whether introduction of a new servingCellMO is needed.

Observation 10: The UE may need more time than given by the regular MAC CE timeline for aperiodic signals to be ready to process OD-SSB.

Observation 11: The additional processing time may depend on the spectrum (FR1 or FR2), the time domain and/or frequency domain location of OD-SSB and always-on SSB and other configuration parameters of OD-SSB.

Proposal 8: RAN4 should introduce UE capability for MAC CE based OD-SSB activation in a single carrier with values of 2 ms and 5 ms.

Observation 12: It should be noted that the discussion for the timeline considered only OD-SSB activation for a single carrier. It still needs to be analyzed whether this processing timeline can also be applied for OD-SSB activation of multiple carriers.

Observation 13: For scenario #3B, additional processing time is required in the UE in case the OD-SSB periodicity is changed by MAC CE

Proposal 9: RAN4 should apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates.

Observation 14: In case that network stops transmission, UE has not processed MAC CE deactivation command quickly enough, the UE will measure an OD-SSB time location that does not carry OD-SSB anymore. This will mess up measurements.

Proposal 10: To keep the specification simple, RAN4 should define the same timeline for activating and deactivating OD-SSB based on MAC CE.

Proposal 11: RAN4 should focus next on OD-SSB based SCell activation delay requirements for multiple SCell activation since multiple SCell activation is a mandatory feature.

# References

- R4-2112508, Work plan for R19 NES, RAN4 #112, August 2024

- R4-2504906, WF on RRM requirements for Netw_Energy_NR_enh_Part1, RAN4 #114bis, April 2025

- R1-2501633, LS on time location of on-demand SSB for SCell, RAN1 #120, February 2025

- R4-2420192, Reply LS on timeline for On-demand SSB operation on SCell, RAN4 #113, November 2024

- R1-2503108, LS to RAN4 on time instance B for on-demand SSB SCell operation, RAN1 #120bis, April 2025

- R2-2503019, LS on on-demand SSB, RAN2, 129bis, April 2025