3GPP TSG-RAN WG4 #115	R4-2505354

St. Julians, Malta, May 19-23, 2025

Agenda item:	7.27.3.2		Adaptation of Common Signal/Channel Transmission

Source:	Qualcomm Incorporated

Title:	On core requirements for adaptation of common signal/channel transmission

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 work on Rel-19 NES has been started. Several agreements have been made for SSD adaptation in RAN4 #114bis in [1] as listed below. The agreements are as follows:

Issue 1-1-1: Scenario clarification based on RAN1 progress

Agreement:

  - Further clarification on the scope of RRM impact for SSB adaption:

    - Existing SSB burst periodicity (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms).

    - SSB adaptation in SCell for NCD-SSB.

    - Time domain positions of SSBs within a burst is same before and after SSB adaptation

  - Note: Above could be further updated based on further RAN1 progress if any.

In this paper we provide our views for the open issues identified in the way forward in [1]

# Discussion

In RAN4 #114bis it has been identified that further discussion is required how to define Tfirst_SSB considering that additional processing/preparation time in the UE may be needed before measurements based on the SSB after the SSB adaptation can be done.

Issue 1-2-1: L1 requirements at transition

Agreement

  - RAN4 FFS how to define Tfirst_SSB considering the processing/preparation time based on further RAN1/2 progress on SSB adaptation command timeline.

  - No impacts on section 9.5.3 of L1 reporting requirements due to SSB adaptation, unless identified by other WGs.

In RAN2 #129bis, it was decided that no MAC CE will be introduced to indicate SSB adaptation in addition to DCI 2-9 based SSB adaptation agreed in RAN1. Hence, RAN4 only needs to consider additional processing/preparation time for DCI 2-9 based SSB adaptation.

Observation 1: Since RAN2 decided that no MAC CE will be introduced for SSB adaptation, RAN4 only needs to consider additional processing/preparation time for DCI 2-9 based SSB adaptation.

Tfirst_SSB should instead be defined as the first SSB that leaves enough processing/preparation time after the reception of the switch command in DCI 2-9. In TS 38.213, the timeline for DCI 2-9 is defined as 3 ms for activation or deactivation of a current cell DTX operation or cell DRX operation for a second serving cell. It is questionable whether this timeline can be directly applied for SSB adaptation since the UE needs to be ready to process the SSB with the new periodicity. Rather than the Rel-18 3ms timeline for DCI 2-9, a timeline like the MAC CE based timeline for OD-SSB that is also under discussion in RAN4 may be applicable for DCI 2-9.

Observation 2: Tfirst_SSB cannot be defined as the first SSB after the reception of the switch command in DCI 2-9, but additional processing time needs to be considered.

Observation 3: Rather than the Rel-18 3ms timeline for DCI 2-9, a timeline like the MAC CE based timeline for OD-SSB may be applicable for DCI 2-9. It remains to be seen by how much the timeline may be shortened since DCI 2-9 is applied.

Proposal 1: RAN4 should discuss the additional processing time required for DCI 2-9 based SSB adaptation. The MAC CE timeline for OD-SSB activation should serve as a starting point, i.e., {[2ms]/[3ms], 5ms}.

For MAC CE based OD-SSB activation/deactivation, RAN4 has agreed to introduce a UE capability that allows to signal the support of shorter or longer processing delays to the network. Since DCI 2-9 can be sent to multiple UEs to adapt SSB burst periodicity, the network may follow in this case the longest processing delay signaled by any UE. Hence, it is not clear whether a UE capability should be introduced or whether a fixed additional processing delay should be added like chapter 11.5 in TS 38.213.

Proposal 2: RAN4 should discuss whether a UE capability for the additional processing delay for DCI 2-9 should be introduced or whether the additional processing delay should be directly captured in the RAN1 specification TS 38.213.

RAN1 agreed in RAN1 #120bis on the following working assumption:

RAN1 #120bis Working Assumption

When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213].

- FFS: how to determine the first [possible] SSB burst.

The working assumption states that the RAN1 timeline is defined based of the view of the gNB when SSB is transmitted but it leaves it FFS how the first possible SSB is determined, i.e., when the first SSB after SSB adaptation can be received. The timeline from a reception point of view should be defined by RAN4. To avoid misunderstandings in RAN1, RAN4 should send an LS to RAN1 about the potential RAN4 agreement on the reception timeline from UE point of view

Proposal 3: Once the DCI 2-9 timeline has been agreed in RAN4 from a reception point of view in the UE, RAN4 should send an LS to RAN1 about the RAN4 agreement.

In RAN4 #114bis it was also discussed whether L3 requirements for SSB adaptation should be defined. It was agreed that for serving cell the legacy L3 measurement framework is applied. In addition, it was concluded that based on RAN2 agreements neighbor cell requirements are not impacted [2].

Issue 1-3-1: L3 requirements for SSB adaptation

Agreement:

- RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation.

    - For neighbour cell L3 measurement, legacy requirements apply.

Open issue for further discuss:

- For SCell with SSB adaptation (deactivated SCell or activated SCell), wait for more RAN2 progress on additional SMTC configuration, and discuss following aspects:

- Whether to define L3 measurement requirement at transition.

In RAN2 #129bis, it was decided that for SSB adaptation multiple additional SMTC configurations are configured via RRC, together with the mapping between SSB burst periodicity and SMTC configuration. It was also decided that upon reception of DCI 2-9, the UE selects one SMTC configuration accordingly.

Observation 4: Based on RAN2 decision in RAN2 #129bis, at one point in time only one SMTC configuration is active. One SSB burst periodicity is mapped to one SMTC configuration and the DCI 2-9 selects one and only one SMTC configuration.

Formally, SSB burst periodicity and SMTC periodicity are independent of each other and a reasonable configuration setting implies SMTC periodicity ≥ SSB burst periodicity. The set of allowed periodicities is {5, 10, 20, 40, 80, 160} ms both for SMTC and SSB burst periodicity. Since the SMTC configuration only defines the SMTC periodicity but not the SSB burst periodicity, the RAN2 agreement above implies that the SMTC periodicity is changed.

Observation 5: Since the SMTC configuration only defines the SMTC periodicity but not the SSB burst periodicity, the RAN2 agreement to map the SSB burst periodicity to an SMTC configuration implies that the SMTC periodicity is changed.

The open question is whether to define L3 requirements for the transition period. So far, the assumption is that the transition period is defined as Tfirst_SSB + meas_period(P2), where Tfirst_SSB is the time till the first SSB with the new period P2 is received and meas_period(P2) is the measurement period based on the new periodicity P2. RAN4 should investigate whether this definition can also be applied for L3 measurements.

Proposal 4: RAN4 should investigate whether the definition of the transition period, i.e., Tfirst_SSB + meas_period(P2) can also be applied for L3 measurements.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #114bis. Our observations and proposals are summarized as follows:

Observation 1: Since RAN2 decided that no MAC CE will be introduced for SSB adaptation, RAN4 only needs to consider additional processing/preparation time for DCI 2-9 based SSB adaptation.

Observation 2: Tfirst_SSB cannot be defined as the first SSB after the reception of the switch command in DCI 2-9, but additional processing time needs to be considered.

Observation 3: Rather than the Rel-18 3ms timeline for DCI 2-9, a timeline like the MAC CE based timeline for OD-SSB may be applicable for DCI 2-9. It remains to be seen by how much the timeline may be shortened since DCI 2-9 is applied.

Proposal 1: RAN4 should discuss the additional processing time required for DCI 2-9 based SSB adaptation. The MAC CE timeline for OD-SSB activation should serve as a starting point, i.e., {[2ms]/[3ms], 5ms}.

Proposal 2: RAN4 should discuss whether a UE capability for the additional processing delay for DCI 2-9 should be introduced or whether the additional processing delay should be directly captured in the RAN1 specification TS 38.213.

Proposal 3: Once the DCI 2-9 timeline has been agreed in RAN4 from a reception point of view in the UE, RAN4 should send an LS to RAN1 about the RAN4 agreement.

Observation 4: Based on RAN2 decision in RAN2 #129bis, at one point in time only one SMTC configuration is active. One SSB burst periodicity is mapped to one SMTC configuration and the DCI 2-9 selects one and only one SMTC configuration.

Observation 5: Since the SMTC configuration only defines the SMTC periodicity but not the SSB burst periodicity, the RAN2 agreement to map the SSB burst periodicity to an SMTC configuration implies that the SMTC periodicity is changed.

Proposal 4: RAN4 should investigate whether the definition of the transition period, i.e., Tfirst_SSB + meas_period(P2) can also be applied for L3 measurements.

# References

- R4-2504907, WF on RRM requirements for Netw_Energy_NR_enh_Part2, RAN4 #114bis, April 2025

- R2-2501484, Reply LS on SSB adaptation, RAN2 #129, February 2025