3GPP TSG-RAN WG4 #115	R4-2505356

St. Julians, Malta, May 19-23, 2025

Agenda item:	7.27.4		RRM performance requirements

Source:	Qualcomm Incorporated

Title: 						On RRM performance requirements for Rel-19 NES

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 work on Rel-19 NES was started and the work plan has been agreed in [1]. According to the work plan, the work on RRM performance requirements will start in RAN4 #115.

In this paper we provide our preliminary thoughts on the RRM performance test cases for Rel-19 NES.

# Discussion

We provide a high-level list of scenarios that should be considered in the Rel-19 RRM performance requirements and in the design of the corresponding test cases.

- OD-SSB

  - L3 measurement: Main goal should be to test the fast measurement mode. Focus could be on unknown SCell with always-on SSB. Then the test could be that L3 measurement report is sent out within duration of fast measurement window once OD-SSB gets activated the first time and FMW gets triggered.

  - SCell activation: Improvements in SCell activation may only be visible if there is a large delta between always-on SSB and OD-SSB. The test case should be defined in such a way that the requirement for OD-SSB cannot be met in case always-on SSB is present.

- OD-SIB1: It is sufficient to test UL WUS transmission and OD-SIB1 reception as part of cell reselection. No separate tests for UL WUS and OD-SIB1 reception are needed.

  - Option 1 (preferred): OD-SIB1 reception is not related to RRM performance but rather to protocol or functional performance testing. This can be more adequately tested in RAN5 protocol testing rather than RRM RAN4 performance testing.

  - Option 2: if RAN4 believes that a RRM performance test case should be introd8ved, it may be tested that the cell re-selection delay to a newly detectable cell and to an already detected cell is within Δ1 and Δ2 seconds.

    - Default assumption for TSI-NR could be 1280 ms for FR1 intra-frequency, perhaps adding an additional Δ. There is no need to specify more details on UL WUS transmission and OD-SIB1 reception.

    - It should be discussed whether test cases for FR2 and inter-frequency scenarios should be included.

- SSB adaptation: During the discussion of the core requirements, it has been discussed to define requirements for L1-RSRP, L1-SINR, candidate beam detection, SCell activation, L3 measurements.

  - The tests should cover FR1 and FR2.

  - TSSB and TSMTC periodicity updates should be tested based on DCI 2-9.

  - Since for L1-RSRP and L1_SINR reporting, the requirement when the measurement report is sent is an upper bound, differences may only be visible for large deltas in the periodicity before and after SSB adaptation.

  - The same argument holds for SCell activation. In this case, the focus could be on unknown SCells since here more SSBs are required. Then differences in activation delays may be visible for short and large periodicities.

# Conclusions

In this paper we provided our preliminary thoughts on the RRM performance test cases for Rel-19 NES. RAN4 should consider these proposals for the definition of the performance requirements tests.

# References

- R4-2112508, Work plan for R19 NES, RAN4 #112, August 2024