3GPP TSG-RAN WG4 #115	R4-2505358

St. Julians, Malta, May 19-23, 2025

Agenda item:	7.24.6		Demodulation performance and CSI reporting requirements

Source:	Qualcomm Incorporated

Title:	On demodulation performance requirements for Rel-19 MIMO Phase 5

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 started working on the RAN1-led WI for NT MIMO Phase 5. The objectives of the WI as defined in [1] are as follows:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1

  - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

  - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook

    - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format

    - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

The status of the work item is reported in [2]. In this paper, we share our views on the impact of NR MIMO Phase 5 topics on demodulation / CSI performance requirements

# Discussion

The work item has five main topics. In the table below we summarize our high-level view whether the topic has RRM impact and which specification document should cover the corresponding requirements.

| Topic | Demod / RRM performance impact | Specification |
| --- | --- | --- |
| UE-initiated/event-driven beam management, targeting FR2 | RRM impact | TS 38.133 |
| 128 CSI-RS ports, targeting FR1 | Demod impact | TS 38.101-4 |
| reporting enhancement for CJT deployments | RRM impact | TS 38.133 |
| non-coherent UL codebook for three antenna ports | UL demod impact? | 38.104? |
| enhancement for asymmetric DL sTRP/UL mTRP deployment | RRM impact | TS 38.133 |

In the following sections, we provide further details for our view

## General Considerations

UE-initiated / even triggered beam management deals with overhead and potentially latency reduction of beam reporting. This topic impacts RRM core and performance requirements but no demodulation requirements should be introduced.

Proposal 1: RAN4 should not introduce demodulation and/or CSI reporting requirements for UE-initiated / even triggered beam management.

Reporting enhancements for coherent joint-transmission deployments covers delay, phase and frequency misalignments between TRPs under non-ideal synchronization and backhaul assumptions. The measurement and reporting accuracy of the phase, frequency and delay misalignment estimations is crucial for this feature to show performance gains. There are already detailed discussions in RRM what core and performance requirements for reporting delay and measurement accuracy should be defined. However, the final gain of this feature depends very much how the network will apply the measurements reports for doing pre-compensation of phase, frequency and delay misalignments. This can be hardly covered in demod tests since it would require specifying how the test equipment should apply the reports for transmission.

Observation 1: There are already detailed discussions in RRM what core and performance requirements for reporting delay and measurement accuracy should be defined.

Proposal 2: RAN4 should not introduce demodulation and/or CSI reporting requirements for reporting enhancements for CJT deployments.

Non-coherent uplink codebook for three antenna ports does not have downlink impact. It should be discussed further in RAN4 whether base station performance requirements should be defined in TS 38.104.

Observation 2: Non-coherent uplink codebook for three antenna ports does not have downlink impact.

Proposal 3: RAN4 should not introduce demodulation and/or CSI reporting requirements for non-coherent uplink codebook.

Proposal 4: It should be discussed further in RAN4 whether base station performance requirements should be defined in TS 38.104.

For the topic of enhancement for asymmetric DL sTRP/UL mTRP deployment no demodulation and CSI reporting enhancements need to be defined since there is now change in DL transmission or CSI reporting.

Proposal 5: RAN4 should not introduce demodulation and/or CSI reporting requirements for asymmetric DL sTRP/UL mTRP deployment.

## 128 CSI-RS Ports

This section discusses relevance of demodulation and CSI requirements relevant and corresponding proposals with greater than 32 ports (up to 128 ports).

### 2.2.1 Demodulation requirements

The demodulation procedure of the UE does not change with the precoder applied at gNB. Moreover, in RAN4, random PMI is used to specify PDSCH demodulation requirements. Higher number of ports at gNB with random PMI does not change the PDSCH throughput performance. Therefore, we think PDSCH demodulation requirements are not required.

Proposal 6: RAN4 not to consider PDSCH demodulation requirements.

### CQI requirements

The purpose of CQI requirements is to select the highest transport format possible while tracking channel variations. In RAN4, this aspect is measured by a throughput ratio with a constant PMI. Since higher number of ports is not expected to change the CQI performance, new CQI requirements are not required.

Proposal 7: RAN4 not to consider new CQI requirements with higher number of CSI-RS ports.

### PMI requirements

With right PMI reported by UE and appropriately selected by gNB, better DL throughput performance can be achieved. Since a narrower and more directed beam with higher number of ports achieves higher precoder gains, it is required to specify requirements for Rel-19 codebooks with greater than 32 ports.

Although there are multiple Rel-19 codebooks available, the advantages of higher number of ports can be demonstrated with Type I single panel codebook. Specifically, Type I codebook is sufficient to measure the higher relative gain in throughput by following reported PMI compared to a random PMI. Since 64 port support is the basic UE feature, we propose the following:

Proposal 8: RAN4 to specify PMI requirements for Rel-19 single panel Type I codebook with 64 ports.

The number of CSI-RS resources (2 x 32 or 4 x 16) and their allocation in a single slot can be discussed further.

Since narrower beams are possible with (N1, N2) = (16,2) than with (8, 4), higher relative throughput is achievable with (16 2) antenna port configuration.

Proposal 9: RAN4 to use (16,2) as (N1, N2) for Rel-19 codebook PMI requirements.

Rank restriction: Test settings with rank restriction 2 with 32 ports in section 6.3.3.1.4 of [3] can be a starting point for Rel-19 codebook requirements. With rank2 selection, there is no difference in the spatial dimension basis vectors of Scheme-A and Scheme-B.

Proposal 10: RAN4 to use rank2 for Rel-19 codebook PMI requirements.

### RI requirements

Since the number of layers possible does not change whether Tx ports are 32 or higher, no new rank requirements are needed.

Proposal 11: RAN4 not to specify new RI requirements for Rel-19 codebook.

## SRS Port Grouping

The SRS port grouping for 8Rx devices enables low complexity receivers supporting up to 8 layers with 8Rx. Each set of 4Rx is used to demodulate a codeword up to 4 layers.  This scheme involves new SRS port grouping for antenna switching and appropriate beamforming at gNB based on channel reciprocity.

Observation 3: Please note that for a 4L single codeword case, the UE demodulation performance with 8Rx is like a simplified 8Rx receiver and/or UE demodulation with 4Rx is like a 4Rx receiver.

For 2 codeword case (rank4 + rank4), the zero-forcing precoder to suppress interference across codewords shall suboptimal than achievable performance assuming 8Rx UE simplified MMSE receivers.

With full CSI possible in TDD, the gNB can choose Singular Value Decomposition (SVD) based precoders that can fully diagonalize the channel matrix to achieve better performance than PMI feedback based precoder selection which provides limited channel feedback. Therefore, the minimum performance requirements for this scheme can be derived based on existing RAN4 demod requirements.

Although there is no precedent in RAN4 to have demod requirements for SRS based precoders, we would like RAN4 to discuss reuse of existing demod requirements for the new SRS port grouping scheme. Reusing the existing requirements does not involve additional simulation work too.

Proposal 12: RAN4 to discuss reuse of existing demod requirements for SRS port grouping scheme.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #114bis. Our observations and proposals are summarized as follows:

General Consideration:

Proposal 1: RAN4 should not introduce demodulation and/or CSI reporting requirements for UE-initiated / even triggered beam management.

Observation 1: There are already detailed discussions in RRM what core and performance requirements for reporting delay and measurement accuracy should be defined.

Proposal 2: RAN4 should not introduce demodulation and/or CSI reporting requirements for reporting enhancements for CJT deployments.

Observation 2: Non-coherent uplink codebook for three antenna ports does not have downlink impact.

Proposal 3: RAN4 should not introduce demodulation and/or CSI reporting requirements for non-coherent uplink codebook.

Proposal 4: It should be discussed further in RAN4 whether base station performance requirements should be defined in TS 38.104.

Proposal 5: RAN4 should not introduce demodulation and/or CSI reporting requirements for asymmetric DL sTRP/UL mTRP deployment.

128 CSI-RS Ports

Proposal 6: RAN4 not to consider PDSCH demodulation requirements.

Proposal 7: RAN4 not to consider new CQI requirements with higher number of CSI-RS ports.

Proposal 8: RAN4 to specify PMI requirements for Rel-19 single panel Type I codebook with 64 ports.

Proposal 9: RAN4 to use (16,2) as (N1, N2) for Rel-19 codebook PMI requirements.

Proposal 10: RAN4 to use rank2 for Rel-19 codebook PMI requirements.

Proposal 11: RAN4 not to specify new RI requirements for Rel-19 codebook.

SRS Port Grouping

Observation 3: Please note that for a 4L single codeword case, the UE demodulation performance with 8Rx is like a simplified 8Rx receiver and/or UE demodulation with 4Rx is like a 4Rx receiver.

Proposal 12: RAN4 to discuss reuse of existing demod requirements for SRS port grouping scheme.

# References

- RP-242395, WID revision: NR MIMO Phase 5, RAN #105, September 2024

- RP-250053, Status Report NR MIMO Phase 5, RAN #107, March 2025