3GPP TSG-RAN WG4 Meeting #115	R4-2505368

Malta, May 2025

Source	:	Qualcomm Incorporated

Title	:	On UE RF requirements for the LPWUR

Agenda item	:	7.28.3.3

Document for	:	Approval

# 1.	Introduction

In this contribution we present the derivation for the proposed ACS requirement, as well as more general proposals.

# 2. 	Discussion

## 2.1	High level aspects

### 2.1.1	The fragmentation problem

RAN1 has already defined separate capabilities for idle vs connected mode support and ability to demodulate OFDM content in LPWUS. See partial excerpt below from current state of RAN1 UE feature list [1]

| Features | Index | Feature group |
| --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM [overlaid] sequence |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence |

These capabilities already provide considerable flexibility to tailor implementation to target scenarios for the UE. For example, an inexpensive IoT type device that is mostly IDLE and very cost sensitive may only support 62-1. On the other hand, a handheld UE that is mostly in CONNECTED state may choose 62-2 or 62-2a. An extremely cost sensitive device may choose 62-1a, etc. If RAN4 now decides to add another dimension to these, there would be 8 variants for which to set requirements, which makes the requirement framework unnecessarily complicated, and complicates handling of UEs in a real deployment.

It is therefore proposed that:

Proposal 1: RAN4 type 1 UEs are mapped to support of FG 62-1 and 62-2, type 2 UEs are mapped to support of FG 62-1a and 62-2a. No fragmentation beyond RAN1 FG 62-1 is justified from RAN4 perspective.

If any further fragmentation is required, technical arguments supporting system benefit must be first agreed on.

### 2.1.2	Spec. wording

The following guidance is captured in the last WF [3]:

Issue 4-1-1: Whether LR can be tested directly Agreements: - Down select below two approaches for LP-WUR UE RF requirements: - Option 1: stick to 1% MDR LP-WUS as performance metric, but with a note in TS that LP-WUR ON is not mandated. - Option 2: update it to 1% MDR of PDCCH triggered by LP-WUR

In a previous contribution [5], we have argued that:

Observation 2: MDR of LPWUS is not directly testable, because the network or TE cannot enforce or verify that a feature-compliant UE engages its LPWUR for the entire duration of the test.

This means that option 1 is directly testable only after one of the conditions listed above is guaranteed by some non-RAN1/2 intended behavior (AKA test mode). A quick comparison is provided below incorporating this conclusion:

|  | 1% MDR LPWUS | 1% MDR of PDCCH triggered by LPWUR |
| --- | --- | --- |
| Principle | Test LPWUR directly | Ensure PDCCH reliability is not compromised by this power saving feature, no special behavior for test required |
| Relationship between LR performance and spec. wording | Good, direct verification possible | Only indirect verification possible, requires dependence on additional processes like PDCCH decoding and UL ACK/NACK |
| Precedent in 38.101? | No | Yes (see PDCCH decoding with energy saving in 38.101-4) |
| Test mode implication | *TBD how LPWUR can be forced to stay ON* LPWUS cannot be directly tested without test mode | No implication of test mode, verification depends only on native behavior as defined by RAN1 and RAN2 |
| New simulations required? | No, agreed sim. assumptions cover | No, test method can be configured so additional processes are not expected to add significant MU (left up to RAN5) |

It is evident that both options have drawbacks. The most significant of those is the implication of a test mode in the core requirement for option 1(LPWUS MDR must be directly verified). The UE RF core requirement cannot specify some behavior not explicitly designed by RAN1 or RAN2. A test mode is sometimes justifiable, but its definition (including applicability) is under the purview of RAN5 who define the rest of the test procedure. In this case, option 2 does provide a method to specify core requirements without relying on a test mode and therefore is the preferred direction.

We note that option 2 does not imply any change to existing RAN4 procedure beyond the wording used in the specification.

Proposal 2: Option 2 wording (‘PDCCH MDR triggered by LPWUS’) is used for the UE RF core requirements for the Rel-19 LPWUS feature, rather than ‘MDR of LPWUS’ which mandates a test mode for the feature.

## 2.2	REFSENS

### 2.2.1	Threshold SNR

#### 2.2.1.1	‘OFDM Rx’

The simulated threshold SNR vs MDR characteristic for OFDM capable receivers is captured in the figure and tables below for < 1% FAR and M=4, L=4, 5 info bits.

Several cases are simulated for the sake of understanding impact of time/frequency: ideal t/f with no correction, idle mode t/f correction, connected mode t/f correction. The idle and connected mode curves are different owing to larger expected time and frequency drift in idle mode, itself due to the longer reference signal (LPSS) periodicity compared to connected mode (SSB). Also included is a short sequence, or the minimum length of LPWUS to convey the 5-bit subgroup id. For L = 4, this requires 3 ON chips. For longer sequences, the LPWUS subgroup id is assumed to be repeated in the ON chips (3 ON chips at a time). The repetition detail is not resolved in RAN1 as of this writing.

Figure 2.2.1.1 – 1: OFDM-capable receiver performance projections

The 1% MDR values are captured in the table below:

Proposal 3: Consider the following threshold SNR values for the OFDM-capable LPWUR for the UE RF requirements

|  | OFDM-capable Rx, SNR threshold for 1% MDR, with 1% FAR (dB) |  |  |
| --- | --- | --- | --- |
| Sequence length | Ideal (no error, no t/f correction) | CONN. mode (w/ t/f correction) | IDLE mode (w/ t/f correction) |
| 32 | -13.8 | -12.0 | -9.5 |
| 16 | -11.9 | -10.5 | -8.0 |
| 8 | -9.9 | -8.7 | -6.6 |
| Short (3) | -6.8 | -5.5 | -4.5 |

Figure 2.2.1 – 1: ED receiver performance projections, no t/f error or tracking

#### 2.2.1.2	‘OOK Rx’

The simulated SNR vs MDR for envelope detector (ED) receivers only capable decoding OOK is captured in the figure and tables below for < 1% FAR.

Only the ideal case is simulated, where the signal is assumed to have no time or frequency offset and the UE does not implement a correction loop. Also, compared to the OFDM case, there is no concept of a short sequence, so this case is not simulated.

For realistic cases, data from the ideal case is extrapolated using the observed degradations in the OFDM Rx from implementing the time/frequency loops. Note that using degradation from the OFDM Rx is more conservative due to higher sensitivity to time and frequency errors.

The SNR results for 1% MDR are tabulated below:

Proposal 4: Consider the following threshold SNR values for the envelope detector based LPWUR (‘OOK-based’) for any relevant UE RF requirements

|  | Envelope detector Rx, SNR threshold for 1% MDR, with 1% FAR (dB) |  |  |
| --- | --- | --- | --- |
| Sequence length | Ideal (no error, no t/f correction) | CONN. mode (w/ t/f correction) | IDLE mode (w/ t/f correction) |
| 32 | -9.2 | -7.4 | -4.9 |
| 16 | -7.5 | -6.1 | -3.6 |
| 8 | -5.9 | -4.7 | -2.6 |

#### 2.2.1.3	Summary of SNR simulation

The simulated SNR threshold ranges from -4 to -12 dB for OFDM capable receivers and -2 and -8 dB for OOK receivers. The windows can be simultaneously contracted and aligned by choosing the correct LPWUS configurations for the two receiver types, for example 16 bit for OOK (-3 to -6) and short sequence for OFDM-based (-4 to -6). For the OFDM-capable receiver, a short sequence is especially attractive because whether and how to specify repetitions is still under discussion in RAN1. Consequently, the gain with longer OFDM sequences is uncertain as of this writing.

Finally, adopting different LPWUS for different LPWUR types is likely to reflect deployments, where network may group UEs with similar capabilities into subgroups. The LPWUS for each subgroup can then be optimized for most economical use of resources for those set of UEs. For example, a short sequence for OFDM capable receivers, and a longer one for OOK Rx.

Proposal 5: For FR1, the requirement is written around a 16-bit OOK sequence for envelope detector based LPWUR and a ‘short’ OOK sequence (3 bits long for L=4 and number of bits = 5) for OFDM capable LPWUR.

### 2.2.2	Refsens impact of desensitization due to legacy RBs

RAN4 have already communicated to RAN1, targeted NF + IM values without any warning about further desensitization due to adjacent non-zero power DL RBs in the system.

To satisfy the coverage criterion in field operation, typically in the presence of legacy DL RBs, it is self-evident that the sensitivity value derived from the agreed signal configuration (no legacy RBs) will be optimistic by the desensitization caused by the legacy RBs. Now, the agreed ASCS desens values are minor, and are comparable to the rounding error in the LS to RAN1. Consequently, there ought to be no real-world impact to implementation, but it is important to capture this correction explicitly in the requirement derivation process for future traceability.

Proposal 6: For future traceability as well as consistency with information sent to RAN1 via LS, the sensitivity equation is modified as:

REFESNS = -174 dBm/Hz + 10*log(RxBW_Hz) + NF + SNR + IM – desens_ASCS

## 2.3	ACS

### 2.3.1	ACS agreements

At a high level, RAN4 have made the following agreements pertaining to ACS:

[2] Issue 2-4-2: General ACS requirements value and guard RB Agreements: - For ACS evaluation, companies are encouraged to provide the simulation results by assuming - Baseline: the same interference experienced by LP-WUR and main radio. - Optional: the different interference level for LP-WUR and main radio [4] Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB Agreements - Same interference signal level as MR for LP-WUR ACS requirements - FFS at least for one set of Rx requirements. - The LP-WUS level is LP-WUS REFSENS+[X]dB, X=14 as starting point - FFS whether two sets of LP-WUS wanted signal level can be defined. [4] Issue 2-4-2: Whether adopt the same wanted power level for Set-2 LR? Agreement: - The same set of the interference signal level defined for both two types of the LP-WUS requirements.

Using the highlighted agreements as a basis, it is possible to construct an ACS requirement for the LPWUS feature. The first task is to translate ‘same interference level’ to mathematical form so it can be reproduced for all channels in all frequency ranges. The next step would be to use RAN4 assumptions for various parameters, so the actual ACS requirement as well as the ACS test conditions can be specified.

### 2.3.2	‘Same interferer’ in equation form

In a previous contribution we worked out the basic formulation for ACS [5]. Here we develop it for diversity gain. The absolute interferer level for the legacy interferer at each UE input can be calculated as (see Annex1)

Intrfrlegacy=-174+NFlegacy-DG+10logCBW+10log101410-1+ACSlegacy

A similar equation can be written for the absolute level of the interferer for the LPWUS case, assuming the LPWUS (wanted) signal is also 14 dB stronger than the LPWUS refsens, and there is no diversity gain (1Rx case, see Annex1):

IntrfrLPWUS= ACSLPWUS+ NLPWUS+10logBWLPWUS+10log101410-1

If the absolute interferer power levels are the same for both cases (legacy and LPWUR), then the legacy interferer per connector and LPWUS interferer are equal, i.e. the right-hand sides of the 2 equations can be directly compared:

NFLPWUS+10logBWLPWUS+ACSLPWUS= -DG+NFlegacy+10logCBW+ACSlegacy

Or,

ACSLPWUS= NFlegacy-NFLPWUS-DG+10logCBWBWLPWUS+ACSlegacy

This expression can be used as the high-level principle for specifying the ACS requirement for the LPWUR. Note that the bandwidth ratio is more accurately the ratio of the RBs, as shown below.

Observation 1: Based on existing RAN4 LPWUS ACS agreements, the ACS requirement translates to:

ACSLPWUS=  NFlegacy-NFLPWUS+10logNRBRBLPWUS+ACSlegacy-DG

### 2.3.3	ACS requirements

The expression in obs.1 is used to create the LPWUR ACS requirement table from that of the legacy PDSCH case by incorporating the following:

- NFlegacy = 9.0 dB

- NFLPWUS (type 1) = 15.5 dB

- NFLPWUS (type 2) = 11.0 dB

- DG(2Rx) = 3.0 dB

- RBLPWUS = 11

The guiding equation then specializes for types 1 and 2 LR as:

ACSLPWUS_type1=  10logNRB11+ACSlegacy-9.5

ACSLPWUS_type2=  10logNRB11+ACSlegacy-5.0

These specializations translate to the following ACS requirements.

Figure 2.3.3 – 1: ACS requirement table for LPWUR

The detailed ACS side conditions can now be constructed based on these tabulated requirement values. The ACS requirement proposal in its entirety, including wording, is provided in draft form in an accompanying contribution [6]. The ACS requirements above are graphically represented in Annex2.

### 2.3.4	ACS case 1

ACS case 1 is a ‘small signal case’, and the associated interferer for the legacy requirement is specified relative to the legacy REFSENS value. Referencing both, the legacy REFSENS as well as the LPWUS REFSENS value allows the requirement to be a compact modification from the legacy PDSCH case in 38.101-1:

Figure 2.3.4 – 1: ACS case 1 test parameters for LPWUR

### 2.3.5	ACS case 2

ACS case 2 is the ‘large signal case’, where the interferer level is specified at an absolute level. The LPWUS signal strength is therefore back calculated from the ACS requirement derived in section 2.3.3 to derive the LPWUR requirement:

Figure 2.3.5 – 1: ACS case 2 test parameters for LPWUR

## 2.4	ASCS

Since RAN4 have already agreed on an acceptable desensitization value, the ASCS requirement can be written more along the lines of a RIB rather than ACS. This format is chosen for the CR proposal [6]. The remaining detail is whether RAN4 want to specify the non-LPWUS RBs on a per RB basis, or a power basis. The disadvantage of the latter is that the power in LP-WUS RBs varies greatly depending on NRB, and it makes the specification complicated. Both versions are presented in the CR proposal for evaluation.

## 2.5	Max. input power

Legacy practice for this parameter is to define the requirement for all possible channel bandwidths. The challenge for the LPWUR is that input power can be differently distributed between LPWUS and legacy DL RBs, depending on channel bandwidth. Moreover, since competitive implementations are expected to use AGC, the additional challenge is to set the AGC correctly, i.e. not saturate components while leaving enough LPWUS power for detection.

All these considerations can be simultaneously addressed if the test signal for the max. input power case includes NZP (non zero power) legacy RBs.

Proposal 7: The max. input power level is specified for all channel bandwidths, in the presence of legacy RBs with the same PSD as the LPWUS RBs.

It can be left up to RAN5 how to streamline test cases.

## 2.6	FR2-specific LPWUR parameters

### 2.6.1	NF+IM assumption

RAN4 have agreed that combining gain for FR2 receivers is -3 dB [3].

FR2 unlike FR1 does not lend itself to making a standalone ‘low power’ radio for LPWUS monitoring. Practical implementations will generally require significant front end sharing with the MR, and one of the benefits of that is good noise figure even for the OOK-based receiver. Consequently, both receiver types can share the same NF as OFDM FR1 type, at least as a baseline, 9dB. Also, for FR2, implementation margin was 2.0 dB.

Proposal 8: For FR2, NF+IM is 11.0 dB for both LPWUR types.

### 2.6.2	Requirements applicability direction

Using the same ‘front-end re-use’ argument from the preceding section, it is reasonable to reuse the beam peak direction derived during legacy connected mode Rx verification.

Proposal 9: All FR2 directional LPWUS requirements apply in the same direction as the corresponding legacy Rx requirement.

# 3.	Conclusions

Proposal 1: RAN4 type 1 UEs are mapped to support of FG 62-1 and 62-2, type 2 UEs are mapped to support of FG 62-1a and 62-2a. No fragmentation beyond RAN1 FG 62-1 is justified from RAN4 perspective.

Proposal 2: Option 2 wording (‘PDCCH MDR triggered by LPWUS’) is used for the UE RF core requirements for the Rel-19 LPWUS feature, rather than ‘MDR of LPWUS’ which mandates a test mode for the feature.

Proposal 3: Consider the following threshold SNR values for the OFDM-capable LPWUR for the UE RF requirements

|  | OFDM-capable Rx, SNR threshold for 1% MDR, with 1% FAR (dB) |  |  |
| --- | --- | --- | --- |
| Sequence length (bits) | Ideal (no error, no t/f correction) | CONN. mode (w/ t/f correction) | IDLE mode (w/ t/f correction) |
| 32 | -13.8 | -12.0 | -9.5 |
| 16 | -11.9 | -10.5 | -8.0 |
| 8 | -9.9 | -8.7 | -6.6 |
| Short (3) | -6.8 | -5.5 | -4.5 |

Proposal 4: Consider the following threshold SNR values for the envelope detector based LPWUR (‘OOK-based’) for any relevant UE RF requirements

|  | Envelope detector Rx, SNR threshold for 1% MDR, with 1% FAR (dB) |  |  |
| --- | --- | --- | --- |
| Sequence length | Ideal (no error, no t/f correction) | CONN. mode (w/ t/f correction) | IDLE mode (w/ t/f correction) |
| 32 | -9.2 | -7.4 | -4.9 |
| 16 | -7.5 | -6.1 | -3.6 |
| 8 | -5.9 | -4.7 | -2.6 |

Proposal 5: For FR1, the requirement is written around a 16-bit OOK sequence for envelope detector based LPWUR and a ‘short’ OOK sequence (3 bits long for L=4 and number of bits = 5) for OFDM capable LPWUR.

Proposal 6: For future traceability as well as consistency with information sent to RAN1 via LS, the sensitivity equation is modified as:

REFSENS = -174 dBm/Hz + 10*log(RxBW_Hz) + NF + SNR + IM – desens_ASCS

Observation 1: Based on existing RAN4 LPWUS ACS agreements, the ACS requirement translates to:

ACSLPWUS=  NFlegacy-NFLPWUS+10logNRBRBLPWUS+ACSlegacy-DG

Proposal 7: The max. input power level is specified for all channel bandwidths, in the presence of legacy RBs with the same PSD as the LPWUS RBs.

Proposal 8: For FR2, NF+IM is 11.0 dB for both LPWUR types.

Proposal 9: All FR2 directional LPWUS requirements apply in the same direction as the corresponding legacy Rx requirement.

# 4.	References

[1]	R1-2501397, ‘Initial RAN1 UE features list for Rel-19 NR after RAN1 #120’

[2] 	R4-2420394, ‘WF on LP-WUS UE RF requirements’, vivo, RAN4#113

[3]	R4-2505104, ‘WF for [114bis][131] NR_LPWUS_UERF’, vivo RAN4#114-Bis

[4] 	R4-2503028, ‘WF on LP-WUS UE RF requirements’, vivo, RAN4#114

[5] 	R4-2503170, ‘On UE Rx requirements for the LPWUR’, Qualcomm, RAN4#114-Bis

[6]	R4-2505370, ‘dCR for ACS and ASCS for LPWUS’, Qualcomm et al., RAN4#115

# 5.	Annex1, derivation of ACS interferer level

## 5.1	Legacy 2Rx baseline ACS treatment

Assume:

- Rlegacy is the refsens PSD for the legacy requirement at each connector

- Nlegacy is the PSD of the antenna input referred noise (= -174 + NFlegacy) also at each connector

- SNRlegacy is the SNR that causes the receiver to run at threshold condition for legacy requirements

- CBW is the channel NRB BW in Hz

- ACSlegacy is the legacy ACS requirement for that CBW

- DG is 10log(Nchains) , where Nchain is the number of Rx chains

- Intrfrlegacy is the absolute interferer level per connector in dBm

For the legacy 2Rx reception case without interferers, and assuming MRC, the baseband signal level (coherent combining) and baseband noise level (power combining) can be written as (to some additive constant):

SBBlegacy=Rlegacy+20logNchains+10log⁡(CBW)

NBBlegacy=Nlegacy+10logNchains+10log⁡(CBW)

SNRlegacy=Rlegacy-Nlegacy+10logNchains

The value of SNRlegacy for UE RF requirements is +1.5 dB, and it is derived by application of an implementation margin (legacy agreement) of 2.5 dB to the theoretical expectation of -1.0 dB for threshold SNR for MCS4.

The legacy 2Rx radio under ACS conditions have the following conditions:

TestSignalACSlegacy=Rlegacy+14.0+10log⁡(CBW)

At the baseband, post MRC, to some fixed additive constant:

TestSignalBBACSlegacy=Rlegacy+14.0+10log⁡(CBW)+20logNchains

NBBACSlegacy=Nlegacy+10log⁡(CBW)+10logNchains (same as case without interferers)

IBBACSlegacy=Ix+20logNchains

Ix=Intrfrlegacy-ACSlegacy

Note here that the interferer at baseband is assumed to add in-phase across all chains(coherent combining), just like the signal. This is a reasonable assumption when the relative phase of the wanted signals across the Rx connectors matches that of the interferer. This assumption is verified later when the analysis value is compared with the specification in the standard.

SINRBBACSlegacy=TestSignalBBACSlegacy-10log{linearNBBACSlegacy+linearIBBACSlegacy}

For a marginally successful ACS test outcome:

SINRBBACSlegacy= SNRlegacy

So,

Rlegacy+14.0+10logCBW+20logNchains-10logCBW.10Nlegacy10+Nchains.10Ix10-10logNchains= Rlegacy-Nlegacy+10logNchains

This simplifies to:

Nlegacy+14.0+10logCBW= 10log⁡( CBW.10Nlegacy10+Nchains.10Ix10)

This simplifies to:

CBW.101410-1.10Nlegacy10= Nchains. 10Ix10

This simplifies to:

Ix= Nlegacy+10logCBW+10log101410-1- 10logNchains

Using the previously defined relationship:

Ix=Intrfrlegacy-ACSlegacy

Intrfrlegacy= ACSlegacy+ Nlegacy+10logCBW+10log101410-1- 10logNchains

Intrfrlegacy= ACSlegacy+ Nlegacy+10logCBW+10log101410-1- DG

The derivation above is made from first principles, and it is useful to cross-check the outcome above with the requirement in the standard. Now, the REFSENS expression can be derived from the previously derived expression for PSD of REFSENS

SNRlegacy=Rlegacy-Nlegacy+10logNchains

Rlegacy=SNRlegacy+Nlegacy-DG

REFSENSlegacy=SNRlegacy+Nlegacy-DG+10log⁡(CBW)

The equation above subtracted from the highlighted equation:

Intrfrlegacy- REFSENSlegacy= ACSlegacy+ 10log101410-1-SNRlegacy

Assuming SNRlegacy is 1.5 dB (= -1.0 dB + 2.5 dB IM), and noting that 10log(101.4-1) approximates 14, the above simplifies to:

Intrfrlegacy =approx.REFSENSlegacy+ACSlegacy+ 12.5

Take test case 1 for high bands (> 3.3 GHz), chosen for the compactness of the ACS requirement.

The ACSlegacy requirement is 33.0 dB:

Incorporating this value into the first principles analysis yields an interferer level that matches the specification of the per-connector interference in the standard, see excerpt below:

Intrfrlegacy =REFSENSlegacy+33+ 12.5

Intrfrlegacy =REFSENSlegacy+45.5

It is easily confirmed that the consistency observation above applies to all other combinations of channel BWs and band. The expression for the interferer level also confirms that RAN4 have previously assumed that the interferer and signal are treated similarly in baseband (i.e. no diversity gain is available for the interferer)

## 5.2	LPWUR 1Rx treatment

Assume:

- RLPWUS is the refsens PSD for the LPWUS requirement at only connector

- NLPWUS is the PSD of the antenna input referred noise (= -174 + NFLPWUS) also at connector

- SNRLPWUS is the SNR that causes the receiver to run at threshold condition for LPWUS requirements

- CBW is the channel NRB BW in Hz

- ACSLPWUS is the LPWUS ACS requirement for that CBW

- IntrfrLPWUS is the absolute interferer level at connector in dBm

For the LPWUS reception case without interferers, the baseband signal level and baseband noise level can be written as (to some additive constant):

SBBLPWUS=RLPWUS+10log⁡(BWLPWUS)

NBBLPWUS=NLPWUS+10logBWLPWUS

SNRLPWUS=SBBLPWUS- NBBLPWUS= RLPWUS-NLPWUS

The LPWUR radio under agreed ACS conditions have the following conditions at baseband (to some fixed additive constant):

TestSignalBBACSLPWUS=RLPWUS+14.0+10log⁡(BWLPWUS)

NBBACSLPWUS=NLPWUS+10logBWLPWUS (same as case without interferers)

IBBACSLPWUS=Ix= IntrfrLPWUS-ACSLPWUS

SINRBBACSLPWUStype1=TestSignalBBACSLPWUStype1-10log{linearNBBACSLPWUS+linearIBBACSLPWUS}

For a marginally successful ACS test outcome:

SINRBBACSLPWUS= SNRLPWUS

So,

RLPWUS+14.0+10logBWLPWUS-10logBWLPWUS.10NLPWUS10+10Ix10= RLPWUS-NLPWUS

This simplifies as:

NLPWUS+14.0+10logBWLPWUS= 10log⁡( BWLPWUS.10NLPWUS10+10Ix10)

BWLPWUS.101410-1.10NLPWUS10= 10Ix10

Ix= NLPWUS+10logBWLPWUS+10log101410-1

Using the previously defined relationship: Ix=IntrfrLPWUS-ACSLPWUS

IntrfrLPWUS= ACSLPWUS+ NLPWUS+10logBWLPWUS+10log101410-1

The yellow highlighted equations in this section are used in section 2.3.

# 6.	Annex2, LPWUS ACS requirement trends

Graphic representation of LPWUS ACS requirements is below. Note that the representation does not imply any specific SCS applicability, the latter will be decided elsewhere.

As is well understood, the fundamentally different trends come from different interferer assumptions between the low band group (<= 2.7 GHz) and the high band group (> 2.7 GHz).