3GPP TSG-RAN4 Meeting #115	R4-2505369

# Malta, Malta, 19th May 2025 - 23rd May 2025

Source	:	Qualcomm Incorporated

Title	:	Some test skipping proposals for LPWUS

Agenda item	:	7.28.3.4

Document for	:	Approval

# 1.	Introduction

In this contribution we consider the test method to measure LPWUS detection reliability.

# 2. 	Discussion

## 2.1	FR2 spherical coverage

In a previous contribution [3], we proposed that LPWUS spherical coverage verification of an FR2 UE be considered for automatic validation based on ‘front-end reuse’. The reasoning is that in FR2, it is impractical to implement a LPWUR with unique front-end components. A more likely implementation is re-use of the MR front end, perahps with simplified baseband processing. Under this practical assumption, spherical coverage ‘gain drop’ of legacy PDSCH sensitivity is expected to be indicative of the same for LPWUS.

Proposal 1: RAN4 to consider the FR2 LPWUR sensitivity spherical coverage requirement met if it meets the LPWUS REFSENS requirement and the legacy spherical coverage requirement for sensitivity (‘EIS’).

## 2.2	Idle vs connected

RAN1 has already agreed to creating distinct capabilities for support of the feature in idle mode and connected mode. In a previous contribution [3], we have therefore proposed that:

Proposal 9: Two sets (each set further broken out into OOK-based and OFDM-based) of LPWUS requirements are specified, one set applies to UEs supporting LPWUS in idle/inactive mode and another for UEs supporting LPWUS in connected mode.

In effect, RAN4 must first set separate requirements for the two capabilities and only then decide if any applicability rule can eliminate low-value additional testing for those UEs that support both. Now, the motivation for UE RF requirements is to ensure RF parameters of sufficient quality. As such, verifying requirements in both CONNECTED and IDLE modes for UEs that support both does not add any value or new information. It is therefore logical to consider test skipping. A further consideration is that using only native behavior as a consideration for core requirements, verification of CONNECTED mode requirements are expected to proceed at a faster time scale than for IDLE mode. Consequently, CONNECTED mode requirements can be used as a representative of the UE’s LPWUR capabilities.

Proposal 2: For a UE that supports LPWUS in both IDLE and CONNECTED modes, RAN4 to consider the IDLE mode LPWUS requirement met if it meets all the LPWUS requirements in CONNECTED mode.

## 2.3	Test mode

RAN1 and RAN2 jointly define UE behavior and RAN4 should be careful not to inadvertently also define UE behavior in some way. Unfortunately, a test mode is implied in much of the discourse on LPWUS requirements. A test mode is a last resort, and unless no verification is possible otherwise. Fortunately, in the LPWUS case, there is a precedent in the form of Rel-16 DCP:

Proposal 3: RAN4 UE RF core requirements are defined by assuming no test-mode is available, and it is left to RAN5 to introduce test mode for simplification and timeline compression.

# 3.	Conclusions

Proposal 1: RAN4 to consider the FR2 LPWUR sensitivity spherical coverage requirement met if it meets the LPWUS REFSENS requirement and the legacy spherical coverage requirement for sensitivity (‘EIS’).

Proposal 2: For a UE that supports LPWUS in both IDLE and CONNECTED modes, RAN4 to consider the IDLE mode LPWUS requirement met if it meets all the LPWUS requirements in CONNECTED mode.

Proposal 3: RAN4 UE RF core requirements are defined by assuming no test-mode is available, and it is left to RAN5 to introduce test mode for simplification and timeline compression.

# 4.	References

[1] 	R4-2503028, ‘WF on LP-WUS UE RF requirements’, vivo, RAN4#114

[2]	R4-2502600, ‘WF on RRM requirements for NR_LPWUS’, vivo RAN4#114

[3] 	R4-2503170, ‘On UE Rx requirements for the LPWUR’, Qualcomm, RAN4#114-Bis