3GPP TSG-RAN4 Meeting #115	R4-2505370

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | dCR to 38.101-1, LPWUS ACS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS |  |  |  |  |  | Date: |  |  | 2025-05-04 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | ‘Feature’ CR component: ACS requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on RAN4 agreements: - The same interference is experienced by the LPWUR and the MR (R4-2420394, ‘baseline’ assumption) - Same interference signal level as MR for LP-WUR ACS requirements (R4-2503028) - The same set of the interference signal level defined for both two types of the LP-WUS requirements. (R4-2503028) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE RF feature not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5M |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/* Begin changes*/

## 7.5M	Adjacent channel selectivity [for LP-WUS]

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured RBs inside the configured channel, in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the configured channel. ACS is the ratio of the receive filter attenuation on the LP-WUS frequency to the receive filter attenuation on the adjacent channel(s).

The UE that supports either [type1 LR] or [type2 LR] shall fulfil the minimum requirements specified in Table 7.5M-1 for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz and the minimum requirements specified in Table 7.5M-2 for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz.

These requirements apply for all values of an adjacent channel interferer up to -25 dBm and for any SCS specified in table [5.3M.2-1]. However, it is not possible to directly measure the ACS; instead the lower and upper range of test parameters are chosen as in Table 7.5M-3 and Table 7.5M-4 for verification of the requirements specified in Table 7.5M-1, and as in Table 7.5M-5 and Table 7.5M-6 for verification of the requirements specified in Table 7.5M-2. For these parameters, the MDR criterion shall be met with the side conditions in clause [7.1M].

Table 7.5M-1: ACS for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz for UEs supporting [type1 LR] or [type2 LR]

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 7, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| ACS for UEs supporting [type1 LR] | dB | [23.5 + 10log10(NRB/ NRB,LP-WUS)] | [20.5 + 10log10(NRB/ NRB,LP-WUS)] | [17.5 – 10log10(BWChannel /20) + 10log10(NRB /NRB,LP-WUS)] |
| ACS for UEs supporting [type2 LR] | dB | [28.0 + 10log10(NRB/ NRB,LP-WUS)] | [25.0 + 10log10(NRB/ NRB,LP-WUS)] | [22.0 – 10log10(BWChannel /20) + 10log10(NRB /NRB,LP-WUS)] |
| NOTE1: ACS value is rounded to the next higher 0.5dB value |  |  |  |  |

Table 7.5M-2: ACS for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz for UEs supporting [type1 LR] or [type2 LR]

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| ACS for UEs supporting [type1 LR] | dB | [23.5 + 10log10(NRB /NRB,LP-WUS)] |
| ACS for UEs supporting [type2 LR] | dB | [28.0 + 10log10(NRB /NRB,LP-WUS)] |

Table 7.5M-3: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 1 for UEs supporting [type1 LR] or [type2 LR]

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 7, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in LP-WUS [RBs] | dBm | [REFSENSLP-WUS] 5 + 14.0 dB |  |  |  |
| Pinterferer4 | dBm |  | REFSENS5 + 45.5 dB | REFSENS5 + 42.5 dB | REFSENS5 + 39.5 – 10log10(BWChannel /20) |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset) | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: 10log10(x) is rounded to the next higher 0.5dB value. NOTE 5: REFSENS is the value specified for 2 antenna port UEs in clause 7.3.2, REFSENSLP-WUS is the value specified in clause [7.3M.2] |  |  |  |  |  |

Table 7.5M-4: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 2 for UEs supporting [type1 LR] or [type2 LR]

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 7, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in LP-WUS [RBs] for UEs supporting [type1 LR] 4 | dBm | -47.0 - 10log10(NRB /NRB,LP-WUS) |  | -44.0 - 10log10(NRB /NRB,LP-WUS) | -41.0 + 10log10(BWChannel /20) - 10log10(NRB /NRB,LP-WUS) |
| Power in LP-WUS [RBs] for UEs supporting [type2 LR] 4 | dBm | -51.5 - 10log10(NRB /NRB,LP-WUS) |  | -48.5 - 10log10(NRB /NRB,LP-WUS) | -45.5 + 10log10(BWChannel /20) - 10log10(NRB /NRB,LP-WUS) |
| Pinterferer | dBm | -25.0 |  |  |  |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset) | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 NOTE 4: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

Table 7.5M-5: Test parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1 for UEs supporting [type1 LR] or [type2 LR]

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in LP-WUS [RBs] 4 | dBm | [REFSENSLP-WUS] + 14.0 dB |
| Pinterferer | dBm | REFSENS + 45.5 dB |
| BWinterferer | MHz | BWChannel |
| Finterferer (offset) | MHz | BWChannel / -BWChannel |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: REFSENS is the value specified for 2 antenna port UEs in clause 7.3.2, REFSENSLP-WUS is the value specified in clause [7.3M.2] |  |  |

Table 7.5M-6: Test parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2 for UEs supporting [type1 LR] or [type2 LR]

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in LP-WUS [RBs] for UEs supporting [type1 LR] 4 | dBm | -47.0 - 10log10(NRB /NRB,LP-WUS) |
| Power in LP-WUS [RBs] for UEs supporting [type2 LR] 4 | dBm | -51.5 - 10log10(NRB /NRB,LP-WUS) |
| Pinterferer | dBm | -25.0 |
| BWinterferer | MHz | BWChannel |
| Finterferer (offset) | MHz | BWChannel / -BWChannel |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |

/* End changes*/

/* Begin changes*/

## 7.11M	Adjacent subcarrier selectivity [for LP-WUS]

Adjacent subcarrier selectivity (ASCS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured RBs, in the presence of adjacent subcarrier NR signal(s) and zero-power PRB between LP-WUS and NR (guard RB), inside the configured channel. The MDR criterion in clause [7.1M] shall be met with the side conditions in [7.1M] and the parameters specified in Table 7.11M-1.

(ALT1)

Table 7.11M-1: ASCS requirement for UE supporting [type1 LR] or [type2 LR]

| RX parameter | Units | UEs supporting [type1 LR] | UEs supporting [type2 LR] |
| --- | --- | --- | --- |
| Applicable channel BW (MHz) |  | 3, 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
| Power in LP-WUS RBs1 | dBm | [REFSENSLP-WUS] + 0.5 | [REFSENSLP-WUS] |
| Guard RB2 | PRB | 1 | 0 |
| Power of each non-LPWUS DL NR RB2,3 | dBm | [REFSENSLP-WUS] + 0.5 – 10log10(NRB,LP-WUS) | [REFSENSLP-WUS] – 10log10(NRB,LP-WUS) |
| NOTE 1: REFSENSLP-WUS is the value specified in clause [7.3M.2] NOTE 2: Guard RBs are zero-power and exist when NRB,LP-WUS > NRB NOTE 3: The interfering NR RBs consists of CP-OFDM QPSK with the same SCS as LP-WUS, in all PRB in the DL transmission BW configuration not occupied by either LP-WUS or Guard RB |  |  |  |

(ALT2)

Table 7.11M-1: ASCS requirement for UE supporting [type1 LR] or [type2 LR]

| RX parameter | Units | UEs supporting [type1 LR] | UEs supporting [type2 LR] |
| --- | --- | --- | --- |
| Applicable channel BW (MHz) |  | 3, 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
| Power in LP-WUS RBs | dBm | [REFSENSLP-WUS] + 0.5 | [REFSENSLP-WUS] |
| Guard RB2 | PRB | 1 | 0 |
| Power of DL NR RB excluding LP-WUS RBs and Guard RBs2,3 | dBm | [REFSENSLP-WUS] + 0.5 + 10log10((NRB - X* Guard RB- NRB,LP-WUS)/NRB,LP-WUS) (4) | [REFSENSLP-WUS] + 10log10((NRB - NRB,LP-WUS)/NRB,LP-WUS) (6) |
|  | dBm | -inf(5) | -inf(5) |
| NOTE 1: REFSENSLP-WUS is the value specified in clause [7.3M.2] NOTE 2: Guard RBs are zero-power and exist when NRB,LP-WUS > NRB NOTE 3: The interfering NR RBs consists of CP-OFDM QPSK with the same SCS as LP-WUS, in all PRB in the DL transmission BW configuration not occupied by either LP-WUS or Guard RB NOTE 4: ‘X’, the number of Guard RBs present in the UE’s configured channel, depends on the LPWUS allocation relative to the DL transmission BW configuration NOTE 5: No interfering NR RB(s) present NOTE 6: Non-zero number of interfering NR RB(s) present |  |  |  |

(note to editor: Need to verify whether Guard RB are illustrated in section 5)

/* End changes*/