3GPP TSG-RAN WG4 Meeting #115                        R4-2505381

Malta, May 19 - 23 2025

Source:	China Telecom

Title:	Discussion on BS demodulation performance requirements for MMSE-IRC

Agenda Item:	7.17.4

Document for:	discussion

Introduction

In RAN#104 meeting, the Rel-19 new WID on NR demodulation performance: Phase 5 is approved.

The following objectives are approved for BS requirements:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions:

  - Receiver type: MMSE-IRC receiver

  - Scenarios:

    - FDD synchronous deployments

    - TDD synchronous deployments with aligned UL:DL configurations among cells

    - Slot-based transmission and aligned SCS among cells

  - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

The WF was approved in [1]. In this contribution, our views on the open issues are given.

Discussion

DMRS configuration - Ratio of PUSCH EPRE to DM-RS EPRE

Before this meeting, company has brought up this issue which was not discussed in the previous meetings.

- Option 1: Ratio of PUSCH EPRE to DM-RS EPRE - -3dB with Number of DM-RS CDM group(s) without data = 2 - Samsung: With opt 1, there will be 3dB mismatch between data and the estimated interference covariance matrix, under the agreed DMRS based MMSE-IRC receiver assumption. - Other options are not precluded

In the existing PUSCH requirements, DMRS power boosting is always enabled. And it is also widely used in the practical network for better UL coverage.

Although this issue was not discussed before, we confirm that our simulation results submitted in the previous meetings are based on option1 as above, which means 3dB mismatch between data and the estimated interference covariance matrix does exist.

In our simulation results, enough performance gain of MMSE-IRC over MMSE is shown. In addition, we also did simulation for some agreed cases and disabled DMRS power boosting:

Table 1: Simulation results for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, 15 kHz SCS (with and w/o DMRS power boosting)

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix |  | INR Set | SNR with DMRS power boosting (dB) | SNR w/o DMRS power boosting (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer |  |  |  |
| 1 | 2 | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 11.4 | 12.0 |
|  |  | TDLA30-10 Low |  | Set 2 | 12.5 | 12.8 |
|  | 4 | TDLC300-100 Low |  | Set 1* | 8.0 | 8.5 |
|  |  | TDLA30-10 Low |  | Set 2 | 8.5 | 8.9 |
|  | 8 | TDLC300-100 Low |  | Set 1* | 5.8 | 6.5 |
|  |  | TDLA30-10 Low |  | Set 2 | 6.0 | 6.3 |

Our observation is that MMSE-IRC is not very sensitive to the 3dB mismatch between the estimated interference power and the PUSCH interference data. Therefore, we propose to enable DMRS power boosting for BS MMSE-IRC tests.

Observation 1: DMRS power boosting is always enabled in the existing PUSCH requirements. And it is also widely used in the practical network for better UL coverage.

Observation 2: MMSE-IRC is not sensitive to the 3dB mismatch between the estimated interference power and the PUSCH interference data.

As for the necessity to consider other MMSE-IRC receiver assumption rather than the agreed DMRS based interference covariance matrix estimation. We would like to point it out that the DMRS-based interference covariance matrix estimation is also the agreed receiver assumption for UE MMSE-IRC as per TR38.833. And DMRS power boosting is a feature that can be used for both UL and DL. Thus, we do not see the necessity to consider other MMSE-IRC receiver assumptions.

Observation 3: The DMRS-based interference covariance matrix estimation is also the agreed MMSE-IRC receiver assumption for UE. DMRS power boosting is a feature that can be used for both UL and DL.

Proposal 1: Ratio of PUSCH EPRE to DM-RS EPRE - -3dB with Number of DM-RS CDM group(s) without data = 2. There is no need to modify the agreed MMSE-IRC receiver assumption.

Applicability for different BS classes and test scenarios

- Online agreement: - Follow the LTE approach – Only specifies that HomNet requirements are not applicable for Local Area BS. - FFS whether to consider additional applicability rule on how the test is performed: - Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped - Option 2: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

Regarding the test requirement applicability for different BS classes and test scenarios, we propose to follow the LTE approach and other than that, we do not see the reason to introduce additional applicabilities.

Proposal 2: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

SNR requirement value deriving rule for BS MMSE-IRC requirements

Considering this meeting is going to endorse draft CRs with test requirements in [], we will need to make decision on the SNR requirement deriving rule, to assist the implementation of the draft CRs.

We propose to reuse the Rel-15 BS demodulation SNR requirement derivation procedure as agreed in [2], which was also used in Rel-17 PUSCH 256QAM demodulation and Rel-17/Rel-18 coverage enhancement requirements.

Proposal 3: Reuse the SNR requirement value deriving rule for Rel-15 BS demodulation SNR requirement derivation procedure as agreed in [2]:

Procedure to derive the performance requirements: – Only inputs that consist of a pair of ideal and impaired results can be taken into account. – If the ideal span <= [2]dB: • The AVERAGE impairment results can be used for the performance requirement with [] in the draftCRs/CRs; – Else if the ideal span is larger than [2]dB: • The results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available: – The ultimate AVERAGE impairment results with corresponding ideal span <=2dB can be used for performance requirement with [] in the draftCRs/CRs. • Otherwise put TBD for the related performance requirements. – If the span of the impairment results after removal the outliers (if any) are larger than 4dB, then the procedure cannot be applied, related performance requirement remain TBD.

Conclusion

Observation 1: DMRS power boosting is always enabled in the existing PUSCH requirements. And it is also widely used in the practical network for better UL coverage.

Observation 2: MMSE-IRC is not very sensitive to the 3dB mismatch between the estimated interference covariance matrix.

Observation 3: The DMRS-based interference covariance matrix estimation is also the agreed MMSE-IRC receiver assumption for UE. DMRS power boosting is a feature that can be used for both UL and DL.

Proposal 1: Ratio of PUSCH EPRE to DM-RS EPRE - -3dB with Number of DM-RS CDM group(s) without data = 2. There is no need to modify the agreed MMSE-IRC receiver assumption.

Proposal 2: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

Proposal 3: Reuse the SNR requirement value deriving rule for Rel-15 BS demodulation SNR requirement derivation procedure as agreed in [2]:

Procedure to derive the performance requirements: – Only inputs that consist of a pair of ideal and impaired results can be taken into account. – If the ideal span <= [2]dB: • The AVERAGE impairment results can be used for the performance requirement with [] in the draftCRs/CRs; – Else if the ideal span is larger than [2]dB: • The results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available: – The ultimate AVERAGE impairment results with corresponding ideal span <=2dB can be used for performance requirement with [] in the draftCRs/CRs. • Otherwise put TBD for the related performance requirements. – If the span of the impairment results after removal the outliers (if any) are larger than 4dB, then the procedure cannot be applied, related performance requirement remain TBD.

Reference

R4-2504768, Way Forward for [114bis][321] NR_demod_Ph5_Part1_General_BS, China Telecom. RAN4#114bis, April 2025

R4-1904713, WF for requirement SNR derivation procedure, Nokia, Nokia Shanghai Bell, ZTE, Ericsson, RAN4 #90bis, April 2019.

R4-2503204, On BS demodulation performance requirements for MMSE-IRC: Simulation Results, China Telecom, RAN4#114bis, Apr 2025