3GPP TSG-RAN WG4 Meeting #115                      R4-2505385

Malta, May 19 - 23 2025

Source:	China Telecom

Title:	Discussion on UE demodulation performance requirements for 6Rx

Agenda Item:	7.1.4

Document for:	discussion

Introduction

The RAN4#114bis is the first meeting we discuss the UE demodulation performance requirements for 6Rx. The following objectives are approved in the latest WID [1]:

6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - 6 MIMO layers performance requirements will be only defined for FWA. - 4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices. - Specify the SDR requirements to support 6Rx General for all the areas Specify release independence requirements for performance part in TS 38.307/36.307, if needed and feasible.

According to the WID, RAN4 will define 6-layer performance requirements for FWA and 4-layer performance requirements for FWA and handheld UE devices. During RAN4#114bis, the Way Forward for [114bis][325] NR_ENDC_RF_Ph4_Demod_6Rx is approved in [2]

In this contribution, our views on the remaining open issues are given.

Discussion

Test applicability rule for 6Rx test cases

- Way forward - Option 1: RAN4 can introduce UE declaration on the 6Rx UE type (e.g., handheld or FWA) without capability signaling and design proper test applicability in the specification. - Option 2: - 6 MIMO layers performance requirements will be only applicable for FWA devices - 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices.

The test applicability as captured in option 2 is aligned with the RAN4 agreement in the Core part, which should be followed in the Perf part.

Regarding how to specify that ‘6 MIMO layers performance requirements will be only applicable for FWA devices’, we think it can be further discussed pending RAN2 conclusion on the 6Rx MIMO layer capability definition.

If it is finally confirmed that there is no explicit capability signaling to differentiate UE type, RAN4 needs to introduce UE declaration and design proper test applicability in the specification.

Proposal 1: The test applicability should follow Core part agreements:

- 6 MIMO layers performance requirements will be only applicable for FWA devices

- 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices.

Proposal 2: If RAN2 finally confirmed that there is no explicit capability signaling to differentiate UE type, RAN4 needs to introduce UE declaration and design proper test applicability in the specification.

Specification structure

- Proposals - Option 1: Add a new clause for 6Rx - Other options - Recommended WF - Postpone the discussion to the CR drafting stage

There is only one thing we would like to point it out is that the current requirements for 1/2/4/8Rx has been introduced in 38.101-4 without a Void section for 6Rx, which means 6Rx related sections may need to be specified under 8Rx.

We would like to check if it is possible to modify the section numbering and put 6Rx requirements between 4Rx and 8Rx in the Rel-19 specification.

Proposal 3: RAN4 to discuss whether it is possible to modify the section numbering and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification.

Test scenarios

- Way forward - Option 1: Test scenarios without interference: PDSCH Mapping Type A - Option 2: Test scenarios with interference - PDSCH demodulation requirements with inter-cell interference - PDSCH demodulation requirements with intra-cell inter-user interference

It is worth mentioning that in the last meeting, MMSE-IRC has been agreed as the receiver type for all 6Rx UEs. For MMSE-IRC, tests under interference scenarios are critical to verify the UE capability to deal with practical interference. That is why RAN4 has specified requirements with inter-cell interference and intra-cell inter-user interference for 2/4/8Rx UEs, which are mandatory and are also release independent.

Observation 1: In the last meeting, MMSE-IRC has been agreed as the receiver type for all 6Rx UEs.

Observation 2: RAN4 has specified requirements with inter-cell interference and intra-cell inter-user interference for 2/4/8Rx UEs, which are mandatory and are also release independent.

Based on the above observations, we propose to specify 6Rx UE performance requirements with inter-cell interference and intra-cell inter-user interference.

Proposal 4: RAN4 to specify 6Rx UE performance requirements with inter-cell interference and intra-cell inter-user interference.

Propagation channel and MIMO correlation for 6 MIMO layers

- Way forward - Option 1: TDLA 30-10 ULA Low - Option 2: TDLD 30-5 ULA Low

TDLD30 (LOS) channel model was introduced for FR2 256QAM testing to avoid testability issue on the highest achievable SNR [3], which is not the issue we face in FR1 testing.

In addition, TDLD channel is used for FR1 1024QAM testing because of the high SNR value, i.e., 26.3~29.5dB, which can be avoided by carefully choosing MCS.

Observation 3: LOS channel model was introduced for FR2 256QAM testing to avoid testability issue on the highest achievable SNR for FR2 only. And TDLD channel is used for FR1 1024QAM testing because of the high SNR value, i.e., 26.3~29.5dB.

After all, we do not see the reason to use LOS channel modeling for 6Rx demodulation tests.

Proposal 5: Do not use LOS channel model for 6Rx demodulation tests.

CQI test for 6Rx

- Way forward - Option 1: Not define CQI test - Option 2: Define CQI test - Option 2a: Only define CQI requirements with rank 4 under static channel - Option 2b: Define the following CQI test - Periodic CQI reporting requirements under AWGN - Wideband CQI reporting requirements with inter-cell interference under fading conditions

RI test

- Way forward - Option 1: Not define RI test for 6 layers - Option 2: Define RI test for 6 layers

For CQI and RI testing for 6Rx, we support to follow the same test coverage for 2/4Rx, i.e., Periodic CQI reporting requirements under AWGN, Wideband CQI reporting requirements with inter-cell interference under fading conditions and RI test is defined.

Proposal 6: Define periodic CQI reporting requirements under AWGN, Wideband CQI reporting requirements with inter-cell interference under fading conditions and RI test for 6Rx.

Conclusion

Proposal 1: The test applicability should follow Core part agreements:

- 6 MIMO layers performance requirements will be only applicable for FWA devices

- 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices.

Proposal 2: If RAN2 finally confirmed that there is no explicit capability signaling to differentiate UE type, RAN4 needs to introduce UE declaration and design proper test applicability in the specification.

Proposal 3: RAN4 to discuss whether it is possible to modify the section numbering and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification.

Observation 1: In the last meeting, MMSE-IRC has been agreed as the receiver type for all 6Rx UEs.

Observation 2: RAN4 has specified requirements with inter-cell interference and intra-cell inter-user interference for 2/4/8Rx UEs, which are mandatory and are also release independent.

Proposal 4: RAN4 to specify 6Rx UE performance requirements with inter-cell interference and intra-cell inter-user interference.

Observation 3: LOS channel model was introduced for FR2 256QAM testing to avoid testability issue on the highest achievable SNR for FR2 only. And TDLD channel is used for FR1 1024QAM testing because of the high SNR value, i.e., 26.3~29.5dB.

Proposal 5: Do not use LOS channel model for 6Rx demodulation tests.

Proposal 6: Define periodic CQI reporting requirements under AWGN, Wideband CQI reporting requirements with inter-cell interference under fading conditions and RI test for 6Rx.

Reference

RP-250780, Revised WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12-14, 2025

R4-2504750, Way Forward for [114bis][325] NR_ENDC_RF_Ph4_Demod_6Rx, Wuhan, China, Apr 7th – 11th, 2025

R4-2100880, Offline e-mail discussion summary on the TDLD30 channel simplification, China Telecom, RAN4#98-e