3GPP TSG-RAN WG4 Meeting #115                      R4-2505387

Malta, May 19 - 23 2025

Source:	China Telecom

Title:	Discussion on demodulation aspects for NR FR1 7 MHz Channel Bandwidth

Agenda Item:	7.8.6

Document for:	discussion

Introduction

RAN4#115 is the first meeting for the UE demodulation performance part for WI: NR channel BW less than 5MHz for FR1 Phase 2. According to the latest approved WID [1], the Core part of this WI include:

- Specify the generic core requirements to support 7 MHz NR FR1 channel bandwidth [RAN4] - Introduce the channel bandwidth 7MHz - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements for UL and DL - Specify the band specific core requirements to introduce 7 MHz for NR band n5, n26. [RAN4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed: - MPR (relative bandwidth criteria) - Additional Maximum Power Reduction (A-MPR) - Enable, if feasible, optional UE support of 7 MHz channel bandwidth from Rel-15 onwards in a release independent manner. [RAN2, RAN4] - No impact to 38.307 is expected. - Once RAN4 has introduced the new channel bandwidth, RAN2 will introduce the necessary signalling for this channel bandwidth. [RAN2] Note: This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN2 will be informed by RAN4 via LS. No additional RAN2 TUs will be requested for this objective. - Once RAN4 has introduced the new channel bandwidth, RAN3 will introduce the necessary signalling for this channel bandwidth [RAN3] Note: This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN3 will be informed by RAN4 via LS. No additional RAN3 TUs will be requested for this objective. NOTE: Once this WI is completed, the 7 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI.

And the Perf part of this WI include:

Specify necessary UE/BS performance requirements for NR operation in dedicated FDD FR1 spectrum allocations for irregular channel bandwidths, corresponding to the core requirements: - Specify necessary RRM performance requirements if needed (RAN4) - Specify, if necessary, UE demodulation performance and CSI reporting requirements for the 7 MHz channel bandwidth (RAN4) - Specify, if necessary, BS demodulation performance requirements (RAN4) - Specify necessary BS conformance tests (RAN4)

In this contribution, our views are given.

Discussion

Test scope

According to [1], RAN4 need to discuss the impact on UE demodulation, CSI reporting and BS demodulation for the new 7MHz channel bandwidth.

For the UE demodulation and CSI reporting, in the existing TS38.101-4, almost all FDD single carrier PDSCH and CSI reporting requirements are defined based on 10MHz CHBW with 15kHz SCS. In addition, unlike the Rel-18/19 3MHz WI that specifies new PDCCH and new SSB position, the existing system design for 5MHz can be reused for the new 7MHz CHBW.

Observation 1: As per TS38.101-4, almost all existing FDD single carrier PDSCH and CSI reporting requirements are defined based on 10MHz CHBW with 15kHz SCS.

Observation 2: Unlike the Rel-18/19 3MHz WI, no new NR system design is introduced for 7MHz CHBW.

Considering the above, we do not have a strong desire to define new requirements for single carrier PDSCH, PDCCH, PBCH and CSI reporting particularly for a 7MHz CHBW.

As for CA demodulation requirements, we notice that although no new CA configuration that including a 7MHz CC is going to be introduced (no new BCS), RAN4 have agreed that general BS RF requirements for 7MHz CHBW are going to be introduced for CA [2]:

BS RF Agreement: - Conducted requirements shall be covered within this WI. - Muti-carrier (including MSR) and CA related requirements are considered for new added 7MHz WID, but no new BCS is specified within this WI. - …

Observation 3: RAN4 have agreed that general BS RF requirements for 7MHz CHBW are going to be introduced for CA.

Based on the above RAN4 conclusion in the BS RF part, we propose RAN4 to discuss the necessity of defining new CA and SDR requirements for the new 7MHz CC. If introduced, similar as the Rel-19 3MHz WI, the same test parameters for 5MHz CC should be used.

Proposal 1: RAN4 to discuss the necessity of defining new CA and SDR requirements for the new 7MHz CC. If introduced, similar as the Rel-19 3MHz WI, the same test parameters for 5MHz CC should be used.

For the BS demodulation, we notice that requirements for multiple CHBWs (5MHz, 10MHz, 20MHz) are defined for 15kHz FDD with the following test applicability rule in TS38.141-1:

8.1.2.1.2 Applicability of requirements for different channel bandwidths For each subcarrier spacing declared to be supported, the test requirements for a specific channel bandwidth shall apply only if the BS supports it (see D.14 in table 4.6-1). Unless otherwise stated, for each subcarrier spacing declared to be supported, the tests shall be done only for the widest supported channel bandwidth. If performance requirement is not specified for this widest supported channel bandwidth, the tests shall be done by using performance requirement for the closest channel bandwidth lower than this widest supported bandwidth; the tested PRBs shall then be centered in this widest supported channel bandwidth.

Following the existing test applicability, if new requirements for 7MHz is introduced, it will be tested only if the BS support 7MHz as the largest CHBW.

Observation 4: For the BS part, requirements for multiple CHBWs (5MHz, 10MHz, 20MHz) are defined for 15kHz FDD with the applicability rule that ‘the tests shall be done only for the widest supported channel bandwidth’.

Proposal 2: RAN4 to discuss the necessity of defining new BS PUSCH requirements for the new 7MHz CC. If introduced, the same test parameters for 5MHz CC should be used.

Conclusion

Observation 1: As per TS38.101-4, almost all existing FDD single carrier PDSCH and CSI reporting requirements are defined based on 10MHz CHBW with 15kHz SCS.

Observation 2: Unlike the Rel-18/19 3MHz WI, no new NR system design is introduced for 7MHz CHBW.

Observation 3: RAN4 have agreed that general BS RF requirements for 7MHz CHBW are going to be introduced for CA.

Proposal 1: RAN4 to discuss the necessity of defining new CA and SDR requirements for the new 7MHz CC. If introduced, similar as the Rel-19 3MHz WI, the same test parameters for 5MHz CC should be used.

Observation 4: For the BS part, requirements for multiple CHBWs (5MHz, 10MHz, 20MHz) are defined for 15kHz FDD with the applicability rule that ‘the tests shall be done only for the widest supported channel bandwidth’.

Proposal 2: RAN4 to discuss the necessity of defining new BS PUSCH requirements for the new 7MHz CC. If introduced, the same test parameters for 5MHz CC should be used.

Reference

RP-250373, Revised WID on NR FR1 7 MHz Channel Bandwidth, T-Mobile USA, China Telecom, RAN#107, March 2025

RP-250374, Status report of WI: NR FR1 7 MHz Channel Bandwidth rapporteur: China Telecom, T-Mobile USA, RAN#107, March 2025