3GPP TSG-RAN WG4 Meeting #115												R4-2505397

Malta, 19 – 23 May, 2025

Agenda item:			7.14.3.2

Source:	Huawei, HiSilicon

Title:	on NTN OTA tests

Document for:	approval

# Introduction

The WF from the last meeting [1] lists the following issues. This contribution addresses them.

Issue 2-1-2: performance metric for NR-NTN and IoT-NTN only support scenario 2

Agreements:

o   For IoT-NTN and NR-NTN, typical usage scenarios are:

  Scenario 1: both head+hand mode and hand only mode.

  Scenario 2: hand only mode targeted for specific UE orientation usage

  Scenario 3: hand only mode targeted for arbitrary UE orientation usage

o   UE shall declare support for only one scenario per band.  The declaration details of each scenario should be further discussed.

o   For IoT-NTN and NR-NTN handheld UEs declared Scenario 2, the performance metrics are partial sphere metric. FFS details

o   For IoT-NTN and NR-NTN handheld UEs declared Scenario 3, the performance metrics are full sphere TRP and TRS

# Discussion

Scenario 1 and 3 are for arbitrary orientation and the associated performance metrics should be measured over full sphere using the existing formulae and measurement grids. While the performance in scenario 2 aims for specific direction and is measured over a partial sphere. The first step for scenario 2 is to define a reference coordinate for manufacturers to declare a specific direction over which the performance is assessed. Figure 6.1-1 in TR 38.870 [2] can be re-used for this purpose and is reproduced below.

Proposal 1: use the reference coordinate in Figure 6.1-1 of TR 38.870 to declare performance direction in scenario 2.

Once a reference coordinate relative to a device is defined, performance direction can be declared using () in spherical coordinate.

Proposal 2: use using () of spherical coordinate under reference coordinate in Figure 6.1-1 of TR 38.870 to declare performance orientation in scenario 2

As agreed in WF [1] from the last meeting, scenario 2 should be measured over a partial sphere. This necessitates definition of a solid angle centred at the declared () direction to form a partial sphere. At FR1 frequency range, the beam width from mobile phones should be comparable to or broader than 30 degrees. Namely, a solid angle of 30 degrees seems to be adequate for the partial sphere in scenario 2. Another reason for 30 degree solid angle is to use 15 degree grid points.

Proposal 3: use [30] degree solid angle centred at the declared direction of () to form a partial sphere in scenario 2.

The TRP and TRS formulae in TR 38.870 [2] can be modified as follows to adapt to a partial sphere for scenario 2 assuming the declared orientation is (dd). The first step is to transform the existing coordinate to a new one in the direction of (dd). The second step is to integrate using the formulae below with 15 degrees being half of 30 degrees. The third step is to replace 4 by 2(1-cos(/2)), which is the solid angle subtended by a cone of degrees.

TRP= 12π(1-cos(Δθ/2))θ=0θ=Δθ/2ϕ=0ϕ=2πEIRPθθ,ϕ+EIRPϕθ,ϕsinθdϕdθ

TRS=2π(1-cos(Δθ/2))θ=0θ=Δθ/2ϕ=0ϕ=2π1EISθθ,ϕ+1EISϕθ,ϕsinθdϕdθ

# Conclusion

This contribution has the following proposals.

Proposal 1: use the reference coordinate in Figure 6.1-1 of TR 38.870 to declare performance direction in scenario 2.

Proposal 2: use using () of spherical coordinate under reference coordinate in Figure 6.1-1 of TR 38.870 to declare performance orientation in scenario 2

Proposal 3: use [30] degree solid angle centred at the declared direction of () to form a partial sphere in scenario 2.

# Reference

- R4-2504763 WF for [114bis][328] TRP_TRS_MIMO_OTA_Ph3

- TR 38.870 v18.4.0 Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS)