3GPP TSG-RAN WG4 Meeting #115												R4-2505399

Malta, 19 – 23 May, 2025

Agenda item:			7.14.3.1

Source:	Huawei, HiSilicon

Title:	on XR OTA tests

Document for:	approval

# Introduction

The WF [1] captures the discussion on alternative split grid method for XR devices. This contribution presents a way of estimating measurement uncertainty for split measurement grid method.

# Discussion

The split measurement grid method proposed in R4-2502367 [2] performs the same test four times with different grid positions and combines the results. Each time the XR device, e.g. a pair of glasses, would be removed from the phantom and put back after fully charging its battery. Measurement Uncertainty (MU) related to use of phantom in 38.870 [3] has several components, one of which is “Positioning Uncertainty from using Phantoms” in clause B.2.11.5. Assuming the MU value for this component is MUXR_positioning and using the principle of Root Sum Square to account for four positioning MUs during the split measurement grid process, the final MU after four positioning would be

MUXR_positioning_split_grid = sqrt (4 x MUXR_positioning2) = 2 x MUXR_positioning

The actual value of MUXR_positioning is not yet available. The proposal below is on how to calculate the composite positioning MU for split measurement grid method.

Proposal 1: the composite positioning MU value in split measurement grid method should be twice of the “positioning uncertainty from using phantom”.

# Conclusion

This contribution makes the following proposal for the split measurement grid method.

Proposal 1: the composite positioning MU value in split measurement grid method should be twice of the “positioning uncertainty from using phantom”.

# Reference

- R4-2504763 WF for [114bis][328] TRP_TRS_MIMO_OTA_Ph3

- R4-2502367 CR for split measurement grids method for TRP/TRS measurements

- TR 38.870 v18.4 Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS)