3GPP TSG-RAN WG4 Meeting #115	R4-2505405

# Malta, Malta,  19th May - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.741 | CR | 0005 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_LSband-Core) Additional power back-off simulation results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_LSband-Core |  |  |  |  |  | Date: |  |  | 2025-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 38.741 captures power back-off simulation results for the combined L-/S-band submitted by different proponents. This CR provides additional results for this band obtained during power back-off analysis. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A set of additional power back-off results is added. These results do not impact already harmonised power back-off values and agreed A-MPR tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.2.4a(new), 6.2.1.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Power back-off results are taken from R4-2504259 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 6.2.1.2.4a	Additional maximum power reduction (company #4)

The following common parameters are assumed for the power back-off simulation:

-	Operating band 1610 MHz – 1626.5 MHz

-	Power class PC3

-	WOLA processing

-	I/Q image -28 dBc

-	Carrier leakage -28 dBc

-	CIM3 -60 dBc, CIM5 -70 dBc

-	Only additional emission requirements from FCC (NS_03N) and ETSI (NS_04N, NS_05N) regulations are simulated.

-	Other gating factors (ACLR, SEM, etc.) are excluded because they are handled by MPR.

###### 6.2.1.2.4a.1	Power back-off results for FCC

###### 6.2.1.2.4a.1.1	5MHz channel

Figure 6.2.1.2.4a.1.1-1: 5MHz channel at Fc = 1612.5 MHz.

Figure 6.2.1.2.4a.1.1-2: 5MHz channel at Fc = 1613.9 MHz.

Figure 6.2.1.2.4a.1-3: 5MHz channel at Fc = 1615.7 MHz.

###### 6.2.1.2.4a.1.2	10MHz channel

Figure 6.2.1.2.4a.1.2-1: 10MHz channel at Fc = 1615MHz.

Figure 6.2.1.2.4a.1.2-2: 10MHz channel at Fc = 1620.1MHz.

Figure 6.2.1.2.4a.1.2-3: 10MHz channel at Fc = 1621.5MHz.

###### 6.2.1.2.4a.1.3	15MHz channel

Figure 6.2.1.2.4a.1.3-1: 15MHz channel at Fc = 1617.5 MHz.

Figure 6.2.1.2.4a.1.3-2: 15MHz channel at Fc = 1619 MHz.

###### 6.2.1.2.4a.2	Power back-off results for ETSI sub-range 1

The NS_04N results are identical to NS_03N results in section 6.2.1.2.4a.1.1.

###### 6.2.1.2.4a.3	Power back-off results for ETSI sub-range 2

###### 6.2.1.2.4a.3.1	5MHz channel

Figure 6.2.1.2.4a.3.1-1: 5MHz channel at Fc = 1624 MHz.

###### 6.2.1.2.4a.3.2	10MHz channel

Figure 6.2.1.2.4a.3.2-1: 10MHz channel at Fc = 1615 MHz.

Figure 6.2.1.2.4a.3.2-2: 10MHz channel at Fc = 1620.1 MHz.

Figure 6.2.1.2.4a.3.2-3: 10MHz channel at Fc = 1621.5 MHz.

###### 6.2.1.2.4a.3.3	15MHz channel

Figure 6.2.1.2.4a.3.3-1: 15MHz channel at Fc = 1617.5 MHz.

Figure 6.2.1.2.4a.3.3-2: 15MHz channel at Fc = 1619 MHz.

##### 6.2.1.2.5	Harmonised A-MPR values

Tables 6.2.1.2.5-1 - 6.2.1.2.5-3 below present "harmonised" A-MPR values for 5, 10 and 15MHz channels at reference frequencies considered for the power back-off simulations and measurements. For the sake of simplicity only "full" RB allocation cases are used to compare results from different sources.

Table 6.2.1.2.5-1: Harmonised A-MPR values for the 5MHz channel.

|  | 5MHz channel centre frequency |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1612.5MHz |  |  | 1613.9MHz |  |  | 1615.5MHz |  |  | 1620.5MHz |  |  | 1622.4MHz |  |  | 1624MHz |  |  |
|  | CP QP | DFT |  | CP QP | DFT |  | CP QP | DFT |  | CP QP | DFT |  | CP QP | DFT |  | CP QP | DFT |  |
|  |  | QP | PI |  | QP | PI |  | QP | PI |  | QP | PI |  | QP | PI |  | QP | PI |
|  | ETSI regulatory domain |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #1 | 6 | 3.5 | 2.5 | 4 | 2.5 | 1 | 3 | 1 | 0.5 | 3 | 1 | 0.5 | 3 | 1 | 1 | 3 | 1.5 | 1.5 |
| #2 | 6 | 4 | 3 | - | - | - | 3 | 1.5 | 0.1 | 3 | 0.8 | - | - | - | - | 3 | 1.2 | 0.7 |
| #3 | 5.5 | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #4 | 9 | 7.5 | 6.5 | 7 | 6 | 5 | 5 | 3.5 | 2 |  |  |  |  |  |  |  |  | 1 |
| => | 6 | 4 | 3 | 4 | 2.5 | 1 |  |  |  |  |  |  | 3 | 1 | 1 | 3 | 1.5 | 1.5 |
|  | FCC regulatory domain |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #1 | 6 | 3.5 | 2.5 | 4 | 2.5 | 1 | 3 | 1 | 0.5 | 3 | 1 | 0.5 |  |  |  | 3 | 1 | 0.5 |
| #3 | 5.5 | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #4 | 9 | 7.5 | 6.5 | 7 | 6 | 5 | 5 | 3.5 | 2 |  |  |  |  |  |  |  |  |  |
| => | 6 | 4 | 3 | 4 | 2.5 | 1 |  |  |  |  |  |  |  |  |  | 3 | 1 | 0.5 |

Table 6.2.1.2.5-2: Harmonised A-MPR values for the 10MHz channel.

|  | 10MHz channel centre frequency |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1615MHz |  |  | 1620MHz |  |  | 1621.5MHz |  |  |
|  | CP-OFDM QPSK | DFT-s-OFDM |  | CP-OFDM QPSK | DFT-s-OFDM |  | CP-OFDM QPSK | DTT-s-OFDM |  |
|  |  | QPSK | PI/2 BPSK |  | QPSK | PI/2 BPSK |  | QPSK | PI/2 BPSK |
|  | ETSI regulatory domain |  |  |  |  |  |  |  |  |
| #1 | 12.5 | 7 | 6 | 4 | 3 | 2 | 4 | 2.5 | 2 |
| #2 | 7.5 | 5 | 3.5 |  |  |  | 4.5 | 2.5 | 2 |
| #3 | 7 | 5 |  |  |  |  |  |  |  |
| #4 | 14 | 13 | 12.5 | 8.5 | 7 | 6 | 7.5 | 6 | 5.5 |
| => | 9 | 6 | 4 | 4.5 | 3 | 2 | 4.5 | 2.5 | 2 |
|  | FCC regulatory domain |  |  |  |  |  |  |  |  |
| #1 | 12.5 | 7 | 6 | 4 | 2 | 1.5 | 3 | 2 | 1.5 |
| #3 | 7 | 5 |  |  |  |  |  |  |  |
| #4 | 14 | 13 | 12 | 8.5 | 7 | 6 | 7.5 | 6 | 5.5 |
| => | 9 | 6 | 4 | 4 | 2 | 1.5 | 3 | 2 | 1.5 |

Table 6.2.1.2.5-3: Harmonised A-MPR values for the 15MHz channel.

|  | 15MHz channel centre frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1617.5MHz |  |  | 1619MHz |  |  |
|  | CP-OFDM QPSK | DTF-s-OFDM |  | CP-OFDM QPSK | DTF-s-OFDM |  |
|  |  | QPSK | PI/2 BPSK |  | QPSK | PI/2 BPSK |
|  | ETSI regulatory domain |  |  |  |  |  |
| #1 | 13.5 | 11.5 | 11.5 | 11 | 9.5 | 7.5 |
| #2 | 7.5 | 5 | 6.5 | 7 | 4.5 | 3.5 |
| #3 | 8.5 | 5.5 |  |  |  |  |
| #4 | 14.5 | 13.5 | 13 | 14 | 13 | 12.5 |
| => | 10 | 7 | 6.5 | 9 | 6 | 5.5 |
|  | FCC regulatory domain |  |  |  |  |  |
| #1 | 13.5 | 9.5 | 9 | 11 | 8.5 | 6 |
| #3 | 8.5 | 5.5 |  |  |  |  |
| #4 | 14.5 | 13.5 | 13 | 14 | 13 | 12.5 |
| => | 10 | 7 | 6.5 | 9 | 6 | 5.5 |