3GPP RAN WG4 Meeting #115																	R4-2505407

Malta, Malta, 19th – 23rd May 2025

Agenda item:	5.2

Source:	Apple, Globalstar

Title:	Clarification for the PSD limits for a UE operating in 1610-1626.5 MHz frequency range

WI/SI:	NR_NTN_LSband-Core

Release:	Rel-18

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. After the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band where the UL is on the L-band and the DL is on the S-band [2], which finally was specified as the NTN band 254/n254. And during the RAN#105 meeting another Rel-19 spectrum WI was agreed [3], aim of which is to standardise the IOT TDD band that will operate in the UL part of the combined L-/S-band.

The 3GPP combined L-/S-band is defined to support both FCC and ETSI standards, the description of which is captured in 3GPP TRs. And as for the ETSI standards, 3GPP core specifications capture additional ETSI in-band and out-of-band emission requirements, as well as PSD limits, for a UE operating in 1610-1626.5 MHz frequency range. It is worth noting that the ETSI standards define two PSD thresholds for this frequency range, both of which are captured accordingly in the core specifications, but it is not entirely clear which one should be applied and when.  Thus, in this discussion paper we present an overview of the existing regulations for these PSD limits suggesting 3GPP to consider further clarifications.

# 2	PSD limits for 1610-1626.5MHz frequency range

## 2.1	Background of WRC and ETSI standards

As already mentioned in the Introduction part, existing ETSI standards define two PSD limits, as presented below in the excerpt from EN 301 441. However, while the ETSI standard mentions both -3 dBW/4kHz and -15 dBW/4kHz thresholds, it is not clear which one is enforced and under which conditions.  For instance, a more stringent threshold of -15dBW/4kHz corresponds to approximately 31.5dBm/180kHz, which should not be an issue for the existing 3GPP NTN handheld power classes even in a scenario when all power is focused in one UL resource block of 180kHz. On the other hand, 3GPP has been contemplating even higher power classes. Furthermore, NB-IOT supports the operational mode when fewer than 12 sub-carriers can be scheduled to a UE, which may restrict maximum output power with the -15dBW/4kHz PSD threshold. As an example, -15dBW/4kHz would correspond to 21.7dBm/15kHz PSD, which will be exceeded even with the 23dBm PC3 power class if the NB-IOT UE is scheduled with one sub-carrier. And since 3GPP already plans to standardise up to 31dBm power class for the NB-IOT devices, the more stringent PSD threshold will become quite a limiting factor.

Observation 1a:	ETSI has two PSD thresholds for the combined L-/S-band, -3dBW/4kHz and -15dBW/4kHz.

Observation 1b:	While a more stringent PSD threshold of -15dBW/4kHz (i.e. approx. 31.5dBm/180kHz) should not be an issue for the existing 3GPP NTN power classes, that threshold can be reached with potentially new higher power classes and/or when fewer than 12 sub-carriers are scheduled.

4.2.3 EIRP density within the operational band

4.2.3.1 Justification

To ensure that the maximum EIRP spectral density within the band 1 610 MHz to 1 626,5 MHz does not exceed the limits defined by the Final Acts of WRC-95 [6], S5.364.

4.2.3.2 Technical requirements

In any frequency sub-band of the band 1 610 MHz to 1 626,5 MHz where the MES is declared to operate, one of two following requirements shall apply under specific operating conditions:

a) The MES shall not produce a mean EIRP density exceeding -3 dB (W/4 kHz), (mean limit).

NOTE: In this context, the mean is the mean over time whilst the MES is in the carrier-on mode. Or b) The MES shall not produce a peak EIRP density exceeding -15 dB (W/4 kHz), (peak limit).

The specific frequency sub-band(s) and operating conditions for which the two different limits apply shall be specified and declared by the applicant in the information leaflet.

These requirements apply to all types of MES, for every transmit channel of the MES in its operational band or sub-bands.

The conformance requirements apply for the full range of environmental conditions corresponding to the type of equipment as described in clause 4.1.

Figure 2.1-1: Excerpt from ETSI EN 301 441, sub-clause 4.2.3.

Our understanding is that two PSD thresholds in the ETSI specifications come from the corresponding WRC resolutions accounting for a potential scenario when either mobile satellite (MSS) or radio-determination satellite (RDSS) service is deployed in the same frequency range as aeronautical radionavigation, whereupon a more stringent threshold is applied as agreed by the affected Administration(s); otherwise, a relaxed threshold is used.  For the sake of clarity, we provide below provisions 5.364 and 5.366 as an excerpt from the international radio regulations.

5.364 The use of the band 1 610-1 626.5 MHz by the mobile-satellite service (Earth-to-space) and by the radiodetermination-satellite service (Earth-to-space) is subject to coordination under No. 9.11A. A mobile earth station operating in either of the services in this band shall not produce a peak e.i.r.p. density in excess of -15 dB(W/4 kHz) in the part of the band used by systems operating in accordance with the provisions of No. 5.366 (to which No. 4.10 applies), unless otherwise agreed by the affected administrations. In the part of the band where such systems are not operating, the mean e.i.r.p. density of a mobile earth station shall not exceed –3 dB(W/4 kHz). Stations of the mobile-satellite service shall not claim protection from stations in the aeronautical radionavigation service, stations operating in accordance with the provisions of No. 5.366 and stations in the fixed service operating in accordance with the provisions of No. 5.359. Administrations responsible for the coordination of mobile-satellite networks shall make all practicable efforts to ensure protection of stations operating in accordance with the provisions of No. 5.366.

5.366 The band 1 610-1 626.5 MHz is reserved on a worldwide basis for the use and development of airborne electronic aids to air navigation and any directly associated ground-based or satellite-borne facilities. Such satellite use is subject to agreement obtained under No. 9.21.

Figure 2.1-2: Excerpt from radio regulations, provisions 5.364 and 5.366.

In addition to that, there also exists the provision 5.371 that indicates that in Region 1 countries the RDSS service has the secondary priority to MSS, and thus cannot claim protection from MSS.

5.371 Additional allocation: in Region 1, the band 1 610-1 626.5 MHz (Earth-to-space) is also allocated to the radiodetermination-satellite service on a secondary basis, subject to agreement obtained under No. 9.21. (WRC-12)

Figure 2.1-3: Excerpt from radio regulations, provision 5.371.

Observation 2a:	A more stringent threshold of -15dBW/4kHz is applied when Administration(s) use MSS/RDSS services together with aeronautical radionavigation systems in the same band.

Observation 2b:	The RDSS service is secondary to MSS in Region 1 and thus cannot claim protection from the MSS services in the Region 1 countries.

## 2.2	Potential impact to 3GPP

3GPP specifications have the corresponding NS flags, NS_04N and NS_05N, which are set by the operator to indicate to the UE to follow the ETSI standards while operating on 1610-1626.5 MHz frequency range. And as already discussed in 3GPP, same NS flags can be used with a new IOT TDD band that will operate in the same frequency range. However, existing definition of the NS flags mention both PSD thresholds creating unnecessary ambiguity on which PSD threshold should be followed at the UE side.  Based the considerations presented in the previous sub-section, it is not necessary to apply always the -15dBW/4kHz PSD limit. Thus, we suggest discussing the following approaches:

a)	We can remove a more stringent -15dBW/4kHz threshold from the existing definitions of NS_04N and NS_05N.

b)	To be on a safe side, we can consider defining new NS flag(s) which will be explicitly associated with a more stringent threshold of -15dBW/4kHz so that the 3GPP has a way to activate more stringent requirements.

From the viewpoint of the 3GPP specifications, option b) can be viewed as a complementary step to option a). In other words, we can change existing NS flags NS_04N and NS_05N so that they are associated only with a more relaxed PSD threshold -3dBW/4kHz. An exemplary change to the 3GPP specification can be found in Annex A that can be applied to TS 38.101-5 (NR) and TS 36.102 (IOT).

Proposal 1a:	Clarify the definition of the existing flags, NS_04N and NS_05N, so that they capture only one PSD threshold of -3dBW/4kHz.

Proposal 1b:	Introduce, if needed, new NS flag(s) that will be associated with a more stringent PSD threshold of -15dBW/4kHz.

Proposal 2:	Capture existing background information on international regulations and WRC provisions in the 3GPP TR 38.741.

# 3	Conclusions

In this discussion paper we have presented our considerations on two PSD thresholds associated with the existing NS flags. And as explained in our paper, a more stringent PSD threshold does not have to be always applied because it is enforced only by the affected Administration(s) when MSS/RDSS services are used in the same frequencies as aeronautical radionavigation. Thus, to avoid unnecessary limitations of the UE maximum output power, we suggest removing a more stringent PSD threshold from the existing NS_04N and NS_05N flags. If preferred, new NS flags can be defined which will be explicitly associated with a more stringent requirement.

The corresponding changes to TS 38.101-5 and TS 36.102 can be found in [4] and [5] respectively. And in addition to changes to the 3GPP core specifications, we also propose capturing all related considerations in the corresponding 3GPP TR 38.741 [6].

Proposal 1a:	Clarify the definition of the existing flags, NS_04N and NS_05N, so that they capture only one PSD threshold of -3dBW/4kHz.

Proposal 1b:	Introduce, if needed, new NS flag(s) that will be associated with a more stringent PSD threshold of -15dBW/4kHz.

Proposal 2:	Capture existing background information on international regulations and WRC provisions in the 3GPP TR 38.741.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

- RP-242415, "New WID for introduction of IoT-NTN TDD mode", Iridium Satellite LLC

- R4-2505409, "(NR_NTN_LSband-Core) Clarification for the PSD limits for a UE operating in 1610-1626.5 MHz frequency range", Apple, Globlastar

- R4-2505411, "(IoT_NTN_FDD_LS_band-Core) Clarification for the PSD limits for a UE operating in 1610-1626.5 MHz frequency range", Apple, Globalstar

- R4-2505408, "(NR_NTN_LSband-Core) Additional information on the international regulations and WRC provisions for a UE operating in 1610-1626.5 MHz frequency range", Apple, Globalstar

Annex A: Pseudo CR for 3GPP specifications

## 6.2	Transmitter power

### 6.2.1	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1-1: UE Power Class

| NR satellite band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n256 | 23 | ±2 |
| n255 | 23 | ±2 |
| n254 | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated |  |  |

The UE shall meet the following additional requirements for maximum mean transmission power density specified in Table 6.2.1-2 when NS is signaled and when the configured channel overlaps with any portion of the specified frequency range.

Table 6.2.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum power density |
| --- | --- | --- | --- | --- |
| n254 | NS_04N | 5 | 1610 - 1618.25 | 27dBm/4kHz (mean) |
|  | NS_05N | 5 | 1618.25 - 1626.5 |  |
|  |  | 10, 15 | 1610 – 1626.5 |  |