3GPP RAN WG4 Meeting #115																	R4-2505419

Malta, Malta, 19th – 23rd May 2025

Agenda item:	7.10.3.2.1

Source:	Apple

Title:	Initial power back-off results for PC2 on NTN S-band

WI/SI:	NR_IoT_NTN_req_test_enh

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already that time it was noted that there exist other satellite deployments. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI.

After the RAN#104 meeting a new WI was agreed [2], one of the aims of which is to enable higher power class PC2 for all satellite bands including band n256. Thus, in this discussion paper we present our preliminary power back-off simulation results for the PC2 power class operating in band n256 frequency range.

# 2	Power back-off simulations for NTN S-band

## 2.1	Simulation assumptions and scenarios

Satellite S-band (n256) was standardised during Rel-17, whereupon only the PC3 power class was assumed and for which existing A-MPR requirements from NS_24 were re-used. Here we present further results for the PC2 power class with the common assumptions as presented below:

-	Tx power: PC2 (+26dBm)

-	LO placement: always in the centre of the carrier

-	Regulations: Requirements from NS_24

For the sake of completeness, we list below the 5MHz channel center frequencies, for which we run power back-off simulations:

-	Fc=2002.5MHz

-	Fc=1997.5MHz

-	Fc=1992.5MHz

## 2.2	Power back-off simulations

Figure 2.2-1, Figure 2.2-2 and Figure 2.2-3 present results for the 5MHz channel with the PC2 power class. The results are in line with the existing 5MHz channel A-MPR values for PC3 as defined by NS_24. For instance, while the large allocation with CP-OFDM and PC3 would need up to 13dB A-MPR, in case of PC2 the maximum power baack-off increases up to 15dB. Nevertheless, it is worth noting that large power back-off is needed only for the full channel allocations, which are not likely to be used with the NTN deployments.  Once the 5MHz channel is shifted father away from the upper band edge, one can notice an improvement in the required power back-off values. For instance for the 5MHz channel residing at 1992.5MHz A-MPR would be need only for the large allocations, whereas small allocations do not even need MPR.

Figure 2.2-1: Power back-off results for 5MHz at Fc=2002.5MHz.

Figure 2.2-2: Power back-off results for 5MHz at Fc=1997.5MHz.

Figure 2.2-3: Power back-off results for 5MHz at Fc=1992.5MHz.

# 3	Conclusions

In this discussion paper we have presented our initial power back-off simulation results for the NTN band n256 with the 5MHz channel and the PC2 power class. The obtained results are generally in line with the existing A-MPR requirements already defined for the PC3 power class with the obvious difference that large RB and band edge RB allocations need even larger power back-off.

Proposal 1:	Capture PC2 A-MPR requirements for the NTN band n256.

Proposal 2:	We ask RAN WG4 to discuss further whether we need to simulate PC2 for allocations larger than 25RB, i.e. 5MHz channels, for those cases which are already A-MPR limited even for PC3.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"