3GPP RAN WG4 Meeting #115																R4-2505421

Malta, Malta, 19th – 23rd May 2025

Agenda item:	7.10.4.2

Source:	Apple

Title:	Power back-off values for the 3MHz channel in band n256

WI/SI:	NR_IoT_NTN_req_test_enh

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications.

After the RAN#104 meeting a new WI was agreed [2], one of the aims of which is to enable the 3MHz channel for the NTN bands including the band n256. During the previous RAN WG4 meetings initial power back-off simulation results were submitted to for technical discussions and were captured in [3]; however, there was no final conclusion on which A-MPR regions should be defined with associated power back-off values. Thus, in this discussion paper we present UE power back-off simulation results for the 3MHz channel in the NTN band n256 frequency range.

# 2	Power back-off for band n256

## 2.1	Simulation assumptions and scenarios

The satellite S-band (n256) was standardised already in Rel-17, whereupon the smaller considered channel bandwidth was 5MHz. Here we present power back-off results for the PC3 power class and the 3MHz channel with the following common assumptions:

-	Tx power: PC3 (+23dBm)

-	LO placement: always in the centre of the carrier

-	Regulations: NS_24

The power back-off simulations were conducted for the following center frequencies:

-	the right-most channel with the 5MHz offset, as "guard" band, from the upper edge of the band (Fc=2003.5MHz)

-	the upper channel with the 6.5MHz offset from the upper edge (Fc=2002MHz)

-	the upper channel with the 8MHz offset from the upper edge (Fc=2000.5MHz)

-	the upper channel with the 9.5MHz offset from the upper edge (Fc=1999MHz)

## 2.2	Power back-off simulations

Figures 2.2-1, 2.2-2 and 2.2-3 below present the power back-off simulation results for the 3MHz channel at three different locations as mentioned in the section 2.1. As anticipated, the upper most channel has the highest power back-off values. For instance, the full channel allocation of 15RB might require 6dB power back-off. And as the channel shifts to the left from the upper edge of the band, the power back-off decreases as well.

Figure 2.2-1: Power back-off results for NS_24 (Fc=2003.5MHz).

Figure 2.2-2: Power back-off results for NS_24 (Fc=2002MHz).

Figure 2.2-3: Power back-off results for NS_24 (Fc=2000.5MHz).

Figure 2.2-4: Power back-off results for NS_24 (Fc=1999MHz)

## 2.3	Summary and proposed A-MPR regions

Based on the power back-off simulations we the A-MPR regions for the 3MHz channel can be defined as presented below in Table 2.3-1 and 2.3-2.

Table 2.3-1: A-MPR regions for the band n256 3MHz channel

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region |  |  |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS | LCRB*12*SCS | A-MPR |
| 3MHz | 2002 < fc <= 2003.5 |  | > 1.08 | A1 |
|  |  | >= 2.52 | <= 1.08 > 0.72 | A2 |
|  | 2000.5 < fc <= 2002 |  | >= 1.44 | A3 |
|  | 1999 < fc <= 2000.5 |  | >= 1.8 | A4 |
|  | 1997.5 < fc <= 1999 |  | >= 2.52 | A2 |

Table 2.3-2: A-MPR values for the band n256 3MHz channel

|  | Modulation | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 6.0 | 1.0 | 3.5 | 2.0 |
|  | QPSK | 6.5 | 1.5 | 4.0 | 2.5 |
|  | 16QAM | 7.0 |  | 4.5 | 3.0 |
|  | 64QAM | 7.0 |  | 4.5 |  |
| CP-OFDM | QPSK | 9.0 |  | 6.5 | 5.0 |
|  | 16QAM | 9.0 |  | 6.5 | 5.0 |
|  | 64QAM | 9.0 |  | 6.5 | 5.0 |

## 2.4	Further discussions

As discussed during the RAN4#114 meeting and captured in [3], it might be beneficial to make A-MPR frequency ranges more compact by grouping together some of them. It was concluded in [4] to group frequency regions as follows:

-	1997.5 < fc <= 1999 and 1999 < fc <= 2000.5;

-	2000.5 < fc <= 2002 and 2002 < fc <= 2003.5.

Based on the presented considerations we suggest defining A-MPR values as presented in Table 2.4-1 and Table 2.4-2 below.

Table 2.4-1: A-MPR regions for the band n256 3MHz channel

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region |  |  |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS | LCRB*12*SCS | A-MPR |
| 3MHz | 2000.5 < fc <= 2003.5 |  | > 1.08 | A1 |
|  |  | >= 2.52 | <= 1.08 > 0.72 | A2 |
|  | 1997.5 < fc <= 2000.5 |  | >= 1.8 | A4 |

Table 2.4-2: A-MPR values for the band n256 3MHz channel

|  | Modulation | A1 | A2 | A4 |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 6.0 | 1.0 | 2.0 |
|  | QPSK | 6.5 | 1.5 | 2.5 |
|  | 16QAM | 7.0 |  | 3.0 |
|  | 64QAM | 7.0 |  |  |
| CP-OFDM | QPSK | 9.0 |  | 5.0 |
|  | 16QAM | 9.0 |  | 5.0 |
|  | 64QAM | 9.0 |  |  |

Proposal:	Define A-MPR for the 3MHz channels grouped around the following frequency ranges: 1997.5<fc<=2002.5 and 2000.5<fc<= 2003.5.

# 3	Conclusions

In this discussion paper we have presented our further results for the power back-off values for the 3MHz channel operating in the NTN band n256 frequency range. As can be seen from the results, a smaller 3MHz channel needs lower power back-off when compared to the 5MHz channel. Nevertheless, some of the 3MHz channel allocations still needs A-MPR when it is configured at the upper edge of the band.

Proposal:	Define A-MPR for the 3MHz channels grouped around the following frequency ranges: 1997.5<fc<=2002.5 and 2000.5<fc<= 2003.5.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"

- R4-2502282, "WF for [113][311] NR_IoT_NTN_less_than_5MHz_UERF", Xiaomi

- R4- 2504713, "WF for [114bis][310] NR_IoT_NTN_less_than_5MHz_UERF", Xiaomi