3GPP RAN4 Meeting #115		R4-2505426

Malta, Malta, May 19-23, 2025

Agenda item:	7.14.3.2

Source:	Apple, GIST, NRRA

Title:	On NTN TRP TRS test methodology

WI/SI:	TRP_TRS_MIMO_OTA_Ph3-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

During the RAN #103 meeting a new Work Item on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3 was approved with the following scope related to IoT/NR NTN devices [1]:

- Study and define test methodology and radiated performance metric for FR1 NTN devices (including NR NTN and IoT NTN) - Study the usage scenarios and develop enhanced test methodology - Study and specify the proper performance metric - Develop preliminary Measurement Uncertainty (MU) (RAN5)

During the RAN4 #112bis meeting preliminary agreements were reached on the sub-topic related to NTN TRP/TRS methodology [2].  During the subsequent meetings these were supplemented with [3] and [4]:

Issue 3-1-2: General view on Usage scenarios for handheld IoT-NTN and NR-NTN Agreements: - For IoT-NTN and NR-NTN, typical usage scenarios are: - Scenario 1: both talk mode (head+hand, and hand only) and browsing mode. - Scenario 2: browsing mode (hand only) - For IoT-NTN UEs not supporting voice call or only supporting voice call with loud speaker etc. are only required and tested with hand only mode. - UEs supporting Scenario 1 are tested with head+hand and hand only modes - UEs supporting Scenario 2 only are only tested with hand only mode Issue 3-1-3: Handheld IoT-NTN and NR-NTN performance metric Agreements: - For IoT-NTN and NR-NTN handheld UEs supporting Scenario 1 the performance metrics are full sphere TRP and TRS - For IoT-NTN and NR-NTN handheld UEs supporting Scenario 2 only, further study performance metric Issue 3-1-4: non-Handheld IoT-NTN performance metric Agreements: - FFS on partial sphere metrics, such as the radiated power and receive sensitivity within a certain angular range, should be performance metric for non-handheld IoT NTN devices. - This can be considered as a package with issue 3-1-3. Issue 3-1-5: Whether performance metric should be same for GEO and NGEO for handheld UEs (both NR/IoT NTN) Agreements: - For UEs support both GSO and NGSO, single set of performance metric is required for GSO and NGSO handheld UEs. - For UE support only GSO or NGSO, FFS performance metric should be the same. Issue 3-1-6: How to determine hand only mode for NR/IoT NTN mobile handheld UE (if we call it as Type 1 NTN UEs, usage-mode restriction) Agreements: - RAN4 to discuss how to distinguish UEs which support Scenario 1 (talk and browsing modes) vs. Scenario 2 only (browsing mode only) Issue 3-2-1: detailed test parameters for NR-NTN band n254 Agreements: - The test parameters in Issue 3-3-1 in WF R4-2416590 for NR-NTN n254 OTA testing is confirmed. CR will be provided next meeting. Issue 3-2-2: Detailed test parameters for IoT-NTN Agreements: - The test parameters in Issue 3-3-2 in WF R4-2416590 for IoT-NTN OTA testing is confirmed. CR will be provided next meeting. Issue 3-2-3: Doppler conditions for NTN Agreements: - The following setting can be a starting point for further checking: - NR/IoT NTN radiated requirements shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites. Issue 3-2-4: How to resolve UE antenna circular polarization issue Proposals - Proposal 1: Use circular polarisation test antennas to assess the performance of the UE. This enables us to characterise the exact loss seen by the UE when operating in a system that utilises circular polarisation antennas, as all satcom systems do. - If we ignore circular polarisation testing at this stage, it may preclude the ability of an operator to leverage XPD to increase the capacity of their system. - Proposal 2: The NR FR1 NTN conformance testing should focus on LP rather than CP assessments. - WF - FFS Issue 3-2-6: Testing time reducing method for NTN Agreements: - FFS on the NTN RTT (Round Trip Time) to reduce TRS measurement time. Feedback from NTN operators and TE vendors requested.

Issue 3-1-1: UE manufacturer declaration of supported usage scenario for different NTN UE Agreements: - OEM declaration of scenario applicability can be used to declare the supported usage scenario. - Details on declaration and spec language can be further discussed. Issue 3-1-2: performance metric for NR-NTN and IoT-NTN only support scenario 2 Agreements: - RAN4 further discuss the performance metric for UE declared only support scenario 2. - Details on performance metric data processing should also be discussed. Issue 3-1-5: Whether performance metric should be same for GEO and NGEO for handheld UEs (both NR/IoT NTN) Agreements - Same performance metric for GSO and NGSO handheld UEs. Issue 3-2-1: NTN OTA test system, LP or CP for measurement antenna Agreements: - From a test system perspective, the baseline test method for FR1 NTN is LP-based OTA test system. - For the full sphere performance metric TRP/TRS for NTN scenario 1, it is same as that defined in TR 38.870. Issue 3-2-3: Whether MU is impacted if NTN OTA test system go with LP architecture Agreements: - No additional MU contributor is identified for NTN OTA test system.

RAN4 #115 made the following agreements [5]:

Issue 2-1-2: performance metric for NR-NTN and IoT-NTN only support scenario 2 Agreements: o For IoT-NTN and NR-NTN, typical usage scenarios are: § Scenario 1: both head+hand mode and hand only mode. § Scenario 2: hand only mode targeted for specific UE orientation usage § Scenario 3: hand only mode targeted for arbitrary UE orientation usage o UE shall declare support for only one scenario per band. The declaration details of each scenario should be further discussed. o For IoT-NTN and NR-NTN handheld UEs declared Scenario 2, the performance metrics are partial sphere metric. FFS details o For IoT-NTN and NR-NTN handheld UEs declared Scenario 3, the performance metrics are full sphere TRP and TRS

Additionally, a draft CR was agreed during RAN4 #115, capturing the structure of the agreed usage scenarios and placeholder clauses for the corresponding performance metric [6].

This contribution provides our views related to this objective.

# 2	Discussion

Given the clear delineation of the scenarios applicable to NTN devices (see Issue 3-1-2 in [3]) and the agreement by RAN4 to introduce the OEM declaration (see Issue 3-1-1 in [4]), the OEM declaration of scenario applicability has been formulated for the TR, as captured in the draft CR in [6]:

The applicability of the OTA performance metric for IoT NTN and NR NTN UEs is also determined by this scenario declaration.  For handheld UEs declaring support of Scenario 1 and Scenario 3, the performance metric assumes the angles of arrival of the desired signal are likely from any direction and relative to any orientation of the device.  For this scenario RAN4 has already agreed that the full sphere TRP and TRS performance metrics apply.

Observation 1:	For Scenario 1 and 3 UEs, RAN4 has already agreed that the full-sphere TRP and TRS metrics apply.

For handheld UEs declaring support of Scenario 2, our past contribution described potential design optimizations that can be applicable [7].  Based on these observations, an argument can be made for defining the Scenario 2 radiated power and radiated sensitivity metrics as partial integrals over a subset of elevation angles, as an example, or other similar schemes.  Given this observation, it follows that the design and performance of such implementation-specific schemes can vary widely depending on form factor design, region, band, and satellite constellation.  Additional band-specific aspects, such as the impact of duplexer separation on the accuracy of such device positioning algorithms, should also be considered by RAN4.

Proposal 1:	For handheld IoT/NR NTN devices declaring support of Scenario 2, RAN4 to consider defining the metric to be the surface integral of EIRP/EIS over the specified range of angles and normalized by the isotropic radiator performance over the same angles.

We note that with Issue 3-2-1 [1] RAN4 agreed to a baseline of a full-sphere measurement, and it is our intention to follow this baseline to move the discussion forward.

Considering the implementation-specific nature of the related optimizations, the range of elevation and azimuth angles which have dominant impact on radiated performance of the device under the assumed user interaction scenario can vary by device and band.  RAN4 should consider an additional OEM declaration of the peak direction and range of elevation and azimuth angles over which the Scenario 2 test metric is calculated.

During the RAN4 #114bis discussion test equipment vendors raised a concern that an arbitrary number of declared parameters of the range of elevation and azimuth angles is not preferred due to the potential impact on positioning MU and test system complexity.  Considering this input, we note that the draft CR in [6] already captures three different angular ranges that need to be implemented by a test system:  {θ ∈ [0,180]o, φ ∈ [0,360] o } for the full sphere test, { θ ∈ [0, 90] o, φ ∈ [0, 360] o } for UHRP (Clause 5.3.1.2) and the potential new TBD range for PsRP (Clause 5.3.1.3).

Observation 2:	Based on current agreements, test systems already need to accommodate at least three different sets of angular ranges to be swept for the radiated power and sensitivity tests of NTN devices.

A practical observation of Scenario 2 devices is that even if an antenna design could be optimized to improve performance in hemisphere, it is not possible to fix the user’s position relative to both the horizon and the satellite simultaneously, even with the use of assistance via the user interface of the device.  For the evaluation of the test metric, this implies that the azimuth angle should still be swept over the full 360 degrees.  To keep the total number of possible test configurations smaller, we recommend quantizing the OEM declaration of the range of elevation angles to be in steps of 30 degrees, where a declaration of 90 degrees implies the applicability of the UHRP metric.

With the angular range handled by OEM declaration in this way, the remaining issue is how to select the θ = 0 direction in the test system.  As we had suggested in the last meeting, an OEM declaration of the initial position of the DUT can be one approach.

Proposal 2:	As one alternative, for handheld IoT/NR NTN devices declaring support of Scenario 2, RAN4 to consider defining an OEM declaration with the following parameters: the initial DUT position (θ0, φ0) and the maximum elevation span θM, selected out of the set of [0, 30, 60, 90]o. PsRP is calculated over the surface spanned by θ ∈ [θ0 -  θM  , θ0 +  θM  ]o and φ ∈ [0,360] o.

Another approach can be to rely on the test system to first measure the metric under test (EIRP or EIS) over the full sphere and then to select the θ = 0 direction to be the direction in which the metric under test is maximized.  This procedure can be analogous to the beam peak search in the FR2 test systems.

Proposal 3:	As another alternative, for handheld IoT/NR NTN devices declaring support of Scenario 2, RAN4 to consider defining an OEM declaration for the maximum elevation span θM, selected out of the set of [0, 30, 60, 90]o. PsRP is calculated over the surface spanned by θ ∈ [θ0 -  θM  , θ0 +  θM  ]o and φ ∈ [0,360] o, where (θ0, φ0) is the direction in which the metric under test (EIRP or EIS) is maximized.

# 3	Conclusions

This contribution provides our views on the topic of TRP/TRS methodology development for IoT/NR NTN devices.  The following observation and proposal are made:

Observation 1:	For Scenario 1 and 3 UEs, RAN4 has already agreed that the full-sphere TRP and TRS metrics apply.

Observation 2:	Based on current agreements, test systems already need to accommodate at least three different sets of angular ranges to be swept for the radiated power and sensitivity tests of NTN devices.

Proposal 1:	For handheld IoT/NR NTN devices declaring support of Scenario 2, RAN4 to consider defining the metric to be the surface integral of EIRP/EIS over the specified range of angles and normalized by the isotropic radiator performance over the same angles.

Proposal 2:	As one alternative, for handheld IoT/NR NTN devices declaring support of Scenario 2, RAN4 to consider defining an OEM declaration with the following parameters: the initial DUT position (θ0, φ0) and the maximum elevation span θM, selected out of the set of [0, 30, 60, 90]o. PsRP is calculated over the surface spanned by θ ∈ [θ0 -  θM  , θ0 +  θM  ]o and φ ∈ [0,360] o.

Proposal 3:	As another alternative, for handheld IoT/NR NTN devices declaring support of Scenario 2, RAN4 to consider defining an OEM declaration for the maximum elevation span θM, selected out of the set of [0, 30, 60, 90]o. PsRP is calculated over the surface spanned by θ ∈ [θ0 -  θM  , θ0 +  θM  ]o and φ ∈ [0,360] o, where (θ0, φ0) is the direction in which the metric under test (EIRP or EIS) is maximized.

# 4	References

RP-240841, “New WID: WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3,” Qualcomm, 3GPP RAN #103, March 2024

R4-2416590, “Way Forward for [112bis][323] TRP_TRS_Ph3,” vivo, 3GPP RAN4 #112bis, October 2024

R4-2419808, “WF for [113][325] TRP_TRS_Ph3,” vivo, 3GPP RAN4 #113, November 2024

R4-2502370, “Way Forward for [114][327] TRP_TRS_Ph3,” vivo, 3GPP RAN4 #114, February 2025

R4-2505426, “WF for [114bis][328] TRP_TRS_MIMO_OTA_Ph3,” vivo, 3GPP RAN4 #114bis, April 2025

R4-2504728, “Draft CR to 38.870 on NTN OTA performance metric and test method,” vivo, 3GPP RAN4 #114bis, April 2025

R4-2417757, “On NTN TRP TRS test methodology,” Apple, 3GPP RAN4 #113, November 2024

TR38.821, “Solutions for NR to support non-terrestrial networks (NTN)”