3GPP RAN WG4 Meeting #115																	R4-2505432

Malta, Malta, 19th – 23rd May 2025

Agenda item:	7.1.2.2.1

Source:	Apple

Title:	On Rel-19 power domain enhancement for FR1

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

Power domain enhancement is a re-occurring topic and enters the 4th phase when starting the Rel-19 work. The objectives are covered in the WID [1]. This contribution focuses on the high-level aspects of MPR reduction. The WID lists those goals as:

# 2  Discussion

##### 2.1 Converting outer RB allocations into inner RB allocations

#### Issue 1-1-1: RB based or CBW based extension - WF - Symmetrical/ Asymmetric for maximum extension at each side are determined by 0.5*NRB - If a limited number of additional fixed values are introduced, the extension is determined by X*NRB at each side, where X < 0.5 - The extended RBs are aligned with common RB grid #### Issue 1-1-2: Fixed or flexible extension - Agreement in main session: - Fixed value (0.5*CBW and/or 0.5*NRB) for Symmetrical/ Asymmetric by default for UE supporting the MPR reduction feature - In addition, consider a limited number of additional fixed values with capability

RAN4 found that a viable scheme for the power domain enhancement is the conversion of ‘outer’ RB allocations into ‘inner’ RB allocations. This is achieved via channel extension. It is proposed that the total number of RBs (“original UE channel RBs” + “RB extension”) is an element of Table 5.3.2-1 from TS 38.101-1.

The first part of the agreement considers extending the channel by half of its size to each side. There are two strategies possible. One is to extend lower and upper side by 0.5*NRB and apply the 2*CBW. In case of odd NRB values we propose to use the ceil() function. As shown in Table 1 this will not exceed the extended CBW. Asymmetric extension can be done as well but does not result into regular channel sizes. Table 2 provides the guard band for extending the lower side. To obtain clear boundaries for OOB requirements the total channel bandwidth would need to be specified. Especially, if other factors smaller than 0.5 would be agreed.

The advantage of extending NRB is that all channels can be extended by simply adding ceil(0.5*NRB) to the lower and upper channel edge. On the flip side it is more difficult to generalise extension other than ceil(0.5*NRB) and asymmetric extensions would require specifying dedicated channel sizes in MHz to get clear boundaries for OOB requirements.

The other option is to extend the original CBW by an existing channel size of Table 5.3.2-1 from TS 38.101-1. The maximum transmission bandwidth would be equal to the extended CBW. This works well for asymmetric extension. In case of symmetric extension, it often results into fraction numbers if both sides are extended equally. The ceil() function can be applied in those cases.

The advantage of extending to existing CBW and use the maximum transmission bandwidth of extended CBW is that it results into a clear extension path with good granularity and straight forward applicability for OOB requirements. On the flip side special handling would be required when considering extended CBW beyond 100MHz since there are no channels defined, and the maximum transmission bandwidth is not defined either.

Table 1: Symmetric CBW extension at each side by ceil(0.5*NRB) for SCS of 15kHz

| Original Channel |  | Extended Channel | Guard Band of extended channel for symmetric extension |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW in MHz | # RBs (As defined in TS 38.101-1 Table 5.3.2-1) | RB extension on each side (calculated as ceil(NRB/2)) | Extended CBW in MHz | Lower Guard in kHz | Upper Guard in kHz |
| 5 | 25 | 13 | 10 | 402.5 | 417.5 |
| 10 | 52 | 26 | 20 | 632.5 | 647.5 |
| 15 | 79 | 40 | 30 | 682.5 | 697.5 |
| 20 | 106 | 53 | 40 | 912.5 | 927.5 |
| 25 | 133 | 67 | 50 | 962.5 | 977.5 |
| 30 | 160 | 80 | 60 | 1192.5 | 1207.5 |
| 35 | 188 | 94 | 70 | 1152.5 | 1167.5 |
| 40 | 216 | 108 | 80 | 1112.5 | 1127.5 |
| 45 | 243 | 122 | 90 | 1162.5 | 1177.5 |
| 50 | 270 | 135 | 100 | 1392.5 | 1407.5 |

Table 2: Asymmetric CBW extension at lower side by ceil(0.5*NRB) for SCS of 15kHz

| Original Channel |  | Extended Channel | Guard Band of extended channel for asymmetric extension |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW in MHz | # RBs (As defined in TS 38.101-1 Table 5.3.2-1) | RB extension on lower side (calculated as ceil(NRB/2)) | Total Channel in MHz | Lower Guard in kHz | Upper Guard in kHz |
| 5 | 25 | 13 | 7.5 | 492.5 | 257.5 |
| 10 | 52 | 26 | 15 | 632.5 | 327.5 |
| 15 | 79 | 40 | 22.5 | 772.5 | 397.5 |
| 20 | 106 | 53 | 30 | 912.5 | 467.5 |
| 25 | 133 | 67 | 37.5 | 1052.5 | 537.5 |
| 30 | 160 | 80 | 45 | 1192.5 | 607.5 |
| 35 | 188 | 94 | 52.5 | 1152.5 | 587.5 |
| 40 | 216 | 108 | 60 | 1112.5 | 567.5 |
| 45 | 243 | 122 | 67.5 | 1252.5 | 637.5 |
| 50 | 270 | 135 | 75 | 1392.5 | 707.5 |

Table 3: Symmetric and Asymmetric CBW extension for SCS of 15kHz

| Original Channel |  | Extension + Original Channel |  | Asymmetric Extension |  | Symmetric Extension |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CBW | # RBs (As defined in TS 38.101-1 Table 5.3.2-1) | MHz | # RBs (As defined in TS 38.101-1 Table 5.3.2-1) | One-sided Extension in MHz | # of extended RBs | One-sided Extension in MHz | # of RBs to each side |
| 5 | 25 | 10 | 52 | 5 | 27 | 2.5 | 13.5 → 14 |
| 10 | 52 | 15 | 79 | 5 | 27 | 2.5 | 13.5 → 14 |
|  |  | 20 | 106 | 10 | 54 | 5 | 27 |
| 20 | 106 | 25 | 133 | 5 | 27 | 2.5 | 13.5 → 14 |
|  |  | 30 | 160 | 10 | 54 | 5 | 27 |
|  |  | 35 | 188 | 15 | 82 | 7.5 | 41 |
|  |  | 40 | 216 | 20 | 110 | 10 | 55 |
| 35 | 188 | 40 | 216 | 5 | 28 | 2.5 | 14 |
|  |  | 45 | 242 | 10 | 54 | 5 | 27 |
|  |  | 50 | 270 | 15 | 82 | 7.5 | 41 |
| 40 | 216 | 45 | 242 | 5 | 26 | 2.5 | 13 |
|  |  | 50 | 270 | 10 | 54 | 5 | 27 |
| 45 | 242 | 50 | 270 | 5 | 28 | 2.5 | 14 |

Table 4: Symmetric and Asymmetric CBW extension for SCS of 15kHz

| Original Channel |  | Extension + Original Channel |  | Asymmetric Extension |  | Symmetric Extension |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CBW | # RBs (As defined in TS 38.101-1 Table 5.3.2-1) | MHz | # RBs (As defined in TS 38.101-1 Table 5.3.2-1) | One-sided Extension in MHz | # of extended RBs | One-sided Extension in MHz | # of RBs to each side |
| 5 | 11 | 10 | 24 | 5 | 13 | 2.5 | 6.5 → 7 |
| 10 | 24 | 15 | 38 | 5 | 14 | 2.5 | 7 |
|  |  | 20 | 51 | 10 | 27 | 5 | 13.5 → 14 |
| 15 | 38 | 20 | 51 | 5 | 13 | 2.5 | 6.5 → 7 |
|  |  | 25 | 65 | 10 | 27 | 5 | 13.5 → 14 |
|  |  | 30 | 78 | 15 | 40 | 7.5 | 20 |
| 20 | 51 | 25 | 65 | 5 | 14 | 2.5 | 7 |
|  |  | 30 | 78 | 10 | 27 | 5 | 13.5 → 14 |
|  |  | 35 | 92 | 15 | 41 | 7.5 | 20.5 → 21 |
|  |  | 40 | 106 | 20 | 55 | 10 | 27.5 → 28 |
| 25 | 65 | 30 | 78 | 5 | 13 | 2.5 | 6.5 → 7 |
|  |  | 35 | 92 | 10 | 27 | 5 | 13.5 → 14 |
|  |  | 40 | 106 | 15 | 41 | 7.5 | 20.5 → 21 |
|  |  | 45 | 119 | 20 | 54 | 10 | 27 |
|  |  | 50 | 133 | 25 | 68 | 12.5 | 34 |
| 30 | 78 | 35 | 92 | 5 | 14 | 2.5 | 7 |
|  |  | 40 | 106 | 10 | 28 | 5 | 14 |
|  |  | 45 | 119 | 15 | 41 | 7.5 | 20.5 → 21 |
|  |  | 50 | 133 | 20 | 55 | 10 | 27.5 → 28 |
|  |  | 60 | 162 | 25 | 84 | 12.5 | 42 |
| 35 | 92 | 40 | 106 | 5 | 14 | 2.5 | 7 |
|  |  | 45 | 119 | 10 | 27 | 5 | 13.5 → 14 |
|  |  | 50 | 133 | 15 | 41 | 7.5 | 20.5 → 21 |
|  |  | 60 | 162 | 20 | 70 | 10 | 35 |
| 40 | 106 | 45 | 119 | 5 | 13 | 2.5 | 6.5 → 7 |
|  |  | 50 | 133 | 10 | 27 | 5 | 13.5 → 14 |
|  |  | 60 | 162 | 20 | 56 | 10 | 28 |
|  |  | 70 | 189 | 30 | 83 | 15 | 41.5 → 42 |
|  |  | 80 | 217 | 40 | 111 | 20 | 55.5 → 56 |
| 45 | 119 | 50 | 133 | 5 | 14 | 2.5 | 7 |
|  |  | 60 | 162 | 15 | 43 | 7.5 | 21.5 → 22 |
|  |  | 70 | 189 | 25 | 70 | 12.5 | 35 |
|  |  | 80 | 217 | 35 | 98 | 17.5 | 49 |
| 50 | 133 | 60 | 162 | 10 | 29 | 5 | 14.5 → 15 |
|  |  | 70 | 189 | 20 | 56 | 10 | 28 |
|  |  | 80 | 217 | 30 | 84 | 15 | 42 |
|  |  | 90 | 245 | 40 | 112 | 20 | 56 |
|  |  | 100 | 273 | 50 | 140 | 25 | 70 |
| 60 | 162 | 70 | 189 | 10 | 27 | 5 | 13.5 → 14 |
|  |  | 80 | 217 | 20 | 55 | 10 | 27.5 → 28 |
|  |  | 90 | 245 | 30 | 83 | 15 | 41.5 → 42 |
|  |  | 100 | 273 | 40 | 111 | 20 | 55.5 → 56 |
| 70 | 189 | 80 | 217 | 10 | 28 | 5 | 14 |
|  |  | 90 | 245 | 20 | 56 | 10 | 28 |
|  |  | 100 | 273 | 30 | 84 | 15 | 42 |
| 80 | 217 | 90 | 245 | 10 | 28 | 5 | 14 |
|  |  | 100 | 273 | 20 | 56 | 10 | 28 |
| 90 | 245 | 100 | 273 | 10 | 28 | 5 | 14 |

##### 2.2 On IBE

The discussion on IBE has not settled and the issue on how to properly implement this requirement is still open.

#### Issue 1-2-2: Whether the extended IBE is applied in the GB of original UE CBW - WF: - Case 1 and case 3 could be further down selected - GB of UE CBW may or may not be scheduled to other UEs - Extended IBE can be applied in the GB of UE CBW - Relaxation can be considered for the GB, based on calculation from the existing IBE formula from the last RB of original UE CBW #### Issue 1-2-4: Relaxation of the extended IBE - WF - For case 1 in issue 1-2-2, use BPSK EVM in the IBE formula for all extended RBs - For case 3 in issue 1-2-2, FFS the relaxation

In general, concerns of a UE producing strong unwanted emissions inside the extended channel should not be large. Aside from the channel extension feature the UE is already tested for compliance to ACLR, SEM and Spur. The main difference is that the output power can be up to 1.5dB higher when the channel extension is applied.

- Consider the network configuring a channel being equal to original CBW plus the extension. If the network would only schedule the RB allocations which are part of inner RB allocations then it would have to conform to the IBE of that channel. Concerns that a UE would create significant emissions just because smaller channel is configured together with the channel extension feature do not seem to be justified.

- In case of channel extension feature being enabled the BB filter of the UE would be applied with respect to the original channel which adds additional confidence that emissions should behave gracefully.

The question is whether RAN4 even needs to define dedicated emission requirements inside the extended channel. The same hardware is already tested for regular conformance. This is just a different context where the UE is tested again for a very similar emission performance. There is no need to apply unnecessary burden on a UE which has already been tested for conformance with regular channels and emission requirements (SEM, ACLR, Spur etc.).

Observation 3: Aside from the channel extension feature the UE is already tested for compliance to ACLR, SEM and Spur. The main difference is that the output power is up to 1.5dB higher when the channel extension is applied. Worries about new spurs or other issues popping up do not seem justified. There is no need to apply unnecessary burden on a UE which has already been tested for conformance with regular channels and emission requirements (SEM, ACLR, Spur etc.).

Proposal 1: Select Case 1 from Issue 1-2-2 together with the BPSK EVM in the IBE formula for all extended RBs.

##### 2.3 On signalling

The table outlines the initial approach on the signalling for CBW extension-based MPR reduction in. It includes two entries.

The first one indicates support for a fixed CBW extension of 0.5 CBW/NRB, which can be either symmetric or asymmetric. It is applicable only to FR1 and is optional with capability signalling. The feature is per band, and there is no need for FDD/TDD differentiation.

The second one indicates support for a flexible CBW extension based on a limited number of fixed values, which can also be symmetric or asymmetric. The specific values for the CBW extension size are to be determined. Like the previous entry, it is applicable only to FR1, optional with capability signalling, and the feature is per band. There is no need for FDD/TDD differentiation.

Table 5: Signalling for CBW extension-based MPR reduction

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_ENDC_RF_Ph4 | 46-1 | Support of CBW extension based MPR reduction | Indicates support of CBW extension - Extension is fixed to 0.5 CBW/NRB - Extension can be symmetric or asymmetric |  | Yes |  | The feature cannot be supported. | Per Band | No | FR1 only |  |  | Optional with capability signaling |
| 46. NR_ENDC_RF_Ph4 | 46-2 | Support of CBW extension based MPR reduction | Indicates support of CBW extension - Size of extension is flexible and based on limited number of fixed values - Extension can be symmetric or asymmetric |  | Yes |  | The feature cannot be supported. | Per Band | No | FR1 only |  | TBD values of CBW extension size | Optional with capability signaling |

Proposal 2: Consider Table 5 when discussing the signalling for CBW extension-based MPR reduction.

# 4	Conclusions

This contribution provides certain considerations to the discussion on the MPR reduction which his part of the power domain enhancement WID. The following observations are made:

Observation 1: The advantage of extending NRB is that all channels can be extended by simply adding ceil(0.5*NRB) to the lower and upper channel edge. On the flip side it is more difficult to generalise extension other than ceil(0.5*NRB) and asymmetric extensions would require specifying dedicated channel sizes in MHz to get clear boundaries for OOB requirements.

Observation 2: The advantage of extending to existing CBW and use the maximum transmission bandwidth of extended CBW is that it results into a clear extension path with good granularity and straight forward applicability for OOB requirements. On the flip side special handling would be required when considering extended CBW beyond 100MHz since there are no channels defined, and the maximum transmission bandwidth is not defined either.

Observation 3: Aside from the channel extension feature the UE is already tested for compliance to ACLR, SEM and Spur. The main difference is that the output power is up to 1.5dB higher when the channel extension is applied. Worries about new spurs or other issues popping up do not seem justified. There is no need to apply unnecessary burden on a UE which has already been tested for conformance with regular channels and emission requirements (SEM, ACLR, Spur etc.).

Proposal 1: Select Case 1 from Issue 1-2-2 together with the BPSK EVM in the IBE formula for all extended RBs.

Proposal 2: Consider Table 5 when discussing the signalling for CBW extension-based MPR reduction.

# 4	References

- RP-240828, “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, 3GPP TSG RAN Meeting #103

- R4-2505099, “WF on Power domain enhancement”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #114

- R4-2418170, “Discussion on UE RF power enhancement for NR single carrier”, vivo, 3GPP TSG-RAN4 Meeting #113