3GPP RAN WG4 Meeting #115																	R4-2505433

Malta, Malta, 19th – 23rd May 2025

Agenda item:	7.28.3.2

Source:	Apple

Title:	On LP-WUR Rx requirements

WI/SI:	NR_LPWUS-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

LP-WUS/WUR shall be introduced to NR in Rel-19. The objectives are covered in the WID [1]. During RAN4#114 a fundamental agreement was made with respect to different sets of requirements. It was decided to consider two sets which can feature different performance in noise figure, interference and differ in modulation aspects. Only if those sets turn out to be similar it shall be considered to merge those.

Issue 1-1-1: How to define two LP-WUR types Agreement: - Define two set of RF requirements as the baseline, considering the following aspects including - Noise figures - Interference performance - Modulation - Consider to merge two sets of requirements into one if the requirements are close to each other

# 2  Discussion

Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB Agreements - Same interference signal level as MR for LP-WUR ACS requirements - FFS at least for one set of Rx requirements. - The LP-WUS level is LP-WUS REFSENS+[X]dB, X=14 as starting point - FFS whether two sets of LP-WUS wanted signal level can be defined. - Final ACS will be decided by LP-WUR REFSENS and wanted signal relaxation - FFS number of guard RBs. Issue 3-1-1: IBB, OBB requirements Way forward: - Same interference level as MR - FFS at least for one set of Rx requirements. - FFS on wanted signal in blocking test.

Last meeting a fundamental agreement was reached regarding various sets of requirements. Two sets were chosen that may exhibit different noise figures, interference characteristics, and modulation aspects. Only if these sets are found to be similar will they be considered for merging. A second set of requirements allows to define relaxed requirements. Relaxation for ACS, IBB and OBB is highly important for devices which implement a dedicated LP focusing on low power and low cost. It has been discussed (e.g. in [3]) that it is imperative to consider relaxed requirements for at least ACS, IBB and OBB. Relaxations allow to reduce power consumption which is one of the main goals when considering low power receiver design. However, LP might be mostly used close the BS where interference might be higher if another UE were located at cell edge and configured at neighboring channels. RAN4 should further discuss whether ACS, IBB and OBB relaxation for the second set can be useful in the field.

Observation: Relaxation for applications in the areas of ACS, IBB and OBB is of importance for devices that are designed with a dedicated LP focus on minimizing power consumption and cost. These relaxations enable a reduction in power consumption, which is a primary objective when designing low-power receivers. However, it’s worth noting that LP may be primarily utilized close to the BS, where interference levels tend to be higher if another UE were located at the cell edge and configured on neighboring channels.

Proposal 1: RAN4 should further discuss whether ACS, IBB and OBB relaxation for the second set can be useful in the field.

Proposal 2: Our analysis on physical hardware suggests that an ACS of 22dB is a reasonable requirement for implementation.

Regarding signalling we propose to consider the details provided in Table 1. The crucial aspect is that the UE can select its support of LP-WUR on a per band basis. Designing a dedicated wake-up receiver for all FR1 bands will take more area and power consumption rises due to LO need to operate at high bands. Selectively creating a wake-up receiver for selected bands (e.g. low-mid bands) allows to reduce cost and power consumption.

Table 1: Initial take of signalling for LP-WUR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55.NR_LPWUS | 55-1 | Wake-up receiver | - Indicating support to receive wake-up signals on a certain band - Indicating capability: Type 1 or Type 2 |  | Yes |  | The feature cannot be supported. | Per Band | No | Yes |  |  | Optional with capability signaling |

Proposal 3: When discussing signalling characteristics consider proposal from Table 1.

# 4	Conclusions

This contribution provides a discussion based on last meetings way forward on LP-WUR. The following observations and proposals are made:

IBB and OBB relaxation for the second set can be useful in the field.

Observation: Relaxation for applications in the areas of ACS, IBB and OBB is of importance for devices that are designed with a dedicated LP focus on minimizing power consumption and cost. These relaxations enable a reduction in power consumption, which is a primary objective when designing low-power receivers. However, it’s worth noting that LP may be primarily utilized close to the BS, where interference levels tend to be higher if another UE were located at the cell edge and configured on neighboring channels.

Proposal 1: RAN4 should further discuss whether ACS, IBB and OBB relaxation for the second set can be useful in the field.

Proposal 2: Our analysis on physical hardware suggests that an ACS of 22dB is a reasonable requirement for implementation.

Proposal 3: When discussing signalling characteristics consider proposal from Table 1.

# 4	References

- RP-240801, “Low-power Wake-up Signal and Receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony, 3GPP TSG RAN Meeting #103

- R4-2505234, “WF for [114bis][131] NR_LPWUS_UERF”, vivo, 3GPP TSG-RAN WG4 Meeting #114bis

- R4-2500159, “Further consideration on UE RF other Rx requirements for Rel-19 LP-WUS”, Huawei, HiSilicon, TSG-RAN WG4 Meeting #114

- R4-2503003, “LS on LP-WUS UE RF”, vivo, RAN4, 3GPP TSG-RAN WG4 Meeting #114