3GPP RAN WG4 Meeting #115		R4-2505447

Malta, May 19th – 23rd, 2025

Agenda item:	5.27.1

Source:	Apple, T-Mobile USA, Skyworks Solutions Inc., Telstra, TELUS, Vodafone, Murata Manufacturing Co Ltd., BT Plc

Title:	On UL Tx switching for FR1 intra-band NC UL CA for TEI19

WI/SI:	TEI19

Release:	Rel-19

Document for:	Approval

# 1	Introduction

UL Tx switching was first introduced in 3GPP Rel-16 as a new feature to enhance inter-band EN-DC, inter-band UL CA, and SUL operations, where the switching is either between (1Tx + 1Tx) and (0Tx + 2Tx) or (1Tx + 0Tx) and (0Tx + 2Tx) for two UL carriers in two corresponding bands [1]. The feature provides the benefits of coverage improvement, UL throughput enhancement, and latency reduction. Considering that the same feature can very well be applied to FR1 intra-band non-contiguous (NC) UL CA where UE is typically limited with two Tx paths for simultaneous transmission, in this contribution, we provide the motivation on introducing the UL Tx switching feature for FR1 intra-band NC UL CA. On the other hand, as we anticipate that the RAN4 workload and RAN1/RAN2 impact on introducing this feature would be no more than the 2-band 3Tx UL switching which has been approved to be introduced via TEI19 [2], we also propose to introduce this feature via TEI19 to save the process overhead and delay as opposed to going through a potential Rel-20 work item.

# Discussion

## 2.1	Motivation for UL Tx switching for FR1 intra-band NC UL CA

Similar to UL Tx switching for inter-band UL CA, the UL Tx switching for intra-band NC UL CA allows UE to dynamically switch between the two non-contiguous UL carriers without the RRC reconfiguration process. The switching can be based on the channel conditions of the two carriers or the network UL traffic loading among multiple users.

On the other hand, the UL Tx switching avoids the potential high MPR/A-MPR for simultaneous non-contiguous UL CA transmission due to the clustered out-of-band emissions caused by 2UL inter-modulations (IMD) as conceptually illustrated in Figure 2.1-1. As a result, the UL coverage can be improved yet without compromising UL throughput performance as compared to the non-switched UL CA which will be further elaborated below.

Figure 2.1-1 Potential high MPR for intra-band NC UL CA due to IMD emissions

Observation 1: UL Tx switching for intra-band NC UL CA allows UE to dynamically switch between the two non-contiguous UL carriers without the RRC reconfiguration process.

Observation 2: The UL Tx switching avoids the potential high MPR/A-MPR for simultaneous non-contiguous UL CA transmission due to the clustered out-of-band emissions caused by 2UL inter-modulations.

Table 2.1-1 summarizes the PC3 and PC2 MPR requirements for intra-band NC UL CA based on dualPA implementation to fulfill the general spurious emission requirement at -30 dBm/MHz. It can be seen that the required MPR is at least 9 dB which implies UL coverage range would be severely impacted.

| PC3 |  | PC2 |  | PC1.5 |  |
| --- | --- | --- | --- | --- | --- |
| B (MHz) | MPR (dB) | B (MHz) | MPR (dB) | B (MHz) | MPR (dB) |
| 0 ≤ B < 1.08 | 15.0 | 0 ≤ B < 1.44 | 15.5 | 0 ≤ B < 1.44 | [18.5] |
| 1.08 ≤ B < 2.16 | 14.5 | 1.44 ≤ B < 2.88 | 15.0 | 1.44 ≤ B < 2.88 | [18.0] |
| 2.16 ≤ B < 3.24 | 13.5 | 2.88 ≤ B < 5.76 | 14.0 | 2.88 ≤ B < 5.76 | [17.0] |
| 3.24 ≤ B < 5.04 | 12.5 | 5.76 ≤ B < 10.8 | 12.0 | 5.76 ≤ B < 10.8 | [15.0] |
| 5.04≤ B < 10.08 | 11.5 | 10.8 ≤ B < 23.04 | 10.5 | 10.8 ≤ B < 23.04 | [13.5] |
| 10.08 ≤ B < 16.38 | 10.5 | 23.04 ≤ B | 9.0 | 23.04 ≤ B | [12.0] |
| 16.38 ≤ B < 21.78 | 10.0 |  |  |  |  |
| 21.78 ≤ B | 9.0 |  |  |  |  |
| NOTE: B is the effective aggregated bandwidth in MHz |  |  |  |  |  |

Table 2.1-1 MPR requirements for intra-band NC UL CA with dualPA

For simultaneous intra-band non-contiguous UL CA transmission, due to the UE limitation on number of Tx path at two, only non-MIMO operation can be supported for each carrier, as is shown in Figure 2.1-2. However, with UL Tx switching, each UL carrier can leverage the two Tx paths to support UL MIMO or Tx diversity as conceptually illustrated in Figure 2.1-3, as a result, the UL throughput performance for UL Tx switching is not compromised as compared to non-switched UL CA, or the UL coverage can be further improved.

Figure 2.1-2 Only non-MIMO operation can be supported for intra-band NC UL CA

Figure 2.1-3 UL MIMO can be supported with UL Tx switching

Observation 3: UL Tx switching does not compromise UL throughput performance as compared to UL CA as each switched carrier can support UL MIMO while UL CA can only support non-MIMO operation for each carrier due to the limitation of UL Tx path for simultaneous transmission.

Observation 4: UL Tx switching can further improve UL coverage as compared to UL CA as each switched carrier can support Tx diversity while for UL CA, each carrier can only be supported by 1Tx.

Another potential benefit for intra-band NC UL CA with UL Tx switching is that only one frequency synthesizer (1 LO) is required to support the operation while for simultaneous transmission, it would require two frequency synthesizers (2 LOs) to support the operation when the frequency span between the two carriers is wider than 200 MHz. As a result, the power consumption from the second UL frequency synthesizer can be saved for the UL Tx switching operation.

Observation 5: For intra-band NC UL CA with UL Tx switching, only one frequency synthesizer (1 LO) is required to support the operation, while for simultaneous transmission it would require two frequency synthesizers (2 LOs) to support the operation when the frequency span between the two carriers is wider than 200 MHz.

Observation 6: For intra-band NC UL CA with UL Tx switching operation, the second UL frequency synthesizer power consumption can be saved as compared to non-switched UL CA.

## 2.2	Introducing intra-band NC UL CA Tx switching via TEI19

As we anticipate that the RAN4 workload and RAN1/RAN2 impact on introducing this feature would be no more than the 2-band 3Tx UL switching which has been approved to be introduced via TEI19 [2], we also propose to introduce this feature via TEI19 to save the process overhead and delay as opposed to going through a potential Rel-20 work item.

Proposal: Introduce UL Tx switching feature for intra-band NC UL CA via TEI19 with the following work plan.

- CR for TS 38.101-1 to introduce time mask for switching between two uplink carriers for intra-band NC UL CA.

- Send an LS to RAN1 and RAN2 to trigger the development of UL Tx switching feature for intra-band NC UL CA based on RAN4’s agreement that the switching period uplinkTxSwitchingPeriod2T2T currently specified for inter-band UL CA can be reused for intra-band NC UL CA.

# 3	Conclusion

In this contribution, we provide the motivation on introducing the UL Tx switching feature for FR1 intra-band NC UL CA. We also propose to introduce this feature via TEI19 to save the process overhead and delay as opposed to going through a potential Rel-20 work item.

Observation 1: UL Tx switching for intra-band NC UL CA allows UE to dynamically switch between the two non-contiguous UL carriers without the RRC reconfiguration process.

Observation 2: The UL Tx switching avoids the potential high MPR/A-MPR for simultaneous non-contiguous UL CA transmission due to the clustered out-of-band emissions caused by 2UL inter-modulations.

Observation 3: UL Tx switching does not compromise UL throughput performance as compared to UL CA as each switched carrier can support UL MIMO while UL CA can only support non-MIMO operation for each carrier due to the limitation of UL Tx path for simultaneous transmission.

Observation 4: UL Tx switching can further improve UL coverage as compared to UL CA as each switched carrier can support Tx diversity while for UL CA, each carrier can only be supported by 1Tx.

Observation 5: For intra-band NC UL CA with UL Tx switching, only one frequency synthesizer (1 LO) is required to support the operation, while for simultaneous transmission it would require two frequency synthesizers (2 LOs) to support the operation when the frequency span between the two carriers is wider than 200 MHz.

Observation 6: For intra-band NC UL CA with UL Tx switching operation, the second UL frequency synthesizer power consumption can be saved as compared to non-switched UL CA.

Proposal: Introduce UL Tx switching feature for intra-band NC UL CA via TEI19 with the following work plan.

- CR for 38.101-1 to introduce time mask for switching between two uplink carriers for intra-band NC UL CA.

- Send an LS to RAN1 and RAN2 to trigger the development of UL Tx switching feature for intra-band NC UL CA based on RAN4’s agreement that the switching period uplinkTxSwitchingPeriod2T2T currently specified for inter-band UL CA can be reused for intra-band NC UL CA.

# 4	Reference

RP-192282, "WID revision: RF requirements for NR frequency range 1 (FR1)", Huawei, HiSilicon, China Telecom, 3GPP TSG RAN Meeting #84, Newport Beach, USA, September 16th – 20th, 2019

R4-2505220 “WF on UL Tx switching for TEI19”, MediaTek, 3GPP TSG RAN WG4 Meeting #114bis, Wuhan, China, April 7th – 11th, 2025

# Appendix I: Example RAN4 CR draft

#### 6.3A.3.2	Transmit ON/OFF time mask for intra-band non-contiguous CA

##### 6.3A.3.2.1	General

For intra-band non-contiguous carrier aggregation, the general output power ON/OFF time mask specified in clause 6.3.3.1 is applicable for each component carrier during the ON power period and the transient periods. The OFF period as specified in clause 6.3.3.1 shall only be applicable for each component carrier when all the component carriers are OFF.

##### 6.3A.3.2.2	Time mask for switching between two uplink carriers

In addition to the requirements in 6.3A.3.2.1 and the maximum output power requirement specified in Table 6.2A.1.2-1 with uplink assigned to two carriers, the switching time mask specified in this clause is applicable for an intra-band non-contiguous UL CA configuration when the capability [uplinkTxSwitchingPeriod2T2T] is present, and is only applicable for uplink switching mechanisms specified in clause 6.1.6 of TS 38.214 [10], where both NR UL carriers are capable of two transmit antenna connectors. The UE shall support switching between the two uplink carriers both with two-layer transmission following the scheduling commands and rank adaptation.

The switching periods described in Figure 6.3A.3.2.2-1a and Figure 6.3A.3.2.2-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability [uplinkTxSwitchingPeriod2T2T].

When switching from one carrier to another, if there is no uplink transmission scheduled or configured on the switch-from carrier for at least the duration of the switching period (X µs) before the point in time the UE is scheduled or configured to start the transmission on the switch-to carrier, the switching period is fully contained in the time period between the end of the transmission on the switch-from carrier and the start of the transmission on the switch-to carrier. In addition, the RRC signalling uplinkTxSwitchingPeriodLocation is ignored by the UE and does not take effect in this case.

Figure 6.3A.3.2.2-1a: Time mask for switching between UL carrier 1 and UL carrier 2, where the switching period is located in carrier 1

Figure 6.3A.3.2.2-1b: Time mask for switching between UL carrier 1 and UL carrier 2, where the switching period is located in carrier 2

# Appendix II: Example RAN1 CR draft

#### 6.1.6.2	Uplink switching for carrier aggregation

##### 6.1.6.2.0	Uplink switching with two uplink bands

For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch or uplinkTxSwitchingPeriod2T2T for a band combination, and if it is for that band combination configured with uplink carrier aggregation:

-	If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s):

-	When the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 1-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers.

-	When the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 2-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers.

-	For the UE configured with uplinkTxSwitchingOption set to 'switchedUL', when the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers.

-	For the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on a carrier on the same band and the UE is under the operation state in which 2-port transmission cannot be supported in the same band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers.

-	For the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on another uplink carrier on another band and the UE is under the operation state in which 2-port transmission can be supported in the same band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers.

-	For the UE configured with uplinkTxSwitchingOption set to 'dualUL', if the UE is configured with uplinkTxSwitching-DualUL-TxState set to 'oneT', when the UE is under the operation state in which 2-port transmission can be supported on one carrier on one band followed by no transmission on any carrier on the same band and 1-port transmission on the other carrier on another band the UE shall consider this as if 1-port transmission was transmitted on both uplinks, otherwise the UE shall consider this as if 2-port transmission took place on the transmitting carrier.

-	If uplinkTxSwitching-2T-Mode is configured, when the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 2-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers.

-	The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier on one band, and any transmission on another uplink carrier on another band.

-	In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

##### 6.1.6.2.X	Uplink switching with 1 uplink band involving 2 non-contiguous carriers

For a UE indicating a capability for uplink switching with Intra-Band-UplinkTxSwitch-R19 or Intra-Band-uplinkTxSwitchingPeriod2T2T-R19 for a band, and if it is for that band with intra-band non-contiguous carriers configured with uplink carrier aggregation:

-	If the UE is configured with uplink switching with parameter Intra-Band-uplinkTxSwitching-R19, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s):

-	When the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 1-port transmission on another uplink carrier on the same band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers.

-	When the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 2-port transmission on another uplink carrier on the same band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers.

-	For the UE configured with Intra-Band-uplinkTxSwitchingOption-R19 set to 'switchedUL', when the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on another uplink carrier on the same band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers.

-	For the UE configured with Intra-Band-uplinkTxSwitchingOption-R19 set to 'dualUL', when the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on the same carrier on the same band and the UE is under the operation state in which 2-port transmission cannot be supported on that same carrier on the same band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers.

-	For the UE configured with Intra-Band-uplinkTxSwitchingOption-R19 set to 'dualUL', when the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on another uplink carrier on the same band and the UE is under the operation state in which 2-port transmission can be supported on the same carrier on the same band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers.

-	For the UE configured with Intra-Band-uplinkTxSwitchingOption-R19 set to 'dualUL', if the UE is configured with Intra-Band-uplinkTxSwitching-DualUL-TxState-R19 set to 'oneT', when the UE is under the operation state in which 2-port transmission can be supported on one carrier on one band followed by no transmission on the same carrier on the same band and 1-port transmission on the other carrier on the same band the UE shall consider this as if 1-port transmission was transmitted on both uplinks, otherwise the UE shall consider this as if 2-port transmission took place on the transmitting carrier.

-	If Intra-Band-uplinkTxSwitching-2T-Mode-R19 is configured, when the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 2-port transmission on another uplink carrier on the same band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers.

-	The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier on one band, and any transmission on another uplink carrier on the same band.

-	In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

##### 6.1.6.2.1	void

- 6.1.6.2.2	Uplink switching with 3 or 4 uplink bands