3GPP RAN WG4 Meeting #115		R4-2505450

Malta, May 19th – 23rd, 2025

Agenda item:	6.3.3

Source:	Apple

Title:	Views on 2UL CA_n28A-n71A

WI/SI:	NR_CADC_SUL_R19-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

CA_n28A-n71A with both single UL and 2UL CA_n28A-n71A support was proposed in last RAN4 meeting [1]. Due to the similarity to CA_n28A-n105A which had been shown to be subject to substantial REFSENS impact from 2UL IMD [2], the contribution was flagged by several companies on the practicality of supporting UL CA. Though the TP was revised to only include single UL support, a WF on UL CA_n28A-n71A was also approved to summarize the open issues and trigger further studies on potential solutions for UL CA_n28A-n71A [3]. In this contribution, we share our views on the potential architecture variants based on 2-antenna and 3-antenna implementations and their implications on UE RF requirements for UL CA_n28A-n71A. We also propose to limit CA_n28A-n71A to single UL operation only as was suggested for the similar combination CA_n28A-n105A due to the concern of both Rx and Tx performance impacts to support UL CA.

# Discussion

Figure 2-1 shows the aggregated spectrum allocation for CA_n28A-n71A where no frequency range restriction has been indicated for either of the constituent bands.

Figure 2-1 CA_n28A-n71A spectrum range

Due to that the two UL bands are surrounded by the two DL bands with close frequency proximity, the 2UL IMD3/IM5/IMD7 may fall into either n71 DL or n28 DL or both DL bands simultaneously to cause REFSENS degradation (MSD), as conceptually illustrated in Figure 2-2. The amount of MSD would depend on the UE RF front-end architecture and the isolation between the two UL carriers which also depends on carrier location.

Figure 2-2 Conceptual illustration on 2UL IMD3 impact to DL bands for CA_n28A-n71A

On the other hand, when the two UL carriers are relatively close to each other, the IMD3 can fall within both UL bands, as shown in Figure 2-3. Since there is no filtering within the UL bands, the IMD3 power spectral density (PSD) may fail the general emission requirement at -30 dBm/MHz. As a result, the combination could be subject to MPR comparable to that of intra-band non-contiguous UL CA if there is no filter isolation between the two UL carriers.

Figure 2-3 IMD3 fall within UL bands may fail general emission requirement

To realize how much Rx and Tx performance impact due to 2UL operation, we have performed background analysis on the worst-case IMD3 MSD and MPR for UE architectures based on both 2-antenna and 3-antenna implementations.

For CA_n28A-n71A with 2-antenna implementation, there can be two potential architecture variants, one with a triplexer in the main signal path and a duplexer in the diversity Rx path, as shown in Figure 2-4 (a), the other with a triplexer in both main and diversity paths, as shown in Figure 2-4 (b). Though in this combination there is a 5MHz frequency gap between the two bands, the quadplexer implementation in the main path has not been considered due to the design challenge to provide isolation between the two UL bands in such a small frequency gap.

Figure 2-4 Potential UE architecture variants to support CA_n28A-n71A with 2 antenna

For 2-antenna architecture variant (a), the IMD would be dominated by PA forward mixing. Since there is no isolation between the two UL carriers due to the sharing of common PA and bandpass filter, both Rx and Tx performance impacts are expected to be relatively severe. Our analysis shows that the IMD3 MSD could be as high as 50 dB on either of the DL carriers and MPR is comparable to that of intra-band non-contiguous UL CA without supporting dualPA-Architecture which can be as high as 17.5 dB for PC3 with small RB allocations.

Observation 1: For CA_n28A-n71A with 2-antenna architecture variant (a), the IMD would be dominated by PA forward mixing.

Observation 2: For CA_n28A-n71A with 2-antenna architecture variant (a), the IMD3 MSD could be as high as 50dB on either of the DL carriers.

Observation 3: For CA_n28A-n71A with 2-antenna architecture variant (a), the MPR is comparable to that of intra-band non-contiguous UL CA without supporting dualPA-Architecture.

For 2-antenna architecture variant (b), the IMD would be dominated by PA reverse mixing. As benefiting from the antenna isolation between the two UL carriers, both Rx and Tx performance impacts are expected to be less severe than architecture variant (a). Our analysis shows that the worst-case IMD3 MSD could be as high as 38 dB on either of the DL carriers and MPR is comparable to that of intra-band non-contiguous UL CA with supporting dualPA-Architecture which can be as high as 15 dB for PC3 with small RB allocations.

Observation 4: For CA_n28A-n71A with 2-antenna architecture variant (b), the IMD would be dominated by PA reverse mixing.

Observation 5: For CA_n28A-n71A with 2-antenna architecture variant (b), the IMD3 MSD could be as high as 38 dB on either of the DL carriers.

Observation 6: For CA_n28A-n71A with 2-antenna architecture variant (b), the MPR is comparable to that of intra-band non-contiguous UL CA with supporting dualPA-Architecture.

For the 3-antenna implementation as shown in Figure 2-5, despite there is no conductive IMD coupling in the diversity path, our analysis shows that both Rx and Tx performance impacts are still comparable to 2-antenna variant (b).

Figure 2-5 CA_n28A-n71A UE architecture based on the 3-antenna implementation

Observation 7: For CA_n28A-n71A with 3-antenna implementation, the IMD would be dominated by PA reverse mixing.

Observation 8: For CA_n28A-n71A with 3-antenna implementation, the IMD3 MSD could be as high as 38 dB on either of the DL carriers.

Observation 9: For CA_n28A-n71A with 3-antenna implementation, the MPR is comparable to that of intra-band non-contiguous UL CA with supporting dualPA-Architecture.

Considering the relatively severe Rx and Tx performance impacts and the unprecedented MPR requirements for inter-band UL CA which would render the 2UL operation rather impracticable for this combination, we propose to limit CA_n28A-n71A to single UL operation only as was also suggested for the similar combination CA_n28A-n105A.

Proposal: Only single UL is supported for CA_n28A-n71A to avoid the severe Rx and Tx performance impacts.

# 3	Conclusion

In this contribution, we share our views on the potential architecture variants for CA_n28A-n71A based on 2-antenna and 3-antenna implementations and their implications on UE RF requirements supporting UL CA. We also propose to limit CA_n28A-n71A to single UL operation only as was suggested for the similar combination CA_n28A-n105A due to the concern of both Rx and Tx performance impacts to support UL CA.

Observation 1: For CA_n28A-n71A with 2-antenna architecture variant (a), the IMD would be dominated by PA forward mixing.

Observation 2: For CA_n28A-n71A with 2-antenna architecture variant (a), the IMD3 MSD could be as high as 50dB on either of the DL carriers.

Observation 3: For CA_n28A-n71A with 2-antenna architecture variant (a), the MPR is comparable to that of intra-band non-contiguous UL CA without supporting dualPA-Architecture.

Observation 4: For CA_n28A-n71A with 2-antenna architecture variant (b), the IMD would be dominated by PA reverse mixing.

Observation 5: For CA_n28A-n71A with 2-antenna architecture variant (b), the IMD3 MSD could be as high as 38 dB on either of the DL carriers.

Observation 6: For CA_n28A-n71A with 2-antenna architecture variant (b), the MPR is comparable to that of intra-band non-contiguous UL CA with supporting dualPA-Architecture.

Observation 7: For CA_n28A-n71A with 3-antenna implementation, the IMD would be dominated by PA reverse mixing.

Observation 8: For CA_n28A-n71A with 3-antenna implementation, the IMD3 MSD could be as high as 38 dB on either of the DL carriers.

Observation 9: For CA_n28A-n71A with 3-antenna implementation, the MPR is comparable to that of intra-band non-contiguous UL CA with supporting dualPA-Architecture.

Proposal: Only single UL is supported for CA_n28A-n71A to avoid the severe Rx and Tx performance impacts.

# 4	References

R4-2503687 “TP for TR 38.719-02-01 on introduction of CA_n28A-n71A”, Huawei, HiSilicon, 3GPP TSG RAN WG4 Meeting #114bis, Wuhan, China, April 7th – 11th, 2025

R4-2504354 “CA_n28-n105 UE architecture consideration”, Apple, 3GPP TSG RAN WG4 Meeting #106bis-e, Online, April 17th – 26th, 2023

R4-2505216 “WF for uplink CA n28-n71”, Huawei, HiSilicon, 3GPP TSG RAN WG4 Meeting #114bis, Wuhan, China, April 7th – 11th, 2025