3GPP RAN WG4 Meeting #115		R4-2505451

Malta, May 19th – 23rd, 2025

Agenda item:	6.11.2

Source:	Apple

Title:	Views on CA_n5A-n8A with non-simultaneous Rx-Tx

WI/SI:	LTE_NR_R19_Simult_RxTx-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

Simultaneous Rx/Tx operation in general shall be supported for FDD-FDD band combinations. However, there are combinations where one band UL is overlapping or in close frequency proximity with the other band DL, such as CA_n5A-n8A, it would not be feasible to support simultaneous Rx/Tx between n5 DL and n8 UL due to severe self-desensitization. To enable 2UL/2DL operation for CA_n5A-n8A, one possible operation scheme is to switch between DL CA_n5A-n8A with single UL in n5 and UL CA_n5A-n8A with single DL in n8 to avoid simultaneous Rx/Tx between n5 DL and n8 UL. However, to support such switching scheme between two CA configurations, there may be potential RAN1 and RAN2 impacts.

In this contribution, we share our views on CA_n5A-n8A with non-simultaneous Rx/Tx to enable 2UL/2DL operation. Considering the resemblance to the operation scenarios for LB-LB CA via switching feature [1], we propose to postpone the CA_n5A-n8A 2U/2DL configuration to Rel-20 as one of the new LB-LB CA switching scenarios in the potential Rel-20 5G-A work item for LB-LB CA via switching.

# Discussion

CA_n5A-n8A was first introduced in Rel-18. Due to that the n5 DL band range is overlapping with n8 UL band range, there is no feasible full-band filter to isolate n5 DL and n8 UL. As a result, when n8 UL and n5 DL are operating simultaneously, severe sensitivity degradation in n5 DL carrier is expected. Though during the Rel-18 discussions, there were three operation configurations as below considered to mitigate the n8 UL to n5 DL self-interference issue:

- Option 1: DL CA_n5A-n8A with single UL in n5 only

- Option 2: UL/DL CA_n5A-n8A with non-simultaneous Rx/Tx between n5 DL and n8 UL

- Option 3: UL/DL CA_n5A-n8A with additional dedicated filter for the restricted frequency ranges for specific operator

Due to the potential RAN1 and RAN2 impact for Option 2 and the UE implementation/performance challenge in Option 3 [2], only Option 1 operation was specified in Rel-18.

Nevertheless, owing to the demand for better UL throughput performance, the CA_n5A-n8A proponent company has proposed to revisit Option 2 in Rel-19 via a basket WID, with the assumption that there would be no RAN1 and RAN2 impact, or at most only certain capability signaling needs to be introduced to support the feature. However, in our view, the assumption of no RAN1 impact was an overlook in RAN4.

Observation 1: The assumption of no RAN1 impact to support 2UL/2DL CA_n5A-n8A with non-simultaneous Rx/Tx between n5 DL and n8 UL was an overlook in RAN4.

Now by comparing Rel-19 LB-LB CA via switching feature with 2UL/2DL CA_n5A-n8A with non-simultaneous Rx/Tx between n5 DL and n8 UL, we have found the strong resemblance between the two features and the alternating configurations between 2UL/1DL and 1UL/2DL for CA_n5A-n8A is essentially another LB-LB CA switching scenario which should be subject to RAN1’s provision before the feature can be enabled. In addition, there may also be potential RAN4 RRM impact to support such new CA switching scenario. With that said, we think the intended 2UL/2DL CA_n5A-n8A with non-simultaneous Rx/Tx between n5 DL and n8 UL feature should not be introduced via basket WID and propose to postpone the CA_n5A-n8A 2UL/2DL configuration to Rel-20 as one of the new LB-LB CA switching scenarios in the potential Rel-20 5G-A work item for LB-LB CA via switching.

Observation 2: There is a strong resemblance between Rel-19 LB-LB CA via switching feature and 2UL/2DL CA_n5A-n8A with non-simultaneous Rx/Tx between n5 DL and n8 UL.

Observation 3: The alternating configurations between 2UL/1DL and 1UL/2DL for CA_n5A-n8A is essentially another LB-LB CA switching scenario which should be subject to RAN1’s provision before the feature can be enabled.

Observation 4: There may also be potential RAN4 RRM impact to support the alternating configurations between 2UL/1DL and 1UL/2DL for CA_n5A-n8A.

Observation 5: The intended 2UL/2DL CA_n5A-n8A with non-simultaneous Rx/Tx between n5 DL and n8 UL feature shall not be introduced via basket WID.

Proposal: Postpone the CA_n5A-n8A 2U/2DL configuration to Rel-20 as one of the new LB-LB CA switching scenarios in the potential Rel-20 5G-A work item for LB-LB CA via switching.

# 3	Conclusion

In this contribution, we share our views on CA_n5A-n8A with non-simultaneous Rx/Tx to enable 2UL/2DL operation. Considering the resemblance to the operation scenarios for LB-LB CA via switching feature, we propose to postpone the CA_n5A-n8A 2U/2DL configuration to Rel-20 as one of the new LB-LB CA switching scenarios in the potential Rel-20 5G-A work item for LB-LB CA via switching.

Observation 1: The assumption of no RAN1 impact to support 2UL/2DL CA_n5A-n8A with non-simultaneous Rx/Tx between n5 DL and n8 UL was an overlook in RAN4.

Observation 2: There is a strong resemblance between Rel-19 LB-LB CA via switching feature and 2UL/2DL CA_n5A-n8A with non-simultaneous Rx/Tx between n5 DL and n8 UL.

Observation 3: The alternating configurations between 2UL/1DL and 1UL/2DL for CA_n5A-n8A is essentially another LB-LB CA switching scenario which should be subject to RAN1’s provision before the feature can be enabled.

Observation 4: There may also be potential RAN4 RRM impact to support the alternating configurations between 2UL/1DL and 1UL/2DL for CA_n5A-n8A.

Observation 5: The intended 2UL/2DL CA_n5A-n8A with non-simultaneous Rx/Tx between n5 DL and n8 UL feature shall not be introduced via basket WID.

Proposal: Postpone the CA_n5A-n8A 2U/2DL configuration to Rel-20 as one of the new LB-LB CA switching scenarios in the potential Rel-20 5G-A work item for LB-LB CA via switching.

# 4	References

RP-243317, “New WID on low band carrier aggregation via switching”, Apple et al, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9th -12th, 2024

RP-231970 “Rescoping CA_n5-n8 in UL CA_n5-n8 support”, Apple, 3GPP TSG-RAN Meeting #101, Bengaluru, India, September 11th – 15th, 2023