3GPP RAN WG4 Meeting #115		R4-2505452

Malta, May 19th – 23rd, 2025

Agenda item:	7.1.2.1.1

Source:	Apple

Title:	On Rel-19 HPUE 2UL IMD MSD

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The Rel-19 RAN4 non-spectrum WID/SID package has been approved in RAN #103 meeting. In consideration that the uplink coverage is crucial for 5G and 5G-adavanced roll-out and the continued strong interests from operators in higher UE maximum output power such as PC1.5 for their band combinations, one of the key objectives under the new Rel-19 “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” work item [1] is to specify the generic requirements of HPUE for NR uplink CA in FR1 and EN-DC with NR FR1 bands where the features include,

    - PC1.5 for intra-band contiguous UL CA with and without UL MIMO

    - PC1.5 Intra-band non-contiguous UL CA without UL MIMO

    - PC1.5 2-band inter-band UL CA with 2Tx or 3Tx for handheld UE and FWA

    - PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx for handheld UE and FWA UE

    - Increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band UL CA and EN-DC HPUE

For HPUE inter-band UL CA/EN-DC, RAN4 has agreed to work on the feasible unified equations, LUT, or simplified guidelines for MSD for HPUE band combinations as normative requirements to replace the existing method using HPUE MSD tables [2]. In last RAN4 meeting, the HPUE MSD LUTs for single UL were generally agreed which were captured in the approved “WF on MSD simplification for HPUE” [3], while the LUTs for HPUE 2UL IMD MSD had not been formerly discussed.

In this contribution, we provide our maximum PCx analysis on HPUE 2UL IMD MSD for PC2 with (23dBm + 23dBm), PC1.5 with (26dBm + 26dBm), and (23dBm + 29dBm) by taking into account better PA linearity for PC2 PA than PC3 PA and share our view on the number of LUTs required to cover all cases. We also look into the MSD difference between 1Tx and 2Tx implementations for PC2 FDD band UL using PC1.5 CA_n3A-n78A as an example band combination for the analysis.

# Discussion

For 2UL IMD MSD, prior to Rel-19, there has been 3 UL power configurations specified as summarized in Table 2-1.

| PC | Aggressor UL Power (dBm) | Release |
| --- | --- | --- |
| PC3 | 20 + 20 | Rel-15 |
| PC2 (2Tx or 3Tx) | 23 + 23 | Rel-18 or earlier |
| PC1.5 (3Tx) | 23 + 27.8 | Rel-18 |
| PC1.5 (2Tx or 3Tx) | 26 + 26 | Rel-19 |

Table 2-1 2UL IMD MSD UL power configurations

In Rel-19, we anticipate new PC1.5 configurations with (FDD + TDD) as (PC2 + PC2) and (PC2 + PC1.5) to be introduced with 2Tx or 3Tx. For PC2 FDD band pairing with PC2 TDD band, the FDD band UL may be implemented with 1Tx or 2Tx which could potentially result in different 2UL IMD MSD impact, similar to the UL harmonic MSD with 1Tx or 2Tx.

To quantify the potential 2UL IMD MSD difference between the 1Tx and 2Tx implementations for PC2 FDD band, we used PC1.5 CA_n3A-n78A as the example band combination for the analysis where the 1Tx PC2 PA is assumed to have 3dB better linearity than PC3 PA in the 2Tx implementation. Figure 2-1(a) and Figure 2-1(b) show the reference architectures with 1Tx and 2Tx PC2 n3 UL for our analysis where 10dB antenna isolation was assumed between main and diversity paths.

Figure 2-1(a) Reference architecture with 1Tx PC2 n3 UL for CA_n3-n78 2UL IMD MSD analysis

Figure 2-1(b) Reference architecture with 2Tx PC2 n3 UL for CA_n3-n78 2UL IMD MSD analysis

Table 2-2 summarizes the n3 IMD2 MSD results for PC1.5 UL CA_n3A-n78A based on our link analysis. It can be observed that the MSD difference is 6.5 dB between 1Tx and 2Tx PC2 UL implementations which is quite substantial and shall be differentiated with different LUTs.

| Band Combination | Power Class | Victim Band | MSD Type | MSD (dB) |  |
| --- | --- | --- | --- | --- | --- |
| CA_n3A-n78A | PC1.5 | n3 | IMD2 | 1Tx | 34.9 |
|  |  |  |  | 2Tx | 41.4 |

Table 2-2 2UL MSD difference between 1Tx and 2Tx in PC2 n3 UL

Observation 1: 2UL IMD MSD difference between 1Tx and 2Tx PC2 UL can be quite substantial which shall be differentiated with different LUTs.

However, in this particular combination, the IMD happens to be dominated by n3 PA which would result in MSD difference between 1Tx and 2Tx in PC2 n3 UL. Our further analysis shows that there would be no MSD difference between PC2 1Tx and 2Tx for the non IMD dominant UL band.

Observation 2: There would be no MSD difference between PC2 1Tx and 2Tx for the non IMD dominant UL band.

Unfortunately in the current 2UL IMD MSD specifications, there is no information on the IMD dominant UL band. Therefore, it would not be possible to determine which combination with 2Tx in one of the UL bands needs higher MSD than the (1Tx + 1Tx) combination in LUT approach. In our view, the most sensible solution is to always apply higher PCx for PC1.5 combinations with 3Tx than 2Tx.

Proposal 1: For PC1.5 combinations, apply [6]dB higher PCx for 3Tx than 2Tx.

To determine the minimum number of LUTs to cover all HPUE 2UL IMD MSD scenarios, we have performed the maximum PCx analysis without taking into account the 2Tx impact in the IMD dominant UL. The Results are summarized in Table 2-3, Table 2-4, Table 2-5, and Table 2-6 for IMD2, IMD3, IMD4, and IMD5 versus PC2, PC1.5 with 2Tx and PC1.5 with 3Tx (PC3+PC1.5) respectively.

| Power Class | PA Config | UL Config (dBm) | Max. PCx (dB) |
| --- | --- | --- | --- |
|  |  |  | IMD2 |
| PC3 | PC3+PC3 | 20 + 20 | 0 |
| PC2 | PC3+PC3 | 23 + 23 | 6.0 |
| PC1.5 | PC2+PC2 | 26 + 26 | 9.0 |
| PC1.5 | PC3+PC1.5 | 23 + 27.8 | 9.6 |
| PC1.5 | PC3+PC1.5 | 26(24) + 26 | 8.8 |

Table 2.3 Maximum PCx for IMD2

| Power Class | PA Config | UL Config (dBm) | Max. PCx (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | IMD3 (2,1) | IMD3 (1,2) |
| PC3 | PC3+PC3 | 20 + 20 | 0 | 0 |
| PC2 | PC3+PC3 | 23 + 23 | 9.0 | 9.0 |
| PC1.5 | PC2+PC2 | 26 + 26 | 12.0 | 12.0 |
| PC1.5 | PC3+PC1.5 | 23 + 27.8 | 13.8 | 12.6 |
| PC1.5 | PC3+PC1.5 | 26(24) + 26 | 14.0 | 10.0 |

Table 2.4 Maximum PCx for IMD3

| Power Class | PA Config | UL Config (dBm) | Max. PCx (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | IMD4 (3,1) | IMD4 (2,2) | IMD4 (1,3) |
| PC3 | PC3+PC3 | 20 + 20 | 0 | 0 | 0 |
| PC2 | PC3+PC3 | 23 + 23 | 12.0 | 12.0 | 12.0 |
| PC1.5 | PC2+PC2 | 26 + 26 | 15.0 | 15.0 | 15.0 |
| PC1.5 | PC3+PC1.5 | 23 + 27.8 | 16.8 | 19.1 | 17.4 |
| PC1.5 | PC3+PC1.5 | 26(24) + 26 | 18.0 | 17.5 | 13.0 |

Table 2.5 Maximum PCx for IMD4

| Power Class | PA Config | UL Config (dBm) | Max. PCx (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | IMD5 (4,1) | IMD5 (3,2) | IMD5 (2,3) | IMD5 (1,4) |
| PC3 | PC3+PC3 | 20 + 20 | 0 | 0 | 0 | 0 |
| PC2 | PC3+PC3 | 23 + 23 | 15.0 | 15.0 | 15.0 | 15.0 |
| PC1.5 | PC2+PC2 | 26 + 26 | 18.0 | 18.0 | 18.0 | 18.0 |
| PC1.5 | PC3+PC1.5 | 23 + 27.8 | 19.8 | 24.6 | 17.4 | 22.2 |
| PC1.5 | PC3+PC1.5 | 26(24) + 26 | 22.0 | 24.0 | 14.0 | 16.0 |

Table 2.6 Maximum PCx for IMD5

Based on the above analysis, we can streamline the required LUTs for the 2UL IMD MSD as below:

|  | LUT1 | LUT2 | LUT3 | LUT4 | LUT5 | LUT6 | LUT7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Max PCx (dB) | 6.0 | 9.0 | 12.0 | 15.0 | 18.0 | 21.0 | 24.0 |
| Applicability | PC2 IMD2 2Tx | PC2 IMD3 2Tx | PC1.5 IMD3 2Tx | PC1.5 IMD4 2Tx | PC1.5 IMD5 2Tx | PC1.5 IMD4 3Tx | PC1.5 IMD5 3Tx |
|  |  | PC1.5 IMD2 2Tx | PC2 IMD4 2Tx | PC2 IMD5 2Tx | PC1.5 IMD3 3Tx |  |  |
|  |  |  |  | PC1.5 IMD2 3Tx |  |  |  |

Table 2.7 Proposed LUTs for 2UL IMD MSD

Proposal 2: RAN4 to take the LUT arrangement in Table 2.7 into consideration for HPUE 2UL IMD self-interference MSD.

Proposal 3: For PC1.5 combinations, only (26dBm + 26dBm) UL configuration is applied for MSD verifications.

Notice that the above analysis is for 2UL/2DL HPUE self-interference only. For 2UL IMD interference to the third DL band, further analysis is required before the LUT arrangement can be concluded.

Observation 3: For 2UL IMD interference to the third DL band, the HPUE MSD impact may not be the same as self-band interference and further analysis is required before the LUT arrangement can be concluded.

# 3	Conclusion

In this contribution, we provide our maximum PCx analysis on HPUE 2UL IMD MSD for PC2 with (23dBm + 23dBm), PC1.5 with (26dBm + 26dBm), and (23dBm + 29dBm) by taking into account better PA linearity for PC2 PA than PC3 PA and share our view on the number of LUTs required to cover all cases. We also look into the MSD difference between 1Tx and 2Tx implementations for PC2 FDD band UL using PC1.5 CA_n3A-n78A as an example band combination for the analysis.

Observation 1: 2UL IMD MSD difference between 1Tx and 2Tx PC2 UL can be quite substantial which shall be differentiated with different LUTs.

Observation 2: There would be no MSD difference between PC2 1Tx and 2Tx for the non IMD dominant UL band.

Proposal 1: For PC1.5 combinations, apply [6]dB higher PCx for 3Tx than 2Tx.

Proposal 2: RAN4 to take the LUT arrangement in the Table below into consideration for HPUE 2UL IMD self-interference MSD.

|  | LUT1 | LUT2 | LUT3 | LUT4 | LUT5 | LUT6 | LUT7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Max PCx (dB) | 6.0 | 9.0 | 12.0 | 15.0 | 18.0 | 21.0 | 24.0 |
| Applicability | PC2 IMD2 2Tx | PC2 IMD3 2Tx | PC1.5 IMD3 2Tx | PC1.5 IMD4 2Tx | PC1.5 IMD5 2Tx | PC1.5 IMD4 3Tx | PC1.5 IMD5 3Tx |
|  |  | PC1.5 IMD2 2Tx | PC2 IMD4 2Tx | PC2 IMD5 2Tx | PC1.5 IMD3 3Tx |  |  |
|  |  |  |  | PC1.5 IMD2 3Tx |  |  |  |

Proposal 3: For PC1.5 combinations, only (26dBm + 26dBm) UL configuration is applied for MSD verifications.

Observation 3: For 2UL IMD interference to the third DL band, the HPUE MSD impact may not be the same as self-band interference and further analysis is required before the LUT arrangement can be concluded.

# 4	References

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18th – 21st, 2024

R4-2420380 “WF on MSD rule”, Skyworks et al, 3GPP TSG RAN WG4 Meeting #113, Orlando, USA, November 18th – 22nd, 2024

R4-2505143 “WF on MSD simplification for HPUE”, Skyworks, Qualcomm, Murata, Nokia, Samsung, Huawei, MediaTek, Vivo, ZTE, 3GPP RAN WG4 Meeting #114bis, Wuhan, China, April 7th – 11th, 2025