3GPP RAN WG4 Meeting #115		R4-2505453

Malta, May 19th – 23rd, 2025

Agenda item:	7.1.2.1.2

Source:	Apple

Title:	On PC1.5 intra-band NC UL CA MPR

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The Rel-19 RAN4 non-spectrum WID/SID package has been approved in RAN #103 meeting. In consideration that the uplink coverage is crucial for 5G and 5G-adavanced roll-out and the continued strong interests from operators in higher UE maximum output power such as PC1.5 for their band combinations, one of the key objectives under the new Rel-19 “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” work item [1] is to specify the generic requirements of HPUE for NR uplink CA in FR1 and EN-DC with NR FR1 bands where the features include,

    - PC1.5 for intra-band contiguous UL CA with and without UL MIMO

    - PC1.5 Intra-band non-contiguous UL CA without UL MIMO

    - PC1.5 2-band inter-band UL CA with 2Tx or 3Tx for handheld UE and FWA

    - PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx for handheld UE and FWA UE

    - Increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band UL CA and EN-DC HPUE

In last RAN4 meeting (#114bis), the MPR values for PC1.5 intra-band contiguous UL CA with contiguous allocations were generally agreed, while the MPR values for non-contiguous allocations were still in square brackets, as captured in the approved “WF on HPUE RF requirements” [2]. On the other hand, the MPR values for PC1.5 intra-band non-contiguous (NC) UL CA have not yet been formerly discussed. In this contribution, we reiterate to propose that MPR for PC1.5 intra-band NC UL CA is 3dB higher than that of PC2 intra-band NC UL CA based on our analysis on RAN4 PA configuration assumptions for MPR evaluations for PC3, PC2 and PC1.5 intra-band NC UL as presented in RAN4 #113 meeting [3].

# Discussion

For PC3 intra-band NC UL CA with dualPA-Architecture, it can be certain that the MPR requirements had been evaluated based on (PC3 + PC3) PA configuration where each PA would back off by 3 dB under PC3 maximum output power. On the other hand, for PC2 intra-band NC UL CA with dualPA-Architecture, the PA configuration could potentially be (PC3 + PC3), (PC3 + PC2), or (PC2 + PC2). Since (PC3 + PC3) is the PA configuration for PC3 MPR requirements, if the same PA configuration is applied for PC2, we would expect 3dB higher MPR than PC3. However, this is not the case as observed in Table 2-1 where the specified MPR values are quite comparable between PC2 and PC3 [4]. Therefore, it can be concluded that (PC3 + PC3) cannot be the PA configuration for PC2 intra-band NC UL CA with dualPA-Architecture.

Observation 1: For PC3 intra-band NC UL CA with dualPA-Architecture, MPR has been specified based on (PC3 + PC3) PA configuration.

Observation 2: For PC2 intra-band NC UL CA with dualPA-Architecture, the PA configuration could potentially be (PC3 + PC3), (PC3 + PC2), or (PC2 + PC2).

| PC3 |  | PC2 |  |
| --- | --- | --- | --- |
| B (MHz) | MPR (dB) | B (MHz) | MPR (dB) |
| 0 ≤ B < 1.08 | 15.0 | 0 ≤ B < 1.44 | 15.5 |
| 1.08 ≤ B < 2.16 | 14.5 | 1.44 ≤ B < 2.88 | 15.0 |
| 2.16 ≤ B < 3.24 | 13.5 | 2.88 ≤ B < 5.76 | 14.0 |
| 3.24 ≤ B < 5.04 | 12.5 | 5.76 ≤ B < 10.8 | 12.0 |
| 5.04≤ B < 10.08 | 11.5 | 10.8 ≤ B < 23.04 | 10.5 |
| 10.08 ≤ B < 16.38 | 10.5 | 23.04 ≤ B | 9.0 |
| 16.38 ≤ B < 21.78 | 10.0 |  |  |
| 21.78 ≤ B | 9.0 |  |  |
| NOTE: B is the effective aggregated bandwidth in MHz |  |  |  |

Table 2-1 MPR requirements for intra-band NC UL CA with dualPA for -30dBm/MHz emission

For (PC3 + PC2) PA configuration, though the PC2 PA is more linear, the MPR would be dominated by the emission from the PC3 PA. Using a conceptual 2-tone IM3 analysis as illustrated in Figure 2-1, it can be seen that the IM3 power level at (2fCC1 – fCC2) for (PC2 + PC3) PA configuration would be lower than that of (PC3 + PC3) PA configuration under the same total output power, owing to the better linearity of PC2 PA. However, the IM3 power at (2fCC2 – fCC1) would be the same between (PC2 + PC3) and (PC3 + PC3) PA configurations under the same total output power.

Figure 2-1 A conceptual 2-tone IM3 analysis between (PC3+PC3) and (PC2+PC3) PA configurations

Since both sides of the IM3 need to fulfill the same emission requirement, the MPR is also expected to be the same between (PC3 + PC3) and (PC2 + PC3) PA configurations when both are operating at PC3. However, if (PC2 + PC3) PA configuration is operating at PC2, the MPR would be expected to be 3dB higher than that of PC3. Therefore, to have comparable MPR between PC3 and PC2 as observed in Table 2-1, the only possible PA configuration for PC2 would be (PC2 + PC2) [5]. And that implies MPR for PC1.5 with the same (PC2 + PC2) PA configuration would be 3dB higher than PC2 which also means there is virtually no performance gain for PC1.5 intra-band NC UL CA as compared to its PC2 counterpart.

Observation 3: For PC2 intra-band NC UL CA with dualPA-Architecture, MPR based on both (PC3 + PC3) and (PC2 + PC3) PA configurations would be 3dB higher than that of PC3.

Observation 4: The comparable MPR values between PC3 and PC2 as defined in current RAN4 specifications implies that the only possible PA configuration for PC2 would be (PC2 + PC2).

Observation 5: As PC1.5 intra-band NC UL CA with dualPA-Architecture would be based on the same (PC2 + PC2) PA configuration for PC2, the PC1.5 MPR is expected to be 3dB higher than that of PC2.

Observation 6: There is virtually no performance gain for PC1.5 intra-band NC UL CA as compared to its PC2 counterpart.

Proposal: PC1.5 intra-band NC UL CA MPR is 3dB higher than that of its PC2 counterpart.

# 3	Conclusion

In this contribution, we reiterate to propose that MPR for PC1.5 intra-band NC UL CA is 3dB higher than that of PC2 intra-band NC UL CA based on our analysis on RAN4 PA configuration assumptions for MPR evaluations for PC3, PC2 and PC1.5 intra-band NC UL as presented in RAN4 #113 meeting.

Observation 1: For PC3 intra-band NC UL CA with dualPA-Architecture, MPR has been specified based on (PC3 + PC3) PA configuration.

Observation 2: For PC2 intra-band NC UL CA with dualPA-Architecture, the PA configuration could potentially be (PC3 + PC3), (PC3 + PC2), or (PC2 + PC2).

Observation 3: For PC2 intra-band NC UL CA with dualPA-Architecture, MPR based on both (PC3 + PC3) and (PC2 + PC3) PA configurations would be 3dB higher than that of PC3.

Observation 4: The comparable MPR values between PC3 and PC2 as defined in current RAN4 specifications implies that the only possible PA configuration for PC2 would be (PC2 + PC2).

Observation 5: As PC1.5 intra-band NC UL CA with dualPA-Architecture would be based on the same (PC2 + PC2) PA configuration for PC2, the PC1.5 MPR is expected to be 3dB higher than that of PC2.

Observation 6: There is virtually no performance gain for PC1.5 intra-band NC UL CA as compared to its PC2 counterpart.

Proposal: PC1.5 intra-band NC UL CA MPR is 3dB higher than that of its PC2 counterpart.

# 4	References

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18th – 21st, 2024

R4-2505100 “WF on HPUE RF requirements”, Qualcomm, 3GPP TSG RAN WG4 Meeting #114bis, Wuhan, China, April 7th – 11th, 2025

R4-2417763 “On PC1.5 intra-band non-contiguous UL CA MPR”, Apple, 3GPP TSG RAN WG4 Meeting #113, Orlando, USA, November 18th – 22nd, 2024

3GPP TS 38.101-1 V19.1.0 (2025-03)

R4-2114948 “WF on PC2 intra-band UL NC CA and contiguous CA with 2Tx architecture”, Skyworks Solutions Inc., 3GPP TSG-RAN WG4 Meeting #100e, online, August 16th – 27th, 2021