3GPP RAN WG4 Meeting #115		R4-2505455

Malta, May 19th – 23rd, 2025

Agenda item:	7.6.1

Source:	Apple

Title:	On PC2 RedCap UE FDD band 1Rx REFSENS

WI/SI:	NR_PC2_RedCap_UE-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

A new work item on NR power class 2 (PC2) RedCap UE was approved in RAN #104 meeting [1]. The main objective of the core part is to apply the PC2 RF requirements for non-RedCap UE to RedCap UE based on 1Tx architecture with 26 dBm PA and the following additional clarifications:

- The PC2 RF requirements include: UE maximum output power, Tx power tolerance, MPR, A-MPR and RSD requirements if needed.

- For TDD bands, PC2 RF requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way.

- Check at RAN #106 on whether to define PC2 RF requirements for FDD bands and the applicable form factors for FDD bands (if defined).

In RAN #106 meeting, it was approved to specify RedCap PC2 RF requirements for FDD bands with n1 and n3 as two example bands where the revised objective was captured in the “Revised WID on NR power class 2 RedCap UE” [2]. In last RAN4 meeting (#114bis), it was agreed that the REFSENS for PC2 1Rx FD-FDD needs to be evaluated band-by-band if the PC2 2Rx RSD (REFSENS Degradation) is not zero and all the FDD bands with PC2 specified for non-RedCap UE are to be evaluated for 1Rx REFSENS, as was captured in the approved “WF on PC2 RedCap UE RF requirements” [3].

In this contribution, we share our PC2 1Rx REFSENS analysis and results for all FDD bands with PC2 specified for non-RedCap UE.

# Discussion

For all FDD bands with PC2 specified for non-RedCap UE, not all of them are subject to RSD relative to PC3 REFSENS. For those bands zero RSD, it implies the PC2 Tx self-interference is negligible to Rx thermal noise floor. Therefore, the ΔR1R as specified in Table 7.3I.2-1 in TS 38.101-1 [4] can be used to derive the PC2 FDD band 1Rx REFSENS.

Observation 1: For non-RedCap PC2 FDD bands without RSD specified, the ΔR1R as specified in Table 7.3I.2-1 in TS 38.101-1 can be used to derive the PC2 FDD band 1Rx REFSENS.

For non-RedCap PC2 FDD bands with non-zero RSD specified, due to that the Tx self-interference level would not be balanced between main and diversity Rx paths, the PC2 1Rx REFSENS can no longer be derived by ΔR1R as specified in Table 7.3I.2-1 in TS 38.101-1 [4] and has to be evaluated band by band.

Observation 2: For non-RedCap PC2 FDD bands with non-zero RSD specified, the PC2 1Rx REFSENS has to be evaluated band by band.

In this contribution, we share our PC2 1Rx REFSENS analysis and results for all non-RedCap PC2 FDD bands specified with non-zero RSD. Our analysis was carried out based on the existing PC2 2Rx RSD requirements with 1Tx via reverse engineering, with the assumption that the Tx noise in primary Rx path is effectively 6dB higher than the diversity Rx path.

Using Band n3 as an example for which 0.5dB RSD has been specified for PC2 with 1Tx for all channel bandwidths. Based on the above 6dB Tx noise power imbalance assumption, we can reverse-engineer the PC2 Tx noise into both primary Rx path and diversity Rx path and then derive the PC2 1Rx RSD (RSD1R), PC2 ΔR1R, and 1Rx REFSENS from the primary Rx path sensitivity degradation, as shown in Table 2-1.

| Band n3 | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| PC3 2Rx REFSENS (dBm) | -97.0 | -93.8 | -92.0 | -90.8 |
| PC2 1Tx 2Rx RSD (dB) | 0.5 | 0.5 | 0.5 | 0.5 |
| PC2 1Tx 2Rx REFSENS (dBm) | -96.5 | -93.3 | -91.5 | -90.3 |
| PC3 MRC noise (dBm) | -96.0 | -92.8 | -91.0 | -89.8 |
| PC3 primary Rx noise (dBm) | -93.5 | -89.8 | -88.0 | -86.8 |
| PC3 diversity Rx noise (dBm) | -92.4 | -89.8 | -88.0 | -86.8 |
| PC2 MRC noise (dBm) (REFSENS) | -95.5 | -92.3 | -90.5 | -89.3 |
| PC2 primary Tx noise (dBm) | -101.0 | -97.0 | -95.0 | -94.0 |
| PC2 diversity Tx noise (dBm) | -107.0 | -103.0 | -101.0 | -100.0 |
| PC2 primary Rx + Tx noise (dBm) | -92.8 | -89.0 | -87.2 | -86.0 |
| PC2 diversity Rx + Tx noise (dBm) | -92.3 | -89.6 | -87.8 | -86.6 |
| PC2 MRC noise (dBm) (derived) | -95.5 | -92.3 | -90.5 | -89.3 |
| PC2 1Rx RSD (dB) | 0.8 | 0.8 | 0.8 | 0.8 |
| PC2 ΔR1R (dB) | 2.8 | 3.3 | 3.3 | 3.3 |
| PC2 1Rx REFSENS (dBm) | -93.7 | -90.0 | -88.2 | -87.0 |

Table 2-1 n3 PC2 1Rx REFSENS analysis based on PC2 2Rx RSD reverse engineering

Notice that the PC2 1Rx REFSENS can be calculated by the following two formulas, depending on whether PC2 RSD1R or PC2 ΔR1R would be specified.

PC2 RSD1R specified: PC2 1Rx REFSENS = PC3 2Rx REFSENS + PC3 ΔR1R + PC2 1Rx RSD

PC2 ΔR1R specified: PC2 1Rx REFSENS = PC3 2Rx REFSENS + PC2 2Rx RSD + PC2 ΔR1R

Considering that both PC2 RSD1R and PC2 ΔR1R are mostly band dependent for FDD bands, in our view there would not be any benefit to specify either PC2 RSD1R or PC2 ΔR1R as compared to directly specify PC2 1Rx REFSENS power levels from the spec simplification perspective. Therefore, for RedCap UE PC2 FDD bands 1Rx REFSENS, we propose to directly specify the REFSENS power levels instead of indirectly specifying PC2 RSD1R or PC2 ΔR1R.

Observation 3: PC2 1Rx REFSENS can be calculated by the following two formulas, depending on whether PC2 RSD1R or PC2 ΔR1R would be specified.

PC2 RSD1R specified: PC2 1Rx REFSENS = PC3 2Rx REFSENS + PC3 ΔR1R + PC2 RSD1R

PC2 ΔR1R specified: PC2 1Rx REFSENS = PC3 2Rx REFSENS + PC2 2Rx RSD + PC2 ΔR1R

Observation 4: For FDD bands, both PC2 RSD1R and PC2 ΔR1R are mostly band dependent.

Observation 5: There would not be any benefit to specify either PC2 RSD1R or PC2 ΔR1R as compared to directly specify PC2 1Rx REFSENS power levels from the spec simplification perspective.

Proposal 1: For RedCap UE PC2 FDD bands 1Rx REFSENS, directly specify the REFSENS power levels instead of indirectly specifying PC2 RSD1R or PC2 ΔR1R.

Table 2-2 summarizes the proposed PC2 1Rx REFSENS requirements (SCS = 15kHz) for all FDD bands with PC2 specified for non-RedCap UE based on our analysis.

|  | PC2 1Rx REFSENS (dBm) (SCS = 15kHz) |  |  |  |
| --- | --- | --- | --- | --- |
| Operating Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| n1 | -97.5 | -93.8 | -92.0 | -90.8 |
| n2 | -94.2 | -90.4 | -88.2 | -86.9 |
| n3 | -96.5 | -93.3 | -91.5 | -90.3 |
| n7 | -94.7 | -91.0 | -89.2 | -88.0 |
| n8 | -93.7 | -89.7 | -86.7 | -77.3 |
| n13 | -93.3 | -89.3 |  |  |
| n14 | -93.6 | -89.5 |  |  |
| n25 | -92.8 | -89.0 | -87.0 | -85.5 |
| n28 | -95.1 | -91.4 | -89.2 | -84.8 |
| n66 | -97.0 | -93.3 | -91.5 | -90.3 |
| n70 | -97.5 | -93.8 | -92.0 | -90.8 |
| n71 | -93.9 | -89.5 | -86.8 | -77.7 |
| n85 | -93.6 | -89.2 | -74.8 |  |
| NOTE: The uplink configurations are specified in Table 7.3.2-3 |  |  |  |  |

Table 2-2 Proposed PC2 1Rx REFSENS requirements (SCS = 15kHz) for FDD bands

Proposal 2: RAN4 to take the REFSENS requirements in Table 2-2 into consideration for RedCap UE PC2 FDD bands with 1Rx.

# 3	Conclusion

In this contribution, we share our PC2 1Rx REFSENS analysis and results for all FDD bands with PC2 specified for non-RedCap UE.

Observation 1: For non-RedCap PC2 FDD bands without RSD specified, the ΔR1R as specified in Table 7.3I.2-1 in TS 38.101-1 can be used to derive the PC2 FDD band 1Rx REFSENS.

Observation 2: For non-RedCap PC2 FDD bands with non-zero RSD specified, the PC2 1Rx REFSENS has to be evaluated band by band.

Observation 3: PC2 1Rx REFSENS can be calculated by the following two formulas, depending on whether PC2 RSD1R or PC2 ΔR1R would be specified.

PC2 RSD1R specified: PC2 1Rx REFSENS = PC3 2Rx REFSENS + PC3 ΔR1R + PC2 RSD1R

PC2 ΔR1R specified: PC2 1Rx REFSENS = PC3 2Rx REFSENS + PC2 2Rx RSD + PC2 ΔR1R

Observation 4: For FDD bands, both PC2 RSD1R and PC2 ΔR1R are mostly band dependent.

Observation 5: There would not be any benefit to specify either PC2 RSD1R or PC2 ΔR1R as compared to directly specify PC2 1Rx REFSENS power levels from the spec simplification perspective.

Proposal 1: For RedCap UE PC2 FDD bands 1Rx REFSENS, directly specify the REFSENS power levels instead of indirectly specifying PC2 RSD1R or PC2 ΔR1R.

Proposal 2: RAN4 to take the REFSENS requirements in the table below into consideration for RedCap UE PC2 FDD bands with 1Rx.

|  | PC2 1Rx REFSENS (dBm) (SCS = 15kHz) |  |  |  |
| --- | --- | --- | --- | --- |
| Operating Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| n1 | -97.5 | -93.8 | -92.0 | -90.8 |
| n2 | -94.2 | -90.4 | -88.2 | -86.9 |
| n3 | -96.5 | -93.3 | -91.5 | -90.3 |
| n7 | -94.7 | -91.0 | -89.2 | -88.0 |
| n8 | -93.7 | -89.7 | -86.7 | -77.3 |
| n13 | -93.3 | -89.3 |  |  |
| n14 | -93.6 | -89.5 |  |  |
| n25 | -92.8 | -89.0 | -87.0 | -85.5 |
| n28 | -95.1 | -91.4 | -89.2 | -84.8 |
| n66 | -97.0 | -93.3 | -91.5 | -90.3 |
| n70 | -97.5 | -93.8 | -92.0 | -90.8 |
| n71 | -93.9 | -89.5 | -86.8 | -77.7 |
| n85 | -93.6 | -89.2 | -74.8 |  |
| NOTE: The uplink configurations are specified in Table 7.3.2-3 |  |  |  |  |

# 4	References

RP-241665 “New WID on NR power class 2 RedCap (Reduced Capability) UE in FR1”, China Telecom, MediaTek, 3GPP TSG-RAN Meeting #104, Shanghai, China, June 17th – 20th, 2024

RP-243267 “Revised WID on NR power class 2 RedCap UE”, China Telecom, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9th – 12th, 2024

R4-2505231 “WF on PC2 RedCap UE RF requirements”, China Telecom, 3GPP TSG RAN WG4 Meeting #114bis, Wuhan, China, April 7th – 11th, 2025

3GPP TS 38.101-1 V19.1.0 (2025-03)